IEEE P802.11
Wireless LANs

| TGnSync Proposal MAC Results |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Syed Aon Mujtaba | Agere Systems Inc. | 555 Union Blvd., Allentown, PA 18109, U.S.A. | +1 610 712 6616 | mujtaba@agere.com |

Abstract

This document discloses results required by the 802.11n comparison criteria and functional requirements related to the MAC performance of the TGnSync proposal [1] [2].

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Additional Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Adrian P Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |
| Andrew Myles | Cisco Corporation |  |  | amyles@cisco.com |
| Brian Johnston | Nortel Networks Corporation |  |  | brjohnso@nortelnetworks.com |
| Daisuke Takeda | Toshiba Corporation |  |  | daisuke.takeda@toshiba.co.jp |
| Darren McNamara | Toshiba Corporation |  |  | Darren.McNamara@toshiba-trel.com |
| Dongjun (DJ) Lee | Samsung Electronics Co. Ltd. |  |  | djthekid.lee@samsung.com |
| David Bagby | Calypso Consulting |  |  | david.bagby@ieee.org |
| Dmitry Akhmetov | Intel Corporation |  |  | Dmitry.akhmetov@intel.com |
| Eldad Perahia | Cisco Corporation |  |  | eperahia@cisco.com |
| Huanchun Ye | Atheros Communications Inc. |  |  | hcye@atheros.com |
| Jan Boer | Agere Systems Inc. |  |  | janboer@agere.com |
| Jeff Gilbert | Atheros Communications Inc. |  |  | gilbertj@atheros.com |
| Joe Pitarresi | Intel Corporation |  |  | joe.pitarresi@intel.com |
| Jörg Habetha | Royal Philips Electronics |  |  | joerg.habetha@philips.com |
| John Sadowsky | Intel Corporation |  |  | john.sadowsky@intel.com |
| Jon Rosdahl | Samsung Electronics Co. Ltd. |  |  | jon.rosdahl@partner.samsung.com |
| Luke Qian | Cisco Corporation |  |  | lchia@cisco.com |
| Masahiro Takagi | Toshiba Corporation |  |  | masahiro3.takagi@toshiba.co.jp |
| Monisha Ghosh | Royal Philips Electronics |  |  | monisha.ghosh@philips.com |
| Osama Aboul-Magd | Nortel Networks Corporation |  |  | osama@nortelnetworks.com |
| Paul Feinberg | Sony Corporation |  |  | paul.feinberg@am.sony.com |
| Pen Li | Royal Philips Electronics |  |  | pen.li@philips.com |
| Pieter-Paul Giesberts | Agere Systems Inc. |  |  | pgiesberts@agere.com |
| Richard van Leeuwen | Agere Systems Inc. |  |  | rleeuwen@agere.com |
| Ronald Rietman | Royal Philips Electronics |  |  | ronald.rietman@philips.com |
| Seigo Nakao | SANYO Electric Co. Ltd. |  |  | snakao@gf.hm.rd.sanyo.co.jp |
| Sergey Shtin | Intel Corporation |  |  | Sergey.shtin@intel.com |
| Sheung Li | Atheros Communications Inc. |  |  | sheung@atheros.com |
| Steve Shellhammer | Intel Corporation |  |  | steven.j.shellhammer@intel.com |
| Takushi Kunihiro | Sony Corporation |  |  | kuni@wcs.sony.co.jp |
| Teik-Kheong TK Tan | Royal Philips Electronics |  |  | tktan@philips.com |
| Tomoko Adachi | Toshiba Corporation |  |  | tomo.adachi@toshiba.co.jp |
| Tomoya Yamaura | Sony Corporation |  |  | yamaura@wcs.sony.co.jp |
| Tsuguhide Aoki | Toshiba Corporation |  |  | tsuguhide.aoki@toshiba.co.jp |
| Won-Joon Choi | Atheros Communications Inc. |  |  | wjchoi@atheros.com |
| Xiaowen Wang | Agere Systems Inc. |  |  | xiaowenw@agere.com |
| Yasuhiko Tanabe | Toshiba Corporation |  |  | yasuhiko.tanabe@toshiba.co.jp |
| Yasuhiro Tanaka | SANYO Electric Co. Ltd. |  |  | y_tanaka@gf.hm.rd.sanyo.co.jp |
| Yoshiharu Doi | SANYO Electric Co. Ltd. |  |  | doi@gf.hm.rd.sanyo.co.jp |
| Yuichi Morioka | Sony Corporation |  |  | morioka@wcs.sony.co.jp |

- Introduction

  - Purpose of this Document

This document contains MAC simulation results for the "TGnSync" proposal to IEEE 802.11 TGn in compliance to the TGn call for proposals.

The TGn comparison criteria call for full disclosure of results and conditions.  We present the results at varying levels of detail in the following documents.  The most summarised form is found in the presentation material [5].  This document contains all of the mandatory results – except per-flow results. Documents [3] and [4] contain the detailed per-flow results and parameter settings as well as results for other simulation conditions.

  - Revision History

| Document Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 0 | August 13, 2004 | Adrian Stephens and Yuichi Morioka | Initial Version for 13 Aug 2004 submission |
| 1 | September 12, 2004 | Adrian Stephens and Yuichi Morioka | Minor corrections and updates |
| 2 | November 4, 2004 | Adrian Stephens Yuichi Morioka Tomoko Adachi Dmitry Akhmetov | Updates to 20MHz Results Analysis of Enhanced BA feature added Updates to MAC1 simulation results. Updates to MAC options analysis |
| 3 | January 2, 2005 | Yuichi Morioka Dmitry Akhmetov Adrian Stephens | Updates to MAC1 and MAC2 simulation results. |
| 4 | January 16, 2005 | Yuichi Morioka Tomoko Adachi Dmitry Akhmetov | Updates to MAC1, MAC2 and MAC3 simulation results. |
| 5 | March 4, 2005 | Yuichi Morioka Tomoko Adachi Dmitry Akhmetov | Updates to MAC1, MAC2 and MAC3 simulation results |
| 6 | March 11, 2005 | Tomoko Adachi | Updates to MAC3 simulation results |

  - Simulation Methodology

Document [1] describes in full the methodology used to obtain this result.  This section contains a very brief summary of the methodologies.

Results were obtained using three independent MAC simulations which are called MAC1, MAC2 and MAC3 in this document.

The MAC1 simulation is based on the commercial Opnet ** simulation tool, with a substantially re-written MAC process and PHY pipeline.  The PHY model built into the pipeline is based on PER as a function of post-detection capacity.  The simulation uses three possible protection methods: Standard NAV, the LongNAV method of protection (a MAC-layer technique described in [2, section 8.1.7.1]) and pairwise spoofing (described in [1]).

Results reported here from that model use the LongNAV protection method. The simulation supports MAC Header compression, reverse direction data and periodic reverse direction data MAC options Implicit BAR and Compressed BAR as described in [1, section 2.1.8.5 and 2.1.8.6], A-MSDU aggregation as described in [1, section 2.1.8.7]

For the MAC1 simulation, variants of the standard SS with increased TCP offered load set to the equivalent of 100Mbps per TCP application in ss#1 and of 30Mbps per TCP application in ss#4 and ss#6 were also simulated.  These are shown as "SS1 +", "SS4 +" and "SS6 +" below.

The MAC2 simulation is a discrete event-based simulator written in "C".  The PHY model is based on PER as a function of SNR.  The simulation supports two protection methods:  Standard NAV and pairwise spoofing (described in [1]).  Results reported here from that model use the pairwise spoofing protection method.

The MAC3 simulation is based on the commercial Opnet simulation tool. The simulation supports Implicit BAR and Compressed BA (described in [2, section 8.4]). The purpose of this simulation was to investigate the performance benefits of the Enhance BA feature.

  - Model Credits

The following people created the models and provided the PHY simulation results embodied in the MAC simulations.

| Model | MAC Model | PHY Curves |
| --- | --- | --- |
| MAC1 | Dmitry Akhmetov, Intel Adrian Stephens, Intel | John Sadowsky, Intel Sergey Shtin, Intel |
| MAC2 | Yuichi Morioka, Sony Kenzoh Nishikawa, Sony Kazuyuki Sakoda, Sony | Darren McNamarra, Toshiba John Sadowsky, Intel |
| MAC3 | Tomoko Adachi, Toshiba | Daisuke Takeda, Toshiba |

- Results for Mandatory Comparison Criteria and FR

  - Summary of Configurations

### MAC1 2x2x20

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 20 MHz |
| PHY Options enabled | SGI where allowed |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |
| BlockAck policy | Implicit BA, Short BA |
| A-MSDU usage | ON |
| A-MPDU usage | ON |
| MRMRA | OFF |
| Reverse data flow | ON |

### MAC1 2x2x40

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 40 MHz |
| PHY Options enabled | SGI where allowed |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |
| BlockAck policy | Implicit BAR, Short BA |
| A-MSDU usage | ON |
| A-MPDU usage | ON |
| MRMRA | OFF |
| Reverse data flow | ON |

### MAC2 2x2x20

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 20 MHz |
| PHY Options enabled | None (Short GI disabled for Channel D and E) |
| TCP Model | New Reno |
| Protection Method | Pairwise Spoofing |
| Channel access method | EDCA |
| QoS Parameter Optimization | Optimized EDCA Contention Window Settings were used to satisfy QoS Requirements. |
| BlockAck policy | Implicit BAR, Compressed BA |
| A-MSDU usage | OFF |
| MRMRA | OFF |
| Reverse data flow | OFF |

### MAC2 2x2x40

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 40 MHz |
| PHY Options enabled | None |
| TCP Model | New Reno |
| Protection Method | Pairwise Spoofing Note, Pairwise Spoofing vs LongNAV comparisons derived from the MAC2 results are shown in section 4.1 below. |
| Channel access method | EDCA |
| QoS Parameter Optimization | Optimized EDCA Contention Window Settings were used to satisfy QoS Requirements. |
| BlockAck policy | Implicit BAR, Compressed BA |
| A-MSDU usage | OFF |
| MRMRA | OFF |
| Reverse data flow | OFF |

  - Results relating to MAC simulations

Note, the channel access mechanism used to obtain the results is indicated with these colours:

| Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs | Optimized EDCA Contention Window Settings |
| --- | --- |

| CC# | Name | Result |  | MAC1 |  | MAC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2x2x20 | 2x2x40 | 2x2x20 | 2x2x40 |
| CC3 | List of goodput results for usage models 1, 4 and 6. | SS1 (Mbps) |  | 85.51 | 85.38 | 63.85 | 79.78 |
|  |  | SS1 + |  | 103.06 | 163.32 | n/a | n/a |
|  |  | SS4 |  | 104.19 | 218.00 | 58.54 | 81.20 |
|  |  | SS4 + |  | 105.13 | 223.44 | n/a | n/a |
|  |  | SS6 |  | 65.40 | 66.15 | 47.78 | 57.58 |
|  |  | SS6 + |  | 90.71 | 179..19 | n/a | n/a |
| CC15 | Sharing of medium with legacy devices | T1 (Mbps) |  | 87.98 | n/a | n/a | n/a |
|  |  | T2 (Mbps) |  | 34.76 | n/a | n/a | n/a |
|  |  | T3 (Mbps) |  | 41.59 | n/a | n/a | n/a |
|  |  | T4 (Mbps) |  | 20.59 | n/a | n/a | n/a |
| CC18 | HT Usage Models Supported (non QoS) | SS1 (Mbps/ratio) |  | 33.02/1.0 | 32.90/1.0 | 11.76/0.38 | 27.72/0.89 |
|  |  | SS4 |  | 95.07/0.21 | 208.91/0.46 | 49.58/0.11 | 72.27/0.16 |
|  |  | SS6 |  | 20.67/1.0 | 21.38/1.0 | 4.65/0.23 | 14.75/0.73 |
| CC19 | HT Usage Models Supported (QoS) | SS1 |  | 17/17 | 17/17 | 16/17 | 17/17 |
|  |  | SS4 |  | 18/18 | 18/18 | 18/18 | 18/18 |
|  |  | SS6 |  | 39/39 | 39/39 | 39/39 | 39/39 |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | SS1 | M1 | 85.51 | 85.26 | 63.85 | 79.78 |
|  |  |  | M2 | 85.51 | 85.26 | 63.85 | 79.78 |
|  |  |  | M3 | 85.51 | 85.26 | 63.36 | 79.78 |
|  |  | SS4 | M1 | 104.19 | 218.02 | 58.54 | 81.20 |
|  |  |  | M2 | 104.17 | 218.00 | 58.54 | 81.20 |
|  |  |  | M3 | 104.19 | 218.02 | 58.54 | 81.20 |
|  |  | SS6 | M1 | 65.40 | 66.15 | 47.78 | 57.58 |
|  |  |  | M2 | 65.40 | 66. 15 | 47.78 | 57.58 |
|  |  |  | M3 | 65.40 | 66. 15 | 47.78 | 57.58 |
| CC24 | MAC Efficiency | SS1 |  | 0.62 | 0.33 | 0.72 | 0.60 |
|  |  | SS4 |  | 0.80 | 0.80 | 0.55 | 0.46 |
|  |  | SS6 |  | 0.50 | 0.24 | 0.50 | 0.38 |
| CC27 | Throughput / Range |  |  | n/a | See section 2.4 |  |  |
| CC28 | Throughput / Range in 20MHz |  |  | See section 2.5 | n/a |  |  |
| CC58 | HT Spectral Efficiency | bps/Hz |  | From the CC28 curve, goodput is ~107 Mbps at a PSDU PHY rate of 135 Mbps for channel Models B and D. (GI=0.8us) Spectral Efficiency = 112.4/20 = 5.62 bps/Hz (for channel models B and D) | From CC27, goodput is 237.75Mbps at a PSDU PHY rate of 283.5Mbps for Channel Models B and D. (GI=0.8us) Spectral Efficiency = 237.75/40 = 6.04 bps/Hz (for channel models B and D) |  |  |
| Note1: Refer to [3] for CC18/19 per-flow MAC1 results |  |  |  |  |  |  |  |
| Note2: Refer to [4] for CC18/19 per-flow MAC2 results |  |  |  |  |  |  |  |

  - MAC Comparison Criteria not related to simulation results

| CC# | Name | Disclosure |
| --- | --- | --- |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Backwards compatibility is provided at the PHY level [2, section 12.3.1.2] essentially by retaining the legacy short and long training fields and the legacy signal field. Backward compatibility is managed within the MAC through the definition of an operating mode [2, section 9.1.1] and rules [2, sections 9.1.3 and 9.1.4] for operation in each operating mode. Protection mechanisms are defined using both PHY-level ("spoofing") [2, section 8.1.7.2] and MAC-level techniques [2, section 8.1.7.1]. |
| CC46 | MAC Compatibility and parameters. | See section 2.6 |
| CC47 | MAC extensions | See section 2.7 |

  - CC27

Figure 1 shows results for CC27 from the MAC1 simulation, truncated at 100m range for Channel B and Channel D.

The MAC and PHY parameters used are defined in [3, CC27 2x2x40 tab].   Any parameters (e.g. SIFS spacing and slot timings) not specified there take on the 802.11a values.

Figure 1 - CC27 Results

  - CC28

Figure 2 shows results for CC28 from the MAC1 simulation, truncated at 100m range for Channel B and Channel D.

The MAC and PHY parameters used are defined in [3, CC28 2x2x20 tab]. Any parameters (e.g. SIFS spacing and slot timings) not specified there take on the 802.11a values.

Figure 2 - CC28 Results

  - CC46 – Baseline options

"Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used.  Include references to the sections in the technical proposal document where the complete details are given."

IEEE 802.11e

| Item | Feature | Brief Description | Sections in [2] |
| --- | --- | --- | --- |
| CF12 | QoS Supported | The proposal requires the use of features introduced in the IEEE 802.11e amendment. | 7.3 |
| QB4 | Block Acknowledgement | TGnSync MAC proposal requires support for Block Acknowledgment in the context of frame aggregation. The proposal also defines a compressed variant of the 802.11e Block Ack. | 7.4 |
| QB5 | Automatic Power Save Delivery | Modified in the context of received aggregate PPDU | 7.1.6 |
| QD3 | Continuation of EDCA TXOP Support | Under LongNAV rules, a TXOP can be truncated by use of CF-END | 7.1.7.1.2 |

  - CC47

"Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given."

| Feature | Brief Description | Section in [2] |
| --- | --- | --- |
| A-MPDU Aggregation | Allows the transmission of multiple MPDU in a single PSDU | 6.3, 7.1 |
| A-MSDU Aggregation | Allows the transmission of multiple MSDU within a single Data MPDU | 6.3 |
| Bi-directional data flow | Allows the aggregation of control MPDUs in one direction with Data MPDUs in the reverse direction under control of the TXOP owner | 7.1.10 |
| Multiple Receiver Frame Aggregation | Allows aggregated frame to be destined to different receivers | 6.1.3, 7.1.11 |
| MAC Protection | Three additional procedures for MAC protection, Long NAV, Pair wise spoofing, and Single Ended spoofing | 7.1.7 |
| Coexistence Mechanisms | Definition of a BSS operating mode (pure, mixed, managed mixed, 20-base managed mixed) for a BSS based on its environment and rules for legacy protection and management of channel width. | 8.1 |
| MAC Support for Closed Loop Link adaptation | Allows the adaptation of modulation coding scheme according to channel conditions | 7.1.8 |
| Enhanced BA | A compressed variant of the 802.11e Block Ack is defined. Also defined are rules for implicit BA that reduce the number of BAR frames transmitted. | 6.1.6, 7.4 |

  - FR1

"Demonstrate at least one set of conditions under which 100 Mbps at the top of the MAC SAP can be achieved.  Provide all relevant information to document this."

In 2x2x40 mode of operation, the MAC1 simulation demonstrates a goodput of >250Mbps for SS16 using the mandatory features only of the MAC and PHY, simulation methodology described in [1] and the MAC and PHY parameters reported in [3,  "ss#16 CC27 2x2x40"].

  - FR2

"Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. Provide all relevant information to document this."

The mandatory minimum PHY mode of operation is 2x2, 64 QAM.

This corresponds to 135Mbps instantaneous rate at the PHY (using GI=0.8).

A maximum goodput of 107. Mbps is reported in [3, "ss#16 CC28 2x2x20" tab]. The curve crosses 100Mbps at about 8m range.

- Required Additional Disclosures

| Number | Name | Disclosure |
| --- | --- | --- |
| AD2 | TCP Model Parameters | Refer to [3, "Common" tab]. Refer to [1, section 3.1] (MAC2 TCP parameters) |
| AD3 | MAC simulation methodology | This is described in [1], please refer to [1]. |
| AD4 | MAC simulation occupied channel width | For nxnx40 results, a nominal channel width of 40MHz was occupied. For nxnx20 results, a nominal channel width of 20MHz was occupied. |
| AD5 | Justification of low PLR rates achieved | This disclosure relates specifically to the MAC1 2x2x40 simulation. MAC1 simulation targets < 10% PER at the top of the PHY. The fast link adaptation method described in [1, section 2.2.4] achieves the performance shown in [1, section 5]. Typically PER is < 1% except at extreme range. Assuming a PER of 1%, the specified residual PLR of 10-7 can be reached by permitting 4 transmission attempts (3 retries). Assuming uncorrelated errors, these retry attempts will include (with high probability) only a single Data MPDU. At the PHY rates observed in simulation for the Video flows (average 230Mbps), a single Data MPDU / BAR / BA retry exchange takes ~300us. 3 retries takes ~1ms. The average transport delay observed with the Video traffic is <30ms. Attributing this entirely to channel access delay, 3 retries in a separate TXOP would take ~31ms, which is comfortably less than the 200ms delay bound. |

- Characterisation of MAC Features

  - Protection Mechanisms

Selected CC results are presented from the MAC2 simulation to show the relative performances of the two protection modes described in [1] relating to the MAC2 simulation.

| CC# | Result |  | MAC2 2x2x40 Standard Nav | MAC2 2x2x40 Pairwise Spoofing |
| --- | --- | --- | --- | --- |
| CC3 | SS1 (Mbps) |  | 71.70 | 79.87 |
|  | SS4 |  | 74.89 | 81.20 |
|  | SS6 |  | 51.67 | 57.58 |
| CC18 | SS1 (Mbps/ratio) |  | 19.66/0.63 | 27.72/0.89 |
|  | SS4 |  | 65.95/0.15 | 72.27/0.16 |
|  | SS6 |  | 8.72/0.44 | 14.75/0.73 |
| CC19 | SS1 |  | 16/17 | 17/17 |
|  | SS4 |  | 18/18 | 18/18 |
|  | SS6 |  | 39/39 | 39/39 |
| CC20 | SS1 | M1 | 71.71 | 79.78 |
|  |  | M2 | 71.60 | 79.78 |
|  |  | M3 | 71.21 | 79.78 |
|  | SS4 | M1 | 74.89 | 81.20 |
|  |  | M2 | 74.89 | 81.20 |
|  |  | M3 | 74.89 | 81.20 |
|  | SS6 | M1 | 51.67 | 57.58 |
|  |  | M2 | 51.67 | 57.58 |
|  |  | M3 | 51.67 | 57.58 |
| CC24 | SS1 |  | 0.55 | 0.60 |
|  | SS4 |  | 0.47 | 0.46 |
|  | SS6 |  | 0.38 | 0.38 |

  - Enhanced BA

The Enhanced BA feature was introduced in the November 2004 meeting as a mandatory feature of the TGnSync proposal.  This section shows the performance results from the MAC3 simulator with/without the use of this feature, and with Enhanced BA combined with the bi-directional or piggy-back feature.

### Summary of Enhanced BA benefit

From the SS4 results of 20 MHz, it can be seen that the aggregate throughput is increased by approximately 9% when the Enhanced BA feature is used. Moreover, the aggregate throughput is increased by approximately 13% when both Enhanced BA and bi-directional features are used.

### TCP Parameters

| TCP Model Parameters for CC18, CC19, CC20. CC24 |  |
| --- | --- |
| MSS | Ethernet (1500) |
| Receive Buffer (bytes) | 65535 |
| Receive Buffer Adjustment | None |
| Delayed ACK Mechanism | Segment/Clock based |
| Maximum ACK Delay (sec) | 0.200 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Disabled |
| Selective AKC (SACK) | Disabled |
| ECN Capability | Disabled |
| Segment Send Threshold | Byte Boundary |
| Active Connection Threshold | Unlimited |
| Karn's Algorithm | Enabled |
| Nagle Algorithm | Disabled |
| Initial Sequence Number | Auto Complete |
| Initial RTO (sec) | 3.0 |
| Min RTO (sec) | 1.0 |
| Max RTO (sec) | 64.0 |
| RTT Gain | 0.125 |
| Deviation gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity | 0.5 |

### MAC3 2x2x20 Parameters

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 20 MHz |
| PHY Options enabled | None |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS Parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |
| BlockAck policy | Implicit BAR, Compressed BA |
| A-MSDU usage | OFF |
| MRMRA | OFF |
| Reverse data flow | ON |

### MAC3 2x2x40 Parameters

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 40 MHz |
| PHY Options enabled | 5/6 coding rate |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |
| BlockAck policy | Implicit BAR, Compressed BA |
| A-MSDU usage | OFF |
| MRMRA | OFF |
| Reverse data flow | ON |

### MAC3 PHY Model

The PER vs. average SNR curves underlying this simulator were supplied by Daisuke Takeda of Toshiba. The simulations that generated these curves included the impairments required by the TGn CC. Link adaptation is not applied.

### Enhanced BA Results

| CC# | Name | Result |  | MAC3 |  |  | MAC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2x2x20 |  |  | 2x2x40 |  |  |
|  |  |  |  | w/o Enhanced BA | wi. Enhanced BA | wi. Enhanced BA and bi-directional | w/o Enhanced BA | wi. Enhanced BA | wi. Enhanced BA and bi-directional |
| CC3 | List of goodput results for usage models 1, 4 and 6. | SS1 (Mbps) |  | 72.9 | 75.2 | 76.2 | 84.0 | 84.0 | 84.0 |
|  |  | SS4 |  | 68.6 | 75.0 | 77.5 | 101.1 | 110.2 | 113.1 |
|  |  | SS6 |  | 52.2 | 55.5 | 58.1 | 64.1 | 65.6 | 66.2 |
| CC18 | HT Usage Models Supported (non QoS) | SS1 (Mbps/ratio) |  | 20.3/ 0.66 | 22.7/ 0.73 | 23.7/ 0.76 | 31.5/1.02 | 31.5/1.02 | 31.5/1.02 |
|  |  | SS4 |  | 59.4/ 0.13 | 65.9/ 0.15 | 68.4/ 0.15 | 92.0/0.20 | 101.1/0.22 | 104.0/0.23 |
|  |  | SS6 |  | 7.3/ 0.36 | 10.6/ 0.53 | 13.3/ 0.66 | 19.2/0.96 | 20.8/1.04 | 21.3/1.07 |
| CC19 | HT Usage Models Supported (QoS) | SS1 |  | 17/17 | 17/17 | 17/17 | 17/17 | 17/17 | 17/17 |
|  |  | SS4 |  | 18/18 | 18/18 | 17/18 | 18/18 | 18/18 | 17/18 |
|  |  | SS6 |  | 39/39 | 39/39 | 39/39 | 39/39 | 39/39 | 39/39 |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | SS1 | M1 | 72.9 | 75.2 | 76.2 | 84.0 | 84.0 | 84.0 |
|  |  |  | M2 | 72.9 | 75.2 | 76.2 | 84.0 | 84.0 | 84.0 |
|  |  |  | M3 | 72.9 | 75.2 | 76.2 | 84.0 | 84.0 | 84.0 |
|  |  | SS4 | M1 | 68.6 | 75.0 | 77.5 | 101.1 | 110.2 | 113.1 |
|  |  |  | M2 | 68.6 | 75.0 | 77.5 | 101.1 | 110.2 | 113.1 |
|  |  |  | M3 | 68.6 | 75.0 | 76.5 | 101.1 | 110.2 | 112.1 |
|  |  | SS6 | M1 | 52.2 | 55.5 | 58.1 | 64.1 | 65.6 | 66.2 |
|  |  |  | M2 | 52.2 | 55.5 | 58.1 | 64.1 | 65.6 | 66.2 |
|  |  |  | M3 | 52.2 | 55.5 | 58.1 | 64.1 | 65.6 | 66.2 |
| CC24 | MAC Efficiency | SS1 |  | 0.71 | 0.74 | 0.75 | 0.52 | 0.52 | 0.52 |
|  |  | SS4 |  | 0.54 | 0.59 | 0.61 | 0.39 | 0.42 | 0.43 |
|  |  | SS6 |  | 0.65 | 0.71 | 0.75 | 0.44 | 0.45 | 0.45 |

  - Reverse direction data feature (including periodic RDR)

### Status of these results

This section was introduced in the November 2004 meeting (revision 2 of this document).  The results have not been updated in the current revision.   This means that the goodput results do not compare directly with the results of section 2.2.  However the differences introduced by use of this feature will not be significantly affected by the improvements made to the protocol and simulation reflected in section 2.2, so the performance gains from the use of this feature are still considered to be valid.

### Performance gain on simulation scenarios

The table below shows the benefit of reverse direction data support on example of ss#1 bis, ss#4, ss#6 bis.

(The "bis" scenario is created by increasing the offered load of all TCP flows to 30Mbps).

Table 1. Bi-direction support performance results

| Scenario | Aggregate Goodput Reverse Direction not enabled (Mbps) | Aggregate Goodput Reverse direction enabled |  | Performance gain, % |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Without Periodic RDR | With Periodic RDR | Without Periodic RDR | With Periodic RDR |
| SS#1 Normal GI | 82.08 | N/A | N/A | N/A | N/A |
| SS#1 BIS Normal GI | 87.26 | 94.67 | N/A | 8% | N/A |
| SS#1 BIS Short GI | 92.18 | 100.43 | N/A | 8% | N/A |
| SS#4 | 90.60 | 123.28 | 142.12 | 36% | 56% |
| SS#4 bis | 126.91 | 141.02 | 160.12 | 11% | 26% |
| SS#6 | 62.56 | 66.00 | N/A | 5% | N/A |
| SS#6 bis | 66.96 | 96.24 | N/A | 43% | N/A |

In the context of the simulation scenarios, it can be seen that reverse direction data shows a benefit that varies significantly across the simulation scenarios.   In SS1 we see 8% benefit from the use of this feature.

In SS4 this increases to 36%.

When periodic RDR is used for the VoIP flows, the benefit is substantially increased to 56% in the case of SS4 and 26% in the case of SS4 bis.

### Reverse direction data benefit for additional simulation scenarios

A number of additional simulations were performed in order to estimate the performance benefit of reverse-direction data flow support.

The following scenarios were created in OpNet simulation tool:

- Peer to Peer connection using TCP protocol. Each station generate traffic stream of the offered load of 125Mb/s

- Peer to Peer connection using UDP protocol. Each station generate traffic stream of the offered load of 125Mb/s

- Single TCP stream from one STA to another of 200Mb/s offered load.

- VoIP support. A ring of VoIP clients around AP.

- TCP ring. A ring of TCP clients around AP.

- Mixture traffic. A ring of TCP and VoIP client (10 station of each type) around AP.

Full details of the methodology and detailed results are contained in [1].

Using the scenarios described in [1] the following results are obtained:

- Peer to Peer connection. In case of single peer to peer connection the use of reverse data flow will allow a small performance benefit in the range of 1-3 % both for TCP and UDP traffic. The biggest performance benefit can be achieved using TCP traffic with small MPDU size (~100-200 bytes) and MSS size equal to that value. In this case performance benefit is up to 10 %

- VoIP support. Reverse support in case of EDCA to increase number of supported users from 20 to 30.

- Pure TCP traffic support. The scenario with a ring of TCP clients around AP, each doing 30 Mb/s stream shows a small performance increase of roughly 3%.

- In the case of a mixture of TCP and VoIP clients (10 TCP and 10 VoIP), the use of reverse direction obtains a significant performance increase – up to 60% aggregate throughput.

  - Header Compression Feature

This feature was withdrawn from the TGn Sync proposal in March 2005.

Although the feature gave significant improvements in the absence of A-MSDU aggregation,  when A-MSDU aggregation was added, this benefit was reduced, and so the feature was removed.

  - Comparison of 20/40 MHz Coexistence Methods

[6] Contains simulation results relating to a comparison of 20/40 MHz Coexistence methods.  It compares system goodput of the base-20 MHz managed mixed mode with an unmanaged mode in which both 20MHz halves of a 40MHz channel are overlapped by 20MHz devices.

It is demonstrated that the use of base-20 MHz managed mixed mode reduces the probability of collisions in the extension channel and results in higher throughput when there are more than 1 contending 20MHz STA in the extension channel.

- References

[1] IEEE 802.11-04/895, "TGnSync Proposal MAC Simulation Methodology"

[2] IEEE 802.11-04/889, "TGnSync Proposal Technical Specification"

[3] IEEE 802.11-04/893, "TGnSync Proposal MAC1 Simulation Results"

[4] IEEE 802.11-04/894, "TGnSync Proposal MAC2 Simulation Results"

[5] IEEE 802.11-04/888, "TGnSync Proposal"

[6] IEEE 802.11-05/1633, "Comparison of 20/40 MHz Coexistence Methods"

** Other names and brands may be claimed as the property of others.

Note that the PHY model used for this simulation (results T1-T4) was the same as that used for the other MAC simulations.  As such it included impairments IM1, IM5 and IM6.  This is not expected to significantly affect these results.