IEEEP 802.11 Wireless LANs

| TGn Sync TGn Proposal MAC Simulation Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-04-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation |  |  | morioka@wcs.sony.co.jp |
| Kenzoh Nishikawa | Sony Corporation |  |  | knishi@wcs.sony.co.jp |
| Kazuyuki Sakoda | Sony Corporation |  |  | sako@wcs.sony.co.jp |
| Adrian P Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |
| Dmitry Akhmetov | Intel Corporation |  |  | dmitry.akhmetov@intel.com |
| Sergey Shtin | Intel Corporation |  |  | sergey.shtin@intel.com |
| Changwen Liu | Intel Corporation |  |  | changwen.liu@intel.com |

Abstract

This document contains a description of the methodology used in TGn Sync MAC simulations.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Purpose of this Document

This document contains a description of the methodology used in the MAC simulations for TGn Sync.

## Revision History

| Document Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 0 | August 13, 2004 | Adrian Stephens and Yuichi Morioka | Initial Version for 13 Aug 2004 submission |
| 1 | September 12, 2004 | Adrian Stephens and Yuichi Morioka | Minor corrections and updates |
| 2 | November 4, 2004 | Adrian Stephens Yuichi Morioka Dmitry Akhmetov | Updates on 20MHz results Updates to MAC1 features |
| 3 | December 22, 2004 | Dmitry Akhmetov; Sergey Shtin | Updates to simulation methodology; Updates to PHY simulation results |
| 4 | March 4, 2005 | Yuichi Morioka Dmitry Akhmetov Adrian Stephens | Updates to MAC2 features Updates to PHY simulation results Updates to MAC1 features Updates to PHY simulation results Removal of Header Compression Results |
| 5 | April 5, 2005 | Changwen Liu | Updates to MAC1 description. Section 2.1.8.8.1 added. |

## Glossary

| TC | Traffic category |
| --- | --- |
| AC | Access category |
| STA | Usual wireless station |
| TX_BUFFER | A buffer to store MPDUs |
| CTS/RTS | Clear to send/ready to send a set of frames to speed up frame exchange between 2 STAs |
| TBD | To be destroyed |
| TI | Training information |
| WB | Wide Band |
| OFDM | Orthogonal Frequency Division Multiplexing |
| GI | Guard Interval |
| IAC/RAC | Initiator aggregate control/responder aggregate control |
| BPL | Bit and Power Loading |
| UBL | Uniform Bit Loading |
| MIMO | Multiple Input Multiple Output |
| BURST | Aggregation. The PHY guys don’t like this term. |
| Aggregation | The joining of multiple MAC frames into a single PHY packet |
| SRA | Single-receiver aggregate |
| MRMRA | Multi response multi-receiver aggregate |

## References

[1] TGn Channel Models, 11-03-0940-01-000n-tgn-channel-models.doc

[2] TGn Usage Models, 11-03-0802-10-000n-usage-models.doc

[3] TGnSync MAC1 simulation results, doc: IEEE 802.11-04/0892r3

# MAC1 Simulator Methodology

## MAC1 Process Description

### TX sequence generation

A STA can perform transmission if:

- STA has won the competition for the channel/receives a poll frame from AP

- STA received a frame that required response to the originator of that frame

- STA received permission for reverse direction data flow

STA willing to transmit in obtained TXOP has to perform this set of actions:

- Decide about TX sequence type

- Initiate TX sequence by transmitting start frame of TX sequence

- Upon the reception of response frame continue TX sequence or in case of absence of response frame start recovery process.

When in TXOP STA uses a set of TXOP usage rules which allow STA to:

- continue TX sequence if response frame is received and there is time for that

- initiate immediate/standard retransmission procedure if response frame is not received

- stop TX sequence if no time left

- Start new TX sequence if previous is completed and there are time and data to form new TX sequence.

### TXOP usage

#### TXOP usage heuristics

There is a set of parameter which has impact on TXOP usage and TX sequence behavior. They are:

- TXOP continuation. The purpose of it is to give the STA the ability to generate (form) as long TX sequences as it can (in current TXOP size constraints). IF the STA completed, for example, TRAINED_BURST transmission, i.e. it got BA which acknowledges all MPDUs of the BURST it may attempt, if there are time and data frames to send, to form new TX sequence to newly selected destination address. This behavior is valid both for EDCA TXOP and polled TXOP. Newly generated TX sequence is transmitted in a SIFS after decision to continue the current TXOP.  Note: channel access constraints are applied.  (i.e. access under EDCA is for only a single AC).

- Retransmission behavior:

  - Immediate start retry: Allow STA to retry first frame of TX sequence, i.e. if STA has transmitted RTS frame and failed to get CTS frame STA may retry it in a SIFS period of time. This also makes sense for HCCA operation mode. This behavior is also useful in case when STA is sure that starting frame was lost because of bad channel but not because of collision. This is done by checking “does STA have finished first transmission sequence in current TXOP”. If yes, than STA with high probability may say that start frame of next transmission sequence was lost because of bad channel and immediately retransmit the frame

  - Immediate in-time retry. If STA failed to get BlockAck for transmitted BlockAckReq or it failed to get Ack on third fragment of SINGLETON TX sequence STA may retry this frame in a SIFS period of time. This makes sense because since STA is managed to transmit intermediate frame than nobody transmit with it simultaneously and there is no “active” interference. This also makes sense for HCCA operation mode, moreover I’d say this is recommended retransmit behavior in case of polled TXOP

  - A more capable solution should include both retry MPDUs and new MPDUs in a burst together.  This behavior is not implemented yet and it is our intention to support this in the future.

- Reverse data flow support - An optional feature to allow STA having obtained TXOP to reserve some time for recipient to allow it to send back some data with response frame.  Support for Reverse data flow is being added to the simulator.

#### TXOP continuation options:

- “Full TXOP”. STA transmits data from selected AC until it runs out of TXOP duration or data. After each successful transmission sequence STA select new destination address as described in 2.1

- “One transmission sequence only”. STA behave as usual wireless station of 802.11 standard, i.e. after successful transmission of transmission sequence of any type it initiates new channel access procedure.

### RA selection heuristic

There are four variables that influence the selection algorithm and medium access time:

- Size of queue

- “Age” of queue (earliest MPDU arrival time to the queue)

- Min size of an aggregation in MPDUs.

- MAX size of an aggregation in MPDUs.

The following is applied at STA that obtained TXOP and identified access category.

To select destination address the following selection heuristic is used:

- STA examine transmission queue taking into account two parameters:

  - number of buffered MPDUs to the particular destination

  - age of MPDU ( identified as difference between MSDU arrival from LLC and current time)

- STA identifies the oldest MPDU in queue. If such MPDU is in transmission queue more than a defined time then destination of that MPDU is selected as destination address of the transmission sequence

- Otherwise, if no “too old” MPDUs are found then STA selects destination address with the largest number of buffered MPDUs.

### Basic/Operation rate adaptation

A STA sends control frames at a basic rate and data frames using an operational rate. At the beginning of simulation, the user defines the basic rate to be used within the simulation and an operational rate to be used for initial TX sequence type definition.

During simulation, the STA uses as the operational rate the transmission rate obtained via IAC/RAC frame exchange (perform training to find out the best possible transmission rate for the following transmission using the data rate). If there are no legacy devices present in the BSS, the STA uses a slow link adaptation algorithm to define the best possible transmission rate for the control frames.

### TX sequence selection

MAC process transmits data using transmission sequences. Transmission sequence is a sequence of frames and time intervals between them to conduct data transmission to the selected destination address.

There are 9 types of TX sequences defined within MAC process:

- BURST – Initiator transmit aggregation (SRA) and a wait for the response BlockAck a SIFS after of BURST transmission

- SINGLETON – usual transmission sequence of DATA frame followed by Ack frame

- PROTECTED SINGLETON (RTS/CTS+DATA/ACK)

- TRAINED BURST (IAC/RAC+SRA)

- PROTECTED BURST (RTS/CTS+SRA).

- TRAINED SINGLETON (IAC/RAC + DATA/ACK).

- FINISHING MOVE  - transmission of CF_End frame at the end of polled TXOP

- POLLING – transmission of CF_POLL frame followed by Ack frame. Used to grant TXOP

- MULTI_DST_BURST – Transmission of MRMRA. After end of transmission expect response BlockAck from each RA

The TX sequence type is selected at the start of every transmission opportunity or upon competition of previous TX sequence. The input to the TX sequence type determination is a number of MPDUs available for the transmission and remaining TXOP size.

The STA selects the TX queue that contain largest number of buffered MPDUs and estimates the number of MPDUs that might be transmitted using one of the defined above transmission sequence using the estimated rate resulting from the previous transmissions to the selected destination. If STA has not transmitted any frames to the selected destination before, it uses the operational data rate (specified by user as MAC parameter at the beginning of simulation).

Note: if transmission sequence type assumes training request/response generation then STA can only estimate this number, actual size of transmission sequence is known only after reception of training response frame.

Note: User is able to configure MAC to force it to select one transmission only, i.e. MAC will check if it can send only trained aggregation

### Single receiver aggregation (SRA)

The MAC1 results presented use only Single Receiver Aggregation (SRA).  This section describes heuristics particular to SRA.

#### SRA transmission

The following is applied at STA that obtained TXOP and identified access category.

In order to transmit SRA STA has to perform following actions:

- Select destination address (receiver address (RA). (as described in 2.1.3)

- Decide to send SRA to the selected destination. STA identifies the number of an MPDUs it can send to the selected destination using trained SRA by the following algorithm:

- STA estimates number of MPDUs it can send in remained part of TXOP. STA uses operational rate for that (defined by a user at the beginning of a simulation). Note: STA takes into account time required for the transmission of IAC, RAC and response BA (plus SIFS intervals)

- If the resulting number of MPDUs less than “Min Aggregation size” constant, then SRA won’t be sent, otherwise it will be.

- If SRA is selected, STA performs training using IAC/RAC frame exchange

- On receipts of a RAC response, update number of MPDUs to send using latest received training information and transmit SRA of defined length.

#### SRA retransmission behavior

A STA that had transmitted SRA waits for the response BlockAck. If such response frame is not received, the STA retransmits a BlockAckReq frame.  Following reception of a response BlockAck frame, the STA retransmits any lost MPDUs, if required and if time permits.

If a STA receives a BA frame that indicates some MPDUs were not delivered, the STA may retransmit these undelivered MPDUs if there is time in the TXOP left without an additional IAC/RAC frame exchange using previous training information. The STA can add more MPDUs to the retransmitted ones if it has additional MPDUs for transmission and time allows it to do this.

### Bi-direction data flow support

The implemented algorithm described below uses the following parameters to be handled:

- Minimum reverse size. Indicates minimum number of MPDUs required to form reverse flow

- Maximum reverse size. Indicates maximum number of MPDUs in the reverse flow

Two stations (initiator and responder) willing to organize bi-direction data flow has to perform the following actions:

- Once the Initiator has obtained a TXOP it starts the transmission with IAC frame to get latest training information from the Responder.

- Upon reception of IAC frame Responder estimates the channel condition and evaluates best possible transmission rate.

- Responder responds with RAC frame to the Initiator filling the following fields of the frame:

  - RDR request. Responder sends the request for the reverse if has available data to be sent and reverse option is operational.

  - RDR size. The Responder put the size of reverse flow satisfying MIN/MAX reverse conditions.

  - Next PPDU default MCS. The Responder put the estimated transmission rate to be used for the reverse transmission. Here Responder strongly relies on the channel reciprocity.

- Upon reception of the response RAC frame Initiator estimates the size of the aggregation it can send to the Responder and calculates the transmission time of that aggregation (TTA). It also estimates the time required for the transmission of the reverse flow (TTR) on the basis of the values reported in the RAC frame. If TXOP time – TTA > TTR then the Initiator grants the right to respond with the reverse to the Responder.

- Initiator forms the following aggregation (assuming all above is true) : IAC+BURST+BAR

- Upon reception of the aggregation Responder analyses its structure and values of the fields of aggregation frames. If reverse direction request allowed in IAC frame it forms and transmits a reverse flow using values reported to the initiator in the previous RAC frame. i.e. responder transmit a following aggregation: BA+BURST+BAR

- Initiator receives the reverse data flow and reacts according to the status of received aggregation as follows:

  - Aggregation is received. Initiator responds with the BA to any BAR and continues the TXOP if has data to be send and time left in the TXOP

  - Aggregation is not received (timeout happened). Initiator initiates recovery procedure.

  - Aggregation received, but BA is not received. Initiator responds with BA and than initiates recovery procedure

  - Aggregation received, but BAR is not received. Initiator responds with implicit BA.

  - Aggregation is received, but none of the data frames received. Initiator responds with the a BA

### AP static schedule and POLL generation

#### Schedule description

A simple method is used to configure the AP with TSPEC information. The AP has a static schedule that is defined by the simulation user. The user has to provide this static schedule at the beginning of simulations. The AP gets required information like destination address, TID and TXOP size from this schedule and sends a CF-Poll frame every n-th ms as defined by the static schedule.

The schedule contains the following information per destination address:

- Destination address

- AC

- TXOP size

- Start time

- Interval between current and next poll

- POLL type

The AP reads data from the table and initiates polling by creating a poll event at the time specified in the “Start time” entry. When the scheduled poll event happen STA schedules the next poll event using “Interval between current and next poll” value.

The AP behaves differently in accordance of the scheduled POLL type. It can be

- Self POLL. Scheduled event of that type will cause AP to send its own data using prioritized channel access mechanism

- External POLL. A CF-POLL frame will be issued to the STA

Note, that current static schedule easily allows sharing time between HCCA/EDCA channel access functions, as it would otherwise be implemented using Beacon frame

#### Poll processing at AP

The AP uses a privileged channel access after a PIFS interval to deliver a poll frame to the selected destination. If AP at the time of the poll event is busy with any transmission sequence it will wait for the reception of any response frame for that sequence, stop transmission sequence and send the poll frame using PIFS interval.

AP waits for a CF-Ack frame acknowledging the transmitted poll frame.

Note: CF-Ack frame is selected just to simplify code structure and to get rid of additional chain of conditions in Ack reception branch.  This is an additional overhead in our results that is not present in the actual protocol.

If CF_Ack is not received the AP retransmits the poll frame after a PIFS interval

IF CF_Ack is received the AP schedules a poll timeout event to regain channel access after expiry of the TXOP. This event is cancelled if the STA sends to the AP QoS NULL frame to return any unused portion of the TXOP.

#### Poll processing at STA

A STA that receives a poll frame checks whether it can send any data frames during the granted period of time. If the STA can’t send any data frames it immediately responds with QoS NULL frame to the access point.

#### Support of Periodic reverse direction request.

To support Periodic Reverse Direction Request the following modification to the static scheduler was made:

The static schedule has additional parameter “Poll Type”. It can be either “External Poll” when AP send Poll frame to the STA or “Self Poll” when AP access the medium using PIFS period to transmit it’s own data to the STA. This “Poll Type” variable allows to organize process of Periodic Reverse Direction Request if:

- Poll Type is set to “Self Poll”

- The traffic stream to which self poll will be issued has bi-directional nature.

- Reverse traffic of that stream has priority > than 7, i.e. the traffic stream is excluded from the contention process

- Reverse option has to be enabled.

If the described above is true for a traffic stream than its reverse part will be transmitted using a periodic reverse data flow request without the MAC/PHY overhead of polling/training.

#### Implicit BA mechanism

The Implicit Block Acknowledgement mechanism is implemented as follows (on example of SRA)

- The Originator sends an aggregate (SRA) to the responder. The SRA has a BAR frame encapsulated at the end. The Originator sends MPDUs within the SRA in order of their sequence numbers. The number of frames per SRA is limited to 64 frames because of the BA bitmap limitation. The originator can’t send a new SRA (or to retransmit the old one) until it receives a BA from the responder or understand that this is not going to happen.

- The responder, upon reception of an SRA, responds to it if it detects the response address from this aggregation. The originator tracks during Rx process which MPDUs of the SRA were received and marks this as zero or one in an implicit BA bitmap. There is strict correspondence between position of MPDUs in SRA and the status in BA bitmap.

- If the responder fails to receive a BAR frame that carries a starting sequence window, the responder responds with BA using the BA bitmap generated during the RX proces, otherwise it generates a bitmap in accordance with parameters within the BAR frame.

- The originator upon the reception of BAR frame deletes from transmission queues acknowledged MPDUS and forms a new SRA that include unacknowledged MPDUs and new MPDUs. MPDUs within the SRA are kept in strict sequence number order.

Note1: Although the .11e protocol assumes transmission of unaggregated multiple frames and transmission of BAR, this is not used in our implementation and simulation. The BAR/BA pair exists per aggregation only, so BA bitmap reflect state of one current aggregation only.

Note2: TGnSync specs require  a BAR to be transmitted to solicit a BA  if an  expected BA is not received. In the current simulation model this is not performed. If the implicit BA is not received, the originator retransmits the whole SRA to the responder.   This approximatation of the TGnSync proposal is likely to be removed in later revisions of the model.

#### Short/Negotiated BlockAck

The MAC process supports three possible type of Block Acknowledgement:

- Standard BA ( has length defined as in .11e)

- Short BA. Implies usage of non-fragmented MSDUs only. 16 times shorter than Standard BA

- Negotiated BA. Has variable length that has to be negotiated duration BA protocol setup

These three options are implemented in a following manner:

- Standard BA. Has length of Block Ack bitmap exactly as specified in .11e standard

- Short BA.

- Negotiated BA. Since we do not have the ADDBA mechanism implemented, and in a light of Note1 in 2.1.8.5. Originator and responder can always implicitly negotiate on BA size. The algorithm is simple: Originator exactly knows size of SRA it is transmitting. Responder exactly knows size of SRA in receiving. Using this knowledge, Originator expect BA with bitmap of exactly same size as SRA and responder will generated BA with bitmap that covers only current (just received SRA). Note: This is an approximation of the TGnSync protocol that will result in an insignificant performance improvement.

#### A-MSDU

A-MSDU aggregation implemented as follows:

A layer at the top of the MAC receives and buffers MSDUs into one A-MSDU. An A-MSDU is formed either when the size of waiting packets reaches the maximal A-MSDU size or the maximal delay of the oldest packet reaches 1ms. Once an A-MSDU is formed, the timer is restarted when a new packet for the flow arrives.

- The A-MSDU is limited to 4Kb

- The A-MSDU timer can be set for every access category and is specific to every simulation scenario

.

On receiving an A-MSDU, the MAC process de-assembles the A-MSDU and then indicates the received frames in order to the LLC.

#### Delayed Channel Access (DCA) Algorithm for EDCA

##### Description of the Algorithm

Network traffic containing TCP and UDP flows is bursty and self-similar.  Self-similarity is manifested in the absence of a natural length of a "burst"; at every time scale ranging from few milliseconds to minutes and even hours, bursts consist of bursty subperiods separated by less bursty subperiods.  A fundamental perspective is that a flow can be thought as consisting of consecutive bursts at all time scales. In this document, a burst of a flow is defined as a sequence of packets bounded by the first and last packets.  Let A(i, j) be the arrival time of the jth packet of the ith burst for the flow and mi be the total number of packets in the ith burst.  Then the burst is uniquely identified by an inter-arrival time  for the flow that satisfies the following

- Packets in the ith burst satisfy a jitter timing constraint.  Namely for j = 1, 2, …, mi-1,  the inter-arrival time between any two neighboring packets is less than the expected inter-arrival time

A(i, j+1) – A(i, j) < 

- Packets in different bursts satisfy a separation timing constraint.  Namely for i≥1,

A(i+1, 1) – A(i, mi) ≥  

Another fundamental concept is the MAC channel access delay for a MAC frame. It is defined as the amount of time between the frame’s arrival at the front of the frame flow and the successful transmission to the intended receiving station, excluding the frame transmission time. Notice that when the channel load is high, the channel access delay is large and when the channel load is low, the channel access delay is small.  So the channel access delay provides a good measurement for the channel load observed by a station independently.

The DCA algorithm is to identify an inter-arrival time proportional to the last MAC channel access delay and aggregate packets belonging to the burst characterized by the inter-arrival time.  The packets for the burst are held in an aggregation buffer before being aggregated.  Each station shall maintain six state variables for each outgoing flow defined by the next table:

| Variable | Initial Value | Description |
| --- | --- | --- |
|  | 0.000001 | The inter-arrival time for the current burst. |
| i | 0 | The number of packets from upper layer that are still in the aggregation buffer. |
| Af | N/A | The arrival time of the first packet from upper layer in the current aggregation buffer. |
| Al | N/A | The arrival time of the last packet from upper layer. |
| Tca | N/A | The channel-access starting time for an aggregate. |
| Ttx | N/A | The transmission starting time for an aggregate. |
| t | N/A | The current time. |

There are three algorithm parameters related to the QoS constraints defined in the next table

|  | A positive constant that is the ratio of the inter-arrival time to the channel access delay. We recommend  takes a value no less than than 2, e.g.  = 10. |
| --- | --- |
|  | A constant that is the maximal waiting time for packets in the aggregation buffer. For example, for a video flow with 200ms maximal delay, we recommend τ = 100ms for low packet loss ratio requirement (e.g. 10-2) and 50ms for high packet loss ratio requirement (e.g. 10-4). |
|  | A constant that determines the maximal number of packets in the aggregation buffer before aggregation is triggered. We recommend σ = 32. |

There are a few events that affect the values of the state variables:

| Event | Description | Effect |
| --- | --- | --- |
| Packet arriving from upper layer | Receive a packet from upper layer belonging to the flow. | if ( i == 0 ) Af = t; endif Al = t; i++; if ( i ≥ σ ) Tca = t; i = 0; Form an aggregate out of the packets in the aggregation buffer, and start channel access and transmission of the aggregate; endif |
| Timer expiration | Any of the following - The waiting time for the first packet reaches the threshold value τ, i.e. i > 0 and t – Af >= τ. - The idling time since receiving the last packet reaches the value i.e. t – Al ≥ | Form an aggregate out of the packets in the aggregation buffer, and start channel access and transmission of the aggregate; Tca = t; i = 0; |
| Transmission of an aggregate as an initiator | Physically transmit the current aggregate to the destination as the TXOP initiator. | Ttr = t; |
| TXOP acquisition as a responder | Obtain TXOP in reverse direction flow as a TXOP responder | Form an aggregate out of the packets in the aggregation buffer, and start channel access and transmission of the aggregate, i = 0. |
| Tx success as an initiator | Any of the followings as a TXOP initiator - The successful reception of an Ack in response to a unicast MPDU - The successful reception of a BlockAck or ACK in response to a BlockAckReq - Transmission of an MPDU that has a multicast RA or has the “no acknowledgement” policy | = ×( Ttr – Tca ); |

##### Configuration of the DCA Algorithm in Simulation

The DCA algorithm is implemented to improve A-MPDU performance by delaying packets, thereby preventing them from contention for the medium and creating aggregations of larger size (as much a possible, satisfying protocol constraints).

A simple method is used to configure the Delayed Aggregation Scheduling Algorithm support for EDCA. Each station, including AP, has a set of parameters for enabling the DCA algorithm with proper protocol parameters. The simulation user has to provide their preferred parameter values like aggregation maximal aggregation delay for each traffic category at the beginning of simulations if they do not want the default values. (In practice this would be related to TXOP parameters).

The run-time simulation takes the following user input for configuring the algorithm:

- Maximal aggregate delay for best efforts and backgraound traffic categories

- Maximal aggregate delay for the video traffic category

- Maximal aggregate delay for the audio traffic category

- Aggregation size threshold

It is the maximal un-acked packet number.

- Aggregation inter-arrival time relaxation coefficient

- Aggregation scheduling algorithm

To enable the DCA algorithm, this value should be set to DELAYED_AGGREGATION.

Each station reads data from the input table. If the aggregation scheduling algorithm is set to DELAYED_AGGREGATION (DCA), it will read and apply other protocol parameters as listed above. Otherwise, it will ignore the other parameters and run the default aggregation scheduling algorithm.

## MAC features simulation methodology

### Status of simulation results

This section was introduced in the November 2004 meeting (revision 2 of this document).  The results have not been updated in the current revision.  This means that the goodput results do not compare directly with the results of section 2.2. of [3] However the differences introduced by use of this feature will not be significantly affected by the improvements made to the protocol and simulation reflected in section 2.2 of [3], so the performance gains from the use of this feature are still considered to be valid.

### Reverse direction feature simulation methodology

#### TCP/UDP p2p scenario

##### Simulation methodology

A specific scenario was prepared to estimate the performance gain of reverse directional data flow support both in UDP and TCP case. It includes two stations each running application of the same type (TCP or UDP) of the same offered load – 125 Mb/s. Distance between stations – 10 meters.

For each traffics type it was performed 6 set of simulations, 9 simulation runs in each set. The following parameters were performed from set to set:

- Each set of simulation contain results for one MSDU size. Six sets, MSDU size from  1500 bytes to 125 bytes

- Within each set MAX reverse size was modified from 5 MPDUs to 63 MPDUs.

Constant parameters for each simulation run:

Table 1. Common parameters for each set of simulations

| Parameter name | Value |
| --- | --- |
| TCP MSS size | Ethernet (1500 bytes) |
| TXOP size | 0.008 sec. |
| Data flow TID | 7 (AC_VO) |
| CwMin/CwMax | 3/7 |
| AIFS[AC_VO] | 2 |
| Max Direct Aggregation size | 63 MPDU |
| Offered load | 125Mb/s in each direction |

Table 2. Per simulation set

| Parameter name | Value |
| --- | --- |
| Frame size on the top of the protocol stack | 125/250/500/750/1000/1500 bytes |
| MAX reverse size | 0/5/10/15/25/35/45/55/63 |

The use of such large TXOP for the transmission is defined by the following: STA may form direct aggregation of the maximum size and responder may fit to the remained portion of TXOP the reverse of the maximum possible size.

##### Simulation results

###### TCP source

Picture 1. TCP performance. Throughput vs Reverse Size

Each curve represents a set of simulations for the fixed frame size at the application level.

###### UDP source

Picture 2. UDP performance. Throughput vs. Reverse Size

Each curve represents a set of simulations for the fixed frame size at the application level.

###### TCP vs. UDP, % performance gain

Picture 3. TCP performance gain, %

Each curve represents a set of simulations for the fixed frame size at the application level. The curve “125 bis” shows the performance benefit of in case when MSS size is equal to the size of the frame size.

Picture 4. UDP performance gain, %

Each curve represents a set of simulations for the fixed frame size at the application level.

#### Conclusion

In case of pure point-to-point connection both for TCP and UDP traffic sources, the maximum performance gain of reverse data flow support is up to 12%. These values can be obtained when size of the aggregation is small because of “small MPDUs”, i.e. bi-direction is good for the application with low offered traffic load.

#### TCP single stream performance

##### Simulation methodology

A scenario was prepared to estimate the performance gain of the reverse in case of single TCP stream. It includes two stations one running an application of the offered load of 200 Mb/s, another generates TCP ACK responses Distance between stations – 10 meters.

4 simulation runs were performed with the Reverse feature turned on and off on the station

Table 3. Common parameters for all simulations

| Parameter name | Value |
| --- | --- |
| TCP MSS size | Ethernet (1500 bytes) |
| TXOP size | 0.008 sec. |
| Data flow TID | 7 (AC_VO) |
| CwMin/CwMax | 3/7 |
| AIFS[AC_VO] | 2 |
| Max Direct Aggregation size | 63 MPDU |
| Offered load | 200Mb/s |
| MAX Reverse size | 63 MPDUs |
| Frame size on the top of the protocol stack | 1500 bytes |
| Protocol type | TCP |

Table 4. Per simulation

| Parameter name | Value |
| --- | --- |
| Reverse | On/Off |

Reverse support was consequently turned on/of in the following manner:

Table 5. Reverse values per STA per simulation run

| Simulation run | Initiator | Responder |
| --- | --- | --- |
| 1 | Off | Off |
| 2 | On | On |
| 3 | Off | On |
| 4 | On | Off |

#### Simulation results

Table 6. Single TCP stream performance

| Simulation run | MSDU Size | TXOP Size | Direct Aggregation size | Reverse Aggregation Size | Achieved Throughput | % change from run 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1500 | 0.008 | 63 | 63 | 154.016 |  |
| 2 | 1500 | 0.008 | 63 | 63 | 158.087 | 2.6432 |
| 3 | 1500 | 0.008 | 63 | 63 | 157.732 | 2.4126 |
| 4 | 1500 | 0.008 | 63 | 63 | 155.843 | 1.1859 |

#### VoIP performance

##### Simulation methodology

Special artificial scenario was prepared to estimate the performance gain of bi-directional data flow support both the set of VoIP clients. It includes one access point and a ring of VoIP clients around it. The radius of the ring is 10 meters. Three simulation runs were performed to show performance benefit of reverse for the support of VoIP traffic.

Constant parameters for each simulation run:

Table 7. Constant parameters

| Parameter name | Value |
| --- | --- |
| TXOP size | 0.002 sec. |
| Data flow TID | 7 (AC_VO) |
| CwMin/CwMax | 3/31 |
| AIFS[AC_VO] | 2 |
| Max Direct Aggregation size | 63 MPDU |
| Channel access function | EDCA |
| MAX reverse size | 10 |

Three types of simulations were performed varying number of VoIP clients and Reverse option only:

- 20 Clients, Reverse Off/On

- 30 Clients, Reverse Off/On

- 40 Clients, Reverse On

The results presented in a table form showing number of clients meeting VoIP QoS objectives for each simulation run.

##### Simulation results

The table below shows the number of supported VoIP clients

| Number of VoIP clients | Reverse Option | Number of stream failed to met QoS objectives |
| --- | --- | --- |
| 20 | OFF | 0 of 40 |
| 20 | ON | 0 of 40 |
| 30 | OFF | 57 of 60 |
| 30 | ON | 0 of 60 |
| 40 | On | 80 of 80 |

##### Summary

As it can be seen from the tables above reverse data flow support allow increasing number of supported VoIP client by ~ 50% from 20 to 30 and improving QoS values in case of 20 clients. The presented values are for EDCA channel access function.

##### TCP ring

###### Simulation methodology

The following simulations were performed to show the benefit of reverse data flow in case of multiple streams.

An artificial scenario was created: a ring of nodes around access point, ring radius – 10 meters. Two types of simulation were performed:

- Client downloads 30 Mb/s from AP; AP generates response TCP ACK traffic.

- Client uploads 30 Mb/s to AP.

Table 8. Common parameters for all simulations

| Parameter name | Value |
| --- | --- |
| Channel access function | EDCA |
| TCP traffic | 0.002 sec. |
| Data flow TID | 7 (AC_VO) |
| CwMin/CwMax | 7/15 |
| AIFS[AC_VO] | 2 |
| Max Direct Aggregation size | 63 MPDU |
| MAX reverse size | 63 |

##### Simulation results

| Simulation description | Aggregate Goodput (Mbps) |
| --- | --- |
| TCP clients upload 30 Mb/s steam to the AP, Reverse OFF | 107.89 |
| TCP clients upload 30 Mb/s steam to the AP, Reverse On | 107.59 |
| TCP clients download 30 Mb/s steam from the AP, Reverse OFF | 105.24 |
| TCP clients download 30 Mb/s stream from the AP, Reverse On | 108.05 |

##### Summary

As it is seen from the table above reverse data flow provides a modest benefit for saturated pure TCP traffic – around 3%.

##### Mixture traffic

###### Simulation methodology

Four simulations were performed to show the performance benefit in case of mixture traffic. The following scenario was created for this purpose: 10 TCP clients doing 30 Mb/s traffic stream and 10 VoIP clients are located on the circle of radius of 10 meters around access point. The following options were changed from simulation to simulation:

- Reverse is OFF

- Reverse is On

- Reverse On for the traffic in up  link direction only

- Reverse On for the traffic in down link direction only.

Table 9. Common for all simulations parameters

| Parameter name | Value |
| --- | --- |
| Channel access function | EDCA |
| TCP traffic | 0.002 sec. |
| Data flow TID | 7 (AC_VO) |
| CwMin/CwMax | 7/31 |
| AIFS[AC_VO] | 2 |
| Max Direct Aggregation size | 63 MPDU |
| MAX reverse size | 63 |

###### Simulation results.

Table 10. Aggregate goodput of mixture traffic load

| Simulation description | Aggregated Goodput |
| --- | --- |
| TCP clients download 30 Mb/s steam from the AP + 10 VoIP clients, Reverse OFF | 46.28032 |
| TCP clients download 30 Mb/s steam from the AP + 10 VoIP clients, Reverse ON for all streams | 77.69908 |
| TCP clients download 30 Mb/s steam from the AP + VoIP clients, Reverse allowed from VoIP clients and upload TCP Ack streams only | 64.95788 |
| TCP clients download 30 Mb/s steam from the AP + VoIP clients, Reverse allowed from VoIP clients and download TCP streams only | 76.3684 |

###### Summary

As it is seen from the table above reverse provide significant performance benefit in case of mixture traffic load (TCP and UDP). – up to 65% in this case.

### Header compression simulation methodology

MPDU Header compression was removed in the March 2005 proposal.

## PHY Process

This section describes the PHY model built into the MAC1 simulator.

The PHY evaluates post-detection capacity, and then uses this value to lookup PER from PER vs. capacity tables supplied by John Sadowsky of Intel.  The simulations that generated these curves included the impairments required by the TGn CC.

A link adaptation algorithm is also defined, which is described below.

### Sub-channel capacity definition

This sub-clause describes a system with M Tx antennas and L Rx antennas.

Assume that the receiver’s input signal for i-th sub-carrier has following form:

y(i)=H(i)x(i)+j=1KHj(i)xj(i)+n(i)               (1)

Where:

- H(i) is a channel matrix of  LXM dimension.

- x(i) is an transmitting signal, which satisfies to following requirements: x(i)(x(i))H=I

- j=1KHj(i)xj(i)=z is an interfering signal from K interferers.

- n(i) is additive Gaussian noise.

The sub-carrier’s capacity is:

C(i)=l=1NSSlog21+1W(i)(σ2I+j=1KHj(i)xj(i)xj(i)HHj(i)H)W(i)HllC(i)=l=1NSSlog21+1W(i)(σ2I+zzH)W(i)Hll (2)

Where:

- W(i)=H(i)HH(i)−1H(i)H

- NSS is a number of spatial streams.

- []ll  is a (l,l)-th matrix diagonal element.

- σ2≈1SNR, where SNR=PS/(N*NSS) is calculated per spatial stream basis

- N is a noise figure, N≈TN*B*NF,

- TN is a thermal noise figure (-174 dBm)

- B is the subcarrier bandwidth (312500 Hz)

- NF is a noise figure (10 dB)

### PER vs. length interpolation

Two tables are used for calculation of PER value. The first table is calculated for data length 1000 B, the second one is calculated for 100 B packets. PER values for other lengths are calculated using a linear approximation between these points.

### PHY performance results for 2x2

This section contains simulation results for 2X2 antenna configuration. 2 stations are in the scenario. PER results relate to MPDUs of length 1000B.  Throughput is defined as (1-PER) * PSDU PHY rate.

Parameters:

- Bandwidth: 20/40 MHz.

- Data sub-carriers: 48/108.

- Transmit power: 17 dBm.

- Noise figure: 10 dB.

- CCA threshold 0 dB

Note: in all simulation scenarios which perform measurements per ensemble average SNR the LOS channel models are used for simulation.

### Link Adaptation

This section defines the fast link adaptation (FLA) method used by the receiver to propose an MCS value.

Assuming that the max number of spatial streams NSS ≤L it is necessary to calculate instantaneous average channel capacities for 1,…, NSS streams as follows:

- Perform channel capacity calculations for 1,…,NSS streams.

- For each capacity perform calculation of PERs for all possible MCSs using table lookup. The PER values are calculated for 1000B packets.

- Select MCS that maximizes throughput and that also satisfies any predefined PER bound (currently 1%)

# MAC2 Simulation Methodology

## Higher Layer Properties

Traffic is generated by Constant distribution corresponding to given load.

The type and parameter of the TCP model is as follows.

| TCP Factor | Parameter |
| --- | --- |
| TCP Model | Simplified New Reno |
| Maximum Window Size | 65,535[Bytes] without window scale option |
| Retransmission Time Out | Based on SRTT measurement |

## MAC Modes Simulated

This section describes the different “MAC Modes” defined to compare each proposed mechanisms.  The proposed mechanisms include, fast link adaptation, Short NAV, and Pairwise Spoofing.

### Mode 1: Standard NAV with Rate Adaptation

This scenario is defined as a reference point of simulation.  IAC/RAC/Block ACK packets are sent at a basic rate of 54[Mbps].  Rate selection is basically done at the receiver.

The initiator will set its NAV according to the predicted rate X.  The responder could override this default rate from the SINR of the IAC packet.  The initiator will send as many packets as it can during the set duration, leaving time for SIFS and BA.  If the TX buffer for that responder becomes empty, or if the next packet does not fit within the indicated duration, the remaining duration is unused.

The protection method used is Standard NAV because the NAV protects multiple packets.  However we only attempt to protect a single aggregate.  NAV reset is not used.

Figure 1 Standard NAV with Rate Adaptation

To summarize, the following operation is assumed

  - Duration value is delivered via MAC header.

  - Duration value is set to for Standard NAV operation without NAV reset

  - Duration may be wasted due to rate adaptation

  - IAC/RAC are sent at basic rate of 54[Mbps]

  - Block ACK is sent at basic rate of 54[Mbps]

### Mode 2: Pairwise Spoofing with Rate Adaptation

Spoofing is another way to achieve protection against legacy devices.  By using the spoofing mechanism the IAC/RAC packets do not have to be sent at a basic rate but can be sent at higher rates that the responder can decode.  And at the same time, can achieve higher level of protection via spoofing from third party nodes.  The Block ACK packets are sent at a basic rate of 54[Mbps] to maintain the same level of error protection.

Figure 2 Pairwise NAV with Rate Adaptation with Spoofing

To summarize, the following operation is assumed

  - Duration value is delivered via PLCP header.

  - Duration value is set to for Pairwise NAV operation : Protection of Single Response Packet

  - Duration will NOT be wasted due to rate adaptation

  - IAC/RAC are sent at predicted rates

  - Block ACK is sent at basic rate of 54[Mbps]

## MAC Layer Parameters

The following section describes the MAC layer assumptions used in the simulator.

### Frame Size

Frame Sizes are based on the TGn Sync MAC Specification draft, D.10.0.

| Frame Size | Parameter |
| --- | --- |
| MAC Header | 16[Bytes] |
| IAC Payload | 18[Bytes] |
| RAC Payload | 11[Bytes] |
| Compressed Block ACK Payload | 16[Bytes] |
| MPDU Delimiter | 4[Bytes] |

### Frame Aggregation

MAC Level aggregation is implemented in the simulator.  For simplification, the maximum number of aggregated frames is 16 MPDU per PPDU.

| Aggregation Factor | Parameter |
| --- | --- |
| Frame Aggregation | A-MPDU |
| Maximum # of Aggregate | 64[MPDU] |

### Rate Adaptation

IAC/RAC/ACK packets are sent at unified rate of 54[Mbps], (MCS3) when spoofing is NOT used.  The duration for the IAC packet is set so that it can send up to 64[MPDUs] at a predicted rate for DATA, as far as the predicted duration is less than Max TXOP duration (currently set to 3.12[msec] for all the access categories).  This predicted rate is chosen from previous transaction with the communicating node.  The responder may override this predicted DATA rate from the SINR of the IAC packet.

| Rate Adaptation Factor | Parameter |
| --- | --- |
| Rate for IAC/RAC | Uniform(54[Mbps]) *ONLY for Standard NAV |
| Rate for BA | Uniform(54[Mbps]) |
| Target PER | 10[%] |

### 802.11e Support

Parts of 802.11e are implemented inside the simulator.

| 802.11e Factor |  | Parameter |
| --- | --- | --- |
| Block ACK |  | Compressed BA |
| EDCF | AIFS | Incorporated (See Below) |
|  | CW | Incorporated (See Below) |
|  | TXOP | One Transmission Sequence Only |
| HCCA |  | Not Used |
| DLP |  | Used(Contention Based Access) |

AIFS is used as described below.  In some evaluation set, Contention Window size is alternated for each usage model for optimization purposes.  The size of individual parameter setting is noted in the results document.

| Priority | AC | AIFSN[] | AIFS[us] |
| --- | --- | --- | --- |
| Lowest ↓ ↓ Highest | AC_BK | 7 | 79 |
|  | AC_BE | 3 | 43 |
|  | AC_VI | 2 | 34 |
|  | AC_VO | 2 | 34 |

*AIFS[AC] = AIFSN[AC] x aSlotTime + a SIFSTime

### Additional MAC Parameters

Additional MAC parameters are as follows. In case of buffer overflow, transmitting MSDU will be discarded in tail-drop manner.

| MAC related parameters | Value |
| --- | --- |
| Transmission Buffer Depth | AP : 2048[MSDU], MT : 512[MSDU] |
| De-queue policy | MAX Amount with Urgent Alert (See Section 3.3.6) |
| Block ACK window size | 32[MSDU] |
| RTS Threshold | 160.0[usec] |
| aSlotTime | 9.0[usec] |
| MRMRA | Not Implemented |
| Bi-Directional | Disabled |
| Fragmentation | Disabled |

### RA Selection Heuristic

When a STA obtains a TXOP for an Access Category with the EDCA mechanism, the Receiver Address is selected using the following rules;

If there are no MPDUs of the Access Category that has been in the transmission buffer for more than a defined time, then the RA is set to the one that has the most number of MSDUs buffered.

If there are MPDUs that have been held in the transmission buffer for more than a defined duration, the Urgent Alert will be set to the RA of this MPDU(s), and among them the RA with the oldest MPDU will be selected.

## PHY Layer Modeling

The following physical layer characteristics and parameters are assumed for all stations.

| PHY Model Parameters | Parameter |
| --- | --- |
| Frequency Channel | 5.25[GHz] |
| Multi-path fading | Enabled |
| Shadow fading | Disabled |
| Signal Bandwidth | 20.0/40.0[MHz] |
| Transmit Power | 50.0[mW] |
| Antenna Gain (TX/RX) | 0.0[dBi] |
| Antenna Pattern (TX/RX) | Omni-Directional |
| Noise Figure | 10.0[dB] |
| CCA accuracy (energy detection) | RSSI>=-65[dBm] |
| CCA accuracy (preamble detection) | SINR>=3.0[dB] |
| CCA delay | 6.0[usec] |

The following 50[Byte] Block Error Rate Curves are used for the simulation.

The Block Error Rates (BLERs) are derived from PER vs. instantaneous SNR curve of 100[Byte] packets provided by Darren McNamara, Toshiba.  Note that one BLER curve is used for all links in one scenario (does not use different tables for LOS and NLOS).

Figure 3 Block Error Rate 50[Bytes]: 2x2x40MHz, Ch.B, and NLOS

Figure 4 Block Error Rate 50[Bytes]: 2x2x40MHz, Ch.D, and NLOS

Figure 5 Block Error Rate 50[Bytes]: 2x2x40MHz, Ch.E, and NLOS

Figure 6 Block Error Rate 50[Bytes]: 2x2x20MHz, Ch.B, and NLOS (with Short GI)

Figure 7 Block Error Rate 50[Bytes]: 2x2x20MHz, Ch.D, and NLOS (without Short GI)

Figure 8 Block Error Rate 50[Bytes]: 2x2x20MHz, Ch.E, and NLOS (without Short GI)

The appropriate BLER curve is selected according to the scenario defined in the usage model document.

The following figure shows relationship between Rx. SNR vs. distance between transmitter and receiver, based on channel model and physical layer model described.  (Instantaneous fading component is not included here.)

Figure 9 RxSNR when BW=40[MHz], TxPower=50[mW], NF=10[dB]

An example above is a graph showing the Rx SNR vs. Distance.   The breakpoints of the curves are dBP = 5[m], 10[m], 20[m] for channel models B, D, E respectively.

In this evaluation, distance loss is calculated based on the parameters, break pointdBP, and wave length of 0.057[m] assuming channel frequency of 5.25[GHz]. Interference from other system or adjacent bands is not taken into account.

The RxSNR is determined using the fading model described in 3.4.1.1.

### Channel model

Radio propagation behavior is modeled by following three components.

- Distance loss

- Shadow fading

- Multi-path fading

Distance loss is uniquely calculated as a function of distance between transmitter and receiver.

Shadow fading is calculated randomly according to given standard deviation, corresponding to each “link”.

Multi-path fading is calculated randomly according to given received power fluctuation versus time, corresponding to each “link” and time. Considering multi-path fading characteristic, received power is varying both on time domain and frequency domain (per sub-carrier).

However, in order to make calculation simpler in the MAC simulation, frequency domain fluctuation is not reproduced within MAC simulation, and channel model provides total SINR value of the OFDM symbol at the receiver as a function of transmitterφ, receiverϕ, and timet, assuming following abstraction.

SINRφ,ϕ,t=i=0NSC−1dφ,ϕ,ti=0NSC−1uiφ,ϕ,t+ni=i=0NSC−1dφ,ϕ,ti=0NSC−1uiφ,ϕ,t+i=0NSC−1ni

Where d is the received power of desired signal at the i-th subcarrier, ui is the received power of un-desired signal (sum of interference transmitted from other stations) at the i-th subcarrier, ni is the noise power at the i-th subcarrier, and NSC−1 is the number of subcarrier of the OFDM signal.

#### RX SINR Calculation

As shown at the formula above, received SINR value is calculated from ratio of desired signal power versus undesired signal power plus noise.

The desired signal power at the receiverϕ is calculated by following formula.

Dφ,ϕ,t≡i=0NSC−1dφ,ϕ,t=i=0NSC−1PφLφ,ϕ,t=PφLφ,ϕ,t

Where

- Pφ is transmit power of transmitterφ,

- Lφ,ϕ,t is pathloss between transmitterφ and receiverϕ at timet.

Further, pathloss Lφ,ϕ,t is calculated by following formula, considering the 3 major components (distance loss, shadow fading, multi-path fading).

Lφ,ϕ,t[dB]=DistLossdistφ,ϕ+Shadowφ,ϕ,t+Fadeφ,ϕ,t

Where, distance loss DistLossdistφ,ϕ[dB] is calculated as defined in the channel model document [1], shadow fading Shadowφ,ϕ,t[dB] is set to zero as usage model document [2] specifies, and multi-path fading Fadeφ,ϕ,t is calculated based on RiceanFading model as described in the following sub-section.

The undesired signal power at the receiverϕ is calculated by following formula.

Uφ,ϕ,t≡i=0NSC−1uφ,ϕ,t=i=0NSC−1k≠φ,k≠ϕNSTA−1PikLik,ϕ,t=k≠φ,k≠ϕNSTA−1i=0NSC−1PikLik,ϕ,t=k≠φ,k≠ϕNSTA−1PkLk,ϕ,t

The noise power at the receiverϕ is calculated by following formula.

Nϕ≡i=0NSC−1n=TN×BW×NF

Where:

- TN is thermal noise equal to -174[dBm/Hz],

- BW is signal band width equal to 20[MHz] or 40[MHz], and

- NF is noise figure set to 10[dB].

#### Instantaneous fading model

As described above, multi-path fading is categorized as instantaneous fading component, and given asFadeφ,ϕ,t in the pathloss calculation. This component is utilized intended to model time varying channel. Though received power is fluctuating corresponding to each subcarrier respectively, the received power summing total subcarrier is used as fading component in MAC simulation.

Fadeφ,ϕ,t=1NSCi=0NSC−1fadeiφ,ϕ,t

Where, fadeiφ,ϕ,t=fs2express attenuation of signal caused by multi-path fading in power at i-th subcarrier.

In the MAC simulator, this component is modeled as Ricean fading based on Jake’s Rayleigh fading model. Following graphs summarize the distribution of Fadeφ,ϕ,t observed at each channel models, and distribution of Ricean fading generated in MAC simulator. Both of the distribution is close enough to utilize Ricean fading as a simplified model of MIMO channels for each environment in the MAC simulation. In following graphs, red line denotes Fadeφ,ϕ,t distribution extracted from PHY simulation reproducing MIMO channel based on the parameters defined in channel model document [1], while blue line denotes approximation used in MAC simulation.

20MHz 2x2 MIMO Channel-B	40MHz 2x2 MIMO Channel-B

20MHz 2x2 MIMO Channel-D	40MHz 2x2 MIMO Channel-D

20MHz 2x2 MIMO Channel-E	40MHz 2x2 MIMO Channel-E

For each channel, following K-factor is adopted as a parameter of Ricean fading.

| Channel | K-factor |
| --- | --- |
| CM-B 20MHz 2x2 MIMO | 5.0 |
| CM-B 40MHz 2x2 MIMO | 7.0 |
| CM-D 20MHz 2x2 MIMO | 12.0 |
| CM-D 40MHz 2x2 MIMO | 22.0 |
| CM-E 20MHz 2x2 MIMO | 28.0 |
| CM-E 40MHz 2x2 MIMO | 48.0 |

Doppler frequency is defined by environmental speed and carrier frequency. As specified in channel model document [1], 1.2[km/h] is assumed as environmental speed, and 5.25[GHz] as carrier frequency is assumed.

Following graph shows one example of time varying fluctuation of Fadeφ,ϕ,t generated by above approximation.

Figure 10 Fading Fluctuation

### Physical Layer Characteristics

In the MAC simulation, transmit power is set to 17dBm] uniquely for every stations with 0[dBi] gain omni-directional antenna. Receiving antenna gain is set to same as transmit antenna.

#### CCA accuracy

PHY_CCA.ind(STATUS=busy) is issued either of following two condition is true.

- RSSI exceeds given threshold (-65.0[dBm]).
RSSI measurement result is forwarded to MAC layer with 2.0[usec] delay considering propagation delay due to filter, etc.

- PLCP preamble (STS part) is continuously received with SINR of equal or higher than given threshold (3.0[dB]) for 6.0[usec].

#### Error Modeling

In MAC simulation, bit error behavior at the receiver is modeled by block error. Since, MAC layer behavior will be different depending on error occurrence “in which part of PSDU”, error occurrence per some block is simulated, based on the assumption that error occurrence probability is identical for any part of the PSDU. For this evaluation, error occurrence per 50[Byte] are examined referring error occurrence probability at the Rx SINR value, according to predetermined given BLER vs. SNR curve.

BLER vs. SNR curve is generated from the following approximation using PER vs. SNR performance results derived from PHY simulations, following the formula below.

PERL=1−1−BLERNLN

Where, PERL is the Packet Error Rate of PSDU length of L, and BLERN is the Block Error Rate of block length of N (50[Byte]).

Though this approximation using BLER provides optimistic assumption for the environment where PERL is larger number, the difference becomes less than 1.0[dB] at typical operating environment (SNR range).

#### Link Quality Measurement

In order to achieve fast rate adaptation, PHY layer report measured SNR value to MAC layer. MAC layer will decide MCS based on this information. PHY layer is assumed to provide SINR value of PCLP header part with perfect accuracy.

As for MCS selection, MAC layer counts packet error rate in MSDU, and try to stabilize PER to some targeted PER (in this evaluation around 10%).

# Cross-Comparison of MAC1 and MAC2 simulators

In order to check the basic performance of the two simulators was similar, we performed a comparison using a deliberately simplified simulation scenario.

## MAC Mode

LongNAV operation was selection for this comparison.

## Simulation Scenario

This section describes the simulation scenario used in the simulation.

This model is defined to check the validity of the two PHY assumptions made between the simulators.

Figure 11 Point to Point Test

## Simulation Results

This section describes the simulation results.

### Point to Point Test (5[m])

Throughput results for both models for Point to Point Test 5[m] are compared in this section.

#### 2x2x40MHz, Channel Model D, NLOS

| Tx Node | Rx Node | Offered Load [Mbps] | Achieved Load [Mbps] |  |
| --- | --- | --- | --- | --- |
|  |  |  | MAC1 | MAC2 |
| 0 | 1 | 100 | 80.3904 | 81.02760 |
| 1 | 0 | 100 | 77.9712 | 79.69920 |
| Total |  | 200 | 158.3616 | 160.7268 |

### Point to Point Test (20[m])

#### 2x2x40MHz, Channel Model D, NLOS

| Tx Node | Rx Node | Offered Load [Mbps] | Achieved Load [Mbps] |  |
| --- | --- | --- | --- | --- |
|  |  |  | MAC1 | MAC2 |
| 0 | 1 | 100 | 45.3312 | 46.87800 |
| 1 | 0 | 100 | 46.0608 | 47.11440 |
| Total |  | 200 | 91.392 | 93.9924 |

## Conclusion

It can be seen that the rates achieved using the simple point-point scenario are sufficiently similar (~2% error) that the basic operation of the MAC layer is considered to be validated.  Other points of comparison may be observed from the CC results for more complex simulation scenarios.

# 2x2 Performance results from MAC1 PHY Model

This section shows the PHY layer behavior results of the MAC1 simulator for various channel models and bandwidths.

Each section presents a throughput vs. range curve, and also shows the PER vs. range curve.  Results are shown for each of the mandatory PHY modes and for the fast link adaptation algorithm using these modes.

## Simulation results, Channel D, Bandwidth 40 MHz

## Simulation results, Channel model B, Bandwidth 40 MHz

## Simulation results, Channel E, 40 MHz

## Simulation results, Channel D, Bandwidth 20 MHz

## Simulation results, Channel B, Bandwidth 20 MHz

## Simulation results, Channel E, Bandwidth 20 MHz

The sensitivity at interference free environment is equivalent to -88[dBm], with detection probability of 100%.

Typically, this assumption is not correct. Error occurrence probability is varying depending on the OFDM symbol, according to the PHY layer demodulation scheme, such as channel tracking algorithm, etc.