IEEE P802.11
Wireless LANs

Partial Proposal Response to the Comparison Criteria

Date:	August 13, 2004

Author:	Takashi Fukagawa, et al
	Panasonic
	BCC Matsushita Electric Industrial Co. Ltd.

4-5-15 Higashi shinagawa Shinagawa-ku Tokyo 140-8632 Japan
	Phone: +81-3-5460-2725
	Fax: +81-3-5460-2836
	e-Mail: fukagawa.takashi@jp.panasonic.com

Abstract

This document describes the response of the partial proposal, refered to in this document as (IEEE 802.11-04/956r0) to the TGn Comparison Criteria as described in IEEE 802.11-03/814r31.

# ADDITIONAL DISCLOSURES

## AD1

Table 1 contains the list of IEEE submissions and documents, referred to by the proposal:

| [1] | “Local and Metropolitan Area Networks – Specific Requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications”, IEEE Std 802.11-1999, IEEE, August 1999. |
| --- | --- |
| [2] | “Draft Supplement to LAN/MAN Specific Requirements – Part 11: Wireless Medium Access Control (MAC) and Physical Layer (PHY) specifications: MAC Enhancements for Quality of Service (QoS)”, IEEE Std 802.11e/D8.0 March 2004. |
| [3] | “Local and Metropolitan Area Networks – Specific Requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Higher-Speed Physical Layer Extension in the 5 GHz Band”, IEEE Std 802.11a-1999, IEEE, September 1999. |
| [4] | Y. Kim et al, “Throughput Enhancement via Frame Aggregation – A Sequel”, IEEE 802.11-03/567r1 |

Table 1: List of References

## AD2

Table 2 contains the list of TCP parameters used by the proposal.

| Maximum Segment Size | Calculated based on MTU of 1st IP interface on surrounding node |
| --- | --- |
| Receive Buffer (Bytes) | 8760 |
| Receive Buffer Adjustment | None |
| Receive Buffer Usage Threshold | 0.0 |
| Delayed ACK Mechanism | Segment/Clock Based (Every 2 segments or Maximum ACK Delay) |
| Maximum ACK Delay (sec) | 0.200 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Disabled |
| Selective ACK | Disabled |
| ECN Capability | Disabled |
| Segment Sent Threshold | Byte Boundary (During fast retransmission slow start threshold will be set to half of the current congestion window) |
| Active Connection Threshold | Unlimited |
| Nagle Algorithm | Disabled |
| Karn’s Algorithm | Enabled |
| Timestamp | Disabled |
| Initial Sequence Number | Auto Compute (Random number is chosen) |
| Retransmission: Mode | Attempts Based (Maximum number of attempts before retransmission is aborted) |
| Retransmission: Maximum Connect Attempts | 3 |
| Retransmission: Maximum Data Attempts | 6 |
| Initial RTO (sec) | 3 (Initial Retransmission TimeOut) |
| Minimum RTO (sec) | 1 |
| Maximum RTO (sec) | 64 |
| RTT Gain | 0.125 |
| Deviation Gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity (sec) | 0.5 |
| Persistence Timeout (sec) | 1.0 |

Table 2: TCP Parameters

## AD3

As this is a partial proposal, the MAC simulation makes use of an error-free transmission medium. The following are simulation setting for each scenario:

|  | Scenario 1 | Scenario 4 | Scenario 6 | Scenario 17 - 19 |
| --- | --- | --- | --- | --- |
| Beacon Interval | 100 ms |  |  |  |
| AIFS Setting | Default (as specified in 802.11e draft v8.0) |  |  |  |
| Cwmin[0] | 31 | 63 | 127 | 15 |
| Cwmin[1] | 31 | 63 | 127 | 15 |
| Cwmin[2] | 31 | 31 | 15 | 7 |
| Cwmin[3] | 15 | 15 | 31 | 3 |
| Cwmax[0] | 1023 | 1023 | 1023 | 1023 |
| Cwmax[1] | 1023 | 1023 | 1023 | 1023 |
| Cwmax[2] | 31 | 31 | 63 | 15 |
| Cwmax[3] | 31 | 31 | 31 | 7 |
| TXOP Limit | Default (as specified in 802.11e draft v8.0) |  |  |  |
| Data Tx Rate | 126 Mbps OFDM PHY |  |  |  |
| QoS Poll Tx Rate | 24 Mbps OFDM PHY |  |  |  |
| Maximum Service Interval | Delay bound <= 50, 10ms |  |  |  |
|  | Delay bound > 50, 20ms |  |  |  |
| Ack Policy Setting | Block ACK Size = 10 | Block ACK Size = 5 | Block ACK Size = 10 | Block ACK Size = 10 |
|  | AP -> STA1 | AP -> STA7 | AP -> STA11 |  |
|  | AP -> STA3 | AP -> STA8 | AP -> STA12 |  |
|  | AP -> STA4 | STA7 -> AP | AP -> STA13 |  |
|  | AP -> STA4 | STA8 -> AP | AP -> STA14 |  |
|  | The rest using No ACK | Block ACK Size = 10 | AP -> STA15 |  |
|  |  | AP -> STA9 | AP -> STA16 |  |
|  |  | AP -> STA10 | AP -> STA17 |  |
|  |  | The rest using No ACK | AP -> STA18 |  |
|  |  |  | AP -> STA19 |  |
|  |  |  | The rest using No ACK |  |

## AD4

The MAC simulation assumes PHY rates achieved through an occupied channel width of 20MHz

# COMPARISON CRITERIA

## CC2

The proposers are unaware of any regulatory issues arising through the implementation of this proposal in any regulatory domains.

## CC3

Table 3 lists the goodput results (CC20, metric 1) for scenarios 1,4 and 6.

| Scenario | Aggregate Goodput (Mbps) |
| --- | --- |
| Scenario 1 | 63.0084 |
| Scenario 4 | 23.3968 |
| Scenario 6 | 55.9434 |

Table 3: Aggregate Goodput results (CC20, metric 1)

## CC11

- The proposal maintains backward compatibility to legacy 802.11 systems by making use of the legacy timing infrastructure – inter-frame spaces andbackoffs; and legacy access mechanisms specified by 802.11 and 802.11e.

- Additional inter-frame spaces are specified as extensions to existing definitions (Refer pg: 9).

- Additional frame types and subtypes are defined using the existing conventions (Refer pg: 3).

## CC15

T1: 100Mbps

T2: 33.5667 Mbps (with .11 MAC)

T3: 16.7677 Mbps

T4: 62.3892 Mbps

## CC18

Tables 4, 5 and 6, respectively report the goodput values and associated results for the non-QoS flows in simulation scenarios 1, 4 and 6.

| STA/Flow | Goodput |
| --- | --- |
| AP -> STA4 | 884579 bps |
| STA4 -> AP (TCP Ack) | 14025 bps |
| STA4 -> STA10 | 9237984 bps |
| STA10 -> STA4 (TCP Ack) | 115697 bps |

Table 4: Goodput for non-QoS flows in SS1

Aggregate non-QoS Goodput in SS1: 10252284 bps

Ratio of aggregate Goodput to Offered load (non-QoS) in SS1: 0.331

| STA/Flow | Goodput (bps) |
| --- | --- |
| AP -> STA1 | 370667 bps |
| STA1 -> AP (TCP Ack) | 18501 bps |
| AP -> STA2 | 412373 bps |
| STA2 -> AP (TCP Ack) | 18439 bps |
| AP -> STA3 | 496920 bps |
| STA3 -> AP (TCP Ack) | 24601 bps |
| AP -> STA4 | 467338 bps |
| STA4 -> AP (TCP Ack) | 23567 bps |
| AP -> STA5 | 468655 bps |
| STA5 -> AP (TCP Ack) | 23208 bps |
| AP -> STA6 | 4012800 bps |
| STA6 -> AP (TCP Ack) | 188366 bps |
| AP -> STA11 | 322259 bps |
| STA11 -> AP (TCP Ack) | 2844 bps |
| AP -> STA12 | 187934 bps |
| STA12 -> AP (TCP Ack) | 1652 bps |
| AP -> STA13 | 16264 bps |
| STA13 -> AP (TCP Ack) | 167 bps |
| AP -> STA14 | 539708 bps |
| STA14 -> AP (TCP Ack) | 4768 bps |
| AP -> STA15 | 312018 bps |
| STA15 -> AP (TCP Ack) | 2760 bps |
| AP -> STA16 | 202391 bps |
| STA16 -> AP (TCP Ack) | 1788 bps |
| AP -> STA17 | 190344 bps |
| STA17 -> AP (TCP Ack) | 1641 bps |
| AP -> STA18 | 259614 bps |
| STA18 -> AP (TCP Ack) | 2258 bps |
| AP -> STA19 | 296960 bps |
| STA19 -> AP (TCP Ack) | 2572 bps |
| AP -> STA20 | 348762 bps |
| STA20 -> AP (TCP Ack) | 3074 bps |
| STA1 -> AP | 105099 bps |
| AP -> STA1 (TCP Ack) | 29850 bps |
| STA2 -> AP | 100168 bps |
| AP -> STA2 (TCP Ack) | 26097 bps |
| STA3 -> AP | 97181 bps |
| AP -> STA3 (TCP Ack) | 26942 bps |
| STA4 -> AP | 2354834 bps |
| AP -> STA4 (TCP Ack) | 31431 bps |
| STA5 -> AP | 1750117 bps |
| AP -> STA5 (TCP Ack) | 16261 bps |
| STA6 -> AP | 79464 bps |
| AP -> STA6 (TCP Ack) | 22966 bps |
| STA21 -> AP | 21082 bps |
| AP -> STA21 (TCP Ack) | 198 bps |
| STA22 -> AP | 70475 bps |
| AP -> STA22 (TCP Ack) | 658 bps |
| STA23 -> AP | 145769 bps |
| AP -> STA23 (TCP Ack) | 1401 bps |
| STA24 -> AP | 143360 bps |
| AP -> STA24 (TCP Ack) | 1357 bps |

Table 5: Goodput for non-QoS flows for SS4

Aggregate non-QoS Goodput in SS4: 14249923 bps

Ratio of aggregate Goodput to offered load (non-QoS) in SS4: 0.0316

| STA/Flow | Goodput (bps) |
| --- | --- |
| AP -> STA1 | 1136999 bps |
| STA1 -> AP (TCP Ack) | 13072 bps |
| AP -> STA2 | 1225811 bps |
| STA2 -> AP (TCP Ack) | 14033 bps |
| AP -> STA3 | 1164612 bps |
| STA3 -> AP (TCP Ack) | 13366 bps |
| AP -> STA4 | 1148981 bps |
| STA4 -> AP (TCP Ack) | 13430 bps |
| AP -> STA5 | 1171555 bps |
| STA5 -> AP (TCP Ack) | 13323 bps |
| AP -> STA6 | 1167867 bps |
| STA6 -> AP (TCP Ack) | 13219 bps |
| AP -> STA7 | 1138929 bps |
| STA7 -> AP (TCP Ack) | 13009 bps |
| AP -> STA8 | 1213204 bps |
| STA8 -> AP (TCP Ack) | 13783 bps |
| AP -> STA9 | 1143675 bps |
| STA9 -> AP (TCP Ack) | 13134 bps |
| AP -> STA10 | 1223401 bps |
| STA10 -> AP (TCP Ack) | 13668 bps |

Table 6: Goodput for non-QoS flows in SS6

Aggregate non-QoS Goodput in SS6: 11869069 bps

Ratio of aggregate Goodput to Offered load (non-QoS) in SS6: 0.5935

## CC19

Tables 7, 8 and 9, respectively report the goodput values and associated results for the QoS flows in simulation scenarios 1, 4 and 6.

| STA/Flow | PLR (Target) | PLR (Actual) | Pass/Fail |
| --- | --- | --- | --- |
| AP -> STA1 | 10^-7 | 0 | Pass |
| AP -> STA3 | 10^-7 | 0 | Pass |
| AP -> STA4 | 5 * 10^-7 | 0 | Pass |
| AP -> STA7 | 5% | 0 | Pass |
| AP -> STA8 | 5% | 0 | Pass |
| AP -> STA9 | 5% | 0 | Pass |
| AP -> STA10 | 10^-4 | 0 | Pass |
| AP -> STA11 | 10^-4 | 0 | Pass |
| STA1 -> AP | 10^-2 | 0 | Pass |
| STA3 -> AP | 10^-2 | 0 | Pass |
| STA5 -> STA6 | 10^-2 | 0 | Pass |
| STA6 -> STA5 | 10^-2 | 0 | Pass |
| STA7 -> AP | 5% | 0 | Pass |
| STA8 -> AP | 5% | 0 | Pass |
| STA9 -> AP | 5% | 0 | Pass |
| STA10 -> AP | 10^-4 | 0 | Pass |
| STA11 -> AP | 10^-4 | 0 | Pass |

Table 7: QoS objectives for flows in SS1

Number of flows satisfying the QoS objectives in SS1: 18

Fraction of flows satisfying the QoS objectives in SS1: 1

| STA/Flow | PLR (Target) | PLR (Actual) | Pass/Fail |
| --- | --- | --- | --- |
| AP -> STA7 | 10^-4 | 0 | Pass |
| AP -> STA8 | 10^-4 | 0 | Pass |
| AP -> STA8 | 10^-4 | 0 | Pass |
| AP -> STA10 | 10^-4 | 0 | Pass |
| AP -> STA25 | 5% | 0 | Pass |
| AP -> STA26 | 5% | 0 | Pass |
| AP -> STA27 | 5% | 0 | Pass |
| AP -> STA28 | 5% | 0 | Pass |
| AP -> STA29 | 5% | 0 | Pass |
| AP -> STA30 | 5% | 0 | Pass |
| STA7 -> AP | 10^-4 | 0 | Pass |
| STA8 -> AP | 10^-4 | 0 | Pass |
| STA25 -> AP | 5% | 0 | Pass |
| STA26 -> AP | 5% | 0 | Pass |
| STA27 -> AP | 5% | 0 | Pass |
| STA28 -> AP | 5% | 0 | Pass |
| STA29 -> AP | 5% | 0 | Pass |
| STA30 -> AP | 5% | 0 | Pass |

Table 8: QoS objectives for flows in SS4

Number of flows satisfying the QoS objectives in SS4: 18

Fraction of flows satisfying the QoS objectives in SS4: 1

| STA/Flow | PLR (Target) | PLR (Actual) | Pass/Fail |
| --- | --- | --- | --- |
| AP -> STA11 | 10^-4 | 0 | Pass |
| AP -> STA12 | 10^-4 | 0 | Pass |
| AP -> STA13 | 10^-4 | 0 | Pass |
| AP -> STA14 | 10^-4 | 0 | Pass |
| AP -> STA15 | 10^-4 | 0 | Pass |
| AP -> STA16 | 10^-4 | 0 | Pass |
| AP -> STA17 | 10^-4 | 0 | Pass |
| AP -> STA18 | 5 * 10^-7 | 0 | Pass |
| AP -> STA19 | 5 * 10^-7 | 0 | Pass |
| AP -> STA20 | 5% | 0 | Pass |
| AP -> STA21 | 5% | 0 | Pass |
| AP -> STA22 | 5% | 0 | Pass |
| AP -> STA23 | 5% | 0 | Pass |
| AP -> STA24 | 5% | 0 | Pass |
| AP -> STA25 | 5% | 0 | Pass |
| AP -> STA26 | 5% | 0 | Pass |
| AP -> STA27 | 5% | 0 | Pass |
| AP -> STA28 | 5% | 0 | Pass |
| AP -> STA29 | 5% | 0 | Pass |
| AP -> STA30 | 5% | 0 | Pass |
| AP -> STA31 | 5% | 0 | Pass |
| AP -> STA32 | 5% | 0 | Pass |
| AP -> STA33 | 5% | 0 | Pass |
| AP -> STA34 | 5% | 0 | Pass |
| STA20 -> AP | 5% | 0 | Pass |
| STA21 -> AP | 5% | 0 | Pass |
| STA22 -> AP | 5% | 0 | Pass |
| STA23 -> AP | 5% | 0 | Pass |
| STA24 -> AP | 5% | 0 | Pass |
| STA25 -> AP | 5% | 0 | Pass |
| STA26 -> AP | 5% | 0 | Pass |
| STA27 -> AP | 5% | 0 | Pass |
| STA28 -> AP | 5% | 0 | Pass |
| STA29 -> AP | 5% | 0 | Pass |
| STA30 -> AP | 5% | 0 | Pass |
| STA31 -> AP | 5% | 0 | Pass |
| STA32 -> AP | 5% | 0 | Pass |
| STA33 -> AP | 5% | 0 | Pass |
| STA34 -> AP | 5% | 0 | Pass |

Table 9: QoS objectives for flows in SS 6

Number of flows satisfying the QoS objectives in SS6: 39

Fraction of flows satisfying the QoS objectives in SS6: 1

## CC20

Table 10 reports the metrics – 1, 2 and 3 called out by the CC document for simulation scenarios 1, 4 and 6.

| Simulation Scenario | Metric#1 (Mbps) | Metric#2 (Mbps) | Metric#3 (Mbps) |
| --- | --- | --- | --- |
| Scenario 1 | 62.76886 | 52.516576 | 52.516576 |
| Scenario 4 | 23.39733 | 9.147392 | 9.147392 |
| Scenario 6 | 56.750925 | 44.881856 | 44.881856 |

Table 10: BSS Aggregate Goodput at the MAC data SAP

## CC46

The proposal requires the use of the Block ACK (BA) mechanism, specified in [2]. The BA mechanism is used to facilitate acknowledgement of individual MSDUs (with ACK policy set to BA), when the MAC-frame aggregation described in the proposal is used.

Individual Compartment MSDUs are identified by a unique sequence number, TID and source and destination addresses. Acknowledgements of packets sent to a particular STA are sought using the block-ACK request frame and delivered using the BA response frame. (For details, refer to pg: 6, of the specification).

## CC47

To facilitate MAC aggregation, the proposal requires the creation of a new frame-type – Extended and subtype – Aggregated Data in the baseline 802.11 specification [1]. (For details, refer to pg: 3, of the technical proposal document).

To facilitate the use of reduced inter-frame spaces, the proposal defines two new inter-frame spaces. The use of the new inter-frame spaces is signalled using the proposed RCE bit in the TXVECTOR and RXVECTOR interface (For details, refer to pg: 9, of the technical proposal document).

## CC51

| PHY Layer Data Rate (Mbps) | Modulation Technique | Number of TX Antennas | Coding Scheme | Coding Rate | Bandwidth (MHz) | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | BPSK | 1 | CC | ½ | 20 | Mandatory |
| 12 | QPSK | 1 | CC | ½ | 20 | Mandatory |
| 24 | 16-QAM | 1 | CC | ½ | 20 | Mandatory |
| 48 | 64-QAM | 1 | CC | 2/3 | 20 | Mandatory |
| 54 | 64-QAM | 1 | CC | ¾ | 20 | Mandatory |
| 36 | QPSK | 2 | CC | ¾ | 20 | Mandatory |
| 72 | 16-QAM | 2 | CC | ¾ | 20 | Mandatory |
| 96 | 64-QAM | 2 | CC | 2/3 | 20 | Mandatory |
| 108 | 64-QAM | 2 | CC | ¾ | 20 | Mandatory |
| 115 | 64-QAM | 2 | LDPC | 4/5 | 20 | Mandatory |
| 126 | 64-QAM | 2 | CC | 7/8 | 20 | Mandatory |
| 108 | 16-QAM | 3 | CC | ¾ | 20 | Optional |
| 144 | 64-QAM | 3 | CC | 2/3 | 20 | Optional |
| 172 | 64-QAM | 3 | LDPC | 4/5 | 20 | Optional |

Table 11: BSS Aggregate Goodput at the MAC data SAP

## CC51.5

This proposal assumes a 20MHz channelization.

## CC59

Shown in Figure 1, Annex.

## CC67

Part 1 of CC67 shown in Figures 2, 3 and 4, respectively of Annex.

## CC67.2

E-NLOS (Antenna spacing=0.5) environment

SNR@PER=10%

PHY rate		108Mbps(2x2 MIMO 3/4 64QAM)	72Mbps(2x2 MIMO 3/4 16QAM)

No offset                    40 dB					31.5 dB

Offset +40ppm	41.5 dB (+1.5dB)			32 dB (+0.5dB)

Offset -40ppm	40.7 dB (+0.7dB)			32 dB (+0.5dB)

E-LOS (Antenna spacing=0.5) environment

PER@SNR=50dB

PHY rate		108Mbps(2x2 MIMO 3/4 64QAM)	72Mbps(2x2 MIMO 3/4 16QAM)

No offset		0.0632					0.0056

Offset +40ppm	0.068					0.0192

Offset -40ppm	0.0664					0.012

## CC80

[Summary]

- 2x2MIMO or 3x3 MIMO

- Scattered and Staggered pilots subcarriers

- Varying Interleave Patterns

[Reference]

- Shown in proposal document, pages (15 through 28)

ANNEX

Figure 1: Performance in AWGN

Figure 2: Performance in Channel B-NLOS

Figure 3: Performance in Channel D-NLOS

Figure 4: Performance in Channel E-NLOS