IEEE P802.11
Wireless LANs

Comparison Criteria and Functional Requirements: I2R Submssion

Date:	September 09, 2004

Author:	Sumei SUN, Chin Keong Ho, Patrick Fung, Yuan Li, Yan Wu, Zhongding Lei, Woon Hau Chin, Ying-Chang Liang, Francois Chin

Institute for Infocomm Research
	21 Heng Mui Keng Terrace, Singapore 119613
	Phone: 65-68747588
	Fax: 65-67768109
	e-Mail: sunsm@i2r.a-star.edu.sg

Abstract

This document discloses the information according to the document “IEEE TGn comparison criteria” (IEEE 802.11-03/814r31) and “802.11 TGn Functional Requirements” (IEEE 802.11-03/813r12), based on the proposal submiited “TGn MIMO-OFDM PHY Partial Proposal-Specifications” [1].

# Comparison Criteria

| Number | Name | Definition | Disclosure |
| --- | --- | --- | --- |
| CC2 | Regulatory compliance | The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. | Requires the use of 40MHz bandwidth. |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Provide a summary description of the means, if any, by which backward compatibility with 802.11-1999 (Rev 2003) and 802.11g is achieved in the band(s) covered by the proposal, including references to the sections in the technical proposal document where the complete details are given. | Support concurrent transmissions of two 11a signals in downlink by inserting 11 subcarriers at DC (see Section 2.5.9, [1]) Majority of the PHY such as scrambler, FEC, bit interleaver is unchanged (see Sections 2.2.4,2.2.5,2.5.4, [1]) |
| CC51 | Data rates | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. For adaptive rate proposals, specify the range of achievable rates or, if possible, state the achievable rates in a closed form expression. | Tables are found at the end of the document. Mandatory Mode: Table 1 - Rate-dependent parameters for 2×2 Spatial Multiplexing (SM) MIMO-OFDM Optional Modes: Table 2 - Rate-dependent parameters for 2×2 STBC MIMO-OFDM Table 3 - Rate-dependent parameters for 4×2 GSTBC MIMO-OFDM Table 4 - Rate-dependent parameters for 4×2 STBC MIMO-OFDM |
| CC42 | Preambles | Specify the proposed preambles. Summarize the important properties of each part the proposed preambles. Include references to the sections in the technical proposal document where the complete details are given. Specify how the use of any new preamble affects reception by legacy STA. | See Section 2.3, [1]. 1) Mandatory mode: PAPR = 9.9188 2) Optional mode PAPR = 7.5547 |
| CC51.5 | Channelization | Specify the channelization – i.e. the adjacent channel spacing. | 0.3125 MHz (= 40 MHz/128) |
| CC67 | PER performance in non AWGN channels | Show either or both of the following two sets of performance curves: 1.) Show the PER curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Plot PER versus SNR averaged over time per receive antennas for PSDUs of length 1000B. Averaging should occur over a minimum of 100 MPDU errors down to 1% PER. Each packet should use an independent channel realization. All models should be run without the fluorescent effect but additionally model D should be run with the fluorescent effect on the highest data rate. The interferer to carrier energy ratio of the fluorescent effect shall be equal to its average value (0.0203). The shadowing variance should be 0. These simulations are performed using the NLOS version of the specified channel models. Note: SNR is defined in section 2. | Data Rates of 48, 96, 216 Mbps are provided for 2x2 SM and 4x2 GSTBC architecture. The coded PER are plotted against SNR in Fig. 1 for Channel model B and Fig. 2 for Channel model E. Fig. 3 illustrates the performance when LDPC is used instead of convolutional codes. Fig. 4 plots the coded PER for Channel B with LMMSE channel estimation, frequency offset estimation and phase compensation. Fig. 5 plots the coded PER for Channel E with LMMSE channel estimation, frequency offset estimation and phase compensation. The imperfect stated in IM1-6 are not entirely implemented. |
| CC80 | Required changes to 802.11 PHY | Give a summary description of changes to a legacy 802.11 PHY. Give references to sections in your specification that give the complete details. | OFDM modulation over 40MHz channel with FFT size of 128; Peak data rate of 216 Mbps; 2×2 MIMO supporting both spatial multiplexing and orthogonal STBC in mandatory mode; 4×2 MIMO from access point to terminal station (downlink) supporting groupwise STBC (GSTBC), orthogonal STBC, antenna beamforming, and antenna selection in optional mode; Linear pre-transform in both frequency and spatial domain to exploit the frequency and spatial diversities; Efficient training signal design (preambles) that supports robust frequency and timing synchronization and channel estimation; Low-density parity check (LDPC) code as optional mode; Support of concurrent transmissions of two 11a signals in downlink with novel subcarrier arrangement. |

# Coverage of Functional Requirements

Each requirement shall be declared to be covered or not covered.  For each requirement relating to measurements, a reference to the document/page showing the appropriate performance measurements should be added.

| Number | Name | Coverage (Yes/No) | Results Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes |  |
| R2 | HT rate supported in 20MHz channel | No |  |
| R3 | Supports 5GHz bands | Yes |  |
| R4 | .11a backwards compatibility | Yes |  |
| R5 | .11g backwards compatibility | N.A. |  |
| R6 | Control of support for legacy STA from .11n AP | Yes |  |
| R7 | .11e QoS support | N.A. |  |
| R8 | Spectral Efficiency | Yes |  |
| R9 | Compliance to PAR | Yes |  |

# References

[1] IEEE 802.11-04-0875-01-000n, TGn MIMO-OFDM PHY Partial Proposal-Specifications

# Tables and Figures

Table 1 - Rate-dependent parameters for 2×2 Spatial Multiplexing (SM) MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | BPSK | 1/2 | 1 | 96 | 48 | 192 | 96 |
| 36 | BPSK | 3/4 | 1 | 96 | 72 | 192 | 144 |
| 48 | QPSK | 1/2 | 2 | 192 | 96 | 384 | 192 |
| 72 | QPSK | 3/4 | 2 | 192 | 144 | 384 | 288 |
| 96 | 16-QAM | 1/2 | 4 | 384 | 192 | 768 | 384 |
| 144 | 16-QAM | 3/4 | 4 | 384 | 288 | 768 | 576 |
| 192 | 64-QAM | 2/3 | 6 | 576 | 384 | 1152 | 768 |
| 216 | 64-QAM | 3/4 | 6 | 576 | 432 | 1152 | 864 |

Table 2 - Rate-dependent parameters for 2×2 STBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | BPSK | 1/2 | 1 | 96 | 48 | 96 | 48 |
| 18 | BPSK | 3/4 | 1 | 96 | 72 | 96 | 72 |
| 24 | QPSK | 1/2 | 2 | 192 | 96 | 192 | 96 |
| 36 | QPSK | 3/4 | 2 | 192 | 144 | 192 | 144 |
| 48 | 16-QAM | 1/2 | 4 | 384 | 192 | 384 | 192 |
| 72 | 16-QAM | 3/4 | 4 | 384 | 288 | 384 | 288 |
| 96 | 64-QAM | 2/3 | 6 | 576 | 384 | 576 | 384 |
| 108 | 64-QAM | 3/4 | 6 | 576 | 432 | 576 | 432 |

Table 3 - Rate-dependent parameters for 4×2 GSTBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | BPSK | 1/2 | 1 | 96 | 48 | 192 | 96 |
| 36 | BPSK | 3/4 | 1 | 96 | 72 | 192 | 144 |
| 48 | QPSK | 1/2 | 2 | 192 | 96 | 384 | 192 |
| 72 | QPSK | 3/4 | 2 | 192 | 144 | 384 | 288 |
| 96 | 16-QAM | 1/2 | 4 | 384 | 192 | 768 | 384 |
| 144 | 16-QAM | 3/4 | 4 | 384 | 288 | 768 | 576 |
| 192 | 64-QAM | 2/3 | 6 | 576 | 384 | 1152 | 768 |
| 216 | 64-QAM | 3/4 | 6 | 576 | 432 | 1152 | 864 |

Table 4 - Rate-dependent parameters for 4×2 STBC MIMO-OFDM

| Data rate (Mbps) | Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Coded bits per MIMO-OFDM symbol (NMCBPS) | Data bits per MIMO-OFDM symbol (NMDBPS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | BPSK | 1/2 | 1 | 96 | 48 | 96 | 48 |
| 18 | BPSK | 3/4 | 1 | 96 | 72 | 96 | 72 |
| 24 | QPSK | 1/2 | 2 | 192 | 96 | 192 | 96 |
| 36 | QPSK | 3/4 | 2 | 192 | 144 | 192 | 144 |
| 48 | 16-QAM | 1/2 | 4 | 384 | 192 | 384 | 192 |
| 72 | 16-QAM | 3/4 | 4 | 384 | 288 | 384 | 288 |
| 96 | 64-QAM | 2/3 | 6 | 576 | 384 | 576 | 384 |
| 108 | 64-QAM | 3/4 | 6 | 576 | 432 | 576 | 432 |

Figure 1: Coded PER performance, Channel B.

Figure 2 Coded PER performance, Channel E.

Figure 3 Coded PER performance, Channel B, 216 Mbps.

48 Mbps

96 Mbps

216 Mbps

Figure 4: Coded PER performance, Channel B, with LMMSE channel estimation, frequency offset estimation and phase compensation.

48 Mbps

96 Mbps

216 Mbps

Figure 5: Coded PER performance, Channel B with LMMSE channel estimation, frequency offset estimation and phase compensation.