IEEE P802.11
Wireless LANs

Response to Call For Proposal for P802.11n

Date:	August, 2004

Authors:	Hervé Bonneville, Bruno Jechoux 
	Mitsubishi ITE.
	1, allee de Beaulieu, 35700 Rennes, France
	Phone: +33-2 23 45 58 58
	Fax: +33-2 23 45 58 59
	e-Mail: {bonneville,jechoux}@tcl.ite.mee.com

Abstract

This document is a response to Call For Proposal for P802.11n standard referenced by document [1]. It includes Functional Requirements table, comparison criteria response and a technical overview. Document 11-04-0916-00-000n-Proposal-Mitsubishi-ITE-DetailedDescription gives additional technical details.

Contributors:

| Hervé Bonneville | Mitsubishi |
| --- | --- |
| Bruno Jechoux | Mitsubishi |
| Romain Rollet | Mitsubishi |

Table of Content

1.	References	3

2.	Functional requirements table	4

3.	Comparison criteria table	4

3.1	Additional disclosures	4

3.2	Comparison Criteria	5

3.3	Detailed Simulation results	9

3.3.1	PHY modes list	9

3.3.2	PHY modes use	9

3.3.3	CC 18 through CC24	10

3.3.4	CC15 (scenarios 17, 18 and 19)	24

4.	Other simulation results	25

4.1	Simulations using complete set of MIMO modes	25

4.1.1	Scenario1bis	25

4.1.2	Scenario 4bis	29

4.1.3	Scenario 6bis	35

4.2	Goodput and MAC efficiency versus PHY data rate	43

4.3	Delays	43

4.4	Mixed traffic handling	44

5.	Simulation conditions and hypothesis	44

5.1	System simulations	44

5.1.1	Simulation parameters	44

5.1.2	PHY abstraction	44

6.	Rationales & Objectives	46

6.1	Need for a new access scheme	46

6.2	Requirements for a CE compatible WLAN	46

6.2.1	Application requirements	46

6.2.2	Environment requirements	47

6.3	MAC features	47

6.4	Some conclusions	48

6.4.1	QoS support	49

6.4.2	Compatibility	49

6.4.3	Efficient resource usage	49

6.4.4	Versatility	49

6.4.5	Flexibility	49

6.4.6	Easy Implementation	49

7.	Technical overview	50

7.1	Presentation	50

7.2	Frame structure	50

7.3	MTF description	51

7.4	MPDU	52

7.5	MPDU overhead reduction	52

7.6	Aggregation at PHY level	52

7.7	Segmentation and Reassembly	53

7.8	MAC Time Frame structure example	53

7.9	Resource management and flow differentiation	54

Annex A: Abbreviations and acronyms	55

Simulation results quick overview

Table 3-1: Performance summary for scenario 1	13

Table 3-2: Performance summary for scenario 4	18

Table 3-3: Performance summary for scenario 6	24

Table 4-1: Performance summary for scenario 1bis	28

Table 4-2: Performance summary for scenario 4bis	34

Table 4-3: Performance summary for scenario 6bis	41

Table of figures

Figure 4-1: TCP throughput and MAC efficiency versus PER in scenarios 1 and 1bis	29

Figure 4-2: TCP throughput and MAC efficiency versus PER in scenarios 4 and 4bis	35

Figure 4-3: TCP throughput and MAC efficiency versus PER in scenarios 6 and 6bis	42

Figure 4-4: MAC efficiency vs. PHY throughput	43

Figure 4-5: Delay distribution (scenario 1)	43

Figure 4-6: Mixed traffic handling	44

Figure 7-1: Extended MAC structure	50

Figure 7-2: Frame structure	51

Figure 7-3: Alternative frame structure	51

Figure 7-4: MAC Time Frame structure	51

Figure 7-5: MPDU general structure	52

Figure 7-6: MPDU structure example with data blocks transmitted to 2 stations	52

Figure 7-7: Segmentation And Reassembly (SAR) sub-layer	53

Figure 7-8: MTF structure example	53

# References

- 11-04-0916-00-000n-Proposal-Mitsubishi-ITE-DetailedDescription

- IEEE 802 11-03/858r7, Call for Proposals for P802.11n

- IEEE 802 11-03/813r12, Functional requirements

- IEEE 802 11-03/814r30, TGn Comparison Criteria

- IEEE 802 11-03/802r23, TGn Usage Models

# Functional requirements table

| Number | Name | Coverage (Yes/No) | Results Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes | See 4.2 |
| R2 | HT rate supported in 20MHz channel | Yes | Since we have a MAC efficiency of about 80%, this requirement is achieved with a PHY rate of 120 Mbit/s, which can be reached in a 20 MHz band with a 2x2 antenna configuration and a 64 QAM ¾ code. |
| R3 | Supports 5GHz bands | Yes |  |
| R4 | .11a backwards compatibility | Yes | All .11a PHY modes can be used. |
| R5 | .11g backwards compatibility | Yes | All .11g PHY modes can be used. |
| R6 | Control of support for legacy STA from .11n AP | Yes |  |
| R7 | .11e QoS support | Yes | EDCA and HCCA can be used without any modification. |
| R8 | Spectral Efficiency | Yes | PHY mode dependent |
| R9 | Compliance to PAR | Yes |  |

# Comparison criteria table

## Additional disclosures

| Number | Name | Disclosure |
| --- | --- | --- |
| AD1 | Reference submissions |  |
| AD2 | TCP Model Parameters | Reno based. TCP window sizes: - Set to 64 Kbytes in scenario 1, 6 - Set to 192 Kbytes in scenarios 4, 4bis, 1bis and 6bis |
| AD3 | MAC simulation methodology | See 5 |
| AD4 | MAC simulation occupied channel width | See simulation results. |

## Comparison Criteria

| Number | Name | Disclosure |
| --- | --- | --- |
| CC2 | Regulatory compliance | 40 MHz physical modes allowed in some countries only. The MAC presented here does not mandate 40 MHz operations. |
| CC3 | List of goodput results for usage models 1,4 and 6. | Referred to as Metric 1 in CC20: Scenario 1: 79.100 Mbit/s (average PHY data rate: 105.40 Mbit/s) Scenario 4: 77.927 Mbit/s (average PHY data rate: 106.24 Mbit/s) Scenario 6: 66.208 Mbit/s (average PHY data rate: 92.58 Mbit/s) Note: hereafter results for scenarios 1, 4 and 6 with fully backlogged TCP sources (denoted 1bis, 4bis and 6bis). TCP flows use as much bandwidth as available. Scenario 1bis: 148.394 Mbit/s (average PHY data rate: 197.80 Mbit/s) Scenario 4bis: 228.611 Mbit/s (average PHY data rate: 340.99 Mbit/s) Scenario 6bis: 124.231 Mbit/s (average PHY data rate: 184.97 Mbit/s) |
| CC6 | PHY complexity | N/A |
| CC7 | MAC processing complexity | Same complexity as 802.11e (including HCCA) |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | - The AP broadcasts beacons legacy devices can decode. - DCF access method is not modified at all inside the Contention Periods, thus legacy devices are not affected and can fully inter-operate - HCCA and EDCA, as defined in .11e, can be used without modification - See chapter 7 in general, paragraph 7.2 more specifically |
| CC15 | Sharing of medium with legacy devices | Case 1: TGn stations use PHY 3x3 64 QAM 2/3 on 1x 20 MHz channel (162 Mbit/s). Legacy station uses 64 QAM. Metric T1 (scenario 17): 100.00 Mbit/s Metric T2 (scenario 18): 30.00 Mbit/s Metric T3 (scenario 19): 100.00 Mbit/s Metric T4 (scenario 19): 9.00 Mbit/s Case 2: TGn stations use PHY 3x3 64 QAM 2/3 on 2x 20 MHz channels (324 Mbit/s). Legacy station uses 64 QAM. Metric T1 (scenario 17): 100.00 Mbit/s Metric T2 (scenario 18): 30.00 Mbit/s Metric T3 (scenario 19): 100.00 Mbit/s Metric T4 (scenario 19): 19.20 Mbit/s See 3.3.4 for details. |
| CC18 | HT Usage Models Supported (non QoS) | Note: hereafter results for scenarios 1, 4 and 6 with minimum PHY mode Scenario 1: Goodput per non-QoS flow: see 3.3.3.1 Average PHY data rate: 105.40 Mbit/s Aggregate goodput for non-QoS flows: 26.57 Mbit/s Ratio for non-QoS flows: 85.73 % Scenario 4: Goodput per non-QoS flow: see 3.3.3.2 Average PHY data rate: 106.24 Mbit/s Aggregate goodput for non-QoS flows: 68.78 Mbit/s Ratio for non-QoS flows: 15.25 % Scenario 6: Goodput per non-QoS flow: see 3.3.3.3 Average PHY data rate: 92.58 Mbit/s Aggregate goodput for non-QoS flows: 16.39 Mbit/s Ratio for non-QoS flows: 81.96 % Note: hereafter results for scenarios 1, 4 and 6 with complete set of PHY modes and fully backlogged TCP sources (denoted 1bis, 4bis and 6bis). TCP flows use as much bandwidth as available. Scenario 1bis: Goodput per non-QoS flow: see 4.1.1 Average PHY data rate: 197.80 Mbit/s Aggregate goodput for non-QoS flows: 95.87 Mbit/s Ratio for non-QoS flows: N/A* % Scenario 4bis: Goodput per non-QoS flow: see 4.1.2 Average PHY data rate: 340.99 Mbit/s Aggregate goodput for non-QoS flows: 219.46 Mbit/s Ratio for non-QoS flows: 48.66 % Scenario 6bis: Goodput per non-QoS flow: see 4.1.3 Average PHY data rate: 184.97 Mbit/s Aggregate goodput for non-QoS flows: 79.36 Mbit/s Ratio for non-QoS flows: N/A* % * N/A: Non applicable because TCP source throughputs are unlimited |
| CC19 | HT Usage Models Supported (QoS) | Scenario 1: PLR of each QoS flow: see 3.3.3.1 Nb of QoS flows satisfying the objectives: 17 % of QoS flows satisfying the objectives: 100 % Scenario 4: PLR of each QoS flow: see 3.3.3.2 Nb of QoS flows satisfying the objectives: 18 % of QoS flows satisfying the objectives: 100 % Scenario 6: PLR of each QoS flow: see 3.3.3.3 Nb of QoS flows satisfying the objectives: 39 % of QoS flows satisfying the objectives: 100 % |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Scenario 1 Scenario 4 Scenario 6 Average PHY data rate (Mbit/s) 92.580 106.240 92.58 Metric 1 (Mbit/s) 66.208 77.927 66.208 Metric 2 (Mbit/s) 66.208 77.927 66.208 Metric 4 (Mbit/s) 66.208 77.927 66.208 Reference 3.3.3.1 3.3.3.2 3.3.3.3 Note: hereafter results for scenarios 1, 4 and 6 with complete set of PHY modes and fully backlogged TCP sources (denoted 1bis, 4bis and 6bis). TCP flows use as much bandwidth as available. Scenario 1bis Scenario 4bis Scenario 6bis Average PHY data rate (Mbit/s) 197.8 340.99 184.97 Metric 1 (Mbit/s) 148.394 228.611 124.231 Metric 2 (Mbit/s) 148.394 228.611 124.231 Metric 4 (Mbit/s) 148.394 228.611 124.231 Reference 4.1.1 4.1.2 4.1.3 |
| CC24 | MAC Efficiency | Scenario 1: 75 % (see 3.3.3.1) Scenario 4: 73 % (see 3.3.3.2) Scenario 6: 72 % (see 3.3.3.3) Note: hereafter results for scenarios 1, 4 and 6 with fully backlogged TCP sources (denoted 1bis, 4bis and 6bis). TCP flows use as much bandwidth as available. Scenario 1bis: 75 % (see 4.1.1) Scenario 4bis: 67 % (see 4.1.2) Scenario 6bis: 67 % (see 4.1.3) |
| CC28 | Throughput / Range | We provide curves of goodput versus PHY rate for different PER See 4.2 |
| CC46 | MAC Compatibility and parameters | No options of 802.11a/b/d/e/g/h/i/j are required. |
| CC47 | MAC extensions | We define a new access method called Extended Centralised Coordinator Function (ECCF), which is an extension of HCCA. This access scheme takes place in the CFP period. Alternatively, it can happen during any period of time AP reserves in the same way .11e defines periods for HCCA. Both methods assure a full backward compatibility with legacy 802.11 and its extensions .e, .h, .i and others. It is based on a centralised resource management, with on-demand resource requests. An overview is given in 7, while document [1] gives a detailed description. |
| CC51 | Data rates | The simulations presented in this proposal suppose the following PHY modes list: 20MHz 40MHz 1Tx (*) 2Tx 3Tx 1Tx 2Tx 3Tx 16QAM½ 24 54 81 54 108 162 16QAM¾ 36 81 121.5 81 162 243 64QAM2/3 48 108 162 108 216 324 64QAM¾ 54 121.5 182.5 121.5 243 365 64QAM5/6 - 135 202.5 135 270 405 (*) Legacy Modes Throughput values in Mbit/s |
| CC42 | Preambles | PHY modes have preambles of following duration: SISO: 20 µs 2x2: 20 µs 3x3: 28 µs |
| CC51.5 | Channelisation | Channelisation is 20 MHz by default, with 40 MHz as an option. |
| CC52 | Spectral Mask | N/A |
| CC58 | HT Spectral Efficiency | N/A |
| CC59 | AWGN PER performance | N/A |
| CC67 | PER performance in non AWGN channels | N/A |
| CC67.2 | Offset Compensation | N/A |
| CC80 | Required changes to 802.11 PHY | A 120 Mbit/s PHY is mandatory to reach target of 100 Mbit/s at MAC-SAP 108 Mbit/s at PHY is enough for completing requirements in scenarios 1 and 6. |

## Detailed Simulation results

### PHY modes list

Table below summarises the PHY mode list that is used among all the system simulations.

|  |  |  | 20MHz |  |  | 40MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1Tx (*) | 2Tx | 3Tx | 1Tx | 2Tx | 3Tx |
|  | 16QAM½ | 24 | 54 | 81 | 54 | 108 | 162 |
| Throughput | 16QAM¾ | 36 | 81 | 121.5 | 81 | 162 | 243 |
| (Mbit/s) | 64QAM2/3 | 48 | 108 | 162 | 108 | 216 | 324 |
|  | 64QAM¾ | 54 | 121.5 | 182.5 | 121.5 | 243 | 365 |
|  | 64QAM5/6 | - | 135 | 202.5 | 135 | 270 | 405 |

(*) Legacy Modes

PHY preambles have the following duration depending on antenna configuration:

| Antenna configuration | Preamble duration (µs) |
| --- | --- |
| SISO | 20 |
| 2x2 | 20 |
| 3x3 | 28 |

### PHY modes use

Table below indicates the rule used to select the different PHY modes in scenarios 1bis, 4 and 6bis depending on the distance between stations.

|  | Range use |
| --- | --- |
| 16QAM½ | < 24m |
| 16QAM¾ | < 16m |
| 64QAM2/3 | < 12m |
| 64QAM¾ | < 8m |
| 64QAM5/6 | < 4m |

### CC 18 through CC24

#### Scenario1

In following results, all stations uses a SISO physical mode 64 QAM 2/3 in two 20 MHz channels, giving a PHY data rate of 108 Mbit/s. It is the lower data rate that allow the system to fulfil scenario's requirements. Similar results can be obtained another PHY combination, a 2x2 64 QAM 2/3 in 20 MHz for example.

The number of contention interval (CTI) per MTF (sub-frame) is equal to 4, corresponding to 2 per channel.

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_SISO_64QAM2/3 | 19199,4 | 2 | 6 | 0 | 19199,7 | 3 | 14 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_SISO_64QAM2/3 | 23997,6 | 6 | 8 | 0 | 23999,6 | 7 | 18 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_SISO_64QAM2/3 | 4000 | 2 | 6 | 0 | 4000 | 2 | 6 | 0 |
| AP | STA4 | Internet File | TCP | 40_SISO_64QAM2/3 | 1066,66 | 2 | 6 | 0 | 1066,67 | 2 | 8 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_SISO_64QAM2/3 | 1999,94 | 2 | 6 | 0 | 2000 | 2 | 8 | 0 |
| AP | STA11 | MP3 Audio | CBR | 40_SISO_64QAM2/3 | 128 | 2 | 4 | 0 | 128 | 2 | 6 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_SISO_64QAM2/3 | 60,01 | 4 | 4 | 0 | 60 | 4 | 20 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_SISO_64QAM2/3 | 60,01 | 4 | 4 | 0 | 60 | 4 | 16 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_SISO_64QAM2/3 | 30397,13 | 6 | 12 | 0 | 28710,43 | 9 | 24 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_SISO_64QAM2/3 | 499,99 | 4 | 4 | 0 | 500 | 4 | 8 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_SISO_64QAM2/3 | 499,99 | 4 | 4 | 0 | 500 | 4 | 12 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_SISO_64QAM2/3 | 999,97 | 3 | 6 | 0 | 999,99 | 4 | 12 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 40_SISO_64QAM2/3 | 499,97 | 4 | 6 | 0 | 500 | 4 | 8 | 0 |
| STA7 | AP | VoIP_UL7 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 96 | 4 | 12 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 96 | 4 | 12 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 40_SISO_64QAM2/3 | 95,97 | 4 | 4 | 0 | 95,99 | 4 | 14 | 0 |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5.10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_SISO_64QAM2/3 | 19199,83 | 4 | 18 | 0 | 19199,9 | 4 | 18 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_SISO_64QAM2/3 | 23999,5 | 8 | 24 | 0 | 23999,47 | 8 | 26 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_SISO_64QAM2/3 | 4000 | 2 | 10 | 0 | 4000 | 2 | 10 | 0 |
| AP | STA4 | Internet File | TCP | 40_SISO_64QAM2/3 | 1066,67 | 2 | 8 | 0 | 1066,67 | 2 | 10 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 10 | 0 | 96 | 2 | 14 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 14 | 0 | 96 | 2 | 14 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 14 | 0 | 96 | 2 | 16 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_SISO_64QAM2/3 | 2000 | 2 | 10 | 0 | 2000 | 2 | 10 | 0 |
| AP | STA11 | MP3 Audio | CBR | 40_SISO_64QAM2/3 | 128 | 2 | 10 | 0 | 128 | 2 | 8 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_SISO_64QAM2/3 | 60 | 6 | 24 | 0 | 60 | 9 | 24 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_SISO_64QAM2/3 | 60 | 7 | 24 | 0 | 60 | 10 | 26 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_SISO_64QAM2/3 | 25510,39 | 11 | 28 | 0 | 22944,25 | 12 | 30 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_SISO_64QAM2/3 | 500 | 4 | 10 | 0 | 500 | 4 | 12 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_SISO_64QAM2/3 | 500 | 4 | 16 | 0 | 500 | 5 | 20 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_SISO_64QAM2/3 | 999,99 | 4 | 14 | 0 | 999,99 | 4 | 18 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 40_SISO_64QAM2/3 | 500 | 4 | 12 | 0 | 499,99 | 4 | 12 | 0 |
| STA7 | AP | VoIP_UL7 | CBR | 40_SISO_64QAM2/3 | 96 | 4 | 14 | 0 | 96 | 4 | 14 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 40_SISO_64QAM2/3 | 96 | 4 | 14 | 0 | 96 | 4 | 14 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 16 | 0 |

PER from 10-1 to 1.5 10-2

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_SISO_64QAM2/3 | 19199,83 | 4 | 16 | 0 | 19199,8 | 5 | 18 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_SISO_64QAM2/3 | 23999,5 | 9 | 22 | 0 | 23999,17 | 10 | 28 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_SISO_64QAM2/3 | 3999,97 | 3 | 16 | 0 | 3999,97 | 4 | 16 | 0 |
| AP | STA4 | Internet File | TCP | 40_SISO_64QAM2/3 | 1066,66 | 2 | 12 | 0 | 1066,66 | 2 | 16 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 20 | 0 | 96 | 2 | 20 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 18 | 0 | 96 | 2 | 20 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 40_SISO_64QAM2/3 | 96 | 2 | 18 | 0 | 96 | 2 | 20 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_SISO_64QAM2/3 | 1999,99 | 3 | 14 | 0 | 2000 | 3 | 16 | 0 |
| AP | STA11 | MP3 Audio | CBR | 40_SISO_64QAM2/3 | 128 | 2 | 14 | 0 | 128 | 2 | 12 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_SISO_64QAM2/3 | 60 | 12 | 30 | 0 | 60 | 13 | 30 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_SISO_64QAM2/3 | 60 | 12 | 30 | 0 | 60 | 13 | 36 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_SISO_64QAM2/3 | 17915,32 | 15 | 42 | 0 | 13596,63 | 20 | 58 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_SISO_64QAM2/3 | 500 | 4 | 14 | 0 | 500 | 5 | 18 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_SISO_64QAM2/3 | 500 | 6 | 28 | 0 | 500 | 7 | 28 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_SISO_64QAM2/3 | 999,99 | 5 | 18 | 0 | 999,99 | 6 | 30 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 40_SISO_64QAM2/3 | 500 | 4 | 18 | 0 | 500 | 5 | 18 | 2,22E-06 |
| STA7 | AP | VoIP_UL7 | CBR | 40_SISO_64QAM2/3 | 96 | 4 | 18 | 0 | 96 | 4 | 22 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 40_SISO_64QAM2/3 | 96 | 4 | 18 | 0 | 96 | 4 | 22 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |

| Scenario 1: Performance Summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1,5.10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 31.463 | 29.777 | 26.577 | 24.010 | 18.981 | 14.663 |
| Ratio for non-QoS flows (CC18) | % | 101.50 | 96.06 | 85.73 | 77.45 | 61.23 | 47.30 |
| Aggregate goodput for QoS flows | Mbit/s | 52.523 | 52.523 | 52.523 | 52.523 | 52.523 | 52.522 |
| Number of QoS flows satisfying the objectives (CC19) |  | 17 | 17 | 17 | 17 | 17 | 17 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 83.984 | 82.300 | 79.100 | 76.534 | 71.505 | 67.186 |
| Metric 2 of CC20 | Mbit/s | 83.984 | 82.300 | 79.100 | 76.534 | 71.505 | 67.186 |
| Metric 3 of CC20 | Mbit/s | 83.984 | 82.300 | 79.100 | 76.534 | 71.505 | 67.186 |
| Average PHY rate | Mbit/s | 105.4 | 105.4 | 105.4 | 105.4 | 105.4 | 105.4 |
| MAC efficiency (CC24) | % | 80 | 78 | 75 | 73 | 68 | 64 |

Table 3-1: Performance summary for scenario 1

A PER of 3%is considered as a mean value in normal conditions. Values corresponding to this PER are reported in Comparison Criteria table.

#### Scenario 4

In following results, all stations uses a SISO physical mode 64 QAM ¾ in two 20 MHz channels, giving a PHY data rate of 121,5 Mbit/s.

The number of contention interval (CTI) per MTF (sub-frame) is equal to 4, corresponding to 2 per channel.

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_SISO_64QAM3/4 | 1066,62 | 18 | 84 | 0 | 1066,65 | 22 | 102 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_SISO_64QAM3/4 | 1066,63 | 18 | 92 | 0 | 1066,59 | 22 | 100 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_SISO_64QAM3/4 | 1066,63 | 18 | 76 | 0 | 1066,6 | 22 | 110 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_SISO_64QAM3/4 | 1066,63 | 38 | 270 | 0 | 1066,62 | 39 | 284 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_SISO_64QAM3/4 | 1066,6 | 38 | 288 | 0 | 1066,6 | 39 | 284 | 0 |
| AP | STA6 | Email_Download | TCP | 40_SISO_64QAM3/4 | 3243,39 | 94 | 256 | 0 | 3186,17 | 88 | 264 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_SISO_64QAM3/4 | 1000 | 2 | 12 | 0 | 1000 | 2 | 14 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_SISO_64QAM3/4 | 1000 | 2 | 12 | 0 | 1000 | 2 | 14 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_SISO_64QAM3/4 | 2000 | 2 | 12 | 0 | 2000 | 3 | 16 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_SISO_64QAM3/4 | 2000 | 2 | 12 | 0 | 2000 | 3 | 16 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_SISO_64QAM3/4 | 3336,5 | 90 | 254 | 0 | 3274,65 | 90 | 264 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_SISO_64QAM3/4 | 3327,38 | 91 | 276 | 0 | 3281,06 | 90 | 290 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_SISO_64QAM3/4 | 3361,73 | 90 | 262 | 0 | 3271,41 | 90 | 264 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_SISO_64QAM3/4 | 3338,05 | 90 | 266 | 0 | 3300,25 | 90 | 264 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_SISO_64QAM3/4 | 3369,33 | 89 | 272 | 0 | 3306,57 | 89 | 278 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_SISO_64QAM3/4 | 3363,69 | 90 | 278 | 0 | 3289,98 | 90 | 282 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_SISO_64QAM3/4 | 3363,99 | 89 | 262 | 0 | 3269,86 | 91 | 280 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_SISO_64QAM3/4 | 3355,01 | 90 | 270 | 0 | 3278,47 | 90 | 262 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_SISO_64QAM3/4 | 3347,71 | 90 | 260 | 0 | 3315,08 | 89 | 256 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_SISO_64QAM3/4 | 3365,1 | 89 | 274 | 0 | 3291,58 | 90 | 288 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 12 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 12 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 12 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 2 | 12 | 0 | 95,98 | 2 | 12 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_SISO_64QAM3/4 | 335,96 | 28 | 84 | 0 | 335,97 | 31 | 114 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_SISO_64QAM3/4 | 335,96 | 28 | 92 | 0 | 335,97 | 31 | 104 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_SISO_64QAM3/4 | 335,96 | 27 | 78 | 0 | 335,97 | 31 | 110 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_SISO_64QAM3/4 | 4746,89 | 70 | 284 | 0 | 4641,13 | 71 | 296 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_SISO_64QAM3/4 | 4736,79 | 69 | 306 | 0 | 4622,79 | 70 | 306 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_SISO_64QAM3/4 | 335,84 | 71 | 256 | 0 | 335,9 | 71 | 264 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 4 | 0 | 999,99 | 4 | 16 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 4 | 0 | 999,99 | 4 | 16 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_SISO_64QAM3/4 | 4835,93 | 70 | 278 | 0 | 4754,05 | 71 | 276 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_SISO_64QAM3/4 | 4820,9 | 70 | 264 | 0 | 4746,42 | 71 | 258 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_SISO_64QAM3/4 | 4812,66 | 70 | 304 | 0 | 4777,83 | 70 | 294 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_SISO_64QAM3/4 | 4778,34 | 71 | 266 | 0 | 4716,96 | 71 | 284 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 14 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 14 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 14 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 14 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 14 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 14 | 0 |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5.10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_SISO_64QAM3/4 | 1066,6 | 27 | 120 | 0 | 1066,6 | 31 | 140 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_SISO_64QAM3/4 | 1066,54 | 27 | 116 | 0 | 1066,59 | 31 | 132 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_SISO_64QAM3/4 | 1066,6 | 27 | 140 | 0 | 1066,59 | 31 | 134 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_SISO_64QAM3/4 | 1066,6 | 41 | 266 | 0 | 1066,6 | 44 | 256 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_SISO_64QAM3/4 | 1066,52 | 41 | 270 | 0 | 1066,6 | 43 | 254 | 0 |
| AP | STA6 | Email_Download | TCP | 40_SISO_64QAM3/4 | 3072,45 | 88 | 290 | 0 | 2959,91 | 92 | 298 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_SISO_64QAM3/4 | 999,99 | 3 | 18 | 0 | 999,99 | 3 | 24 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_SISO_64QAM3/4 | 999,99 | 3 | 16 | 0 | 999,97 | 3 | 22 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_SISO_64QAM3/4 | 1999,97 | 3 | 20 | 0 | 1999,96 | 4 | 22 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_SISO_64QAM3/4 | 1999,99 | 3 | 22 | 0 | 1999,99 | 4 | 24 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_SISO_64QAM3/4 | 3174,33 | 92 | 298 | 0 | 3080,94 | 94 | 298 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_SISO_64QAM3/4 | 3173,82 | 92 | 296 | 0 | 3071,85 | 94 | 296 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_SISO_64QAM3/4 | 3172,24 | 92 | 300 | 0 | 3058,71 | 95 | 280 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_SISO_64QAM3/4 | 3151,86 | 93 | 282 | 0 | 3079 | 94 | 306 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_SISO_64QAM3/4 | 3188,82 | 91 | 278 | 0 | 3043,88 | 95 | 292 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_SISO_64QAM3/4 | 3169,02 | 92 | 298 | 0 | 3050,6 | 95 | 314 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_SISO_64QAM3/4 | 3137,24 | 93 | 288 | 0 | 3069,24 | 94 | 312 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_SISO_64QAM3/4 | 3188,18 | 91 | 294 | 0 | 3067,49 | 94 | 302 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_SISO_64QAM3/4 | 3170,78 | 92 | 276 | 0 | 3059,82 | 95 | 306 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_SISO_64QAM3/4 | 3163,18 | 92 | 262 | 0 | 3067,05 | 94 | 288 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 18 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 18 | 0 | 95,99 | 2 | 18 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM3/4 | 95,98 | 2 | 18 | 0 | 95,99 | 2 | 18 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 16 | 0 | 95,98 | 2 | 16 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 16 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_SISO_64QAM3/4 | 335,94 | 36 | 122 | 0 | 335,95 | 41 | 150 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_SISO_64QAM3/4 | 335,96 | 36 | 128 | 0 | 335,96 | 41 | 132 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_SISO_64QAM3/4 | 335,94 | 36 | 140 | 0 | 335,95 | 41 | 134 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_SISO_64QAM3/4 | 4445,41 | 72 | 298 | 0 | 4236,62 | 74 | 270 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_SISO_64QAM3/4 | 4386,3 | 72 | 292 | 0 | 4175,94 | 74 | 264 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_SISO_64QAM3/4 | 335,89 | 74 | 290 | 0 | 335,86 | 78 | 298 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 20 | 0 | 999,99 | 5 | 20 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 20 | 0 | 999,98 | 5 | 22 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_SISO_64QAM3/4 | 4612,99 | 72 | 264 | 0 | 4482,44 | 74 | 278 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_SISO_64QAM3/4 | 4653,05 | 72 | 274 | 0 | 4479,24 | 74 | 278 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_SISO_64QAM3/4 | 4634,88 | 72 | 274 | 0 | 4505,01 | 73 | 266 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_SISO_64QAM3/4 | 4604,52 | 72 | 270 | 0 | 4479,14 | 74 | 272 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 20 | 0 | 95,99 | 4 | 18 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 18 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 20 | 0 | 95,98 | 4 | 18 | 0 |

PER from 10-1 to 1.5 10-2

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_SISO_64QAM3/4 | 1066,53 | 41 | 176 | 0 | 1066,54 | 49 | 186 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_SISO_64QAM3/4 | 1066,58 | 40 | 162 | 0 | 1066,6 | 49 | 222 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_SISO_64QAM3/4 | 1066,57 | 40 | 156 | 0 | 1066,53 | 49 | 174 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_SISO_64QAM3/4 | 1066,56 | 51 | 304 | 0 | 1066,55 | 58 | 312 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_SISO_64QAM3/4 | 1066,58 | 50 | 274 | 0 | 1066,48 | 58 | 270 | 0 |
| AP | STA6 | Email_Download | TCP | 40_SISO_64QAM3/4 | 2693,83 | 99 | 304 | 0 | 2424,32 | 103 | 334 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 22 | 0 | 999,99 | 5 | 30 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_SISO_64QAM3/4 | 999,99 | 4 | 24 | 0 | 1000 | 5 | 32 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_SISO_64QAM3/4 | 1999,97 | 5 | 28 | 0 | 1999,96 | 6 | 34 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_SISO_64QAM3/4 | 1999,96 | 5 | 26 | 0 | 1999,99 | 6 | 32 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_SISO_64QAM3/4 | 2795 | 102 | 332 | 0 | 2550,48 | 112 | 348 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_SISO_64QAM3/4 | 2817,43 | 101 | 306 | 0 | 2548,93 | 112 | 336 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_SISO_64QAM3/4 | 2787,1 | 103 | 318 | 0 | 2543,29 | 112 | 392 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_SISO_64QAM3/4 | 2807,54 | 102 | 322 | 0 | 2544,91 | 112 | 334 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_SISO_64QAM3/4 | 2796,22 | 102 | 334 | 0 | 2535,89 | 112 | 350 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_SISO_64QAM3/4 | 2806,25 | 102 | 326 | 0 | 2546,05 | 112 | 336 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_SISO_64QAM3/4 | 2794,2 | 102 | 330 | 0 | 2543,33 | 112 | 322 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_SISO_64QAM3/4 | 2804,46 | 102 | 310 | 0 | 2552,38 | 111 | 352 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_SISO_64QAM3/4 | 2785,48 | 103 | 322 | 0 | 2538,76 | 112 | 344 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_SISO_64QAM3/4 | 2815,94 | 101 | 330 | 0 | 2531,74 | 112 | 348 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 26 | 0 | 95,99 | 3 | 26 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 20 | 0 | 95,98 | 3 | 22 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 24 | 0 | 95,99 | 3 | 26 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 20 | 0 | 95,99 | 3 | 22 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM3/4 | 95,99 | 2 | 20 | 0 | 95,99 | 3 | 24 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 2 | 20 | 0 | 95,98 | 3 | 26 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_SISO_64QAM3/4 | 335,94 | 50 | 176 | 0 | 335,91 | 57 | 190 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_SISO_64QAM3/4 | 335,93 | 50 | 166 | 0 | 335,94 | 58 | 222 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_SISO_64QAM3/4 | 335,95 | 49 | 156 | 0 | 335,93 | 57 | 192 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_SISO_64QAM3/4 | 3782,87 | 81 | 334 | 0 | 3409,73 | 88 | 302 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_SISO_64QAM3/4 | 3731,63 | 81 | 274 | 0 | 3349,53 | 88 | 292 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_SISO_64QAM3/4 | 335,76 | 86 | 314 | 0 | 335,7 | 93 | 352 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_SISO_64QAM3/4 | 999,98 | 6 | 26 | 0 | 999,98 | 7 | 30 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_SISO_64QAM3/4 | 999,98 | 6 | 30 | 0 | 999,99 | 7 | 34 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_SISO_64QAM3/4 | 4141,19 | 79 | 288 | 0 | 3819,56 | 85 | 272 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_SISO_64QAM3/4 | 4135,85 | 79 | 278 | 0 | 3816,75 | 85 | 302 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_SISO_64QAM3/4 | 4146,73 | 79 | 308 | 0 | 3819,89 | 85 | 294 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_SISO_64QAM3/4 | 4156,52 | 79 | 298 | 0 | 3813,61 | 85 | 308 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 22 | 0 | 95,99 | 4 | 24 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 24 | 0 | 95,99 | 4 | 22 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 22 | 0 | 95,99 | 4 | 24 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM3/4 | 95,99 | 4 | 22 | 0 | 95,99 | 4 | 30 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 24 | 0 | 95,98 | 4 | 28 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM3/4 | 95,98 | 4 | 24 | 0 | 95,98 | 4 | 28 | 0 |

| Scenario 4: Performance Summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1,5.10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 72.180 | 71.001 | 68.775 | 66.643 | 61.474 | 56.565 |
| Ratio for non-QoS flows (CC18) | % | 16.00 | 15.74 | 15.25 | 14.78 | 13.63 | 12.54 |
| Aggregate goodput for QoS flows | Mbit/s | 9.151 | 9.151 | 9.151 | 9.151 | 9.151 | 9.151 |
| Number of QoS flows satisfying the objectives (CC19) |  | 18 | 18 | 18 | 18 | 18 | 18 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 81.332 | 80.152 | 77.927 | 75.795 | 70.626 | 65.717 |
| Metric 2 of CC20 | Mbit/s | 81.332 | 80.152 | 77.927 | 75.795 | 70.626 | 65.717 |
| Metric 3 of CC20 | Mbit/s | 81.332 | 80.152 | 77.927 | 75.795 | 70.626 | 65.717 |
| Average PHY rate | Mbit/s | 106.31 | 106.28 | 106.24 | 106.2 | 106.08 | 105.95 |
| MAC efficiency (CC24) | % | 77 | 75 | 73 | 71 | 67 | 62 |

Table 3-2: Performance summary for scenario 4

A PER of 3%is considered as a mean value in normal conditions. Values corresponding to this PER are reported in Comparison Criteria table.

#### Scenario 6

In following results, all stations uses a SISO physical mode 64 QAM 2/3 in two 20 MHz channels, giving a PHY data rate of 108 Mbit/s, except for high quality video streaming flows that use 64 QAM3/4 (12.5 Mbit/s).

The number of contention interval (CTI) per MTF (sub-frame) is equal to 6, corresponding to 3 per channel.

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_SISO_64QAM2/3 | 2133,26 | 24 | 142 | 0 | 1984,47 | 79 | 508 | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_SISO_64QAM2/3 | 2133,24 | 24 | 144 | 0 | 1983,56 | 79 | 508 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_SISO_64QAM2/3 | 2133,23 | 24 | 144 | 0 | 1994,92 | 78 | 508 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_SISO_64QAM2/3 | 2133,27 | 24 | 152 | 0 | 1960,53 | 79 | 508 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_SISO_64QAM2/3 | 2133,28 | 24 | 138 | 0 | 2010,17 | 78 | 508 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_SISO_64QAM2/3 | 2133,27 | 24 | 142 | 0 | 1971,44 | 79 | 508 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_SISO_64QAM2/3 | 2133,28 | 24 | 138 | 0 | 1997,03 | 78 | 508 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_SISO_64QAM2/3 | 2133,26 | 24 | 144 | 0 | 1971,72 | 79 | 508 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_SISO_64QAM2/3 | 2133,28 | 24 | 140 | 0 | 1996,97 | 79 | 508 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_SISO_64QAM2/3 | 2133,23 | 24 | 144 | 0 | 1986,7 | 78 | 508 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_SISO_64QAM2/3 | 1999,87 | 2 | 6 | 0 | 1999,87 | 2 | 14 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_SISO_64QAM2/3 | 1999,86 | 2 | 6 | 0 | 1999,86 | 2 | 14 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_SISO_64QAM2/3 | 1999,85 | 2 | 6 | 0 | 1999,85 | 2 | 10 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_SISO_64QAM2/3 | 1999,84 | 2 | 6 | 0 | 1999,83 | 2 | 12 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_SISO_64QAM3/4 | 7999,29 | 2 | 6 | 0 | 7999,26 | 2 | 14 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_SISO_64QAM3/4 | 7999,25 | 2 | 6 | 0 | 7999,25 | 2 | 14 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_SISO_64QAM3/4 | 7999,2 | 2 | 6 | 0 | 7999,2 | 2 | 14 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_SISO_64QAM2/3 | 4999,5 | 2 | 8 | 0 | 4999,5 | 2 | 14 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_SISO_64QAM2/3 | 4999,47 | 2 | 8 | 0 | 4999,47 | 2 | 14 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 4 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 4 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 6 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 4 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 8 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 14 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5.10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_SISO_64QAM2/3 | 1641,71 | 104 | 508 | 0 | 1343,14 | 131 | 300 | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_SISO_64QAM2/3 | 1644,67 | 104 | 494 | 0 | 1348,89 | 130 | 324 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_SISO_64QAM2/3 | 1642,35 | 105 | 508 | 0 | 1347,15 | 130 | 454 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_SISO_64QAM2/3 | 1633,28 | 105 | 456 | 0 | 1347,75 | 130 | 300 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_SISO_64QAM2/3 | 1634,88 | 105 | 508 | 0 | 1346,57 | 130 | 316 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_SISO_64QAM2/3 | 1646,52 | 104 | 258 | 0 | 1341,78 | 131 | 320 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_SISO_64QAM2/3 | 1635,37 | 105 | 474 | 0 | 1343,38 | 131 | 302 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_SISO_64QAM2/3 | 1637,81 | 105 | 508 | 0 | 1346,49 | 130 | 320 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_SISO_64QAM2/3 | 1639,41 | 105 | 456 | 0 | 1343,87 | 131 | 306 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_SISO_64QAM2/3 | 1635,64 | 105 | 474 | 0 | 1346,38 | 130 | 300 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_SISO_64QAM2/3 | 1999,87 | 2 | 18 | 0 | 1999,87 | 3 | 18 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_SISO_64QAM2/3 | 1999,86 | 2 | 14 | 0 | 1999,85 | 3 | 22 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_SISO_64QAM2/3 | 1999,85 | 2 | 14 | 0 | 1999,84 | 3 | 18 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_SISO_64QAM2/3 | 1999,82 | 2 | 14 | 0 | 1999,84 | 3 | 16 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_SISO_64QAM3/4 | 7999,29 | 4 | 16 | 0 | 7999,15 | 5 | 18 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_SISO_64QAM3/4 | 7999,21 | 4 | 18 | 0 | 7999,2 | 5 | 18 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_SISO_64QAM3/4 | 7999,15 | 4 | 20 | 0 | 7999,14 | 5 | 20 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_SISO_64QAM2/3 | 4999,4 | 3 | 16 | 0 | 4999,5 | 4 | 18 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_SISO_64QAM2/3 | 4999,47 | 3 | 14 | 0 | 4999,47 | 4 | 18 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 18 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 10 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 10 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 10 | 0 | 95,98 | 2 | 16 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 12 | 0 | 95,99 | 4 | 14 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 16 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 12 | 0 | 95,98 | 4 | 14 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 16 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 18 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |

PER from 10-1 to 1.5 10-1

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_SISO_64QAM2/3 | 798,86 | 224 | 584 | 0 | 311 | 587 | 1372 | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_SISO_64QAM2/3 | 797,46 | 225 | 540 | 0 | 312,11 | 585 | 1332 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_SISO_64QAM2/3 | 800,86 | 224 | 590 | 0 | 312,6 | 584 | 1332 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_SISO_64QAM2/3 | 803,35 | 223 | 550 | 0 | 311,07 | 587 | 1374 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_SISO_64QAM2/3 | 800,48 | 223 | 536 | 0 | 310,82 | 588 | 1350 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_SISO_64QAM2/3 | 800,11 | 224 | 526 | 0 | 310,25 | 589 | 1332 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_SISO_64QAM2/3 | 795,45 | 225 | 570 | 0 | 310,73 | 588 | 1346 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_SISO_64QAM2/3 | 799,55 | 224 | 564 | 0 | 309,75 | 590 | 1370 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_SISO_64QAM2/3 | 795,83 | 225 | 600 | 0 | 311,18 | 587 | 1448 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_SISO_64QAM2/3 | 804,16 | 223 | 530 | 0 | 312,63 | 584 | 1376 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_SISO_64QAM2/3 | 1999,84 | 4 | 20 | 0 | 1999,84 | 6 | 26 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_SISO_64QAM2/3 | 1999,82 | 4 | 22 | 0 | 1999,84 | 6 | 28 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_SISO_64QAM2/3 | 1999,85 | 4 | 22 | 0 | 1999,83 | 6 | 26 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_SISO_64QAM2/3 | 1999,84 | 4 | 24 | 0 | 1999,8 | 6 | 32 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_SISO_64QAM3/4 | 7999,1 | 7 | 24 | 0 | 7999,19 | 8 | 24 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_SISO_64QAM3/4 | 7999,14 | 7 | 22 | 0 | 7999,07 | 8 | 22 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_SISO_64QAM3/4 | 7999,06 | 7 | 22 | 0 | 7998,98 | 8 | 22 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_SISO_64QAM2/3 | 4999,47 | 6 | 22 | 0 | 4999,4 | 7 | 28 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_SISO_64QAM2/3 | 4999,4 | 6 | 24 | 0 | 4999,4 | 7 | 30 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 24 | 0 | 95,99 | 2 | 30 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 18 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 18 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 22 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 22 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 40_SISO_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 26 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 20 | 0 | 95,98 | 2 | 26 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 20 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 2 | 20 | 0 | 95,98 | 2 | 26 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 22 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 26 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 26 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 22 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 18 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 40_SISO_64QAM2/3 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 22 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 22 | 0 | 95,98 | 4 | 20 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 22 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 22 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 24 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 22 | 0 | 95,98 | 4 | 22 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 40_SISO_64QAM2/3 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 20 | 0 |

| Scenario 6: Performance Summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1,5.10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 21.332 | 19.857 | 16.391 | 13.455 | 7.996 | 3.112 |
| Ratio for non-QoS flows (CC18) | % | 106.66 | 99.29 | 81.96 | 67.28 | 39.98 | 15.56 |
| Aggregate goodput for QoS flows | Mbit/s | 44.875 | 44.875 | 44.875 | 44.875 | 44.875 | 44.875 |
| Number of QoS flows satisfying the objectives (CC19) |  | 39 | 39 | 39 | 39 | 39 | 39 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 66.208 | 66.208 | 66.208 | 58.33 | 52.871 | 47.987 |
| Metric 2 of CC20 | Mbit/s | 66.208 | 66.208 | 66.208 | 58.33 | 52.871 | 47.987 |
| Metric 3 of CC20 | Mbit/s | 66.208 | 66.208 | 66.208 | 58.33 | 52.871 | 47.987 |
| Average PHY rate | Mbit/s | 92.45 | 92.44 | 92.58 | 92.76 | 93.29 | 94 |
| MAC efficiency (CC24) | % | 72 | 72 | 72 | 63 | 57 | 51 |

Table 3-3: Performance summary for scenario 6

A PER of 3%is considered as a mean value in normal conditions. Values corresponding to this PER are reported in Comparison Criteria table.

### CC15 (scenarios 17, 18 and 19)

For scenario 18, we made a theoretical analysis considering a 802.11a station using the 64 QAM physical mode that give a throughput of 54 Mbit/s to the MAC. The goodput of 30 Mbit/s is the theoretical best case goodput a legacy MAC is able to offer.

In case 1, we suppose that the TGn AP and the TGn stations use a PHY mode 3x3, 64 QAM 2/3 on one 20 MHz channel, giving a throughput above the PHY of 162 Mbit/s.

Simulation of scenario 17 gives following results:

Goodput: 	100.00	Mbit/s

Frame occupancy: 	70	%

Thus the AP reserves 30% of the time for legacy devices, leading to results of scenario 19:

Goodput for HT station: 	100.00	Mbit/s

Goodput for Legacy station:	9.00	Mbit/s

In case 2, we suppose that the TGn AP and the TGn stations use a PHY mode 3x3, 64 QAM 2/3 on two 20 MHz channels, giving a throughput above the PHY of 324 Mbit/s.

Simulation of scenario 17 gives following results:

Goodput: 	100.00	Mbit/s

Frame occupancy: 	36	%

Thus the AP reserves 64% of the time for legacy devices, leading to results of scenario 19:

Goodput for HT station: 	100.00	Mbit/s

Goodput for Legacy station:	19.20	Mbit/s

# Other simulation results

## Simulations using complete set of MIMO modes

Previous results () give the minimum PHY rate that shall be used to satisfy scenario's requirement. The aim of following simulations is now to show results when a full set of MIMO mode is selected.

Scenarios 1 and 6 have been slightly modified to have in any case a full physical medium. For that purpose, local file transfer sources have been unleashed, letting TCP sources occupying as bandwidth as available. Indeed, with the fully backlogged TCP traffic, we obtain a simulation of maximum system performances. Scenario 4 is unmodified compared to 3.3.3.2, but the PHY mode is selected in the complete set of MIMO modes.

Physical MIMO modes range from 54 Mbit/s (2x2, 16 QAM1/2 in 20 MHz) up to 365 Mbit/s with a 3x3 antennas configuration and modulation 64 QAM3/4 in a 40 MHz channel. They have been selected upon the distance of each station pair. Table in 3.3.1 summarises the PHY mode list and gives their utilisation range.

### Scenario1bis

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_3x3_64QAM3/4 | 19199,9 | 2 | 6 | 0 | 19199,7 | 3 | 14 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_3x3_64QAM3/4 | 23999,57 | 6 | 8 | 0 | 23999,47 | 7 | 20 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_3x3_64QAM2/3 | 4000 | 2 | 6 | 0 | 4000 | 2 | 8 | 0 |
| AP | STA4 | Internet File | TCP | 40_3x3_64QAM2/3 | 1066,67 | 2 | 6 | 0 | 1066,67 | 2 | 8 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 2 | 0 | 96 | 2 | 10 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_3x3_16QAM3/4 | 2000 | 2 | 6 | 0 | 2000 | 2 | 8 | 0 |
| AP | STA11 | MP3 Audio | CBR | 20_2x2_64QAM2/3 | 128 | 2 | 6 | 0 | 128 | 2 | 6 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 4 | 0 | 60 | 4 | 12 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 4 | 0 | 60 | 4 | 12 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_3x3_16QAM1/2 | 100081,8 | 12 | 20 | 0 | 98107 | 12 | 26 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_2x2_16QAM1/2 | 500 | 4 | 4 | 0 | 500 | 4 | 10 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_2x2_16QAM1/2 | 500 | 4 | 4 | 0 | 500 | 4 | 16 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_3x3_64QAM3/4 | 1000 | 2 | 4 | 0 | 1000 | 2 | 10 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 20_2x2_64QAM3/4 | 500 | 4 | 4 | 0 | 500 | 4 | 10 | 0 |
| STA7 | AP | VoIP_UL7 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 4 | 0 | 96 | 4 | 14 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 4 | 0 | 96 | 4 | 8 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_3x3_64QAM3/4 | 19199,7 | 4 | 18 | 0 | 19199,7 | 4 | 22 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_3x3_64QAM3/4 | 23999,47 | 9 | 22 | 0 | 23999,33 | 9 | 26 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_3x3_64QAM2/3 | 4000 | 2 | 8 | 0 | 3999,97 | 3 | 10 | 0 |
| AP | STA4 | Internet File | TCP | 40_3x3_64QAM2/3 | 1066,67 | 2 | 8 | 0 | 1066,67 | 2 | 8 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 14 | 0 | 96 | 2 | 14 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 10 | 0 | 96 | 2 | 14 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_3x3_16QAM3/4 | 2000 | 2 | 12 | 0 | 2000 | 2 | 12 | 0 |
| AP | STA11 | MP3 Audio | CBR | 20_2x2_64QAM2/3 | 128 | 2 | 8 | 0 | 128 | 2 | 8 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 16 | 0 | 60 | 4 | 16 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 12 | 0 | 60 | 4 | 16 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_3x3_16QAM1/2 | 94804,58 | 13 | 26 | 0 | 91783,09 | 13 | 30 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_2x2_16QAM1/2 | 500 | 4 | 14 | 0 | 500 | 4 | 18 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_2x2_16QAM1/2 | 500 | 4 | 20 | 0 | 500 | 4 | 20 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_3x3_64QAM3/4 | 1000 | 2 | 12 | 0 | 1000 | 2 | 12 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 20_2x2_64QAM3/4 | 500 | 4 | 12 | 0 | 500 | 4 | 14 | 0 |
| STA7 | AP | VoIP_UL7 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 14 | 0 | 96 | 4 | 12 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 14 | 0 | 96 | 4 | 14 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 12 | 0 | 95,99 | 4 | 16 | 0 |

PER from 10-1 to 1.5 10-1

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | HDTV#0 | CBR | 40_3x3_64QAM3/4 | 19199,67 | 6 | 24 | 0 | 19199,67 | 7 | 32 | 0 |
| AP | STA3 | HDTV#1 | CBR | 40_3x3_64QAM3/4 | 23999,23 | 11 | 30 | 0 | 23999,23 | 12 | 30 | 0 |
| AP | STA4 | SDTV#0 | CBR | 40_3x3_64QAM2/3 | 3999,97 | 3 | 14 | 0 | 3999,97 | 4 | 14 | 0 |
| AP | STA4 | Internet File | TCP | 40_3x3_64QAM2/3 | 1066,67 | 2 | 12 | 0 | 1066,67 | 2 | 14 | 0 |
| AP | STA7 | VoIP_DL7 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 16 | 0 | 96 | 2 | 20 | 0 |
| AP | STA8 | VoIP_DL8 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 16 | 0 | 96 | 2 | 20 | 0 |
| AP | STA9 | VoIP_DL9 | CBR | 20_2x2_16QAM1/2 | 96 | 2 | 16 | 0 | 96 | 2 | 20 | 0 |
| AP | STA10 | Internet Streaming | CBR | 40_3x3_16QAM3/4 | 2000 | 3 | 14 | 0 | 1999,99 | 3 | 18 | 0 |
| AP | STA11 | MP3 Audio | CBR | 20_2x2_64QAM2/3 | 128 | 2 | 10 | 0 | 128 | 2 | 12 | 0 |
| STA1 | AP | VoD Control#0 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 22 | 0 | 60 | 5 | 26 | 0 |
| STA3 | AP | VoD Control#1 | CBR | 40_3x3_64QAM3/4 | 60 | 4 | 22 | 0 | 60 | 5 | 28 | 0 |
| STA4 | STA10 | Local file transfer | TCP | 40_3x3_16QAM1/2 | 84666,87 | 14 | 32 | 0 | 77540,72 | 15 | 34 | 0 |
| STA5 | STA6 | Video Phone#0 | CBR | 40_2x2_16QAM1/2 | 500 | 5 | 22 | 0 | 500 | 5 | 26 | 0 |
| STA6 | STA5 | Video Phone#1 | CBR | 40_2x2_16QAM1/2 | 500 | 5 | 24 | 0 | 500 | 6 | 28 | 0 |
| STA10 | AP | Console to Internet | CBR | 40_3x3_64QAM3/4 | 999,99 | 3 | 14 | 0 | 999,99 | 4 | 18 | 0 |
| STA11 | STA10 | Video Gaming Controller | CBR | 20_2x2_64QAM3/4 | 499,99 | 4 | 16 | 0 | 499,99 | 5 | 18 | 2,00E-05 |
| STA7 | AP | VoIP_UL7 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 18 | 0 | 96 | 4 | 22 | 0 |
| STA8 | AP | VoIP_UL8 | CBR | 20_2x2_16QAM1/2 | 96 | 4 | 18 | 0 | 96 | 4 | 20 | 0 |
| STA9 | AP | VoIP_UL9 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 20 | 0 |

| Scenario 1bis: Performance Summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1.5 10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 101.148 | 99.173 | 95.871 | 92.849 | 85.733 | 78.607 |
| Ratio for non-QoS flows (CC18) | % | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* |
| Aggregate goodput for QoS flows | Mbit/s | 52.523 | 52.523 | 52.523 | 52.523 | 52.523 | 52.523 |
| Number of QoS flows satisfying the objectives (CC19) |  | 17 | 17 | 17 | 17 | 17 | 17 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 153.671 | 151.600 | 148.394 | 145.372 | 138.256 | 131.130 |
| Metric 2 of CC20 | Mbit/s | 153.671 | 151.600 | 148.394 | 145.372 | 138.256 | 131.130 |
| Metric 3 of CC20 | Mbit/s | 153.671 | 151.600 | 148.394 | 145.372 | 138.256 | 131.130 |
| Average PHY rate | Mbit/s | 196.34 | 195.27 | 197.8 | 198.62 | 200.64 | 202.92 |
| MAC efficiency (CC24) | % | 78 | 78 | 75 | 73 | 69 | 65 |

Table 4-4: Performance summary for scenario 1bis

* N/A: Non applicable because TCP source throughput is unlimited

Figure 4-1: TCP throughput and MAC efficiency versus PER in scenarios 1 and 1bis

### Scenario 4bis

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_3x3_64QAM3/4 | 1066,66 | 10 | 48 | 0 | 1066,64 | 10 | 48 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_3x3_64QAM3/4 | 1066,65 | 10 | 44 | 0 | 1066,64 | 10 | 44 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_3x3_16QAM3/4 | 1066,64 | 10 | 54 | 0 | 1066,64 | 10 | 48 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_3x3_64QAM3/4 | 1066,64 | 11 | 58 | 0 | 1066,64 | 11 | 60 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_3x3_64QAM3/4 | 1066,65 | 11 | 74 | 0 | 1066,64 | 12 | 72 | 0 |
| AP | STA6 | Email_Download | TCP | 40_3x3_64QAM3/4 | 10666,25 | 16 | 96 | 0 | 10666,03 | 19 | 92 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 2 | 8 | 0 | 1000 | 2 | 14 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_3x3_16QAM3/4 | 999,99 | 2 | 8 | 0 | 999,99 | 2 | 14 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_3x3_64QAM3/4 | 1999,99 | 2 | 8 | 0 | 1999,99 | 2 | 14 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_3x3_64QAM5/6 | 1999,99 | 2 | 8 | 0 | 1999,99 | 2 | 14 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_3x3_64QAM3/4 | 12938,34 | 22 | 118 | 0 | 12926,42 | 23 | 118 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_3x3_64QAM3/4 | 12819,97 | 23 | 118 | 0 | 12971,02 | 23 | 120 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_3x3_64QAM3/4 | 12909,93 | 23 | 126 | 0 | 13066,29 | 23 | 130 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_3x3_64QAM3/4 | 12851,7 | 23 | 126 | 0 | 12909,26 | 23 | 112 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_3x3_64QAM3/4 | 13664,99 | 21 | 114 | 0 | 13645,27 | 22 | 108 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_3x3_64QAM3/4 | 13707,19 | 21 | 120 | 0 | 13546,1 | 22 | 108 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_3x3_64QAM3/4 | 13649,75 | 21 | 108 | 0 | 13482,74 | 22 | 106 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_3x3_64QAM3/4 | 13650,47 | 21 | 124 | 0 | 13649,35 | 22 | 108 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_3x3_64QAM5/6 | 14287,29 | 20 | 134 | 0 | 14048,64 | 21 | 114 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_3x3_64QAM5/6 | 14244,8 | 20 | 128 | 0 | 14022,84 | 21 | 110 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 8 | 0 | 95,99 | 2 | 12 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 6 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 8 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 8 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 6 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 8 | 0 | 95,98 | 2 | 12 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_3x3_64QAM3/4 | 335,99 | 14 | 50 | 0 | 335,99 | 13 | 48 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_3x3_64QAM3/4 | 335,99 | 14 | 48 | 0 | 335,99 | 13 | 46 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_3x3_16QAM3/4 | 335,99 | 14 | 58 | 0 | 335,98 | 14 | 52 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_3x3_64QAM3/4 | 5099,73 | 15 | 66 | 0 | 5099,82 | 16 | 66 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_3x3_64QAM3/4 | 10132,42 | 16 | 82 | 0 | 10132,85 | 18 | 82 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_3x3_64QAM3/4 | 335,99 | 17 | 94 | 0 | 335,98 | 19 | 92 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 4 | 4 | 0 | 999,99 | 4 | 16 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_3x3_16QAM3/4 | 999,99 | 4 | 4 | 0 | 999,99 | 4 | 16 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_3x3_64QAM3/4 | 17750,24 | 17 | 92 | 0 | 15359,73 | 20 | 88 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_3x3_64QAM3/4 | 17702,14 | 17 | 100 | 0 | 15368,92 | 20 | 86 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_3x3_64QAM3/4 | 17720,23 | 17 | 102 | 0 | 15347,1 | 20 | 106 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_3x3_64QAM5/6 | 18256,44 | 17 | 88 | 0 | 15558,35 | 20 | 88 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5.10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_3x3_64QAM3/4 | 1066,64 | 11 | 52 | 0 | 1066,61 | 12 | 56 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_3x3_64QAM3/4 | 1066,66 | 11 | 50 | 0 | 1066,66 | 12 | 62 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_3x3_16QAM3/4 | 1066,64 | 11 | 48 | 0 | 1066,63 | 12 | 52 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_3x3_64QAM3/4 | 1066,66 | 13 | 56 | 0 | 1066,62 | 14 | 60 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_3x3_64QAM3/4 | 1066,64 | 14 | 68 | 0 | 1066,65 | 15 | 66 | 0 |
| AP | STA6 | Email_Download | TCP | 40_3x3_64QAM3/4 | 10665,99 | 22 | 90 | 0 | 10665,76 | 23 | 110 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 2 | 18 | 0 | 999,99 | 3 | 18 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_3x3_16QAM3/4 | 999,99 | 2 | 16 | 0 | 1000 | 3 | 18 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_3x3_64QAM3/4 | 1999,97 | 2 | 18 | 0 | 2000 | 3 | 18 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_3x3_64QAM5/6 | 2000 | 2 | 16 | 0 | 2000 | 3 | 18 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_3x3_64QAM3/4 | 12623,09 | 23 | 124 | 0 | 12023,97 | 24 | 116 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_3x3_64QAM3/4 | 12479,19 | 24 | 104 | 0 | 12144,06 | 24 | 102 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_3x3_64QAM3/4 | 12531,04 | 24 | 106 | 0 | 12039,82 | 24 | 106 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_3x3_64QAM3/4 | 12524,19 | 24 | 104 | 0 | 12075,18 | 24 | 112 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_3x3_64QAM3/4 | 12990,25 | 23 | 106 | 0 | 12430,48 | 24 | 112 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_3x3_64QAM3/4 | 12890,41 | 23 | 106 | 0 | 12482,58 | 23 | 102 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_3x3_64QAM3/4 | 12956,68 | 23 | 114 | 0 | 12444,21 | 23 | 110 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_3x3_64QAM3/4 | 12966,14 | 23 | 106 | 0 | 12480,89 | 23 | 108 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_3x3_64QAM5/6 | 13327,12 | 22 | 100 | 0 | 12785,75 | 23 | 108 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_3x3_64QAM5/6 | 13371,98 | 22 | 116 | 0 | 12758,27 | 23 | 100 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 12 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 12 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 18 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 18 | 0 | 95,98 | 2 | 16 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_3x3_64QAM3/4 | 335,99 | 14 | 54 | 0 | 335,98 | 14 | 54 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_3x3_64QAM3/4 | 335,98 | 14 | 54 | 0 | 335,99 | 14 | 62 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_3x3_16QAM3/4 | 335,99 | 14 | 52 | 0 | 335,99 | 14 | 58 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_3x3_64QAM3/4 | 5099,7 | 18 | 62 | 0 | 5099,74 | 19 | 60 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_3x3_64QAM3/4 | 10132,72 | 20 | 70 | 0 | 10132,42 | 21 | 74 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_3x3_64QAM3/4 | 335,98 | 21 | 90 | 0 | 335,98 | 22 | 110 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 4 | 16 | 0 | 999,99 | 5 | 20 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_3x3_16QAM3/4 | 999,99 | 4 | 20 | 0 | 999,97 | 5 | 24 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_3x3_64QAM3/4 | 14528,43 | 21 | 84 | 0 | 13987,06 | 22 | 80 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_3x3_64QAM3/4 | 14539,88 | 21 | 90 | 0 | 13994,97 | 22 | 86 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_3x3_64QAM3/4 | 14518,43 | 21 | 78 | 0 | 13960,66 | 22 | 90 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_3x3_64QAM5/6 | 14637,06 | 21 | 86 | 0 | 14071,94 | 22 | 92 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 18 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |

PER from 10-1 to 1.5 10-2

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | WebDL1 | TCP | 40_3x3_64QAM3/4 | 1066,61 | 14 | 58 | 0 | 1066,59 | 16 | 72 | 0 |
| AP | STA2 | WebDL2 | TCP | 40_3x3_64QAM3/4 | 1066,64 | 14 | 68 | 0 | 1066,62 | 16 | 84 | 0 |
| AP | STA3 | WebDL3 | TCP | 40_3x3_16QAM3/4 | 1066,64 | 14 | 64 | 0 | 1066,64 | 16 | 80 | 0 |
| AP | STA4 | WebDL#4 | TCP | 40_3x3_64QAM3/4 | 1066,6 | 16 | 92 | 0 | 1066,6 | 19 | 74 | 0 |
| AP | STA5 | WebDL#5 | TCP | 40_3x3_64QAM3/4 | 1066,62 | 17 | 76 | 0 | 1066,65 | 19 | 78 | 0 |
| AP | STA6 | Email_Download | TCP | 40_3x3_64QAM3/4 | 10386,24 | 28 | 106 | 0 | 9449,17 | 31 | 112 | 0 |
| AP | STA7 | VideoConf_DL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 4 | 24 | 0 | 1000 | 5 | 30 | 0 |
| AP | STA8 | VideoConf_DL#2 | CBR | 40_3x3_16QAM3/4 | 1000 | 4 | 24 | 0 | 999,99 | 5 | 26 | 0 |
| AP | STA9 | Streaming#1 | CBR | 40_3x3_64QAM3/4 | 1999,97 | 4 | 26 | 0 | 2000 | 5 | 32 | 0 |
| AP | STA10 | Streaming#2 | CBR | 40_3x3_64QAM5/6 | 1999,99 | 4 | 22 | 0 | 1999,97 | 5 | 30 | 0 |
| AP | STA11 | LocalFileTransfer11 | TCP | 40_3x3_64QAM3/4 | 10998,79 | 26 | 110 | 0 | 10000,96 | 29 | 132 | 0 |
| AP | STA12 | LocalFileTransfer12 | TCP | 40_3x3_64QAM3/4 | 10980,85 | 26 | 122 | 0 | 10060,4 | 29 | 122 | 0 |
| AP | STA13 | LocalFileTransfer13 | TCP | 40_3x3_64QAM3/4 | 11040,19 | 26 | 122 | 0 | 10033,85 | 29 | 120 | 0 |
| AP | STA14 | LocalFileTransfer14 | TCP | 40_3x3_64QAM3/4 | 11003,08 | 26 | 108 | 0 | 10004,34 | 29 | 116 | 0 |
| AP | STA15 | LocalFileTransfer15 | TCP | 40_3x3_64QAM3/4 | 11280,79 | 26 | 110 | 0 | 10233,28 | 28 | 112 | 0 |
| AP | STA16 | LocalFileTransfer16 | TCP | 40_3x3_64QAM3/4 | 11266 | 26 | 118 | 0 | 10226,45 | 28 | 120 | 0 |
| AP | STA17 | LocalFileTransfer17 | TCP | 40_3x3_64QAM3/4 | 11271,94 | 26 | 106 | 0 | 10220,67 | 28 | 114 | 0 |
| AP | STA18 | LocalFileTransfer18 | TCP | 40_3x3_64QAM3/4 | 11259,91 | 26 | 106 | 0 | 10232,9 | 28 | 118 | 0 |
| AP | STA19 | LocalFileTransfer19 | TCP | 40_3x3_64QAM5/6 | 11475,51 | 25 | 104 | 0 | 10411,15 | 28 | 110 | 0 |
| AP | STA20 | LocalFileTransfer20 | TCP | 40_3x3_64QAM5/6 | 11516,4 | 25 | 100 | 0 | 10415,5 | 28 | 110 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 26 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 24 | 0 |
| STA1 | AP | WebRequest1 | TCP | 40_3x3_64QAM3/4 | 335,98 | 16 | 56 | 0 | 335,97 | 17 | 72 | 0 |
| STA2 | AP | WebRequest2 | TCP | 40_3x3_64QAM3/4 | 335,99 | 16 | 70 | 0 | 335,97 | 17 | 78 | 0 |
| STA3 | AP | WebRequest3 | TCP | 40_3x3_16QAM3/4 | 335,99 | 16 | 62 | 0 | 335,99 | 17 | 76 | 0 |
| STA4 | AP | WebUL#4 | TCP | 40_3x3_64QAM3/4 | 5099,72 | 21 | 84 | 0 | 5099,61 | 23 | 76 | 0 |
| STA5 | AP | Email_Upload | TCP | 40_3x3_64QAM3/4 | 10132,62 | 23 | 80 | 0 | 10126,88 | 27 | 88 | 0 |
| STA6 | AP | WebRequest#6 | TCP | 40_3x3_64QAM3/4 | 335,96 | 26 | 106 | 0 | 335,97 | 28 | 112 | 0 |
| STA7 | AP | VideoConf_UL#1 | CBR | 40_3x3_64QAM3/4 | 999,99 | 5 | 24 | 0 | 999,98 | 7 | 28 | 0 |
| STA8 | AP | VideoConf_UL#2 | CBR | 40_3x3_16QAM3/4 | 999,98 | 6 | 36 | 0 | 999,99 | 7 | 32 | 0 |
| STA21 | AP | LocalFileTransfer21 | TCP | 40_3x3_64QAM3/4 | 12770,16 | 23 | 84 | 0 | 11724,71 | 25 | 80 | 0 |
| STA22 | AP | LocalFileTransfer22 | TCP | 40_3x3_64QAM3/4 | 12776,68 | 23 | 80 | 0 | 11720,58 | 25 | 80 | 0 |
| STA23 | AP | LocalFileTransfer23 | TCP | 40_3x3_64QAM3/4 | 12801,37 | 23 | 84 | 0 | 11706,49 | 25 | 80 | 0 |
| STA24 | AP | LocalFileTransfer24 | TCP | 40_3x3_64QAM5/6 | 12841,69 | 23 | 76 | 0 | 11760,1 | 25 | 82 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 24 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 20 | 0 | 95,99 | 4 | 22 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 22 | 0 | 95,99 | 4 | 20 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 20 | 0 | 95,99 | 4 | 26 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 18 | 0 | 95,98 | 4 | 22 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 22 | 0 |

| Scenario 4bis: Performance summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1,5.10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 238.729 | 228.477 | 219.459 | 212.254 | 195.579 | 180.104 |
| Ratio for non-QoS flows (CC18) | % | 52.93 | 50.66 | 48.66 | 47.06 | 43.36 | 39.93 |
| Aggregate goodput for QoS flows | Mbit/s | 9.151 | 9.151 | 9.151 | 9.151 | 9.151 | 9.151 |
| Number of QoS flows satisfying the objectives (CC19) |  | 18 | 18 | 18 | 18 | 18 | 18 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 247.880 | 237.629 | 228.611 | 221.406 | 204.730 | 189.255 |
| Metric 2 of CC20 | Mbit/s | 247.880 | 237.629 | 228.611 | 221.406 | 204.730 | 189.255 |
| Metric 3 of CC20 | Mbit/s | 247.880 | 237.629 | 228.611 | 221.406 | 204.730 | 189.255 |
| Average PHY rate | Mbit/s | 342.84 | 341.71 | 340.99 | 340.45 | 339.06 | 337.62 |
| MAC efficiency (CC24) | % | 72 | 70 | 67 | 65 | 60 | 56 |

Table 4-5: Performance summary for scenario 4bis

Figure 4-2: TCP throughput and MAC efficiency versus PER in scenarios 4 and 4bis

### Scenario 6bis

PER from 0 to 10-2

|  |  |  |  |  |  | No Error |  |  |  | Error = 10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_3x3_16QAM1/2 | 8378,72 | 45 | 162 |  |  |  |  | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_3x3_16QAM1/2 | 8355,58 | 46 | 176 | 0 | 9542,57 | 41 | 148 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_3x3_16QAM3/4 | 10664,59 | 15 | 132 | 0 | 9565,13 | 41 | 156 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_3x3_64QAM2/3 | 10664,11 | 13 | 120 | 0 | 10664,57 | 16 | 132 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_3x3_16QAM1/2 | 8388,88 | 45 | 180 | 0 | 1749,42 | 17 | 106 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_3x3_64QAM3/4 | 10664,89 | 12 | 78 | 0 | 9509,41 | 41 | 162 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_3x3_16QAM1/2 | 8401,39 | 45 | 170 | 0 | 1147,31 | 18 | 98 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_3x3_16QAM3/4 | 10665,18 | 15 | 124 | 0 | 9537,39 | 41 | 186 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_3x3_16QAM1/2 | 8366,27 | 46 | 186 | 0 | 10664,81 | 15 | 108 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_3x3_16QAM1/2 | 8380,48 | 45 | 178 | 0 | 9482,05 | 41 | 160 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_3x3_16QAM1/2 | 1999,87 | 2 | 2 | 0 | 9543,41 | 41 | 162 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_3x3_16QAM1/2 | 1999,86 | 2 | 2 | 0 | 1999,86 | 3 | 30 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_3x3_16QAM1/2 | 1999,85 | 2 | 2 | 0 | 1999,86 | 3 | 44 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_3x3_16QAM1/2 | 1999,84 | 2 | 2 | 0 | 1999,85 | 3 | 38 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_3x3_16QAM1/2 | 7999,29 | 2 | 2 | 0 | 1999,84 | 3 | 30 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_3x3_64QAM5/6 | 7999,25 | 2 | 2 | 0 | 7999,2 | 6 | 44 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_3x3_16QAM3/4 | 7999,2 | 2 | 2 | 0 | 7999,25 | 6 | 44 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_3x3_64QAM3/4 | 4999,5 | 2 | 2 | 0 | 7998,95 | 6 | 44 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_3x3_64QAM3/4 | 4999,47 | 2 | 2 | 0 | 4999,5 | 5 | 36 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 4999,47 | 5 | 34 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 6 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 2 | 0 | 95,99 | 2 | 10 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 2 | 2 | 0 | 95,98 | 2 | 10 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,98 | 2 | 10 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,99 | 4 | 12 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 4 | 4 | 0 | 95,98 | 4 | 12 | 0 |
|  |  |  |  |  |  |  |  | 0 | 95,98 | 4 | 14 |  |

PER from 3.10-2 to 5.10-2

|  |  |  |  |  |  | Error = 3.10-2 |  |  |  | Error = 5.10-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_3x3_16QAM1/2 | 9287,21 | 42 | 210 | 0 | 7877,05 | 50 | 174 | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_3x3_16QAM1/2 | 9280,97 | 42 | 218 | 0 | 7853,52 | 50 | 178 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_3x3_16QAM3/4 | 10664,86 | 20 | 120 | 0 | 10664,11 | 28 | 526 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_3x3_64QAM2/3 | 183,52 | 23 | 102 | 0 | 434,82 | 27 | 98 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_3x3_16QAM1/2 | 9297,58 | 42 | 162 | 0 | 7859,01 | 50 | 194 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_3x3_64QAM3/4 | 10664,71 | 19 | 512 | 0 | 9243,11 | 24 | 512 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_3x3_16QAM1/2 | 9300,32 | 42 | 156 | 0 | 7887,48 | 50 | 176 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_3x3_16QAM3/4 | 2087,44 | 23 | 116 | 0 | 10664,89 | 26 | 512 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_3x3_16QAM1/2 | 9292,46 | 42 | 152 | 0 | 7856,45 | 50 | 200 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_3x3_16QAM1/2 | 9298,04 | 42 | 156 | 0 | 7833,41 | 50 | 190 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_3x3_16QAM1/2 | 1999,8 | 6 | 44 | 0 | 1999,79 | 8 | 56 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_3x3_16QAM1/2 | 1999,84 | 6 | 58 | 0 | 1999,86 | 8 | 72 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_3x3_16QAM1/2 | 1999,85 | 6 | 44 | 0 | 1999,85 | 8 | 58 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_3x3_16QAM1/2 | 1999,84 | 6 | 50 | 0 | 1999,78 | 8 | 58 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_3x3_16QAM1/2 | 7999,18 | 9 | 66 | 0 | 7999,2 | 11 | 64 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_3x3_64QAM5/6 | 7998,95 | 9 | 58 | 0 | 7999,04 | 11 | 58 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_3x3_16QAM3/4 | 7998,69 | 9 | 56 | 0 | 7998,99 | 11 | 70 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_3x3_64QAM3/4 | 4999,5 | 8 | 58 | 0 | 4999,3 | 10 | 64 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_3x3_64QAM3/4 | 4999,4 | 8 | 58 | 0 | 4999,37 | 10 | 58 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 14 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 10 | 0 | 95,99 | 2 | 14 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 16 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 10 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 2 | 14 | 0 | 95,98 | 2 | 14 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 12 | 0 | 95,99 | 4 | 16 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 16 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 12 | 0 | 95,99 | 4 | 14 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 14 | 0 | 95,99 | 4 | 14 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 12 | 0 | 95,99 | 4 | 16 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 12 | 0 | 95,98 | 4 | 14 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 12 | 0 | 95,98 | 4 | 14 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 12 | 0 | 95,98 | 4 | 14 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 4 | 14 | 0 | 95,98 | 4 | 14 | 0 |

PER from 10-1 to 1.510-1

|  |  |  |  |  |  | Error = 10-1 |  |  |  | Error = 1.5 10-1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source STA | Destination STA | Traffic Name | Traffic Type | PHY mode | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR | Throughput (kbit/s) | Mean Delay (ms) | Max Delay (ms) | PLR |
| AP | STA1 | FileTransfer1 | TCP | 40_3x3_16QAM1/2 | 7790,35 | 49 | 178 | 0 | 6085,56 | 63 | 216 | 0 |
| AP | STA2 | FileTransfer2 | TCP | 40_3x3_16QAM1/2 | 7772,23 | 49 | 512 | 0 | 6097,54 | 63 | 226 | 0 |
| AP | STA3 | FileTransfer3 | TCP | 40_3x3_16QAM3/4 | 10432,37 | 35 | 526 | 0 | 4595,68 | 68 | 147640 | 0 |
| AP | STA4 | FileTransfer4 | TCP | 40_3x3_64QAM2/3 | 862,96 | 36 | 520 | 0 | 8470,6 | 43 | 540 | 0 |
| AP | STA5 | FileTransfer5 | TCP | 40_3x3_16QAM1/2 | 7775,41 | 49 | 186 | 0 | 6085,24 | 63 | 512 | 0 |
| AP | STA6 | FileTransfer6 | TCP | 40_3x3_64QAM3/4 | 10663,69 | 30 | 526 | 0 | 6194,16 | 44 | 14702 | 0 |
| AP | STA7 | FileTransfer7 | TCP | 40_3x3_16QAM1/2 | 7775,27 | 49 | 172 | 0 | 6105,75 | 63 | 210 | 0 |
| AP | STA8 | FileTransfer8 | TCP | 40_3x3_16QAM3/4 | 250,68 | 43 | 126 | 0 | 8103,3 | 46 | 534 | 0 |
| AP | STA9 | FileTransfer9 | TCP | 40_3x3_16QAM1/2 | 7784,84 | 49 | 512 | 0 | 6095,82 | 63 | 208 | 0 |
| AP | STA10 | FileTransfer10 | TCP | 40_3x3_16QAM1/2 | 7782,38 | 49 | 162 | 0 | 6108,1 | 63 | 202 | 0 |
| AP | STA11 | LowStreaming11 | CBR | 40_3x3_16QAM1/2 | 1999,79 | 12 | 90 | 0 | 1999,67 | 17 | 84 | 0 |
| AP | STA12 | LowStreaming12 | CBR | 40_3x3_16QAM1/2 | 1999,72 | 12 | 86 | 0 | 1999,82 | 17 | 100 | 0 |
| AP | STA13 | LowStreaming13 | CBR | 40_3x3_16QAM1/2 | 1999,82 | 12 | 82 | 0 | 1999,76 | 17 | 100 | 0 |
| AP | STA14 | LowStreaming14 | CBR | 40_3x3_16QAM1/2 | 1999,76 | 12 | 86 | 0 | 1999,68 | 17 | 96 | 0 |
| AP | STA15 | HighStreaming15 | CBR | 40_3x3_16QAM1/2 | 7998,75 | 18 | 76 | 0 | 7998,95 | 23 | 98 | 0 |
| AP | STA16 | HighStreaming16 | CBR | 40_3x3_64QAM5/6 | 7998,93 | 18 | 82 | 0 | 7998,63 | 23 | 118 | 0 |
| AP | STA17 | HighStreaming17 | CBR | 40_3x3_16QAM3/4 | 7998,58 | 18 | 84 | 0 | 7998,75 | 23 | 100 | 0 |
| AP | STA18 | SDTV18 | CBR | 40_3x3_64QAM3/4 | 4999,37 | 15 | 74 | 0 | 4999,3 | 22 | 100 | 0 |
| AP | STA19 | SDTV19 | CBR | 40_3x3_64QAM3/4 | 4999,27 | 15 | 86 | 0 | 4999,13 | 22 | 88 | 0 |
| AP | STA20 | VoIP_DL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA21 | VoIP_DL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA22 | VoIP_DL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA23 | VoIP_DL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA24 | VoIP_DL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA25 | VoIP_DL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 20 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA26 | VoIP_DL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA27 | VoIP_DL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 20 | 0 |
| AP | STA28 | VoIP_DL28 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 16 | 0 | 95,99 | 2 | 24 | 0 |
| AP | STA29 | VoIP_DL29 | CBR | 20_2x2_16QAM1/2 | 95,99 | 2 | 16 | 0 | 95,99 | 2 | 16 | 0 |
| AP | STA30 | VoIP_DL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 24 | 0 |
| AP | STA31 | VoIP_DL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 24 | 0 |
| AP | STA32 | VoIP_DL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 2 | 20 | 0 | 95,98 | 2 | 24 | 0 |
| AP | STA33 | VoIP_DL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 20 | 0 |
| AP | STA34 | VoIP_DL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 2 | 16 | 0 | 95,98 | 2 | 16 | 0 |
| STA20 | AP | VoIP_UL20 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 18 | 0 |
| STA21 | AP | VoIP_UL21 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 18 | 0 |
| STA22 | AP | VoIP_UL22 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 22 | 0 |
| STA23 | AP | VoIP_UL23 | CBR | 20_2x2_64QAM2/3 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 20 | 0 |
| STA24 | AP | VoIP_UL24 | CBR | 20_2x2_16QAM3/4 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 22 | 0 |
| STA25 | AP | VoIP_UL25 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 20 | 0 |
| STA26 | AP | VoIP_UL26 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 22 | 0 |
| STA27 | AP | VoIP_UL27 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 18 | 0 | 95,99 | 4 | 18 | 0 |
| STA28 | AP | VoIP_UL28 | CBR | 20_2x2_16QAM1/2 | 95,99 | 4 | 16 | 0 | 95,99 | 4 | 16 | 0 |
| STA29 | AP | VoIP_UL29 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 20 | 0 | 95,98 | 4 | 20 | 0 |
| STA30 | AP | VoIP_UL30 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 18 | 0 |
| STA31 | AP | VoIP_UL31 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 18 | 0 |
| STA32 | AP | VoIP_UL32 | CBR | 20_2x2_16QAM1/2 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 18 | 0 |
| STA33 | AP | VoIP_UL33 | CBR | 20_2x2_16QAM3/4 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 18 | 0 |
| STA34 | AP | VoIP_UL34 | CBR | 20_2x2_64QAM3/4 | 95,98 | 4 | 16 | 0 | 95,98 | 4 | 22 | 0 |

| Scenario 6bis: Performance summary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PER |  | 0 | 10-2 | 3.10-2 | 5.10-2 | 10-1 | 1,5.10-1 |
| Name | Unit | Value | Value | Value | Value | Value | Value |
| Aggregate goodput for non-QoS flows (CC18) | Mbit/s | 92.930 | 81.406 | 79.357 | 78.173 | 68.890 | 63.941 |
| Ratio for non-QoS flows (CC18) | % | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* |
| Aggregate goodput for QoS flows | Mbit/s | 44.875 | 44.875 | 44.874 | 44.874 | 44.873 | 44.873 |
| Number of QoS flows satisfying the objectives (CC19) |  | 39 | 39 | 39 | 39 | 39 | 39 |
| % of QoS flows satisfying the objectives (CC19) | % | 100 | 100 | 100 | 100 | 100 | 100 |
| Total aggregate goodput (metric 1 of CC20) | Mbit/s | 137.805 | 126.281 | 124.231 | 123.048 | 113.763 | 108.815 |
| Metric 2 of CC20 | Mbit/s | 137.805 | 126.281 | 124.231 | 123.048 | 113.763 | 108.815 |
| Metric 3 of CC20 | Mbit/s | 137.805 | 126.281 | 124.231 | 123.048 | 113.763 | 108.815 |
| Average PHY rate | Mbit/s | 195.15 | 181.67 | 184.97 | 189 | 186.77 | 192.21 |
| MAC efficiency (CC24) | % | 71 | 70 | 67 | 65 | 61 | 57 |

Table 4-6: Performance summary for scenario 6bis

* N/A: Non applicable because TCP source throughput is unlimited

Figure 4-3: TCP throughput and MAC efficiency versus PER in scenarios 6 and 6bis

## Goodput and MAC efficiency versus PHY data rate

We do not have extensive PHY simulation results allowing us to plot a curve of goodput versus the distance. As a replacement, we drew the goodput for scenario 16 (point-to-point) for a set of physical layers, and considering several PER values. Curves slope corresponds to the MAC efficiency.

Figure 4-4: MAC efficiency vs. PHY throughput

We can see in Figure 4-4 that the goodput at MAC SAP varies as a linear function of the physical data rate, showing that MAC efficiency remains almost constant.

Indeed, the MAC scales very well with the PHY data rate, even for very high bit rates.

An increasing value of PER implies only a little decrease in the efficiency. Retransmissions are not bandwidth wasting, and amount of signaling remains low even for high PER values.

Moreover, since segmentation is applied, these results are valid whatever the application packet size is.

## Delays

Figure 4-5 gives the cumulative distribution function of transmission delays in scenario 1 for TCP and VoIP flows in various channel conditions (different PER).

Even when the medium is full radio condition very harsh, VoIP traffics experience delays that are always below 20 ms.

Figure 4-5: Delay distribution (scenario 1)

## Mixed traffic handling

To assess system capacity to handle a large amount of mixed traffic, we defined a new scenario.

Figure 4-6: Mixed traffic handling

We considered 1 TCP data flow, able to fully backlog the medium, transmitted using a MIMO mode at 126 Mbit/s, and VoIP sessions, each one having duplex traffic (2 x 96 kbit/s) and different PHY modes among simulation runs. We increased the number of VoIP sessions and report how the TCP traffic is impacted.

Figure 4-6 report simulation results. The system is able to handle correctly 30 VoIP sessions while offering room for at least 70 Mbit/s of TCP traffic.

# Simulation conditions and hypothesis

## System simulations

The system simulations have been performed on a C/C++ simulator modelling the MAC layer and the PHY layer abstraction. It is connected to a TCP/IP stack (Reno type, window size 64K or 192K), and is fed by a flow generator on top of it.

The MAC efficiency definition given in the CC is relevant only for a 100% medium occupancy, i.e. if the aggregated throughput of all sources is greater than the system capacity. With the MAC efficiency we are able to show up, this is not achieved in original scenarios such as number 1 and 6.

Thus, we ran two sets of simulations:

- The first one, referred by scenario numbers 1,4 and 6, uses the minimal PHY mode that fulfils scenarios' requirements (scenario 1,6) or that fulfils QoS requirements while still providing some room for non-QoS flows (scenario 4).

- In the second set, we modified slightly the scenarios to remove the throughput limitation of "Local file transfer" sources and numbered them 1bis, 4bis and 6bis. MIMO PHY modes are selected depending on range. Indeed, TCP sources take all the remaining bandwidth and give a good metric of maximum system performances.

### Simulation parameters

- Unique MAC configuration: no knob activation nor parameter tuning depending on the context or scenario

- MAC, EC and segmentation overhead fully taken into account

- Dynamic resource allocation based on requests from STA

- Simple Round Robin scheduler (per priority level), 2 priority classes

- No contention period, 2 ms long MTF

- 2 or 4 CTIs per MTF

- Simulated time: 3 to 20 min.

### PHY abstraction

The PHY abstraction (link adaptation, error model and impairments) have been modelled as follows.

- Link adaptation:

The simulations are processed with an elementary link adaptation model, which selects the PHY mode with respect to the transmission range of the link. This is a simple model, parameterised with very conservative hypothesis (for example 64 QAM 5/6 mode is used only below 4m, 3.3.1).

It must be noted that the link adaptation was activated only in scenarios 1bis, 4bis and 6bis.

- Error model:

- Error distribution: The simulator uses a uniformly distributed error model. The accuracy of such model might differ depending on the considered PHY as the error behaviour depends on several factors such as link adaptation, channel characteristics and PHY design, but it must be noted that this error model is the most stressful for the MAC. Indeed, the performances we disclose correspond to a minimum bound.

- PER value: Simulations have been run for several average PER from 0 to 1,5.10-1 and we selected the PER operating point at 3.10-2 in the comparison criteria. This value is very conservative as it applies to short packet (134 bytes or less) and is equivalent to a PER of 2,9.10-1 for 1500 bytes MPDU and to 6.10-1 for 4K MPDU in uniformly distributed errors case.

- PHY impairments

- IM1, PA non linearities: The effect of the PA non linearities has been taken into account through the inclusion of a 4 dB provision for PA back off in the link budgets. The usage range of the different PHY modes hence encompasses IM1 and the chosen value is sufficient to ensure that the PHY doesn’t violate the 802.11a spectral mask.

- IM5, Noise Figure: The 10dB noise figure is taken into account in the link budget.

- IM6, antenna configuration lambda/2 spacing has been taken into account in the PHY simulations.

# Rationales & Objectives

Applications that are targeted in TGn's PAR include, in addition to typical data-oriented ones, high QoS demanding services like audio/video streaming, high definition video, or VoIP. Moreover, the CFP requires an aggregate throughput of 100 Mbit/s measured at top of the MAC Service Access Point, obtained in a 20 MHz radio bandwidth. These constraints clearly imply the definition of a new physical layer, but include also the MAC in the enhancement loop. We present hereafter the rational and the objectives we had in mind when designing 802.11 MAC's improvements to make it able to cope efficiently with such a mix of traffic flows with a highly variable QoS requirements.

## Need for a new access scheme

While EDCA can hardly be used for STAs with QoS constraints (VoIP & multimedia terminals for example) due to the absence of guaranty on the medium access delay and to the high probability of collision in scenarios with an important number of STAs, HCCA can be foreseen as a solution for QoS Support. Its central controller allocating TxOP in a contention free manner via a polling scheme is expected to ensure bound medium access delay.

However HCCA scheme suffers from severe weaknesses. Its polling can be based either on timers or on instant resource needs.

A polling scheme based on timers faces several challenges since it has to deal with traffics that are generally VBR (pure CBR flows are very rare, even voice traffic is not because of the Operating System and network jitters). The non-synchronism between packet arrival and resources allocation leads to delay and resource waste. Finally the Resource Manager needs to know the traffic profile of each flow. This information is usually unknown by the application itself, and even in case it has this knowledge, there is anyway no standard mean to transmit it to the RM.

- Getting traffic profiles is difficult and can be implemented only for very specific applications. Timer based polling is not efficient

Alternately a STA can indicate its resource needs during polling via piggybacking (QoS Control)

However such a mechanism does not work properly for VBR applications and for medium bandwidth demanding applications, i.e. up to a few Mbit/s in case of TGn. Indeed, the source queue may be emptied in one shot and  then there is no more traffic to request resource for at piggy backing time. Moreover, due to MPDU overhead and to the station-by-station polling protocol itself, this access scheme is not efficient when flow number increases and can't be used for elastic streams.

- HCCA piggybacking is not efficient for low bit rate or bursty applications

To be efficient, resource shall be allocated by  a centralised controller from indications sent by STAs via short Resource Requests signaling messages. There is then no resource wasting and the system can adapt quickly to flow variations. Resource Requests messages shall be sent either via in-band signaling or via a dedicated contention period to provide short allocation latency at low signaling cost. Such a mechanism can deal efficiently with any type of traffic, from the most stringent one like VoIP to the most elastic one like TCP.

- HCF does not provide such a mechanism

## Requirements for a CE compatible WLAN

### Application requirements

- Throughput constraints

There are an infinite number of possible applications in a WLAN and it is quite hard to define exactly the required throughput for each of them. Some applications such as video streaming may require very high throughput up to 25Mbit/s without strong delay constraint (the “usual” zapper can tolerate delays below few hundred ms) while some others will require low bit rates with stringent delay constraints (video conferencing, voice over IP). At last, some classes of applications are able to adapt their data rate according to the network state (TCP, some video codecs). Link and PHY layer shall be able to adapt efficiently to this applications diversity. Moreover, the data rate produced by most of the applications is far from constant and the range of required throughputs is very wide. For instance, a MPEG/2 video stream encoded at a mean data rate of 8 Mbit/s may include peaks above 15 Mbit/s, and still periods below 6 Mbit/s, depending on the scenes and on the encoder. This characteristic of WLAN applications implies a WLAN system able to cope with high throughput range, bursty traffic, while being able to guaranty low transmission delay.

- Error rate

Numerous applications do not support errors. This is the case for traffic output from most of video coders, like MPEG2, but it is true also TCP traffic. Indeed, TCP considers any packet loss as a congestion indication, and automatically reduces its window size, leading to a strong decrease of throughput. Thus, it is mandatory to have methods able to provide an error-free data stream.

These methods can be based either on ARQ, or on Link Adaptation, or, more probably, based on both. Anyway, the consequence is a variable consumption of PHY resources among time in all cases. Thus, from the MAC perspective, which goal is to manage PHY resources, CBR flows do not exist.

- Latency

A low latency (small delay transmission) shall be guarantied for “real-time” applications (voice, video conferencing). Even video streaming has delay constraints, around a few hundreds of ms, to allow a quick control and a smooth zapping. As the WLAN is likely to be only a part of the crossed network, the delay constraint applied to the WLAN Link Layer is stronger than the final maximum delay the application can support.

This constraint has an impact on the resource allocation (MAC) and the EC mechanisms (ARQ). A maximum latency of a few ms is tolerable.

### Environment requirements

- BSS load

Unlike cellular networks, the load of a BSS may stay at a very high level during long periods of time. Therefore, some efficient MAC and EC protocols are required to optimise the PHY resource usage and to maximise the throughput at application level. The overhead produced by each layer (EC, MAC, and PHY) shall be severely limited as they correspond to a waste of the available PHY data rate. PHY overhead is particularly significant for low bit rate applications and for MAC/EC signalling flows, as the amount of time taken by symbols required for time synchronisation and channel estimation processing take up a fixed time period whatever the payload is.

- Home environment

In the home environment, both terminals associated in a cell may need to communicate (for instance, DVD/camcorder with TV). For applications with high data rate, the most efficient way to perform that operation is to open a direct communication between both terminals. Thus the access scheme shall offer efficient direct mode communications.

- Hot spot environment

Hotspots cells are characterised by a potentially great number of low bit rate data flows (essentially generated by low bit rate applications), mixed with an also potentially great number of high bit rate data flows. As mentioned before, low bit rate flows introduce heavy PHY overhead in OFDM systems and creates a real problem when their number becomes important.. Moreover, the system shall be able to cope with this big mix traffic with different profiles and different requirements.

## MAC features

- Dynamic, fast and accurate resource allocation

A fast and dynamic allocation shall be performed in Down Link and Up Link.

Fast allocation shall ensure a low latency especially when ARQ is used (fast retransmission) or when adaptive transmission (fast PHY modes change) is applied. Applications can support a minimum latency of a few milliseconds, which allows using a MAC based on a frame of several milliseconds. Such a MAC frame will allow using a fast ARQ mechanism even for real-time services with strong delay constraint.

Dynamic and accurate allocation shall be performed according to the needs of each terminal, on a per-service basis.. Due to the variable data rate of application sources and to the EC mechanism, the amount of required PHY resource is variable. These variations may be weak (010%) or more important, up to 100%. Furthermore, some flows such as MAC/EC signalling need few sporadic resources.

MAC and PHY layers shall be able to adapt dynamically (based on a few ms frame) with a fine accuracy in order to save PHY resource.

Resources shall be shared between all the terminals of the cell: unused resource shall be re-allocated to terminals that can use it. For instance, some terminals may not use all the PHY resource that has been reserved when the service has been opened. Then, the remaining unused bandwidth can be freely re-allocated to terminals that can use it (TCP applications).

- Support of Error Control mechanisms

EC mechanisms such as ARQ require signalling resource in the reverse direction. At MAC level, terminals need also to indicate to the Resource Manager their resource needs for uplink user traffic and to report the radio downlink quality. These signalling flows are low bit-rate with quite strong delay constraint. For terminals that have user traffic in both directions, the resource for signalling may be aggregated to the traffic (piggy-backing). However, for unidirectional applications, these flows have the same profile as delay constrained low bit rate services

- Support of Link Adaptation

A link adaptation mechanism brings a rough, medium long-term, adaptation of the PHY to the channel while the ARQ can be considered as a fast, fine tuning link adaptation used on top of the long term one. Thus both mechanisms are complementary and shall be used in conjunction.

- Support of direct transmission between

PHY and MAC shall support this feature to avoid an expensive relay procedure, especially for high data rate flows. Similarly to DL and UL cases, the amount of resources that must be transmitted is variable along the time. The resource allocation shall be performed dynamically with a fine accuracy, which involves again a dynamic repartition between DL, UL and direct transmission.

- Power consumption

MAC mechanisms that are based on the usage of a TDM frame with resource announcements offer more possibilities to reduce the power consumption. Indeed, a station has not to stay awake all the time and to listen and decode all packets present on the medium. It knows at each time-frame when it has traffic to receive, and when it will be able to transmit. Then the station may switch off several parts of its hardware.

Since the access scheme has no collision, the probability to have to retransmit a packet is lower, due only to errors in the channel. The average power consumption is then reduced compared to access schemes with collisions.

Moreover, the centralised resource allocation mechanism and the resource announcement make low-power consumption modes available for stations communicating in direct mode without the need of specific signalling.

- PHY efficiency

With a MIMO-based PHY layer, preambles for time synchronisation and channel estimation are necessary equal or longer then the one used for SISO modes. With the increase of PHY data rate, preamble time can become a preponderant part of transmission time. So can become all fixed dead times like guard times and radio turn-around times. It is a necessity to severely minimise their impact.

A first mean to overcome these issues is to aggregate data packets at PHY level, while keeping data units handled by the MAC short. Thus, PHY burst length can be maximised, Packet Error Rate minimised, and retransmissions cost maintained low.

An additional response is given by the centralised resource control scheme with resource announcement, where the Resource Manager is able to allocate resources in a way to minimise all these dead times.

## Some conclusions

The design of our new access scheme for the MAC was driven by the following goals: Offering an effective QoS to various types of applications, in different types of environments, and relaxing the pressure on the PHY layer by improving MAC efficiency, while dealing with backward compatibility issues with legacy 802.11 MAC. Another objective was to achieve these goals with a low complexity, easily implementable solution.

### QoS support

The most efficient way for QoS delivery and optimisation of resource usage in a wireless LAN is obtained with a fast, dynamic (ms) and accurate resource allocation obtained by using a MAC time frame, a centralised allocation process and a resource request / grant scheme to perform allocation in uplink. Indeed, the system is able to adapt to the application needs in the time. Inside a MAC frame, the available PHY resources are shared among the different services in order to respect the QoS constraints attached to each of them.

### Compatibility

A full compatibility with legacy 802.11 systems is ensured by including the enhanced MAC access scheme inside the legacy superframe (use of the CFP or alternatively the CAP as defined in 802.11e).

### Efficient resource usage

To provide a robust support of emerging applications, 802.11n systems must have the capacity to sustain high load, this even in bad channel conditions. Aggregation at PHY level (several MPDUs in a single PPDU) coupled with short MAC-PDUs and an optimised fast selective repeat ARQ using low cost signalling allows a much better MAC efficiency while ensuring robustness.

### Versatility

Thanks to a MAC access scheme based on an accurate centralised on-demand allocation scheme, the AP knows precisely the current resource needs of the STAs, which can then be fulfilled with a single scheduler without relying on any context dependent knob or tuning. It allows an easy deployment of 11n systems with a high level of QoS whatever the context is, in particular for home environment where the end user doesn’t necessarily have system administrator abilities.

### Flexibility

Beyond CBR traffic (if any), this access scheme is able to handle all kind of bursty and elastic traffic flows. Its fast and flexible resource request and access grant mechanisms have been especially designed to support bursty and elastic traffics in a very efficient way.

### Easy Implementation

An accurate centralised on-demand allocation scheme strongly relaxes the pressure on the scheduler, which can then be very simple (we use round robin per priority). This, coupled with the fixed size MAC time frame lead to an easy implementation.

Furthermore, a MAC based on the usage of a fixed duration frame is profitable to implement advanced MAC power saving schemes and more efficient PHY synchronisation algorithms since each receiver knows when it shall receive data.

# Technical overview

## Presentation

An overview of the extended MAC stack we have defined, named ECCF (Extended Centralised Coordination Function) is described in Figure 7-7.

Figure 7-7: Extended MAC structure

The MAC SAP is kept identical to the one defined by legacy 802.11 while the PHY SAP may be modified according to the capabilities of the PHY layer. The MAC layer is constituted of two convergence sub-layers, LLC Convergence Sub-Layer (LLCCS) and Segmentation and Re-assembly (SAR), and two transfer sub-layers, MAC Intermediate Sub-Layer (MIS) and MAC Lower Sub-layer (MLS).

The LLCCS sub-layer maintains the MAC SAP consistency. The MIS embeds the core transfer function of the MAC layer and is based on short fixed-size transfer units. The MIS also integrates the Error and Flow Control functions. The SAR sub-layer performs the adaptation between the variable size packet provided by the LLCCS and the transfer units managed by the MIS. The MLS sub-layer is in charge of building 802.11 compatible MPDUs from MIS transfer unit and signalling information, and delivers them to the PHY layer. In addition, it can implement the encryption support functions.

Backward compatibility is an important issue since the number of already installed devices is now tremendous. Nevertheless, this important issue shall not undermine the effort deployed to enhance the MAC; otherwise new requirements are likely to be impossible to be satisfied. Moreover, each time the legacy functions are modified, it raises a possible backward compatibility issue even if designed carefully. Thus we have preferred to keep the legacy MAC as is, assuring a full backward compatibility, and to add a dedicated function able to cope with new requirements.

This stack conveys a MAC architecture based on:

- Fixed MAC time frame (MTF)

- Connection-less

- Centralised resource management

Short and fixed size MAC data transfer units

- Low latency Error Correction (Selective Repeat ARQ)

- Aggregation at PHY level

## Frame structure

The ECCF mode (see Figure 7-8) can operate concurrently with HCCA and EDCA schemes within the same cell. The legacy MAC super frame is kept. The super frame started with a beacon compatible with legacy MAC that includes additional parameters relative to the new access scheme. Contention Periods (CP) and Contention Free Periods (CFP) alternate in time and room is introduced in the latter for the new access method. A Radio Resource Manager (RRM) is responsible to determine the duration of each period according to various traffic considerations.

The ECCF implements a contention-free access method based on short TDMA/TDD of fixed duration, called MAC Time Frame (MTF).  Each CFP may include any number of them, selected by the RRM.

Figure 7-8: Frame structure

Alternatively (Figure 7-9), ECCF can be inserted into a CAP generated by the HC using mechanisms defined in the 802.11e extension. The RRM reserves a CAP by contending for the medium with its high priority and then by sending a CF-Poll data frame for itself. CAP is split by the RRM into successive MTFs of fixed duration, each being described by a PGPM broadcast at the beginning of the MTF.

.

Figure 7-9: Alternative frame structure

## MTF description

Resource scheduling is performed on a per MTF basis. Time Intervals (TI) of variable duration are dynamically allocated to stations. These allocations are broadcast by the RRM at the beginning of each MTF in the so-called Periodic Grouped Polling (PGPM) structure (see Figure 7-10). For each TI, the RRM indicates the station the slot is granted to for transmission, and the list of stations that are a traffic destination.

Figure 7-10: MAC Time Frame structure

## MPDU

Each Time Interval, including TI number #0, is constituted of one variable length MAC PDU. This is the data unit that is exchanged between MAC and PHY layers, as in legacy 802.11.

A MPDU is composed of a header, a signalling part dedicated to in-band signalling and an optional data part all of them being of variable length. The header describes the MPDU's content, the signalling part is the place for resource requests, error control signalling and data block descriptors. A dedicated check sequence protects header and signalling trunks, two crucial pieces. The data part may aggregate all data blocks sent by a station.

## MPDU overhead reduction

Still in the process of reducing overheads, a new format has been defined for MPDUs (Figure 7-11). The header is shortened down to two octets, the first one assuring a compatibility with legacy 802.11 MAC-PDU frame. This enhancement requires a new protocol version identifier, inserted in the Protocol Version field of this first octet.

Figure 7-11: MPDU general structure

## Aggregation at PHY level

In Legacy IEEE 802.11 MAC, a substantial amount of time is taken by PHY preambles for channel estimation. This amount is going to rise with the coming of more sophisticated PHY like MIMO which requests more precise channel estimation. Moreover, the cost in time of these preambles compared to the time spent in data transmission becomes overwhelming when considering very high performance PHY layers. Consequently, lengthening PHY bursts is a necessity.

In our architecture, each MPDU is the data unit exchanged with the PHY and corresponds to one PHY burst. When present, its data part (Figure 7-12) is composed of one or several data blocks, each of them described in the header by a Data Part Descriptor (DPD). Each data block includes aggregated data packets intended for a single destination station.

The ability to aggregate packets from one originator to one or several destination nodes allows having long PHY bursts, thus limiting PHY overhead. Nevertheless, the MPDU structure lets the possibility to use a separate PHY mode for each individual data block.

Figure 7-12: MPDU structure example with data blocks transmitted to 2 stations

## Segmentation and Reassembly

A common way of lengthening PHY burst is to aggregate traffic at MAC SAP. This approach has several severe drawbacks. Packet Error Rate increases quickly with packet length, leading to poor performances in realistic conditions, while at the same time repeating long packets costs considerable bandwidth waste.  We took another approach by designing a segmentation and reassembly mechanism at MAC level. MAC handles small or medium size packets, which help keeping the PER low when radio condition worsened and allows the definition of a fast and resource-thrifty ARQ mechanism to recover from remaining errors let by the PHY layer. The system is then more robust against errors, more efficient in terms of bandwidth and delay.

Figure 7-13 gives an overview of the Segmentation and Re-assembly (SAR) sub-layer introduced to perform adaptation between LLC and MAC. It allows adapting the variable packets coming from / to LLC interface to fixed size elements that are the data unit the error control deal with. To limit overhead inherent to padding, several different sizes, two for example, are defined.

A short header is added to each segment to hold signalling relative to segmentation. It is composed of a sequence number, numbering SAR-SDUs and a segment sequence number. Each segment is postfixed with an individual check sequence.

Figure 7-13: Segmentation And Reassembly (SAR) sub-layer

## MAC Time Frame structure example

Figure 7-14 gives an example of a MTF with four stations. Resource manager has allocated a TI to Station #1 for data transmission to stations #2 and #3, and another to station #4 which wants transmit to station #4. The Radio Resource Manager listens to all the traffic. CTA and CTB portions correspond to contention based access.

Station #1 transmits its data in one MPDU. Station #4 transmits signalling only, a Resource Request sent to the RRM and an ARQ feedback to station #3. For the sake of simplicity, the example below shows stations transmitting either data or signalling. Nevertheless, in band signalling is always possible.

Figure 7-14: MTF structure example

## Resource management and flow differentiation

PHY resources are allocated with a contention-free, centralised scheme, under the responsibility of the Radio Resource Manager. Each station shall request resource to the RRM before any transmission via a Resource Request message sent in the signalling part of MPDUs (in-band signalling).

A contention access procedure is used for initial requests. It is based on a slotted aloha procedure. The RRM acknowledges the contention accesses in the next emitted PGPM.

The RRM doesn't need to manage connections: PHY resources are allocated per station, per priority. Each station schedules its own flows within each Time Interval that was granted to it.

Annex A: Abbreviations and acronyms

| AKV | Acknowledgement Vector |
| --- | --- |
| ARQ | Automatic Repeat reQuest |
| BM | Bit Map |
| CBC | Cipher-Block Chaining |
| CBC-MAC | CBC Message Authentication Code. |
| CCM | Counter mode with CBC-MAC |
| CFP | Contention Free Period |
| CP | Contention Period |
| CTI | Contention Time Interval |
| DCF | Distributed Coordination Function |
| EC | Error Control |
| EFC | Error and Flow Control |
| ECCF | Extended Centralised Coordination Function |
| ETF | ECCF Time Frame |
| LLCCS | Logical Layer Control Convergence Sub-layer |
| LSN | LLCCS Sequence Number |
| MAC | Medium Access Control |
| MDB | MLS Data Block |
| MIS | MAC Intermediate Sub-layer |
| MLS | MAC Lower Sub-layer |
| MPDU | MAC Packet Data Unit |
| MSF | MAC Super Frame |
| NAV | Network Allocation Vector |
| PDU | Protocol Data Unit |
| PGPM | Periodic Grouped Polling MPDU |
| PHY | PHYsical Layer |
| PS-STA | Power Saving Station |
| QoS | Quality of Service |
| RLC | Radio Link Control |
| RRC | Radio Resource Control |
| RRM | Radio Resource Manager |
| SAR | Segmentation and Re-Assembly |
| SID | Short STA Identifier |
| SDU | Service Data Unit |
| SSN | Segment Sequence Number |
| STA | Station |
| TI | Time Interval |
| TLV | Type-Length Value |
| LTT | LLCCS Translation Table |