IEEE P802.11
Wireless LANs

Response to Call For Proposal for P802.11n

- Detailed Technical Description -

Date:	August, 2004

Authors:	Hervé Bonneville, Bruno Jechoux 
	Mitsubishi ITE.
	1, allee de Beaulieu, 35700 Rennes, France
	Phone: +33-2 23 45 58 58
	Fax: +33-2 23 45 58 59
	e-Mail: {bonneville,jechoux}@tcl.ite.mee.com

Abstract

This document is the detailed technical description of response to Call For Proposal for P802.11n standard ([2]). Response to functional requirements, comparison criteria table and a technical overview may be found in document 11-04-0914-00-000n-Proposal-Mitsubishi-ITE-Response.

Contributors:

| Hervé Bonneville | Mitsubishi |
| --- | --- |
| Bruno Jechoux | Mitsubishi |
| Romain Rollet | Mitsubishi |

Table of Content

1.	References	3

1.	MAC Functional Description	4

1.1	MAC Architecture	4

1.1.1	Extended Centralised Coordination Function (ECCF)	4

1.1.2	Error and Flow Control overview	4

1.1.3	Convergence sub-layers overview	5

1.2	Extended Centralised Coordination Function	5

1.2.1	Frame structure and timing	5

1.2.2	Medium access and transfer procedures	7

1.2.3	Broadcast function	9

1.2.4	Contention access procedure	9

1.3	Coexistence with legacy 802.11	9

1.4	Convergence Sub-layers	10

1.4.1	LLCCS (LLC Convergence SubLayer)	10

1.4.2	SAR (Segmentation And Re-assembly)	10

1.5	Error and Flow Control	11

1.5.1	General	11

1.5.2	Sequence numbers	11

1.5.3	ARQ window	11

1.5.4	ARQ protocol messages	11

1.5.5	Resource allocation	12

1.5.6	Transmitter operations	12

1.5.7	Receiver operations	12

1.6	QoS Support	13

1.7	Power Saving	13

1.8	Dual-channel operations (40MHz support)	14

1.9	Association	15

1.10	Security	15

1.10.1	Authentication	15

1.10.2	Encryption	15

2.	Packet formats	16

2.1	LLCCS-PDU	16

2.2	MIS-PDU	16

2.2.1	Long MIS-PDU	17

2.2.2	Short MIS-PDU	17

2.2.3	Segmentation samples	17

2.3	MPDU	18

2.4	SIE (Signalling Information Element)	18

2.4.1	General Format	18

2.4.2	MAC SIEs	18

2.4.3	EC SIEs	20

2.4.4	RRC SIEs	22

2.4.5	RLC SIEs	22

2.5	Data Type	23

2.5.1	RESOURCE_LOCATION	23

2.5.2	RESOURCE_NUMBER	23

2.5.3	PHY_PARAM_SET	23

2.5.4	Other Data Type	23

Annex A: Abbreviations and acronyms	25

Table 1-1: ECCF Parameter Set Field List	10

Table 1-2: Secure Data Block Encapsulation	15

Table 1-3: Parameters Used for the Nonce Value Calculation	15

Table 2-1: Header size of common protocols	17

Figure 1-1: MAC Protocol Stack Comparison	4

Figure 1-2: Frame Structure and Timing	5

Figure 1-3: Alternative frame structure	6

Figure 1-4: User Data Encapsulation in the Transmitter	7

Figure 1-5: PGPM and MPDUs structure sample	8

Figure 1-6: Structure of MPDUs emitted inside CTI	9

Figure 1-7: ECCF Time Frame.	10

Figure 1-8: Flows of signalling messages between STAs.	12

Figure 1-9: Power Saving procedure example	13

Figure 1-10: Dual-channel operations	14

# References

- 11-04-0914-00-000n-Proposal-Mitsubishi-ITE-Response

- IEEE 802 11-03/858r7, Call for Proposals for P802.11n

- IEEE 802 11-03/813r12, Functional requirements

- IEEE 802 11-03/814r30, TGn Comparison Criteria

- IEEE 802 11-03/802r23, TGn Usage Models

# MAC Functional Description

## MAC Architecture

Figure 1-1: MAC Protocol Stack Comparison

The new MAC access scheme described hereafter enhances the current 802.11 MAC. The MAC SAP is kept identical while the PHY SAP may be modified according to the capabilities of the PHY layer. As shown in Figure 1-1, the enhanced MAC layer is constituted of two Convergence sub-layers, LLC Convergence Sub-Layer (LLCCS) and Segmentation and Re-assembly (SAR), and two transfer sub-layers, MAC Intermediate Sub-Layer (MIS) and MAC Lower Sub-layer (MLS).

The MAC SAP consistency is maintained by the LLCCS sub-layer. The MIS embeds the core transfer function of the MAC layer and is based on short fixed-size transfer units. The MIS also integrates the Error and Flow Control functions. The SAR sub-layer performs the adaptation between the variable size packet provided by the LLCCS and the transfer units managed by the MIS. The MLS sub-layer is in charge of building 802.11 compatible MPDUs from MIS transfer unit and signalling information, and delivers them to the PHY layer. In addition, it can implement the encryption support functions.

### Extended Centralised Coordination Function (ECCF)

The proposed MAC extension defines a centralised access method that is designed to efficiently support QoS applications in hotspots and home environments. This access method relies on a Radio Resource Manager (RRM) function that manages the radio resource in the cell and shares it between the different associated STA (association has the same meaning as in legacy 802.11). The ECCF implements a contention free access method based on short TDMA/TDD MAC time frames of fixed duration. An access priority scheme is implemented to guaranty resource allocation to specific data flows that have strong QoS constraints. Data exchanges are connection-less and therefore do not require any specific set-up procedure.

### Error and Flow Control overview

In order to improve the error resilience of the MAC protocol, ECCF defines an Error and Flow Control mechanism (EFC) that is included in the MAC Intermediate Sub-layer (MIS). It is based on a selective ARQ scheme applied to the short fixed size MIS data units. Selective ARQ allows fast and efficient retransmissions while short data units increase robustness by reducing data unit error rate.

EFC is applied to all data flows except the broadcast ones.

### Convergence sub-layers overview

In order to reduce protocol overhead, ECCF defines Short STA Identifier (SID) used in ECCF MAC protocol messages. However, for compatibility reasons, i.e. to keep the 802.11 MAC-SAP unchanged, ECCF integrates an LLC Convergence SubLayer (LLCCS) that maintains the consistency between SIDs and IEEE 802.2 addresses. The LLCCS allows the exchange of data between STAs belonging to the same ECCF cell as well as between an ECCF STA and any network device located outside the ECCF cell. The LLCCS is also in charge of passing QoS information between the MIS and the upper layers including the application layer.

Fragmentation of a LLCCS-PDU into smaller data units is performed by the Segmentation And Re-assembly Sub-layer (SAR). It produces SAR-PDUs that are then encapsulated into MAC Intermediate Sub-layer PDUs (MIS-PDU). The MIS-PDUs are the basic data unit transmitted by the MAC, of which two types are defined: Long MIS-PDU (L-MIS-PDU) and Short MIS-PDU (S-MIS-PDU). An L-MIS-PDU is twice as long as a S-MIS-PDU.

The process of recombining a series of x-MIS-SDUs into a single LLCCS-PDU is defined as re-assembly. Re-assembly is accomplished upon reception of error-free x-MIS-SDU.

## Extended Centralised Coordination Function

### Frame structure and timing

The ECCF provides contention-free data transfers managed by a centralised RRM function that may reside in any STA. When one of the STA provides an interconnection service with another network (bridge, access point), it may be profitable that this STA hosts the RRM function. In this case, the RRM STA is equivalent to the AP in the 802.11 acceptance.

As ECCF defines some new MAC frame formats, only ECCF compatible STAs are able to communicate with the RRM function. In the following, the term RRM will refer to the STA embedding the RRM function or the RRM function itself.

In order to operate with stations implementing DCF, PCF or HCF functions, the RRM broadcasts a 802.11 beacon that defines two parts: CFP and CP. The time interval between two consecutive beacons is fixed and defines the MAC Super-Frame (MSF). However, because of the inherent jitter introduced by the CSMA/CA access method at the end of the CP, the MSF duration slightly varies. Concerning the CFP, the Beacon indicates the duration of time intervals controlled by either ECCF or PCF/HCCA functions as shown in Figure 1-2. For the ECCF function, ECCF specific global information is added which is used by new non-associated terminals.

Figure 1-2: Frame Structure and Timing

Alternatively (Figure 1-3), ECCF can be inserted into a CAP generated by the HC using mechanisms defined in the 802.11e extension. The RRM reserves a CAP by contending for the medium with its high priority and then by sending a CF-Poll data frame for itself. CAP is split by the RRM into successive MTFs of fixed duration, each being described by a PGPM broadcast at the beginning of the MTF.

Figure 1-3: Alternative frame structure

#### ECCF Time Frame description

Within the CFP period, the RRM schedules several ECCF Time Frames (ETF) of fixed and equal duration. Each ETF contains several Transmission Intervals (TI) allocated by the RRM to the different STAs. The first TI in the ETF is used by the RRM to broadcast the frame description information. Each subsequent TI is allocated to a unique STA to transmit its data toward one or several destination STA. The source STA is in charge of distributing its TI resource among its different data flows.

#### TI description

Each TI contains one MAC PDU (MPDU) that is the data unit exchanged with the PHY layer. An MPDU is constituted of two parts of variable lengths: the signalling and data parts. The data part may possibly be empty. Different PHY modes may be independently selected for the signalling part and the data part.

The first TI of the ETF has to be listen by all STAs and contains a specific MPDU composed of signalling information only. It is notably describing the TIs granted in the current ETF and is called Periodic Grouped Polling MPDU (PGPM) in the following.

In addition, the RRM may include some TIs shared by sets of STA, on a contention access basis, to transmit signalling information. These TIs are called Contention TIs (CTI) in the following. The CTIs have a fixed length that is selected by the RRM and announced in the beacon and are only listen by the RRM. Successful accesses to CTIs are acknowledged by the RRM in the next PGPM.

#### MPDU description

##### MPDU header

The MPDU header contains information fields required for 802.11 compatibility, and the length of the following signalling part.

##### Signalling part

The signalling part is constituted of Signalling Information Elements (SIE). Each SIE is encoded on a TLV basis and may belong to several classes corresponding to the different control and data planes functions of the ECCF MAC. Each SIE indicates, explicitly or implicitly (according to their type), their destination STA. The MPDU header and the whole signalling part is protected by a unique Cyclic Redundancy Code (CRC) located at the end of the signalling part. The MPDU header and the signalling part use the same PHY mode that is retrieved from PHY layer information.

##### Data part

The data part is divided into one or more MLS Data Block(s) (MDB). Each MDB is described by an associated MAC-SIE located in the signalling part that specifies the destination STA, the MDB size and the PHY mode selected for this MDB.

Each MDB contains one or more MIS-PDU(s) intended for the same STA. An MIS-PDU contains a payload provided by the SAR sub-layer to which is pre-pended a header. A CRC is appended to the MIS-PDU to protect the header and payload.

### Medium access and transfer procedures

Figure 1-4: User Data Encapsulation in the Transmitter

#### Resource characterisation

The PHY resource is shared between all STAs by the RRM depending on the STA requirements and the PHY features.

PHY resources are managed according to priority levels attributed to the different MAC data flows. A MAC data flow is defined as the data exchanged between a given pair of source and destination STAs with a specific priority level. Priority levels are defined so that the QoS required by the applications can be provided. Resource allocation is then performed by the RRM according to the relative priorities of the different data flows. Priority levels are defined as per IEEE 802.1D standard (Annex H).

#### Resource Management

##### Overview

Prior to any signalling or data transmission, each STA has to request some PHY resource to the RRM. For that purpose a MAC-SIE-RR message is sent to the RRM using either a CTI or space in the signalling part within a TI already granted to the STA. The MAC-SIE-RR message includes the following information: the SID of the destination STA, the number of MIS-PDUs the source STA needs to transmit, and the priority level of the data to be transmitted. In addition, the requesting STA specifies in this message the PHY parameter set that will be use to transmit further data. One MAC-SIE-RR message has to be sent for each destination-STA / priority-level pair. Since the resource granularity used in the MAC-SIE-RR is too coarse for signalling message, a specific Resource Request for Signalling SIE (MAC-SIE-RRS) is provided that allows the request of resource for signalling usage only.

In response, the RRM grants TIs to the requesting STAs according to the available PHY resource and the contents of the RRs. The RRM dynamically determines the frame composition, i.e. the organisation of the granted TIs, which is announced through specific TI Descriptor SIEs (MAC-SIE-TID) in the PGPM. The RRM shall set the LastGr bit in a MAC-SIE-TID in order to indicate the source STA that the whole requested resource has been allocated, i.e. no more TI will be granted in subsequent ECCF. From that information, the STA is able to determine whether a new MAC-SIE-RR shall be sent to the RRM. Use of the granted TI resource and MPDU contents is determined by the sole source STA.

#### Transmitter operation

For each granted TI, the source STA organises the contents of the transmitted MPDU payload. As illustrated by Figure 1-4, SIEs are inserted first, followed by the MDBs sent to one or several destination STAs. Each MDB contains data intended for the same destination STA. MIS-PDUs of different priorities and different sizes may be inserted in any order within the same MDB. When the data part is not empty, the first SIE must be a Data Part Descriptor SIE (MAC-SIE-DPD) that describes the MDB format.

Order of SIEs within the signalling part, except for the MAC-SIE-DPD, obeys to no particular rule.

#### Receiver operation

##### Signalling decoding

As shown by Figure 1-5, each STA shall decode the MAC-SIE-TIDs contained in the PGPM and listen to the TIs it is the destination. Within each received MPDU, the destination STA shall decode the MPDU header in order to retrieve the signalling part length. It then interprets the MAC-SIE-DPD to extract the MDBs it is the destination. An STA shall decode all SIEs but interprets only the SIEs it is dedicated.

Figure 1-5: PGPM and MPDUs structure sample

#### Resource usage

As mentioned in section 1.2.2.2.1, the source STA is in charge of determining the composition of the MPDU inserted in its allocated TI. The length and the composition of the MPDU shall be determined so that its duration on the PHY medium is equal or shorter to the TI duration. Furthermore, the source STA shall take into account possible Radio Turn-Around times (RTA) required by the PHY layer (i.e. time required by the PHY layer to switch between receiving and transmitting states). The RTA shall be inserted at the end of the MPDU when the last MDB of the MPDU is intended for a given STA that is the transmitter STA in the next TI. It shall also be inserted when the MPDU contains only a signalling part intended for a given STA that is the transmitter STA in the next TI. In any case, every source STA shall be ready to transmit at the beginning of the TI, which induces it shall respect the RTA and leave the receiving state before the TI starts. Consequently, data transmitted by a non-RTA aware STA during the RTA may be lost.

Furthermore, as the RRM is not always able to precisely determine the TI length according to the source STA requirement, it can result that the allocated resource is not fully used (i.e. the MPDU size does not exactly match the TI duration) due to the MIS-PDU’s size granularity. However, the resource left unused can be exploited for other purposes such as PHY layer information insertions used for measurements, synchronisation and channel estimation, or redundancy information insertion.

### Broadcast function

An STA is able to broadcast a data flow by using the specific destination SID 255. The mechanism used for resource request and data transfer is similar to unicast data flows. The broadcast SID is used in the MAC-SIE-RR sent to the RRM and in the returned MAC-SIE-TID. It is also indicated in the MAC-SIE-DPD located in the MPDU containing the broadcast data. Any associated STA of the cell shall decode the broadcast MDBs. No ARQ mechanism is used to control errors affecting the broadcast data flows.

### Contention access procedure

In an ETF, the RRM can schedule some fixed length Contention TIs (CTIs) that can be used by STAs without any granted resource, to send signalling information to the RRM. Access protocol to this resource is based on a slotted ALOHA scheme. Each CTI contains a specific MPDU that includes a signalling part only. In the MPDU header, the Frame Control field indicates the type of the MPDU and the SigLen field is replaced by a SID field that contains the SID of the emitting STA. The length of the signalling and the PHY mode used are deduced from information given by the PHY layer.

As shown in Figure 1-6, the CTIs allocated by the RRM are described in the PGPM via a CTI Block SIE (MAC-SIE-CTB). Contention accesses are acknowledged by the RRM via a CTI Acknowledgement (MAC-SIE-CTA) located in the next emitted PGPM. When collision occurs, the STA can re-transmit the MPDU after a random backoff period as defined by the slotted ALOHA scheme.

CTIs are mainly used by STAs to trigger an ECCF association procedure, to request resource after a long idle or sleeping period, or to send short ARQ feedback message when unexpected error occurs. The type of signalling message can be introduced in the computation of the backoff period in order to accelerate re-transmission of critical messages and to increase the probability of a successful transmission.

Figure 1-6: Structure of MPDUs emitted inside CTI

## Coexistence with legacy 802.11

DCF STAs are supposed to set their NAV to the value specified in the beacon so that they remains in a listening only state until the end of the CFP period. PCF compatible devices expect a CF-Poll message in the CFP period. The RRM might implement functions to schedule PCF devices after the ETF.

We consider that the beacon is sufficient to prevent non ECCF STAs from accessing the CFP, only the first byte (Protocol Version, Type, Subtype fields) of the Frame Control field is actually required. It is the later option that is considered in the current proposition.

In addition, the PHY header provided by the 802.11 legacy physical layers is kept as is and used to determine the PHY mode the MPDU’s signalling part is encoded with.

The 802.11 beacon is modified via the addition of a specific IE called “ECCF Parameter Set”, and defined in Table 1-1. Greyed rows in the table indicate fields required by the legacy IEEE 802.11 standard.

| Length (octets) | Field Name |
| --- | --- |
| 1 | ElementID |
| 1 | Length |
| 2 | ETF duration (µs) |
| 1 | Number of ETFs in the current Super Frame. |
| 6 | Super-frame counter (used by CCM encryption) |
| 1 | CTI length (unit depends on PHY layer) |

Table 1-1: ECCF Parameter Set Field List

Nevertheless, in order to protect the ECCF protocol against eventual DCF accesses performed by any legacy STA that did not correctly decode the previous beacon, a CTS frame can be inserted by the RRM before the CTI block. This CTS control frame shall respect the legacy MAC standard and contains the 802.2 address of the RRM in the RA field and the CTI block duration in the Duration/ID field. Such a mechanism prevents a DCF STA to gain access to the medium during an apparent idle time period within a MTF frame and to interfere with another STA at the RRM antenna.

In the same way, every granted TI shall be protect by the source STA so that any DCF STA cannot gain access to the medium during a partially used TI. If the source STA has neither signalling nor data to send in the granted TI, a CTS frame can be inserted instead of the expected MPDU described above. In this case, the CTS frame contains the CTI block duration in the Duration/ID field. If the emitted MPDU is shorter than the TI, a padding Data Block can be added at the end of the MPDU. In any cases, the remaining idle period shall remain lower than the DIFS (or AIFS for 802.11e). This padding part is indicated through the MAC-SIE-PADDING located in the MPDU signalling part.

Figure 1-7: ECCF Time Frame.

## Convergence Sub-layers

### LLCCS (LLC Convergence SubLayer)

The LLC Convergence Sub-layer plays three roles: it maintains addressing compatibility with IEEE 802.2, provides a mean to map application QoS requirements onto services provided by the MIS sub-layer and finally, performs a subset of the LLCCS-PDU buffer management function.

#### IEEE 802.2 addressing compatibility

MAC-SAP primitives use IEEE 802.2 addresses. When a cell is attached to another network through an Access Point, the later shall act as a level 2 bridge (i.e. a switch/hub in the Ethernet terminology) and shall remain transparent for the LLC layer. Consequently, each STA shall have a 802.2 MAC address that is visible from all STAs whatever the network they belong to.

#### QoS information

- QoS information is handled by the LLCCS: a set of priority levels that are common to all data flows transiting in the cell. This indication is associated with every LLCCS-PDU. It is assumed that the applications with QoS constraints are able to provide this information through the LLC layer (802.2q for example). Alternatively, the priority can be set automatically by each station based on flow information such as the protocol (UDP, TCP), or any other relevant parameter.

- Priority information is passed to the MIS through the SAR sub-layer.

### SAR (Segmentation And Re-assembly)

In the transmit direction the SAR sub-layer fragments LLCCS-PDUs into segments of fixed length. Each segment is associated a Segment Sequence Number (SSN) that is passed to the MIS sub-layer.

In the receive direction, the SAR sub-layer uses the LSN and SSN information passed by the MIS sub-layer to reconstruct LLCCS-PDUs.

The SAR may handle SAR-PDUs of two different lengths depending on the chosen segmentation scheme:

- only short SAR-PDUs fitting into S-MIS-PDUs

- only long SAR-PDUs fitting into L-MIS-PDUs

- long SAR-PDUs for all but the last segment when the remaining LLCCS-PDU part is shorter than the length of a short SAR-PDU.

The usage of long SAR-PDUs decreases the overall MAC protocol overhead since SAR-PDUs are further encapsulated in MIS-PDUs. In addition, the last strategy reduces the ratio of unused resource that would be otherwise due to the use of long MIS-PDUs only.

## Error and Flow Control

### General

The MIS sub-layer implements the Error and Flow Control (EFC) functions. The Error Control (EC) is based on a selective repeat ARQ scheme. The EC function is responsible for:

- generating and checking of the CRC for MIS-PDUs

- detecting of missing MIS-PDUs

- re-transmitting of missing and corrupted MIS-PDUs so that the SAR can deliver complete LLCCS-PDUs.

- generating and analysing ARQ feedback messages.

- discarding LLCCS-PDUs

The Flow Control (FC) function is in charge of:

- generating flow control messages

- stopping/restarting the traffic emission

The EFC functions are performed on a per priority and per STA basis. In the following, a MAC data flow is identified by the source STA, the destination STA and a priority level. EFC is implicitly enabled on all MAC data flows except the broadcast ones.

### Sequence numbers

The sequence numbers are independently assigned for each MAC data flow.

Each MIS-PDU has an assigned LLCCS-PDU Sequence Number (LSN) and a Segment Sequence Number (SSN), both attributed by the Convergence sub-layers. The LSN is incremented by 1 each time an LLCCS-PDU is emitted and is calculated modulo 210. Within each LLCCS-PDU, the SSN of the first MIS-PDU is equal to 0 and then incremented by 1 for the subsequent MIS-PDUs. SSN is 5-bits long allowing LLCCS-PDU of length up to 4096 bytes.

### ARQ window

ARQ is not responsible for the buffer management. However, buffers used by the MIS and SAR sub-layers are shared for retransmission and can be controlled by using the Flow Control function.

In the transmitter and the receiver, a pool of buffers, called ARQ window, is used for each MAC data flow. In the transmitter, the ARQ window contains a copy of the transmitted LLCCS-PDUs so that they can be repeated upon error detection in the receiver. The size of this pool of buffers is dynamically evaluated and depends on the throughput of the data flow, the granted amount of resource, the error rate and the amount of available buffers in the transmitter.

In the receiver, this ARQ window contains the LLCCS-PDUs that have not been delivered to the LLCCS sub-layer by the SAR sub-layer (i.e. any LLCCS-PDU with missing MIS-PDU). The Flow Control function can be used to avoid an ARQ buffer starvation in the receiver.

For QoS data flows, an RLC procedure can be engaged between the transmitter and the receiver that reserves a minimum ARQ window in the receiver in order to guarantee a required data rate.

The maximum size of an ARQ window used by a MAC data flow is half the size of the LSN space (29) to prevent ambiguities in the interpretation of the sequence numbers.

### ARQ protocol messages

The received MIS-PDUs are cumulatively or selectively acknowledged by the ARQ receiver by using EC-SIE-FB messages. Cumulative acknowledgement is used to positively acknowledge consecutive correctly received LLCCS-PDUs. It indicates the lowest LSN among the remaining incomplete LLCCS-PDUs set. When no error occurs on received MIS-PDU, this mechanism is sufficient to fulfil the EC function.

Selective acknowledgement is introduced when corrupted or missing MIS-PDUs are detected within LLCCS-PDUs. For that purpose, the EC-SIE-FB message contains a Bit Map (BM) associated to a particular LLCCS-PDU that indicates the reception status of its constituting MIS-PDUs. A bit of rank n in the BMB reflects the reception status of the MIS-PDU which SSN equals n in the given LLCCS-PDU. When a bit is set, the corresponding MIS-PDU is positively acknowledged. Otherwise, it is negatively acknowledged, that is the corresponding MIS-PDU is corrupted or missing.

LSN and associated BM are encapsulated into an Acknowledgement Vector (AKV).

A Request For Feedback SIE (EC-SIE-RFB) can be sent by the transmitter to request the reception status of given LLCCS-PDUs.

The transmitter can discard all LLCCS-PDUs up to a given LSN value by sending an EC-SIE-DCD signalling message to the ARQ receiver.

Resource allocation

Figure 1-8: Flows of signalling messages between STAs.

The ARQ transmitter shall request resource for MIS-PDU transmission and retransmission by sending MAC-SIE-RR message to the RRM.

The ARQ receiver shall use any further allocated resource to send an EC-SIE-FB to the ARQ transmitter. As shown in Figure 1-8, for each MAC data flow characterised by the triplet (source STA, destination STA, priority), the RRM shall schedule a minimum resource for signalling usage: RR in forward direction and EC-SIE-FB in backward direction. STAs that do not have sufficient resource to send feedback messages may request more resource by sending a MAC-SIE-RR or a specific RR for Signalling (MAC-SIE-RRS) to the RRM.

### Transmitter operations

For each MIS-SDU provided by the SAR sub-layer, the LSN, SSN and priority level are inserted in the MIS-PDU header. A CRC-24 is calculated over the entire MIS-PDU header and payload and is appended to the packet in the MISCS field.

The transmitter can send a maximum number of consecutive LLCCS-PDUs without receiving any acknowledgement. This number is equal to half the LSN upper bound (29) for all data flows.

Upon reception of a MAC-SIE-FB with the FlowControl bit set, the transmitter, for the considered Data Flow, can continue emitting MIS-PDUs that belong to LLCCS-PDUs which transmissions is in progress. Once in this state, the ARQ transmitter stops emitting MIS-PDUs belonging to further LLCS-PDUs but can perform retransmission of MIS-PDUs. The transmitter resumes new LLCS-PDUs transmission upon reception of a MAC-SIE-FB message with the FlowControl bit cleared.

No particular rule is applied in the ARQ transmitter to choose the next MIS-PDU to be transmitted.

The transmitter can request the reception status of particular LLCCS-PDUs by sending an EC-SIE-RFB message to the receiver. This procedure is useful when the transmitter is lacking feedback information.

When the transmitter triggers a discard procedure, it shall first emit an EC-SIE-DCD discard message to the ARQ receiver. Then, it shall wait for a feedback message that acknowledges at least the LLCCS-PDU specified in the discard message. Once the feedback message received, the transmitter can release the corresponding ARQ buffers.

### Receiver operations

The receiver shall decode and check each received MIS-PDU by using the MISCS field. If the control  fails, the MIS-PDU is rejected and considered as corrupted. Otherwise, the receiver checks the LSN and priority consistency, discards the MIS-PDU if invalid or delivers it to the SAR sub-layer.

The receiver indicates the status of its ARQ window by sending EC-SIE-FB message to the transmitter. It informs the transmitter of missing or corrupted MIS-PDUs by adding Acknowledgement Vectors (AKV), each of which containing the LSN of an incomplete LLCCS-PDU and optionally, the BM that describes the reception status of that LLCCS-PDU’s constituting MIS-PDUs. Particular LLCCS-PDUs can be signalled by setting flags so that feedback information can be compacted. The LLCCS-PDU of lowest LSN that contains at least one missing MIS-PDU (bottom of ARQ window) is identified by the FIRST_CORRUPTED flag. The FIRST_RECEIVED flag indicates that the signalled LLCCS-PDU is the LLCCS-PDU of lowest LSN that contains at least one correct MIS-PDU. The LAST_RECEIVED flag indicates that the signalled LLCCS-PDU is the LLCCS-PDU of highest LSN that contains at least one correct MIS-PDU.

If the allocated resource does not allow sending all the required AKVs, an arbitration between data flows has to be performed by the receiver.

If the number of receive ARQ buffers becomes insufficient, the receiver can set the FlowControl bit in the EC-SIE-FB message so that the transmitter stops emitting MIS-PDU that belongs to new LLCCS-PDUs.

On reception of an EC-SIE-RFB message, the receiver shall send an EC-SIE-FB including the AKVs corresponding to the requested LLCCS-PDU status.

On reception of a discard message, the ARQ receiver shall deliver to the LLCCS sub-layer all complete LLCCS-PDUs which LSN is strictly lower than the LSN specified in the discard message. It shall send a cumulative acknowledgement that at least includes this LSN. Incomplete LLCCS-PDUs covered by the cumulative acknowledgement are discarded.

## QoS Support

QoS support is provided through several mechanisms distributed over the different MAC sub-layers: PHY resource and  a variable EFC effort based on priorities.

LLCCS enables QoS by associating a priority level to each received LLC-PDU according to the application data flow it belongs to.

## Power Saving

STA with power saving constraints (PS-STA) shall indicate their capabilities to the RRM during the association procedure. In particular, they indicate the duration of their sleeping period as a number of MSFs. The RRM schedules their data flows in the earliest possible ETF of the MSF.

When no more traffic is scheduled for a PS-STA (source or destination STA), the RRM gives to it the authorisation to sleep by sending a MAC-SIE-SLP message in the PGPM. During the sleeping period, the RRM does not schedule any TI for the sleeping STAs. Consequently, all traffic intended for a sleeping STA shall be buffered in the source STAs. MAC-SIE-RRs are sent by source STAs to the RRM using the usual procedure (source STAs does not necessarily know that the destination STA is sleeping) but the RRM delays the resource grants until the end of the sleeping period.

A sleeping STA must wake up so that it is able to receive and decode the beacon that follows the end of the sleeping period. However, a STA that needs to wake up before the end of the sleeping period can communicate with the RRM by attempting access to a CTI. On reception of the message, the RRM considers the STA as active and can schedule some TIs.

In the example described by Figure 1-9, the RRM manages four PS-STAs with a sleeping period of one MSF. The traffic of these PS-STAs is scheduled in the first ETFs of the MSF. PS-STA #2 can sleep from ETF#1 while other PS-STAs have traffic scheduled in the ETF#1. RRM indicates in the next ETF that PS-STA#1, #3 and #4 can sleep.

Figure 1-9: Power Saving procedure example

## Dual-channel operations (40MHz support)

The ECCF frame can be built in order to support dual-channel operations. Both channels are considered as independent and are called primary and secondary channels. A single RRM is able to control both channels and simultaneously emits the Beacon frames with the same period. In each super-frame, the number of MTF can differ between both channels. The content of each PGPM emitted at the beginning of each MTF also differs.

Stations that support only single channel operations can be associated on either the primary or the secondary channel. All subsequent transmission or reception will be done on the associated channel: Allocated TIs are announced in the PGPM emitted on the channel the station is associated, and are valid only on this channel. Consequently, 20MHz stations need to track only the channel on which they are associated.

Stations that support dual-channel operations can be associated on either the primary or the secondary channel. Allocated TIs are announced in the PGPM emitted on the channel the station is associated, and are valid for both channels.

In order to distribute stations between the primary and secondary channels, it might be useful to modify the association procedure so that the RRM can move the stations from one channel to the second one.

In the following, it is considered a 40MHz station is able to transmit two independent 20MHz PHY bursts on both channels, each of one being intended for a different station.

However, a station cannot receive data from one channel while transmitting data on the other one. In addition; a station is not able to simultaneously receive two different bursts on both channels. Consequently, the frame building must avoid allocating two overlapping TIs on both channel with two different transmitters and a single receiver. These restrictions may lead to waste some resource in particular during pure centralised-mode operations.

The following sample illustrates the MTF composition in order to support 20MHz/40MHz communications by respecting the rules detailed above.

We suppose STA#1 and STA#3 are 20MHz stations associated on the primary channel, while STA#2 and STA#4 are 20MHz stations associated on the secondary channel. STA#5 is a 40MHz station associated on the primary channels. Consequently, STA#1, #3 and #5 listen for the PGPMs emitted on the primary channel, while STA#2 and #4 listen for the PGPMs emitted on the secondary channel.

We decide to build the MTF so that traffic emitted by the RRM at 20MHz follows the PGPMs. The traffic emitted using both channels (i.e. using the 40MHz band) is grouped after. At last, bursts emitted by the different stations using a single channel are organised in order to limit resource waste. Some optimisations can be performed by using advanced algorithm to determine the TI lengths. However, as STAs are associated to a fix channel and may have different requirements along the time, optimisations are not always possible.

Figure 1-10: Dual-channel operations

Such a mechanism does not add any complexity in the terminal. The frame building algorithm running in the RRM can be simple as proposed in this section, and allows to reduce the resource waste because of transmissions on a single channel.

## Association

The ECCF association procedure is used by new STAs to gain access to the ETF. A non- associated STA shall generate an Association Request SIE (RLC-SIE-ASR), that contains its 802.2 MAC address, via a CTI. Upon reception of this message, the RRM acknowledges the CTI through a MAC-SIE-CTA in the next PGPM. If the association is accepted, the RRM inserts an Association Acknowledgement SIE (RLC-SIE-ASA) in a later PGPM that indicates the 802.2 MAC address and the SID allocated to the STA. If the association is denied, the SID field of the RLC-SIE-ASA is forced to 0.

Once associated, the STA is able to request resources to the RRM and perform data transfers with other STAs.

An STA can disassociate by sending a Disassociation Request SIE (RLC-SIE-DSR) to the RRM it is associated to. The RRM acknowledges this request by emitting a Disassociation Acknowledgement SIE (RLC-SIE-DSA) in a next PGPM. Furthermore, the RRM can unilaterally initiate an STA disassociation by sending the RLC-SIE-DSA message.

## Security

### Authentication

Authentication is based on the procedure described in the 802.11i extension. Authentication messages are exchanged by using either the CP or the ECCF period. When ECCF is chosen, prior to any authentication message transfer, the STA must be associated so that the RRM can grant some resources. To avoid malicious resource usage, the RRM may limit the resource allocated for authentication until the procedure succeeds.

### Encryption

Encryption is performed in the MLS sub-layer on a per MDB basis.

The Data Block Vector located in the MAC-SIE-DPD of the signalling part indicates the encryption method used to encode the associated MDB. Encryption information specific to the encryption method is pre-pended and appended to the Secure Data Block (SDB).

For the CCM encryption method (also defined in 802.11i and 802.15.3), the SDB implementation described in Table 1-2is realised.

| 2 octets | N octets | 8 octets |
| --- | --- | --- |
| SecurityID | Data Block payload | Message Integrity Code (MIC) |

Table 1-2: Secure Data Block Encapsulation

The nonce value used for CCM encryption shall be a 13-octets parameter calculated by concatenating the values listed in Table 1-3.

| Length | 6 octets | 1 octets | 1 octets | 2 octets | 3 octets |
| --- | --- | --- | --- | --- | --- |
| Location | Beacon | MAC-SIE-TID | MAC-SIE-DPD | MAC-SIE-TID | MPDU Signalling part |
| Description | A Super-frame counter. | Source SID. | Destination SID | TI position in the ECCF frame. | HSCS field (CRC-24). |

Table 1-3: Parameters Used for the Nonce Value Calculation

Key exchanges required by the encryption method are performed by using dedicated SIEs (TBD).

# Packet formats

## LLCCS-PDU

|  | LLCCS-PDU Header |  |  |  |  |  | Payload |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Length (bits) | 2 | 48 | 48 | 48 | 12 | 2 | 8*Length |
| Field name | NoA | Address1 (optional) | Address2 (optional) | Address3 (optional) | Length | ToP | Data Payload |

The Length field contains the length of the Data Payload expressed in bytes. The NoA field (Number of Addresses) determines the number of subsequent address fields located in the LLCCS-PDU header. At last, the ToP field encodes the Type of Payload as mentioned below:

Type 0: data payload contains an LLC packet.

Type 1: data payload contains an IP packet: the Ethernet and LLC-SNAP headers are removed.

Type 2: data payload contains an Ethernet II packet (no LLC-SNAP header)

Type 3: reserved .

The NoA and ToP fields can be advantageously combined to limit the overhead added by LLC-SNAP layer.

IP packets can be directly embedded into the LLCCS-PDU (ToP=1). For packets emitted from or toward a device located outside the cell, only Address1 is required. It contains the 802.2 MAC address of the external device. For packets that are transmitted between two stations associated to a single RRM, no address is required. In this case, the Length field contains the length of IP packet.

ToP 0 is provided in order to support LLC-SNAP capabilities (tunnelling) and network layer protocols different from IP. The same rules shall be applied concerning the usage of Address fields. ToP 0 is the only one that allows to strictly respect the IEEE 802 protocol stack.

Top 2 is provided in order to support simple bridging between Ethernet and 802.11. In this case, no address is required in the LLCCS-PDU header.

The following table summarises the total overhead generated by LLCCS and LLC-SNAP/Ethernet II for the different ToP.

| ToP | Through Gateway | Direct |
| --- | --- | --- |
| 0 (LLC) | 8+8(LLC-SNAP) | 2+8(LLC-SNAP) |
| 1 (IP) | 8 | 2 |
| 2 (Ethernet II) | 2+14(Ethernet II) | 2+14(Ethernet II) |

## MIS-PDU

Two types of MIS-PDU are defined that share the same structure as described below.

| Length (octets) | 3 | nP | 3 |
| --- | --- | --- | --- |
| Fields | MIS-PDU Header | Data Payload | MISCS |

The format of the MIS-PDU header is the following.

|  | MIS-PDU Header |  |  |  |
| --- | --- | --- | --- | --- |
| Length (bits) | 10 | 6 | 8 |  |
| Fields | LCP-SN | MP-SN | PRIORITY | ECINFO |

In the MIS-PDU header, the LCP-SN field contains the LSN of the LLCCS-PDU while the MP-SN field contains the SSN of the MIS-PDU within the LLCCS-PDU. The PRIORITY field indicates the priority level of the data flow the MIS-PDU belongs to. ECINFO contains information relative to EC.

PRIORITY and ECINFO data types are detailed in section 2.5.4.

The size nP (in octets) of the Data Payload depends on the type of the MIS-PDU. Whatever the type of the MIS-PDU is, the MIS-PDU is protected by a CRC-24 embedded in the MIS-PDU Check Sequence (MISCS) field located at the end of the MIS-PDU.

### Long MIS-PDU

The Data Payload part has a length of 128 octets and the size of the Long MIS-PDU is 134 octets.

### Short MIS-PDU

The Data Payload part has a length of 61 octets and the size of the Short MIS-PDU is 67 octets.

### Segmentation samples

Table 2-4 summarises the size of protocol headers for the IEEE 802.2/802.3 encapsulation (RFC 1042) and the Ethernet encapsulation (RFC 894).

| Header size | 802.2/802.3 | Ethernet |
| --- | --- | --- |
| TCP Header (no data) | 20 | 20 |
| IP Header | 20 | 20 |
| 802.2 LLC+SNAP | 8 | _ |
| 802.3 MAC | 14+4 | 14+4 |
| Total | 66 | 58 (64) |

Table 2-4: Header size of common protocols

#### TCP Acknowledgement

An IP packet carrying a TCP acknowledgement has a 40-byte length. Next table gives the segmentation results.

| LLCCS-PDU ToP | Header Size | LLCCS-PDU Size | #Short MIS-PDU | #Long MIS-PDU |
| --- | --- | --- | --- | --- |
| 0 (LLC) | 2(8)+8 | 50(56) | 1 | 0 |
| 1 (IP) | 2(8) | 42(48) | 1 | 0 |
| 2 (EthII) | 2+14 | 56 | 1 | 0 |

#### ARP/RARP message

A LLC-PDU carrying a ARP/RARP message has a 46-byte length since the 802.3 MAC header and trailer are removed. Next table gives the segmentation results.

| LLCCS-PDU Type | Header Size | LLCCS-PDU Size | #Short MIS-PDU | #Long MIS-PDU |
| --- | --- | --- | --- | --- |
| 0 (LLC) | 2(8)+8 | 48(54) | 1 | 0 |
| 1 | _ | _ | _ | _ |
| 2 | 2+14 | 62 | 0 | 1 |

#### Ethernet Data Frame of maximum size

A LLC-PDU carrying an Ethernet (resp. 802.2) Data Frame with a 1500-byte (resp. 1492-byte with 802.2 LLC-SNAP) payload has a 1514-byte maximum length. Whatever the LLCCS-PDU ToP is, the segmentation generates 12 Long-MIS-PDUs.

#### VoIP packet (as defined in IEEE usage models)

IP packet conveying VoIP has a 120-octet length.

| LLCCS-PDU ToP | Header Size | LLCCS-PDU Size | #Short MIS-PDU | #Long MIS-PDU |
| --- | --- | --- | --- | --- |
| 0 (LLC) | 2(8)+8 | 130(136) | 1 | 1 |
| 1 (IP) | 2(8) | 122(128) | 0 | 1 |
| 2 (EthII) | 2+14 | 136 | 1 | 1 |

## MPDU

| Length (octets) | 2 |  | nS |  |  | 3 | nDB1 | nDB2 | nDBn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part | MPDU Header |  | Signalling |  |  |  | User Data |  |  |
| Fields | Frame Control | SigLen/ SID | SIE #1 | SIE #2 | SIE #m | HSCS | MDB #1 | MDB #2 | MDB #n |

The MPDU header contains a Frame Control field kept for compatibility with the legacy 802.11 (see table below), and the SigLen field that defines the length of the following Signalling part of the MPDU. The unit used for SigLen depends on the PHY layer and is similar to the unit used for the ResNum field in the MAC-SIE-TID. For an OFDM TDMA PHY layer, the unit can be the OFDM symbol.

For an MPDU inserted in a CTI, the SigLen field is replaced by the SID of the source STA.

|  | Frame Control |  |  |
| --- | --- | --- | --- |
| Length (bits) | 2 | 2 | 4 |
| Fields | Protocol Version | Frame Type | Frame Sub-Type |

The Signalling part of the MPDU includes one or more SIEs and the HSCS (Header and Signalling Check Sequence) field containing a CRC-24 which protects both the MPDU header and the Signalling part. This part of the MPDU is encoded with the same PHY mode. Protocol information relative to the PHY mode used for signalling is managed by the PHY layer. For the 802.11a PHY layer, the RATE field of the PLCP header may be used to indicate to the receiver the PHY mode used for the encoding of the signalling part.

The User Data part of the MPDU contains MDBs. Each MDB is destined to a particular STA and is encoded with a specific and constant PHY mode. The User Data part may eventually be empty. The composition of the User Data part is defined by the first SIE of the Signalling part, in the form of a MAC-SIE-TID. MDB are replaced by Secure Data Blocks (SDB) when encryption is enabled.

## SIE (Signalling Information Element)

Signalling information is transported into variable length entities named Signalling Information Elements (SIE).

### General Format

| Field name | Length | Description |
| --- | --- | --- |
| SIE_Type | 8 bits | Type of SIE. |
| SIE-Length | 8 bits | Length of the SIE Data Part in octets. |
| SIE Data | 1-256 octets | Data part |

### MAC SIEs

#### MAC-SIE-TID (TI Descriptor)

Grants to a transmitter STA some resources (TI) in the current frame that are used to transmit user data towards one or more specified receiver STAs.

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Tx | SID | 1 | SID of the source STA. |
| ResLoc | RESOURCE_LOCATION | 1 | Location of the resource in the frame |
| ResNum | RESOURCE_NUMBER | 1 | Amount of allocated resource |
| LastGr | BIT_1 | 1 | Flag indicating that all requested resource has been granted by the RRM. |
| PHYType | BIT_2 | 1 | Indicate the type of PHY burst used in the TI. By this field, the RRM indicates to the receiver the length of the preamble and the number of antenna used by the MIMO transmitter. |
| STA_Rx | SID | 1-n | SID of the destination STAs. |

#### MAC-SIE-CTB (CTI Block)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| ResLoc | RESOURCE_LOCATION | 1 | Location of the resource in the frame. |
| ResNum | BIT_4 | 1 | Number of CTIs. |

By sending this SIE in the PGPM, the RRM allocates a block of consecutive CTI in the current ECCF time frame. The maximum number of CTIs in a block is 16. Only one block can be allocated by the RRM in one ECCF time frame.

#### MAC-SIE-CTA (CTI block Acknowledgement)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| BMB | BIT_16 | 1 | Bitmap block |

By sending this SIE in the PGPM, the RRM acknowledges the MPDUs emitted in the CTI block that was allocated in the previous ECCF time frame.

#### MAC-SIE-DPD (Data Part Descriptor)

Describe the composition of the user data part of the MPDU.

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| DBV | DBV | 1-n | Data Block Vector |
| Padding |  | 1 | Padding to reach an octet boundary |

Data Block Vector

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Rx | SID | 1 | SID of the destination STAs. |
| SMPNum | BIT_9 | 1 | Number of Short MIS-PDU in the MDB |
| PhyMode | PHY_MODE | 1 | Physical Mode used to encode the MDB. |
| Security | BIT_2 | 1 | Specify the encryption mode used for the MDB. |

#### MAC-SIE-RR (Resource Request)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| RR | MAC_SIE_RRV | 1-n | Resource Request Vector |
| Padding |  | 1 | Padding to reach an octet boundary |

The MAC-SIE-RR is sent to the RRM by an STA that needs to emit data and or signalling toward one or several destination STAs including the RRM. A separate RR Vector is used per destination STA and data flow priority.

RR Vector (MAC_SIE_RRV)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Rx | SID | 1 | SID of the destination STA. |
| Priority | PRIORITY | 1 | Priority associated to the requested MIS-PDUs. |
| PhyParam | PHY_PARAM_SET | 1 | Required PHY parameters deduced from information returned by the receiver through the RRC_SIE_PHYPARAMREPORT message. |
| SMPReqNum | BIT_9 | 1 | Number of requested resources expressed as a number of Short MIS-PDU. |

#### MAC-SIE-RRS (Resource Request for Signalling)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| RRS | MAC_SIE_RRSV | 1-n | Resource Request for Signalling Vector |
| Padding |  | 1 | Padding to reach an octet boundary |

The MAC-SIE-RRS is sent to the RRM by an STA that needs to emit signalling toward one or several destination STAs including the RRM. A separate RR Vector is sent for each destination STA.

RR For Signalling Vector (MAC_SIE_RRSV)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Rx | SID | 1 | SID of the STA that requires some resource. It can be different from the STA that emits the MAC-SIE-RSS. |
| ResNum | BIT_8 | 1 | Number of requested resource (unit is identical to the RESOURCE_NUMBER data type). |

This message is useful to get a finer granularity than the one providing by the MAC-SIE-RR message.

#### MAC-SIE-SLP (Sleep)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Slp | SID | 1-n | SID of the STAs to be put to sleep. |

By emitting this SIE, the RRM indicates to the STAs listed in the message that they can enter a sleeping period until the next beacon. During this period, the RRM does not schedule any TI for these STAs.

#### MAC-SIE-PAD

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| PadLen | BIT_9 | 1 | Padding length (unit is identical to the RESOURCE_NUMBER data type). |

This SIE indicates that a padding pattern has been added at the end of the MPDU by the MAC layer so that the duration of the MPDU inserted in the TI is at least greater than TI – DIFS (or AIFS).

### EC SIEs

#### EC-SIE-FB (ARQ Feedback)

EC-SIE-FB is sent by the ARQ receiver STA to the ARQ transmitter STA to acknowledge the received packets. When error occurs, the ARQ receiver STA may request the retransmission of corrupted MIS-PDU by using one or several AcKnowledgment Vector (AKV) appended to the EC-SIE-FB. The receiver is able to indicate to the transmitter the number of AKV that remains to be emitted in further EC-SIE-FB by filling the ReqMoreAKV field. By using this information, the transmitter can perform some resource requests to the RRM instead of the receiver so that this latter can emit EC-SIE-FB messages.

Furthermore, this SIE can be used by the ARQ receiver STA to perform a Flow Control procedure when there is a lack of reception buffer.

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Tx | SID | 1 | SID of the ARQ transmitter STA. |
| Priority | PRIORITY | 1 | Priority associated to the acknowledged MIS-PDUs |
| FlowControl | BIT_1 | 1 | Flow Control Bit; when set, the ARQ transmitter can only retransmit some previously sent MIS-PDUs. |
| ReqMoreAKV | BIT_6 | 1 | Number of additional AKV required to describe the whole receiver ARQ window. |
| LCP-AKV | EC_SIE_AKV | 0-n | List of Acknowledgement Vectors. |

For each LLCCS PDU that contains at least one corrupted MIS-PDU, an Acknowledgement Vector is appended to the EC-SIE-FB. It contains a bitmap block that describes which MIS-PDUs are corrupted.

Acknowledgement Vector (EC_SIE_AKV)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| SN | LCP-SN | 1 | Sequence number of the LLCCS-PDU described by the BMB. |
| BMB_PROVIDED | BIT_1 | 1 | Indicates if the associated Bitmap Block (BMB) is appended at the end of the vector. |
| FIRST_CORRUPTED | BIT_1 | 1 | Indicates if the LLCCS-PDU is the first received packet that contains corrupted MIS-PDUs. |
| FIRST_RECEIVED | BIT_1 | 1 | Indicates if the LLCCS-PDU is the first correctly received packet. |
| LAST_RECEIVED | BIT_1 | 1 | Indicates if the LLCCS-PDU is the last correctly received packet. |
| BMB | LCP-BMB | 1 | BitMap block that indicates which MIS-PDUs are corrupted within the LLCCS-PDU. |

To perform a simple cumulative acknowledgement ( indicate the Bottom Of Window), only one AKV is required: the FIRST_CORRUPTED bit is set and the Sequence Number of the first corrupted LLCCS-PDU is specified in the SN field. Other bits (BMB_PROVIDED and LAST_RECEIVED) are reset and the BMB is not provided.

To perform a simple flow control operation, the collection of AKV may be empty.

When some errors occur on the received MIS-PDUs, an AKV is appended to the EC-SIE-FB for each LLCCS-PDU that contains at least one corrupted MIS-PDU. It shall contain a bitmap block that describes which MIS-PDUs are corrupted (BMB_ PROVIDED bit is set).

When the FIRST_RECEIVED bit is set, it indicates that all MIS-PDUs belonging to LLCCS-PDUs between the Bottom of Window and the specified LLCCS-PDU are corrupted or missing.

When the LAST_RECEIVED bit is set, it indicates that all MIS-PDUs belonging to LLCCS-PDUs after the specified LLCCS-PDU are missing.

These bits can be combined to efficiently signal the receiver window status.

When the FIRST_CORRUPTED and LAST_RECEIVED bits are both set, it indicates that no packets were received after the specified LLCCS-PDU at Bottom of window. That infers that the receiver ARQ window contains only one or more MIS-PDUs of the specified LLCCS-PDU, or is empty.

#### EC-SIE-RFB (ARQ Request for FeedBack)

By sending this message, the ARQ transmitter requests an ARQ FeedBack message (EC-SIE-FB) to the ARQ receiver. It can explicitly ask for the status of particular LLCCS-PDUs.

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Rx | SID | 1 | SID of the ARQ receiver STA. |
| Priority | PRIORITY | 1 | Priority associated to the acknowledged MIS-PDUs |
| HLCP | LCP-SN | 1 | Highest LLCCS-PDU sequence number sent by the ARQ transmitter in the current ETF. |
| LSN | LCP-SN | 0-n | Sequence number of the LLCCS-PDU which reception status is required by the transmitter. |

#### EC-SIE-DCD (Discard)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Tx | SID | 1 | SID of the ARQ transmitter STA. |
| Priority | PRIORITY | 1 | Priority associated to the discarded MIS-PDUs |
| LSN | LCP-SN | 1 | Sequence number of the highest discarded LLCCS-PDU. |

By sending this message, the ARQ transmitter requests for discard of some LLCCS-PDU up to a given LLCCS-PDU sequence number included.

### RRC SIEs

#### RRC-SIE-PHYPARAMREPORT

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| PPRV | RRC_SIE_PPRV | 1-n | Phy Parameter Report Vector. |

Physical Parameter Report Vector (RRC_PPRV)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| STA_Tx | SID | 1 | SID of the source STA. |
| PhyParam | PHY_PARAM_SET | 1 | Proposed PHY parameters. |

The RRC-SIE-PHYPARAMREPORT is sent to the specified STA (STA_Tx) by a destination STA to indicate which PHY Parameter Set should be used by the specified source STA (STA_Tx). The proposed PHY mode is reflected in further MAC-SIE-RR, emitted by the source STA, that conveys a resource request for data flows intended for the destination STA.

### RLC SIEs

#### RLC-SIE-ASR (AsSociation Request)

A new STA shall send this request to the RRM so that it can access the ECCF functions provided by the RRM. By doing this request, the STA implicitly indicates that it supports ECCF functions.

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| Address | IEEE8022_ADDR | 1 | 802.2 address of the STA. |
| PowerSaving | BIT_1 | 1 | Indicates that the terminal requires power saving. |
| PSPeriod | BIT_4 | 1 | Indicates the sleeping period duration required by the terminal expressed as a fixed number of super-frame. |
| NbTxAntenna | BIT_2 | 1 | Number of antenna usable for transmission 00=1;11=2;10=3;11=4 |
| NbRxAntenna | BIT_2 | 1 | Number of antenna usable for reception 00=1;11=2;10=3;11=4 |

#### RLC-SIE-ASA (AsSociation Acknowledgment)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| Address | IEEE8022_ADDR | 1 | 802.2 address of the STA. |
| SID | SID | 1 | STA ID dedicated to the STA. |
| PowerSaving | BIT_1 | 1 | RRM indicates to the terminal that power saving procedure will be applied to schedule its data flows. |

The SID 0 is reserved for the RRM. However, the RRM can return this value to indicate to the STA that the association has been rejected.

#### RLC-SIE-DSR (Dissociation Request)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| SID | SID | 1 | SID of the STA. |

#### RLC-SIE-DSA (Dissociation Acknowledgment)

| Field name | Field Type | Number of occurrences | Description |
| --- | --- | --- | --- |
| SID | SID | 1 | SID of the STA. |

## Data Type

In this section, we detail the data types of the fields previously described. For some fields, they depend on the PHY layer and their length and meanings are listed for each PHY layer.

### RESOURCE_LOCATION

| PHY layer | Length (bits) | Description |
| --- | --- | --- |
| 802.11a | 13 | Time unit expressed as a multiple of 400ns |

### RESOURCE_NUMBER

| PHY layer | Length (bits) | Description |
| --- | --- | --- |
| 802.11a | 9 | Number of OFDM symbols |

### PHY_PARAM_SET

| PHY layer | Length (bits) | Description |
| --- | --- | --- |
| 802.11a | 3 | PHY mode |

### Other Data Type

| Data Type | Length (bits) | Description |
| --- | --- | --- |
| BIT_n | n | Field encoded on n bits. |
| SID | 8 | STAtion Identifier |
| PRIORITY | 3 (2?) | Data flow priority |
| ECINFO | 5 (6?) | Specific information reserved for EC usage (Hybrid ARQ support, variable CRC coverage...). |
| LSN | 10 | LLCCS-PDU Sequence Number |
| SSN | 6 | Segment Sequence Number |
| LCP-BMB | 32 | LLCCS-PDU BitMap Block |
| IEEE8022_ADDRESS | 48 | IEEE 802.2 Address |

Annex 1: Abbreviations and acronyms

| AKV | Acknowledgement Vector |
| --- | --- |
| ARQ | Automatic Repeat reQuest |
| BM | Bit Map |
| CBC | Cipher-Block Chaining |
| CBC-MAC | CBC Message Authentication Code. |
| CCM | Counter mode with CBC-MAC |
| CFP | Contention Free Period |
| CP | Contention Period |
| CTI | Contention Time Interval |
| DCF | Distributed Coordination Function |
| EC | Error Control |
| EFC | Error and Flow Control |
| ECCF | Extended Centralised Coordination Function |
| ETF | ECCF Time Frame |
| LLCCS | Logical Layer Control Convergence Sub-layer |
| LSN | LLCCS Sequence Number |
| MAC | Medium Access Control |
| MDB | MLS Data Block |
| MIS | MAC Intermediate Sub-layer |
| MLS | MAC Lower Sub-layer |
| MPDU | MAC Packet Data Unit |
| MSF | MAC Super Frame |
| NAV | Network Allocation Vector |
| PDU | Protocol Data Unit |
| PGPM | Periodic Grouped Polling MPDU |
| PHY | PHYsical Layer |
| PS-STA | Power Saving Station |
| QoS | Quality of Service |
| RLC | Radio Link Control |
| RRC | Radio Resource Control |
| RRM | Radio Resource Manager |
| SAR | Segmentation and Re-Assembly |
| SID | Short STA Identifier |
| SDU | Service Data Unit |
| SSN | Segment Sequence Number |
| STA | Station |
| TI | Time Interval |
| TLV | Type-Length Value |
| LTT | LLCCS Translation Table |