IEEE 802.11
Wireless LANs

HNS Proposal for 802.11n Physical Layer: Disclosure Document

Date:	August 13, 2004

Authors:         M. Eroz, F. Sun, Y. Jiang, J. Song, K. Karimullah and L. Lee

Hughes Network Systems (DirecTV Groups) 
	11717 Exploration Lane, Germantown MD 20876
	301-601- 4278
	Fax: 
	fsun@hns.com, meroz@hns.com

Abstract

## This document is provided for the IEEE 802.11n attendees in compliance with the requirements under the document No.11-03-0858-06, “Call for Proposals”.  This document addresses only the Physical Layer aspect of the 802.11n High Throughput WLAN standard under development by IEEE.

# Coverage of Functional Requirements

Table 1 sumarizes the compliance of this proposal with the requirements document

Table 1.  Functional Requirements Compliance Summary

| Number | Name | Coverage (Yes/No) | Results Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes | See specification |
| R2 | HT rate supported in 20MHz channel | Yes | See specification |
| R3 | Supports 5GHz bands | Yes | See Note 1 |
| R4 | .11a backwards compatibility | Yes | See note 1 |
| R5 | .11g backwards compatibility | Yes | See note 1 |
| R6 | Control of support for legacy STA from .11n AP | System related – N/A |  |
| R7 | .11e QoS support | MAC related - N/A |  |
| R8 | Spectral Efficiency | Yes |  |
| R9 | Compliance to PAR | Yes | N/A |

- Simlation is performed in complex signal domain with Doppler per the channel model and typical PA model as specified for these frequencies. Specific 5 GHz or 2.4 GHz frequencies not used in simulation. Also 802.11a OFDM Standard (1999) transmission structure used in simulation. A modified preamble, only in the ‘short’ portion (i.e. first 8 usec duration) is used for preamble which is expected to be received satisfactorily by a legacy STA. Other Backward compatibility related MAC topics are not addressed.

# Additional Documents/ Disclosures

Table 2  Related Documents/ Contributions

| Number | Title | Document Number |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Physical Layer Approach for 802.11n |  | 11-04-0746-00-000n | HNS July presentation |  |
| 2. | HNS Physical Layer Proposal Technical Specification |  | 11-04-0941-00-000n | Technial Proposal Doc. |  |
| 3. | HNS Physical Layer Proposal Technical Presentation |  | 11-04-0942-00-000n | .ppt Tech Presentation |  |

# Compliance to PHYSICAL LAYER Comparison Criteria

CC51 : data rates supported

# CC42 Preambles

Preamble is an extension of the 11a (details see the technical specification).  Each antenna uses one set of the 12 subcarriers. It is fully backward compatible and the autocorrelation properties follows exactly those of 11a.

# CC51 channelization

The same as 11a. No changes made in terms of the adjacent channel spacing.

CC52 Spectral mask

The following two figures shows the spectral masks OFDM Signal 16-QAM after Non-linear Amplifier IBO = 3 dB and IBO=6 dB respectively.

spectral mask at IBO=3 dB

spectral mask at IBO=6 dB

CC59 AWGN Performance

CC67 Performance in non AWGN channels