IEEE P802.11 Task Group n (TGn)

High Throughput (HT) Wireless Local Area Networks (WLAN)

| Project | IEEE P802.11 Task Group n (TGn) for High Throughput (HT) Wireless Local Area Networks (WLANs) |  |
| --- | --- | --- |
| Title | Medium Access Control (MAC) Layer Partial Proposal |  |
| Date Submitted | 13 August, 2004 |  |
| Source | [Yukimasa NAGAI, et al.] [Mitsubishi – ITC(JAPAN)] [5-1-1 Ofuna, Kamakura Kanagawa, JAPAN, 2478501] | Voice: [+81-467-41-2488] Fax: [+81-467-41-2519] E-mail: [yuki-n@isl.melco.co.jp] |
| Re: | IEEE P802.11n High Throughput Wireless Local Area Networks -- Call For Proposals |  |
| Abstract |  |  |
| Purpose | Propose an adaptive distributed channel access (ADCA) MAC protocol and a simple hybrid coordinated channel access (SHCCA) MAC protocol to increase the MAC layer throughput above 100 Mbps in the IEEE 802.11 system. |  |
| Notice |  |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.11 TGn. |  |

1.	Introduction	3

2.	Adaptive Distributed Channel Access (ADCA)	6

2.1.	Basic Algorithm	6

2.2.	Adaptive Frame Aggregation and Bursting	7

2.3.	Adaptive Frame Distribution based on channel condition	8

2.4.	Copied Frame (ACK)	10

2.5.	Backward Compatibility and Implementation Issues	12

2.6.	Simulation Results	12

2.6.1.	MAC Throughput Gain using Adaptive Frame Aggregation and Bursting	13

2.6.2.	IEEE802.11n Usage Model and Simulation Results	14

3.	Simple Hybrid Coordinated Channel Access (SHCCA)	14

3.1.	Basic Algorithm	14

3.2.	Backward Compatibility and Implementation Issues	15

3.3.	Simulation Results	15

4.	Frame formats	15

4.1.	PLCP frame format	15

4.1.1.	Reserved field	15

4.1.2.	SERVICE field	16

4.2.	MAC frame format	16

5.	Conclusion	17

6.	References	18

7.	Appendix	19

7.1.	Simulation Results	19

7.1.1.	Scenario 1: Home	19

7.1.2.	Scenario 4: Large Enterprise	21

7.1.3.	Scenario 6: Hot spot	24

7.1.4.	Scenario 16: Point-to-Point Throughput Test	27

7.1.5.	Scenario 17: Point-to-Point HT Goodput Test	29

7.1.6.	Scenario 18: Point-to-Point Legacy Throughput Test	30

7.1.7.	Scenario 19: Point-to-Point Legacy Sharing Throughput Test	31

# Introduction

In legacy IEEE802.11a, although the maximum physical data rate is 54Mbps, the throughput at the medium access control (MAC) service access point (SAP) is less than 30Mbps when transmits 1500byte UDP data because of the MAC overhead using CSMA/CA. Therefore in some situation, the throughput to transmit broadband data like HDTV traffic, Video streaming is insufficient.

Additionally, there are many applications (e.g., HDTV, VoIP, Video streaming, of course data) for using IEEE802.11 wireless local area networks (WLAN) [1]because of diversification of applications at the Home or Office. These enhancing the QoS support at the MAC layer is becoming the standard at Task Group e(TGe)[2].

Task Group n(TGn) mainly intends to boost the network throughput over 100Mbps at the MAC SAP to meet the demand of future applications and environments.

However, even if increasing bandwidth at the physical layer and the bursting time to transmit data spends less time, the overhead of PLCP header, PLCP preamble, random backoff and interframe space(IFS) for carrier sense and bursting time to transmit acknowledge frame(ACK) are constant, therefore the legacy MAC protocol doesn’t meet enough throughput in accordance with increasing bandwidth at the physical layer. Moreover, many legacy IEEE802.11a system limit the maximum length as 1500byte, because 1500byte is maximum length at Wired LAN. The results shown in Table  1, Figure  1.

| Parameter | Value |
| --- | --- |
| SIFS | 16μs |
| DIFS | 34μs |
| Slot time | 9μs |
| PLCP preamble | 16μs |
| Signal | 4μs |
| CWmin | 15 |
| CWmax | 1023 |
| MAC Header | 28byte |
| ACK rate | 6Mbps |

Table  1: IEEE802.11a parameters

Figure  1: 802.11 CSAMA/CA Throughput Limit

In this document, we propose a new MAC architecture, which not only meets the 100Mbps throughput requirement at the top of the MAC SAP, but also maintain the affinity with legacy IEEE802.11, IEEE802.11a and IEEE802.11e.

As shown in Figure  2, our MAC protocol retains the temporal structure of superframe and the channel access alternates between a contention period (CP) and a contention-free period (CFP) to serve QoS for diversification of applications.

The super frame starts with a beacon broadcast by the AP. Then after point (coordination function ) interframe space (PIFS), if there are some QoS frames that need to satisfy delay and throughput during CFP, CF-Poll will broadcast by the AP. The AP terminates the contention-free period by broadcasting a CF-End message. Then, the contention period begins until next beacon broadcasting by the AP.

In our proposal, the contention-free period between first CF-Poll and CF-End is simple hybrid coordinated channel access (SHCCA), the contention period between after CF-end and next beacon broadcasting by the AP is Adaptive Distributed Channel Access (ADCA).

The SHCCA used in the contention-free period is a highly efficient centralized polling access protocol, which similar to a part of IEEE802.11e HCCA and PCF. So the SHCCA is backward compatible with both the HCCA mode of IEEE802.11e and the PCF mode of legacy IEEE02.11. Each STA requests the transmission opportunity (TXOP) for the AP based on their traffic. Then STA can be allocated TXOP from the AP if possible. Additionally, SHCCA use 1 ideas, namely adaptive frame aggregation to improve channel efficiency.

The ADCA used in the contention period is a highly efficient CSMA/CA based random access protocol, which is backward compatible with both the DCF mode of IEEE802.11 and EDCA mode of IEEE802.11e. The ADCA uses 2 main ideas, namely adaptive frame aggregation and bursting based on channel condition for high-rate STAs, to improve channel efficiency.

Figure  2: Super Frame Structure for TGn

In this document, we expand the MAC protocol architecture as shown Figure  3. The Enhanced MAC can control in addition to legacy IEEE802.11a, enable to control multiple channels. We don’t care that the multiple channels mean that using frequency multiple channels on channel bundling or using multiple antennas on MIMO, as shown Figure  4.

Figure  3: Proposed Protocol Architecture

Figure  4: Channel Structure definition for Channel Bundling and MIMO

The document is organized as follows. First, ADCA and SHCCA MAC mechanisms are explained in details in section 2 and 3. Then, the new messages used in our proposed MAC protocols will be defined in section 4. Finally, the document is concluded with section 5.

# Adaptive Distributed Channel Access (ADCA)

In this section, the adaptive distributed channel access protocol will be presented. The basic algorithm is fist introduced in 2.1. Then, more details on the adaptive frame aggregation and bursting are provided in section 2.2, and the frame distribution based on channel condition is explained in section 2.3, and the copied ACK mechanism is explained in section 2.4. The backward compatibility and Implementation issues are provided in section 2.5. Finally, the simulation results are presented in section 2.6.

## Basic Algorithm

The ADCA uses a combination with the Adaptive Frame Aggregation and the Bursting in order to reduce MAC over head and improve overall channel throughput. Additionally, it offers multiple access categories (ACs) to provide service differentiation. We use four arbitration inter frame space (AIFS) to provide four access categories, defined in IEEE802.11e[2].

Therefore, we propose the ADCA, which not only reduces MAC over head and improves overall channel throughput using Adaptive Frame Aggregation and Bursting, but also meets QoS using ACs that a part of IEEE802.11e.

## Adaptive Frame Aggregation and Bursting

The ADCA uses Frame Aggregation that aggregates multiple frames to transmit. In our proposal, the STA (AP) changes “maximum aggregated frame length” based on channel condition and control from AP. Therefore we provide the Adaptive Frame Aggregation that changes maximum aggregated frame length based on channel condition and the control from AP.

When the situation at some channel is changed by channel condition is changed and then the STA (AP) changes transmit rate, we decrease maximum aggregated frame length and enable to reduce the transmission time and channel occupation time. Additionally, we enable to reduce the effect of the fading.

The Adaptive Frame Aggregation Architecture in this document is shown in Figure  5.

Figure  5: Adaptive Frame Aggregation Architecture

The adaptive Frame Aggregation is consist of “Frame Aggregate Unit” that aggregates some frames to transmit as one long frame and “Frame De-Aggregate Unit” that drive one long frame into some frames and “Shared Memory”. Moreover, there is one feed back path called “Aggregate Information” that indicates channel information included channel condition and so on from physical layer via MAC core as shown in Figure  5.

The frame format is shown in section 4.

Then the ADCA uses Bursting to archive highly efficient MAC. As shown in Figure  6, DATA frame and ACK frame are transmitted over and over at interval of short inter frame space (SIFS) for several times after carrier sensing DIFS + Back off at first.

Figure  6: Bursting

When the channel condition is something wrong with fading and so on, the Total Bursting Time is decreased from channel information from physical layer or control from AP.

## Adaptive Frame Distribution based on channel condition

The ADCA can use multiple channels to transmit data. In our proposal, we provide “Frame Distribution Architecture” that distributes one frame for using channels with same bursting time and same OFDM symbols even if the transmit rates differ for channel by channel.

The Frame Distribution Architecture in this document is shown in Figure  7.

Figure  7: Frame Distribution Architecture

In Adaptive Frame Distribution, One long frame is distributed for each channel based on Data octets per OFDM symbol from the beginning of the frame (as written PPDU in physical layer), as shown in Figure  8. The NDBPS means “Data bit per OFDM symbol”. And the NDOPS means “Data octets per symbol”. At the beginning OFDM symbol, the distributed data length is 2byte less than subsequent OFDM symbol because of SERVICE field (2byte).

Figure  8: Frame Distribution Algorism

At the last OFDM symbol as shown in Figure  9, there are the cases that the channel OFDM symbols differ for channel by channel, if distributing one frame for channels unthinkingly. In our proposal, as shown in Figure  10, we add “MAC pad” that make same OFDM symbols.

MAC pad is one octet Null data and inserted by MAC in front of “Tail (6bit)+PHY pad(2bit)” to adjust OFDM symbols for channel by channel. The rest of the OFDM is filled in up by PHY pad.

Additionally, the presence or absence of MAC pad is indicated at the reserved bit in Signal Field or Service Field. The receiver detects MAC pad by reserved bit. In our proposal, the length field indicated in TXVECTOR doesn’t include MAC pad length, so the modulator should work based on MAC pad field using reserved bit as shown in Figure  15 and  Figure  16.

Figure  9: Different OFDM symbols case without MAC Pad

Figure  10: Same OFDM symbols case with MAC Pad

## Copied Frame (ACK)

In this section, the receiving ACK in the situation of using multiple IEEE802.11a channels are presented. In case when channel bundling is used, ACK frame are sent using multiple channels as shown in Figure  11.

In our proposal, as shown in section 2.3, one aggregated data frame is distributed for multiple channels. On the other hand, the contents of ACK frames and rate are same in every channel called “Copied ACK”. So, we don’t have to worry about OFDM symbols.

In “the normal transmission case” as shown in Figure  11, at first the STA(AP) transmits data frames using multiple channels, in this case 2 channels. The Receive STA(AP) check CRC and reply ACK frames using two channels.

In “the fail to receive data case” as shown in Figure  11, at first the STA(AP) transmits data frames using multiple channels, in this case two channels. Unfortunately, the data #1 is dropped because of fading and so on. In this case, there is one CRC field for one aggregated data frame, so if one of transmitted frame is dropped, we should retransmit whole one aggregated data frame again. Therefore, the Receive STA(AP) don’t reply.

In “the fail to receive ACK case” as shown in Figure  11, at first the STA(AP) transmits data frames using multiple channels, in this case 2 channels. The Receive STA(AP) check CRC and reply ACK frames using two channels. Unfortunately, the ACK #1 is dropped because of fading and so on. In this case, same ACK frames are transmitted in 2 channels, so the STA (AP) can receive one ACK#2 frame and recognize the success of transmission.

Therefore, there is some possibility of using higher rate to transmit ACK frame than usual rate.

Figure  11: Copied ACK Architecture

Although we provided only Copied ACK scheme, we can use same mechanism for the control frames of RTS, CTS and so on.

## Backward Compatibility and Implementation Issues

We provide some typical frame sequences among the ADCA AP and the ADCA STA#1 and the legacy STA#1 in Figure  12.

In our proposal, the ADCA uses the Adaptive Frame Aggregation, the Bursting and a part of the EDCA that defined in IEEE802.11e to provide ACs. So, the ADCA is based on CSMA/CA access protocol defined in [1],[2]. Therefore, the legacy IEEE802.11a STA#2 can connect to our proposal ADCA AP. Moreover, the proposed ADCA STA#1 can meet high throughput using the Adaptive Frame Aggregation, the Bursting and ACs.

Figure  12: Interoperability Situation

## Simulation Results

We use NS2 (Network Simulator 2) to evaluate our protocol and compare with Legacy IEEE802.11a. The PHY and MAC parameters for IEEE802.11a used in this simulation are listed in Table  1. And the PHY and MAC parameters for our proposal in this simulation are listed in Table  2

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| SIFS | 16μs | PIFS | 25μs |
| DIFS | 34μs | Slot time | 9μs |
| PLCP preamble | 16μs | Signal | 4μs |
| CWmin | 15 | CWmax | 511 |
| MAC Header | 28byte | Maximum Length | 9100byte |
| Channel Bandwidth | 20MHz | ACK rate | 24Mbps |
| Carrier Frequency | 5.25GHz |  |  |

Table  2: PHY/MAC parameters used in this simulation

### MAC Throughput Gain using Adaptive Frame Aggregation and Bursting

The throughput performance of ACDA at different PHY layer rate is compared with legacy IEEE802.11 MAC transmitting 1500byte length as shown in Figure  13.

In this case, the maximum aggregated data length is less than 9100byte, because of data length from LLC is 1500byte UDP data, then we aggregate up to 6 packets to make long frame and distribute for 2 or 4 channels as shown 2x2 or 4x4 in Figure  13. Each transmission rates are 63Mbps (64QAM, R=7/8), 54Mbps(64QAM, R=3/4), 36Mbps (16QAM, R=3/4), 24Mbps (16QAM, R=1/2), and 18Mbps (QPSK, R=3/4). The Bursting is 2 times. Additionally, ACK rate is 24Mbps and used “Copied ACK”.

Figure  13: Throughput vs. Data Transmission Rate

### IEEE802.11n Usage Model and Simulation Results

The throughput performance of ADCA at Error Fee is shown in Figure  13. In this case, our proposal using the ADCA archives over 100Mbps at the top of the MAC SAP.

Additionally, the throughput performance of ADCA in the situation of Usage Model[4] are shown in Figure  18, Figure  19, Figure  20, Figure  21 . In these cases, the channel models are implemented as 20MHz SISO model written in [5], but just sample data for this document.

# Simple Hybrid Coordinated Channel Access (SHCCA)

In this section, the simple hybrid coordinated channel access protocol will be presented.

## Basic Algorithm

The SHCCA uses a combination with the Adaptive Frame Aggregation and the Bursting that explained in section 2 and a part of the hybrid coordinated channel access (HCCA) that defined in IEEE802.11e in order to reduce MAC over head and improve overall channel throughput and satisfy QoS. As shown in Figure  14, the STA#1 use the Bursting to transmit multiple data during the TXOP for STA#1, and the STA#2 use the Frame Aggregation to transmit during the TXOP for STA#2.

Figure  14: Simple Hybrid Coordinated Channel Access

## Backward Compatibility and Implementation Issues

The SHCCA protocol is similar to the HCCA defined at IEEE802.11e. So, Legacy IEEE802.11a STA can communicate with our proposed AP when polled from AP. Moreover, Adaptive Frame Aggregation makes more MAC efficiency. So we can archive not only Backward Compatibility and Interoperability, but also MAC efficiency.

## Simulation Results

We evaluate the SHCCA within Super frame as shown in Figure  2. so we don’t provide only  SHCCA simulation results. The throughput performance of our proposal in the situation of Usage Model[4] are shown from Table  3 to Table  8.

In these cases, we provide the simulation results of “Error Free using appropriate rate” and “Implemented PHY mode”.

# Frame formats

In this section, the frame format will be presented.

## PLCP frame format

The PLCP frame format that transmitted from physical layer is shown in Figure  15. Figure  15 shows the format for the PPDU including the OFDM PLCP preamble, OFDM PLCP header, PSDU, tail bits, and PHY pad bits. The PLCP header contains the following fields: LENGTH, RATE a reserved bit, an even parity bit, and the SERVICE field.

In Figure  15, the MIMO preamble is omitted, so if necessary we can provide “Extended Preamble” between PLCP Preamble and SIGNAL or SIGNAL and DATA to detect MIMO.

Figure  15: PPDU frame format

### Reserved field

The Reserved field is 1bit in length and is set null in IEEE802.11a[3]. In our proposal, The Reserved field is set to 1 indicating the presence of TGn Mode.

### SERVICE field

The SERVICE field is 16 bits in length, which shall be denote as bits 0-15. The bit 0 shall be transmitted first in time. The bits from 0-6 of the SERVICE field, which are transmitted first, are set to zeros and are used to synchronize the descrambler in the receiver. The remaining 9 bits (7-15) of the SERVICE field shall be reserved for future use in[3].

In our proposal, the remaining 9bits(7-15) of the SERVICE field are set as shown in Figure  16. The SERVICE field of (7) is set to 1 indicating this frame being Copied frame. Therefore, If ACK is transmitted as Copied frame, this field is set to 1. The SERVICE field of (8) is set to 1 indicating the presence of MAC Pad at the end of PSDU, and the Length in the PLCP contains one octet of MAC pad length also. The following 5bit(9-13) contain channel structure, so if using MIMO or Channel Bundling system, this field indicating this channel number and total usage channel number. The last 2bits of SERVICE field (14-15) indicated “Extended Length bit” to extend maximum length at physical layer. So the total length field in our proposal is 14 bits, the Length (12bit) and the Extended Length bits (2bit).

Figure  16: SERVICE field bit assignment

## MAC frame format

The format of the MAC frames is specified in this subsection. The MAC frames consist of the MAC Header, Frame Body, FCS as same as IEEE802.11[1].

In our proposal, the Frame Body is extended for high efficiency MAC using the Adaptive Frame aggregation. The Frame Body consists of the Aggregate Header and the Aggregated Frame Body.

The Aggregate Header is 2 octets in length and is set to frame type and total aggregate length without the aggregate header and total packets number in the aggregated frame body.

The Aggregated Frame Body is variable length up to “maximum aggregate length” and consists of the pair of Length field and Data. The Length field is 16 bits in length and indicates the length of following one data length.

Figure  17: Aggregated Data frame format

# Conclusion

In this document, we proposed a new MAC architecture, which not only meets the 100Mbps throughput requirement at the top of the MAC SAP, but also maintain the affinity with legacy IEEE802.11, IEEE802.11a and IEEE802.11e.

The main schemes are the Adaptive Frame Aggregation, the Bursting, and the Adaptive Frame Distribution for channels to meet the 100Mbps. Additionally, the SHCCA mechanism and the ACs during CP based on IEEE802.11e were implemented in our proposal and satisfied simple QoS, although we haven’t implemented and simulated perfectly yet.

The simulation result in section2 and 3 indicated that our proposal using the Adaptive Frame aggregation, the Bursting, and the Adaptive Frame Distribution for channels are quite effective schemes to archive high MAC efficiency and high throughput at MAC SAP.

# References

- IEEE 802.11, “Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”, Standard, IEEE, August, 1999

- IEEE 802.11e, “Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Medium Access Control (MAC) Enhancements for Quality of Service (QoS)”, Draft v6, IEEE, November 2003

- IEEE 802.11a, “Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: High-speed Physical Layer in the 5GHz Band”, Standard, IEEE, 1999

- IEEE802.11-03/0802r23, “Usage Models”

- IEEE802.11-03/0940r04, “TGn Channel Model”

- IEEE802.11-03/0813r12, “Functional Requirements”

# Appendix

## Simulation Results

### Scenario 1: Home

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HDTV#0 | CBR | AP STA1 | 5 | 4x4 54Mbps | 216 | 19.072 | 19.2 |
| HDTV#1 | CBR | AP STA3 | 5 | 4x4 54Mbps | 216 | 23.74 | 24 |
| SDTV#0 | CBR | AP STA4 | 9.89 | 4x4 36Mbps | 144 | 3.992 | 4 |
| Internet File | TCP | AP STA4 | 9.89 | 4x4 36Mbps | 144 | 0.9904 | 1 |
| VoIP_DL | CBR | AP STA7 | 20 | 4x4 24Mbps | 96 | 0.09728 | 0.096 |
| VoIP_DL | CBR | AP STA8 | 20 | 4x4 24Mbps | 96 | 0.09408 | 0.096 |
| VoIP_DL | CBR | AP STA9 | 20 | 4x4 24Mbps | 96 | 0.09664 | 0.096 |
| Inernet Streaming | CBR | AP STA10 | 14.14 | 4x4 36Mbps | 144 | 2.00158 | 2 |
| MP3 Audio | CBR | AP STA11 | 11.18 | 4x4 36Mbps | 144 | 0.12707 | 0.128 |
| VoD control#0 | CBR | STA1 AP | 5 | 4x4 63Mbps | 252 | 0.06263 | 0.06 |
| VoD control#1 | CBR | STA2 AP | 5 | 4x4 63Mbps | 252 | 0.06193 | 0.06 |
| Local file transfer | TCP | STA4 AP | 24.04 | 4x4 36Mbps | 144 | 27.992 | 30 |
| Video Phone#0 | CBR | STA5 STA6 | 30 | 4x4 18Mbps | 72 | 0.49698 | 0.5 |
| Video Phone#1 | CBR | STA6 STA5 | 30 | 4x4 18Mbps | 72 | 0.49971 | 0.5 |
| Console to Internet | CBR | STA10 AP | 14.14 | 4x4 36Mbps | 144 | 0.99806 | 1 |
| Video Gaming Controller | CBR | STA11 AP | 5 | 4x4 63Mbps | 252 | 0.44933 | 0.5 |
| VoIP_UL | CBR | STA7 AP | 20 | 4x4 24Mbps | 96 | 0.09824 | 0.096 |
| VoIP_UL | CBR | STA8 AP | 20 | 4x4 24Mbps | 96 | 0.09601 | 0.096 |
| VoIP_UL | CBR | STA9 AP | 20 | 4x4 24Mbps | 96 | 0.09760 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 145.894 | 145.895 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 81.0642 | 83.524 |
|  |  | ADCF (Mbit/s) |  |  |  | 31.4265 |  |
|  |  | SHCCA (Mbits/s) |  |  |  | 49.6377 |  |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 55.5635 | 57.249 |

Table  3: Scenario1 (Error Free using appropriate rate)

Not consider AIFS in simulation scenario1,4

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HDTV#0 | CBR | AP STA1 | 5 | 4x4 54Mbps | 216 | 19.0899 | 19.2 |
| HDTV#1 | CBR | AP STA3 | 5 | 4x4 54Mbps | 216 | 23.5386 | 24 |
| SDTV#0 | CBR | AP STA4 | 9.89 | 4x4 36Mbps | 144 | 4.0521 | 4 |
| Internet File | TCP | AP STA4 | 9.89 | 4x4 36Mbps | 144 | 0.1528 | 1 |
| VoIP_DL | CBR | AP STA7 | 20 | 4x4 24Mbps | 96 | 0.09312 | 0.096 |
| VoIP_DL | CBR | AP STA8 | 20 | 4x4 24Mbps | 96 | 0.09696 | 0.096 |
| VoIP_DL | CBR | AP STA9 | 20 | 4x4 24Mbps | 96 | 0.0944 | 0.096 |
| Inernet Streaming | CBR | AP STA10 | 14.14 | 4x4 36Mbps | 144 | 1.96873 | 2 |
| MP3 Audio | CBR | AP STA11 | 11.18 | 4x4 36Mbps | 144 | 0.12707 | 0.128 |
| VoD control#0 | CBR | STA1 AP | 5 | 2x2 63Mbps | 126 | 0.06005 | 0.06 |
| VoD control#1 | CBR | STA2 AP | 5 | 2x2 63Mbps | 126 | 0.05994 | 0.06 |
| Local file transfer | TCP | STA4 AP | 24.04 | 4x4 24Mbps | 96 | 1.06 | 30 |
| Video Phone#0 | CBR | STA5 STA6 | 30 | 4x4 24Mbps | 96 | 0.10513 | 0.5 |
| Video Phone#1 | CBR | STA6 STA5 | 30 | 4x4 24Mbps | 96 | 0.13653 | 0.5 |
| Console to Internet | CBR | STA10 AP | 14.14 | 4x4 24Mbps | 96 | 0.21026 | 1 |
| Video Gaming Controller | CBR | STA11 AP | 5 | 4x4 63Mbps | 252 | 0.46427 | 0.5 |
| VoIP_UL | CBR | STA7 AP | 20 | 4x4 24Mbps | 96 | 0.09130 | 0.096 |
| VoIP_UL | CBR | STA8 AP | 20 | 4x4 24Mbps | 96 | 0.09478 | 0.096 |
| VoIP_UL | CBR | STA9 AP | 20 | 4x4 24Mbps | 96 | 0.09172 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 130.105 | 130.105 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 51.588 | 83.524 |
|  |  | ADCF (Mbit/s) |  |  |  | 2.1290 | - |
|  |  | SHCCA (Mbits/s) |  |  |  | 49.4589 | - |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 39.651 | 64.19 |

Table  4: Scenario1 (implemented PHY model)

### Scenario 4: Large Enterprise

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internet File#1 | TCP | APSTA1 | 10.73 | 4x4 36Mbps | 144 | 0.01886 | 1 |
| Internet File#2 | TCP | APSTA2 | 8.27 | 4x4 54Mbps | 216 | 0.02922 | 1 |
| Internet File#3 | TCP | APSTA3 | 12.1 | 4x4 36Mbps | 144 | 0.01555 | 1 |
| Internet File#4 | TCP | APSTA4 | 4.52 | 4x4 54Mbps | 216 | 0.03599 | 1 |
| Internet File#5 | TCP | APSTA5 | 6.18 | 4x4 54Mbps | 216 | 0.0024 | 1 |
| Internet File, Large email attachments | TCP | APSTA6 | 7.1 | 4x4 54Mbps | 216 | 0.62633 | 10 |
| Video conferencing#1 | TCP | APSTA7 | 10.29 | 4x4 36Mbps | 144 | 1.0008 | 1 |
| Video conferencing#2 | TCP | APSTA8 | 12.02 | 4x4 36Mbps | 144 | 0.9981 | 1 |
| Internet Streaming Video+MP3 audio#1 | CBR | APSTA9 | 10.25 | 4x4 36Mbps | 144 | 1.99031 | 2 |
| Internet Streaming Video+MP3 audio#2 | CBP | APSTA10 | 3.04 | 4x4 54Mbps | 216 | 1.96331 | 2 |
| Local File transfer#1 | TCP | APSTA11 | 8.01 | 4x4 36Mbps | 144 | 4.38 | 30 |
| Local File transfer#2 | TCP | APSTA12 | 0.89 | 4x4 36Mbps | 144 | 4.076 | 30 |
| Local File transfer#3 | TCP | APSTA13 | 5.65 | 4x4 54Mbps | 216 | 4.38 | 30 |
| Local File transfer#4 | TCP | APSTA14 | 7.56 | 4x4 36Mbps | 144 | 4.38 | 30 |
| Local File transfer#5 | TCP | APSTA15 | 3.04 | 4x4 54Mbps | 216 | 4.38 | 30 |
| Local File transfer#6 | TCP | APSTA16 | 10 | 4x4 36Mbps | 144 | 4.38 | 30 |
| Local File transfer#7 | TCP | APSTA17 | 7.5 | 4x4 36Mbps | 144 | 4.312 | 30 |
| Local File transfer#8 | TCP | APSTA18 | 10.01 | 4x4 36Mbps | 144 | 4.38 | 30 |
| Local File transfer#9 | TCP | APSTA19 | 5.14 | 4x4 54Mbps | 216 | 4.38 | 30 |
| Local File transfer#10 | TCP | APSTA20 | 2.06 | 4x4 54Mbps | 216 | 4.38 | 30 |
| VoIP_DL#1 | CBR | APSTA25 | 6.1 | 4x4 54Mbps | 216 | 0.09376 | 0.096 |
| VoIP_DL#2 | CBR | APSTA26 | 13.08 | 4x4 36Mbps | 144 | 0.09248 | 0.096 |
| VoIP_DL#3 | CBR | APSTA27 | 6.5 | 4x4 54Mbps | 216 | 0.09536 | 0.096 |
| VoIP_DL#4 | CBR | APSTA28 | 9.7 | 4x4 54Mbps | 216 | 0.09664 | 0.096 |
| VoIP_DL#5 | CBR | APSTA29 | 3.8 | 4x4 63Mbps | 252 | 0.09632 | 0.096 |
| VoIP_DL#6 | CBR | APSTA30 | 10.12 | 4x4 54Mbps | 216 | 0.09824 | 0.096 |
| Clicking on web lin#1 | TCP | STA1AP | 10.73 | 4x4 54Mbps | 216 | 0.00397 | 0.256 |
| Clicking on web lin#2 | TCP | STA2AP | 8.27 | 4x4 54Mbps | 216 | 0.00602 | 0.256 |
| Clicking on web lin#3 | TCP | STA3AP | 12.1 | 4x4 36Mbps | 144 | 0.00367 | 0.256 |
| Internet file transfer in the upling direction | TCP | STA4AP | 4.52 | 4x4 54Mbps | 216 | 0.07243 | 5 |
| Uploading large email attachments | TCP | STA5AP | 6.18 | 4x4 54Mbps | 216 | 4.68031 | 10 |
| Clicking on web link | TCP | STA6AP | 7.1 | 4x4 54Mbps | 216 | 0.01145 | 0.256 |
| UL | CBR | STA7AP | 10.29 | 4x4 36Mbps | 144 | 0.98987 | 1 |
| UL | CBP | STA8AP | 12.02 | 4x4 36Mbps | 144 | 1.00625 | 1 |
| UL | TCP | STA21AP | 7.15 | 4x4 36Mbps | 144 | 4.688 | 30 |
| UL | TCP | STA22AP | 4.5 | 4x4 54Mbps | 216 | 4.684 | 30 |
| UL | TCP | STA23AP | 7.15 | 4x4 36Mbps | 144 | 4.604 | 30 |
| UL | TCP | STA24AP | 3.9 | 4x4 54Mbps | 216 | 4.648 | 30 |
| VoIP Phone#1 | CBR | STA25AP | 6.1 | 4x4 54Mbps | 216 | 0.09594 | 0.096 |
| VoIP Phone#2 | CBR | STA26AP | 13.08 | 4x4 36Mbps | 144 | 0.0992 | 0.096 |
| VoIP Phone#3 | CBR | STA27AP | 6.5 | 4x4 54Mbps | 216 | 0.10085 | 0.096 |
| VoIP Phone#4 | CBR | STA28AP | 9.7 | 4x4 54Mbps | 216 | 0.09724 | 0.096 |
| VoIP Phone#5 | CBR | STA29AP | 3.8 | 4x4 63Mbps | 252 | 0.09855 | 0.096 |
| VoIP Phone#6 | CBR | STA30AP | 10.12 | 4x4 54Mbps | 216 | 0.09857 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 188.182 | 188.181 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 76.670 | 460.176 |
|  |  | ADCF (Mbit/s) |  |  |  | 71.553 | 455.024 |
|  |  | SHCCA (Mbits/s) |  |  |  | 5.1168 | 5.125 |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 40.742 | 244.538 |

Table  5: Scenario4 (Error Free using appropriate rate)

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internet File#1 | TCP | APSTA1 | 10.73 | 4x4 36Mbps | 144 | 0.0159 | 1 |
| Internet File#2 | TCP | APSTA2 | 8.27 | 4x4 54Mbps | 216 | 0.04779 | 1 |
| Internet File#3 | TCP | APSTA3 | 12.1 | 4x4 36Mbps | 144 | 0.50138 | 1 |
| Internet File#4 | TCP | APSTA4 | 4.52 | 4x4 54Mbps | 216 | 0.57091 | 1 |
| Internet File#5 | TCP | APSTA5 | 6.18 | 4x4 54Mbps | 216 | 0.27830 | 1 |
| Internet File, Large email attachments | TCP | APSTA6 | 7.1 | 4x4 54Mbps | 216 | 0.0096 | 10 |
| Video conferencing#1 | TCP | APSTA7 | 10.29 | 4x4 36Mbps | 144 | 0.98577 | 1 |
| Video conferencing#2 | TCP | APSTA8 | 12.02 | 4x4 36Mbps | 144 | 0.9885 | 1 |
| Internet Streaming Video+MP3 audio#1 | CBR | APSTA9 | 10.25 | 4x4 36Mbps | 144 | 1.98384 | 2 |
| Internet Streaming Video+MP3 audio#2 | CBP | APSTA10 | 3.04 | 4x4 36Mbps | 144 | 1.99883 | 2 |
| Local File transfer#1 | TCP | APSTA11 | 8.01 | 4x4 36Mbps | 144 | 3.2 | 30 |
| Local File transfer#2 | TCP | APSTA12 | 0.89 | 4x4 36Mbps | 144 | 3.164 | 30 |
| Local File transfer#3 | TCP | APSTA13 | 5.65 | 4x4 54Mbps | 216 | 3.164 | 30 |
| Local File transfer#4 | TCP | APSTA14 | 7.56 | 4x4 36Mbps | 144 | 3.088 | 30 |
| Local File transfer#5 | TCP | APSTA15 | 3.04 | 4x4 54Mbps | 216 | 3.088 | 30 |
| Local File transfer#6 | TCP | APSTA16 | 10 | 4x4 36Mbps | 144 | 3.164 | 30 |
| Local File transfer#7 | TCP | APSTA17 | 7.5 | 4x4 36Mbps | 144 | 2.792 | 30 |
| Local File transfer#8 | TCP | APSTA18 | 10.01 | 4x4 36Mbps | 144 | 3.088 | 30 |
| Local File transfer#9 | TCP | APSTA19 | 5.14 | 4x4 54Mbps | 216 | 3.164 | 30 |
| Local File transfer#10 | TCP | APSTA20 | 2.06 | 4x4 54Mbps | 216 | 3.164 | 30 |
| VoIP_DL#1 | CBR | APSTA25 | 6.1 | 4x4 54Mbps | 216 | 0.096 | 0.096 |
| VoIP_DL#2 | CBR | APSTA26 | 13.08 | 4x4 36Mbps | 144 | 0.09536 | 0.096 |
| VoIP_DL#3 | CBR | APSTA27 | 6.5 | 4x4 54Mbps | 216 | 0.09504 | 0.096 |
| VoIP_DL#4 | CBR | APSTA28 | 9.7 | 4x4 54Mbps | 216 | 0.09312 | 0.096 |
| VoIP_DL#5 | CBR | APSTA29 | 3.8 | 4x4 63Mbps | 252 | 0.09408 | 0.096 |
| VoIP_DL#6 | CBR | APSTA30 | 10.12 | 4x4 54Mbps | 216 | 0.09536 | 0.096 |
| Clicking on web lin#1 | TCP | STA1AP | 10.73 | 4x4 54Mbps | 216 | 0.04326 | 0.256 |
| Clicking on web lin#2 | TCP | STA2AP | 8.27 | 4x4 54Mbps | 216 | 0.09034 | 0.256 |
| Clicking on web lin#3 | TCP | STA3AP | 12.1 | 4x4 36Mbps | 144 | 0.01246 | 0.256 |
| Internet file transfer in the upling direction | TCP | STA4AP | 4.52 | 4x4 54Mbps | 216 | 0.07529 | 5 |
| Uploading large email attachments | TCP | STA5AP | 6.18 | 4x4 54Mbps | 216 | 0.23271 | 10 |
| Clicking on web link | TCP | STA6AP | 7.1 | 4x4 54Mbps | 216 | 0.13604 | 0.256 |
| UL | CBR | STA7AP | 10.29 | 4x4 36Mbps | 144 | 0.98577 | 1 |
| UL | CBP | STA8AP | 12.02 | 4x4 36Mbps | 144 | 0.98167 | 1 |
| UL | TCP | STA21AP | 7.15 | 4x4 36Mbps | 144 | 3.32 | 30 |
| UL | TCP | STA22AP | 4.5 | 4x4 54Mbps | 216 | 3.048 | 30 |
| UL | TCP | STA23AP | 7.15 | 4x4 36Mbps | 144 | 3.316 | 30 |
| UL | TCP | STA24AP | 3.9 | 4x4 36Mbps | 144 | 3.32 | 30 |
| VoIP Phone#1 | CBR | STA25AP | 6.1 | 4x4 54Mbps | 216 | 0.09675 | 0.096 |
| VoIP Phone#2 | CBR | STA26AP | 13.08 | 4x4 36Mbps | 144 | 0.09486 | 0.096 |
| VoIP Phone#3 | CBR | STA27AP | 6.5 | 4x4 54Mbps | 216 | 0.09373 | 0.096 |
| VoIP Phone#4 | CBR | STA28AP | 9.7 | 4x4 54Mbps | 216 | 0.08997 | 0.096 |
| VoIP Phone#5 | CBR | STA29AP | 3.8 | 4x4 63Mbps | 252 | 0.09236 | 0.096 |
| VoIP Phone#6 | CBR | STA30AP | 10.12 | 4x4 54Mbps | 216 | 0.09706 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 184.909 | 184.090 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 55.151 | 460.176 |
|  |  | ADCF (Mbit/s) |  |  |  | 50.0349 | 455.024 |
|  |  | SHCCA (Mbits/s) |  |  |  | 5.116 | 5.152 |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 29.826 | 248.866 |

Table  6: Scenario4 (implemented PHY model)

### Scenario 6: Hot spot

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internet File Transfer#1 | TCP | APSTA1 | 31.14 | 4x4 24Mbps | 96 | 0.7128 | 2 |
| Internet File Transfer#2 | TCP | APSTA2 | 26.4 | 4x4 24Mbps | 96 | 0.6848 | 2 |
| Internet File Transfer#3 | TCP | APSTA3 | 13.03 | 4x4 36Mbps | 144 | 0.7184 | 2 |
| Internet File Transfer#4 | TCP | APSTA4 | 9.21 | 4x4 36Mbps | 144 | 0.7056 | 2 |
| Internet File Transfer#5 | TCP | APSTA5 | 29 | 4x4 24Mbps | 96 | 0.728 | 2 |
| Internet File Transfer#6 | TCP | APSTA6 | 5.09 | 4x4 54Mbps | 216 | 0.7 | 2 |
| Internet File Transfer#7 | TCP | APSTA7 | 25.05 | 4x4 24Mbps | 96 | 0.7 | 2 |
| Internet File Transfer#8 | TCP | APSTA8 | 12.08 | 4x4 36Mbps | 144 | 0.7352 | 2 |
| Internet File Transfer#9 | TCP | APSTA9 | 16.15 | 4x4 36Mbps | 144 | 0.7632 | 2 |
| Internet File Transfer#10 | TCP | APSTA10 | 23.08 | 4x4 24Mbps | 96 | 0.7592 | 2 |
| Mid quality streaming audio/video#1 | CBR | APSTA11 | 23.02 | 4x4 24Mbps | 96 | 1.97681 | 2 |
| Mid quality streaming audio/video#2 | CBR | APSTA12 | 23.02 | 4x4 24Mbps | 96 | 1.963.5 | 2 |
| Mid quality streaming audio/video#3 | CBR | APSTA13 | 22.62 | 4x4 24Mbps | 96 | 1.97267 | 2 |
| Mid quality streaming audio/video#4 | CBR | APSTA14 | 24.75 | 4x4 24Mbps | 96 | 1.96166 | 2 |
| High quality streaming audio/video#1 | CBR | APSTA15 | 16.12 | 4x4 24Mbps | 96 | 7.89698 | 8 |
| High quality streaming audio/video#2 | CBR | APSTA16 | 10.44 | 4x4 36Mbps | 144 | 7.89287 | 8 |
| High quality streaming audio/video#3 | CBR | APSTA17 | 15.88 | 4x4 24Mbps | 96 | 7.9557 | 8 |
| SDTV broadcast#1 | CBR | APSTA18 | 4.12 | 4x4 54Mbps | 216 | 4.98 | 5 |
| SDTV broadcast#2 | CBR | APSTA19 | 6 | 4x4 36Mbps | 144 | 4.90411 | 5 |
| VoIP_DL#1 | CBR | APSTA20 | 30.41 | 4x4 24Mbps | 96 | 0.09408 | 0.096 |
| VoIP_DL#2 | CBR | APSTA21 | 21.95 | 4x4 24Mbps | 96 | 0.09664 | 0.096 |
| VoIP_DL#3 | CBR | APSTA22 | 18.02 | 4x4 36Mbps | 144 | 0.09344 | 0.096 |
| VoIP_DL#4 | CBR | APSTA23 | 10.29 | 4x4 36Mbps | 144 | 0.09632 | 0.096 |
| VoIP_DL#5 | CBR | APSTA24 | 12.64 | 4x4 36Mbps | 144 | 0.09536 | 0.096 |
| VoIP_DL#6 | CBR | APSTA25 | 22.09 | 4x4 24Mbps | 96 | 0.09376 | 0.096 |
| VoIP_DL#7 | CBR | APSTA26 | 19.69 | 4x4 36Mbps | 144 | 0.09728 | 0.096 |
| VoIP_DL#8 | CBR | APSTA27 | 22.8 | 4x4 24Mbps | 96 | 0.09504 | 0.096 |
| VoIP_DL#9 | CBR | APSTA28 | 18.78 | 4x4 36Mbps | 144 | 0.09344 | 0.096 |
| VoIP_DL#10 | CBR | APSTA29 | 25.07 | 4x4 24Mbps | 96 | 0.09408 | 0.096 |
| VoIP_DL#11 | CBR | APSTA30 | 21 | 4x4 24Mbps | 96 | 0.09664 | 0.096 |
| VoIP_DL#12 | CBR | APSTA31 | 29.06 | 4x4 24Mbps | 96 | 0.0944 | 0.096 |
| VoIP_DL#13 | CBR | APSTA32 | 22.47 | 4x4 24Mbps | 96 | 0.0944 | 0.096 |
| VoIP_DL#14 | CBR | APSTA33 | 15.65 | 4x4 36Mbps | 144 | 0.09344 | 0.096 |
| VoIP_DL#15 | CBR | APSTA32 | 3.6 | 4x4 63Mbps | 252 | 0.0944 | 0.096 |
| VoIP_UL#1 | CBR | STA20AP | 30.41 | 4x4 24Mbps | 96 | 0.09856 | 0.096 |
| VoIP_UL#2 | CBR | STA21AP | 21.95 | 4x4 24Mbps | 96 | 0.09725 | 0.096 |
| VoIP_UL#3 | CBR | STA22AP | 18.02 | 4x4 36Mbps | 144 | 0.09762 | 0.096 |
| VoIP_UL#4 | CBR | STA23AP | 10.29 | 4x4 36Mbps | 144 | 0.09789 | 0.096 |
| VoIP_UL#5 | CBR | STA24AP | 12.64 | 4x4 36Mbps | 144 | 0.09530 | 0.096 |
| VoIP_UL#6 | CBR | STA25AP | 11.09 | 4x4 24Mbps | 96 | 0.09564 | 0.096 |
| VoIP_UL#7 | CBR | STA26AP | 19.69 | 4x4 36Mbps | 144 | 0.1005 | 0.096 |
| VoIP_UL#8 | CBR | STA27AP | 22.8 | 4x4 24Mbps | 96 | 0.09661 | 0.096 |
| VoIP_UL#9 | CBR | STA28AP | 18.78 | 4x4 36Mbps | 144 | 0.09790 | 0.096 |
| VoIP_UL#10 | CBR | STA29AP | 25.07 | 4x4 24Mbps | 96 | 0.09973 | 0.096 |
| VoIP_UL#11 | CBR | STA30AP | 21 | 4x4 24Mbps | 96 | 0.09728 | 0.096 |
| VoIP_UL#12 | CBR | STA31AP | 29.06 | 4x4 24Mbps | 96 | 0.09661 | 0.096 |
| VoIP_UL#13 | CBR | STA32AP | 22.47 | 4x4 24Mbps | 96 | 0.09561 | 0.096 |
| VoIP_UL#14 | CBR | STA33AP | 15.65 | 4x4 36Mbps | 144 | 0.09729 | 0.096 |
| VoIP_UL#15 | CBR | STA34AP | 3.6 | 4x4 63Mbps | 252 | 0.09824 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 124.897 | 124.897 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 51.595 | 64.88 |
|  |  | ADCF (Mbit/s) |  |  |  | 7.207 | 20 |
|  |  | SHCCA (Mbits/s) |  |  |  | 44.388 | 44.88 |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 41.310 | 51.946 |

Table  7: Scenario6 (Error Free using appropriate rate)

| Flow Name | Type | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) | Mean Rate In UM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internet File Transfer#1 | TCP | APSTA1 | 31.14 | 4x4 24Mbps | 96 | 0.4048 | 2 |
| Internet File Transfer#2 | TCP | APSTA2 | 26.4 | 4x4 24Mbps | 96 | 0.4024 | 2 |
| Internet File Transfer#3 | TCP | APSTA3 | 13.03 | 4x4 36Mbps | 144 | 0.428 | 2 |
| Internet File Transfer#4 | TCP | APSTA4 | 9.21 | 4x4 36Mbps | 144 | 0.4338 | 2 |
| Internet File Transfer#5 | TCP | APSTA5 | 29 | 4x4 24Mbps | 96 | 0.4192 | 2 |
| Internet File Transfer#6 | TCP | APSTA6 | 5.09 | 4x4 54Mbps | 216 | 0.4224 | 2 |
| Internet File Transfer#7 | TCP | APSTA7 | 25.05 | 4x4 24Mbps | 96 | 0.4296 | 2 |
| Internet File Transfer#8 | TCP | APSTA8 | 12.08 | 4x4 36Mbps | 144 | 0.4472 | 2 |
| Internet File Transfer#9 | TCP | APSTA9 | 16.15 | 4x4 36Mbps | 144 | 0.4264 | 2 |
| Internet File Transfer#10 | TCP | APSTA10 | 23.08 | 4x4 24Mbps | 96 | 0.4056 | 2 |
| Mid quality streaming audio/video#1 | CBR | APSTA11 | 23.02 | 4x4 24Mbps | 96 | 1.98901 | 2 |
| Mid quality streaming audio/video#2 | CBR | APSTA12 | 23.02 | 4x4 24Mbps | 96 | 1.98549 | 2 |
| Mid quality streaming audio/video#3 | CBR | APSTA13 | 22.62 | 4x4 24Mbps | 96 | 2.00213 | 2 |
| Mid quality streaming audio/video#4 | CBR | APSTA14 | 24.75 | 4x4 24Mbps | 96 | 1.96449 | 2 |
| High quality streaming audio/video#1 | CBR | APSTA15 | 16.12 | 4x4 24Mbps | 96 | 7.9575 | 8 |
| High quality streaming audio/video#2 | CBR | APSTA16 | 10.44 | 4x4 36Mbps | 144 | 7.91398 | 8 |
| High quality streaming audio/video#3 | CBR | APSTA17 | 15.88 | 4x4 24Mbps | 96 | 7.85657 | 8 |
| SDTV broadcast#1 | CBR | APSTA18 | 4.12 | 4x4 36Mbps | 144 | 4.868 | 5 |
| SDTV broadcast#2 | CBR | APSTA19 | 6 | 4x4 36Mbps | 144 | 5.0361 | 5 |
| VoIP_DL#1 | CBR | APSTA20 | 30.41 | 4x4 24Mbps | 96 | 0.09248 | 0.096 |
| VoIP_DL#2 | CBR | APSTA21 | 21.95 | 4x4 24Mbps | 96 | 0.09664 | 0.096 |
| VoIP_DL#3 | CBR | APSTA22 | 18.02 | 4x4 36Mbps | 144 | 0.09472 | 0.096 |
| VoIP_DL#4 | CBR | APSTA23 | 10.29 | 4x4 36Mbps | 144 | 0.09728 | 0.096 |
| VoIP_DL#5 | CBR | APSTA24 | 12.64 | 4x4 36Mbps | 144 | 0.0944 | 0.096 |
| VoIP_DL#6 | CBR | APSTA25 | 22.09 | 4x4 36Mbps | 144 | 0.09568 | 0.096 |
| VoIP_DL#7 | CBR | APSTA26 | 19.69 | 4x4 36Mbps | 144 | 0.09472 | 0.096 |
| VoIP_DL#8 | CBR | APSTA27 | 22.8 | 4x4 24Mbps | 96 | 0.09312 | 0.096 |
| VoIP_DL#9 | CBR | APSTA28 | 18.78 | 4x4 36Mbps | 144 | 0.0928 | 0.096 |
| VoIP_DL#10 | CBR | APSTA29 | 25.07 | 4x4 18Mbps | 72 | 0.09152 | 0.096 |
| VoIP_DL#11 | CBR | APSTA30 | 21 | 4x4 24Mbps | 96 | 0.09568 | 0.096 |
| VoIP_DL#12 | CBR | APSTA31 | 29.06 | 4x4 24Mbps | 96 | 0.09312 | 0.096 |
| VoIP_DL#13 | CBR | APSTA32 | 22.47 | 4x4 24Mbps | 96 | 0.0944 | 0.096 |
| VoIP_DL#14 | CBR | APSTA33 | 15.65 | 4x4 36Mbps | 144 | 0.09568 | 0.096 |
| VoIP_DL#15 | CBR | APSTA34 | 3.6 | 4x4 36Mbps | 144 | 0.09632 | 0.096 |
| VoIP_UL#1 | CBR | STA20AP | 30.41 | 4x4 24Mbps | 96 | 0.09629 | 0.096 |
| VoIP_UL#2 | CBR | STA21AP | 21.95 | 4x4 24Mbps | 96 | 0.09792 | 0.096 |
| VoIP_UL#3 | CBR | STA22AP | 18.02 | 4x4 36Mbps | 144 | 0.09497 | 0.096 |
| VoIP_UL#4 | CBR | STA23AP | 10.29 | 4x4 36Mbps | 144 | 0.09644 | 0.096 |
| VoIP_UL#5 | CBR | STA24AP | 12.64 | 4x4 36Mbps | 144 | 0.09708 | 0.096 |
| VoIP_UL#6 | CBR | STA25AP | 11.09 | 4x4 36Mbps | 144 | 0.09431 | 0.096 |
| VoIP_UL#7 | CBR | STA26AP | 19.69 | 4x4 24Mbps | 96 | 0.09727 | 0.096 |
| VoIP_UL#8 | CBR | STA27AP | 22.8 | 4x4 24Mbps | 96 | 0.09855 | 0.096 |
| VoIP_UL#9 | CBR | STA28AP | 18.78 | 4x4 36Mbps | 144 | 0.1008 | 0.096 |
| VoIP_UL#10 | CBR | STA29AP | 25.07 | 4x4 24Mbps | 96 | 0.09793 | 0.096 |
| VoIP_UL#11 | CBR | STA30AP | 21 | 4x4 24Mbps | 96 | 0.09758 | 0.096 |
| VoIP_UL#12 | CBR | STA31AP | 29.06 | 4x4 24Mbps | 96 | 0.09956 | 0.096 |
| VoIP_UL#13 | CBR | STA32AP | 22.47 | 4x4 24Mbps | 96 | 0.09758 | 0.096 |
| VoIP_UL#14 | CBR | STA33AP | 15.65 | 4x4 36Mbps | 144 | 0.09582 | 0.096 |
| VoIP_UL#15 | CBR | STA34AP | 3.6 | 4x4 36Mbps | 144 | 0.09790 | 0.096 |
|  |  | Aggregate PHY Rate(Mbits/s) |  |  |  | 116.571 | 116.571 |
|  |  | Aggregated MAC Goodput (Mbits/s) (CC20) |  |  |  | 48.672 | 64.88 |
|  |  | ADCF (Mbit/s) |  |  |  | 4.2184 | 20 |
|  |  | SHCCA (Mbits/s) |  |  |  | 44.4539 | 44.88 |
|  |  | MAC Efficiency (%) (CC24) |  |  |  | 41.753 | 55.6568 |

Table  8: Scenario6 (implemented PHY model)

### Scenario 16: Point-to-Point Throughput Test

| Flow Name | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| UDP sources | APSTA | 0 | 20MHz 4x4 64QAM R=3/4 | 216 | 104.627 |
|  |  | 2 |  |  | 102.344 |
|  |  | 4 |  |  | 101.143 |
|  |  | 6 | 20MHz 4x4 16QAM R=3/4 | 144 | 99.593 |
|  |  | 8 |  |  | 91.710 |
|  |  | 10 |  |  | 81.617 |
|  |  | 12 | 20MHz 4x4 64QAM R=1/2 | 96 | 75.529 |
|  |  | 14 |  |  | 71.403 |
|  |  | 16 |  |  | 62.944 |
|  |  | 18 |  |  | 51.865 |
|  |  | 20 | 20MHz 4x4 QPSK R=3/4 | 72 | 37.734 |
|  |  | 22 |  |  | 30.188 |
|  |  | 24 |  |  | 24.421 |
|  |  | 26 |  |  | 17.692 |
|  |  | 28 |  |  | 15.769 |
|  |  | 30 |  |  | 10.915 |
|  |  | 35 |  |  | 5.431 |
|  |  | 40 |  |  | 0 |

Table  9: Scenario16 (Channel Model B, 4x4)

| Flow Name | Direction | Distance (m) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| UDP sources | AP  STA | 0 | 20MHz 4x4 64QAM R=3/4 | 216 | 103.330 |
|  |  | 2 | 20MHz 4x4 16QAM R=3/4 | 144 | 103.233 |
|  |  | 4 |  |  | 98.103 |
|  |  | 6 |  |  | 91.157 |
|  |  | 8 |  |  | 83.924 |
|  |  | 10 | 20MHz 4x4 64QAM R=1/2 | 96 | 80.239 |
|  |  | 12 |  |  | 78.773 |
|  |  | 14 |  |  | 75.409 |
|  |  | 16 |  |  | 69.665 |
|  |  | 18 |  |  | 60.228 |
|  |  | 20 |  |  | 47.251 |
|  |  | 25 | 20MHz 4x4 QPSK R=3/4 | 72 | 17.211 |
|  |  | 30 |  |  | 4.796 |
|  |  | 35 |  |  | 1.081 |
|  |  | 40 |  |  | 0 |

Table  10: Scenario16 (Channel Model D, 4x4)

Figure  18: Throughput vs. Distance for Scenario16

### Scenario 17: Point-to-Point HT Goodput Test

| Flow Name | Direction | Location (x,y) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| UDP 100Mbps (1500byte) | STA AP | (0.10) | 20MHz 4x4 16QAM R=3/4 | 144 | 81.721 Ave. |

Table  11: Scenario17 (Channel Model B, 4x4)

Figure  19: Throughput Performance for Scenario17

### Scenario 18: Point-to-Point Legacy Throughput Test

| Flow Name | Direction | Location (x,y) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| UDP 100Mbps (1500byte) | STA AP | (0.10) | 20MHz 16QAM R=3/4 | 36 | 21.672 Ave. |

Table  12: Scenario18 (IEEE802.11a)

Figure  20: Throughput Performance for Scenario18

### Scenario 19: Point-to-Point Legacy Sharing Throughput Test

| Flow Name | Direction | Location (x,y) | PHY mode | PHY Rate (Mbps) | Mean Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| UDP 100Mbps (1500byte) (Legacy UDP Source) | STA1 AP | (-10,0) | 20MHz 16QAM R=3/4 | 36 | 10.817 Ave. |
| UDP 100Mbps (1500byte) (HT UDP Source) | STA2AP | (10,0) | 20MHz 4x4 16QAM R=3/4 | 144 | 41.384 Ave. |
| System Throughput Average |  |  |  |  | 52.201 |

Table  13: Scenario19 (Channel Model B, Legacy STA and HT STA)

Figure  21: Throughput Performance for Scenario19