IEEE P802.11
Wireless LANs

Non-Country Entity

Date:	Aug 17, 2004

Authors:		Richard Paine
	Boeing
	1201 58th Pl SW  Everett, Wa  98203
	Phone: 206-854-8199
	E-mail: richard.h.paine@boeing.com

Abstract

This document contains the changes needed to update the Reaff2003 to reflect non-country entities.

Change dot11CountryString on  page 603 of the Reaff2003 document to reflect the following changes in red:

dot11CountryString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute identifies the country or non-country entity in which the station is operating. If it is a country, the first two octets of this string is the two character country code as described in document ISO/IEC 3166-1. The first two octets of a non-country entity will be two ASCII ‘X’ characters (XX). The third octet shall be one of the following:

1. an ASCII space character, if the regulations under

which the station is operating encompass all

environments in the country,

2. an ASCII 'O' character, if the regulations under

which the station is operating are for an Outdoor

environment only, or

3. an ASCII 'I' character, if the regulations under

which the station is operating are for an Indoor

environment only.

4. an ASCII ‘X’ character, if the station is operating under a non-country entity.

::= { dot11StationConfigEntry 23 }