IEEE P802.11 Working Group for Wireless Local Area Networks (WLANs)

Inprocomm’s PHY Proposal MASSDIC-OFDM for 802.11 TGn

Date:	September, 2004

Author:	Kim Wu et al.
	Inprocomm, Inc.

9F, No. 93, Shuei-Yuan St.

Hsinchu, 300 Taiwan, R.O.C.
Phone: +886 572 5050

Fax: +886 572 6060
	e-Mail: kimwu@inprocomm.com.tw

Abstract

This document presents specification of Inprocomm’s PHY proposal for the 802.11 TGn standard. A partial proposal for a high-data-rate physical layer of a Local Area Network, using MASSDIC-OFDM transmission is specified. The air interface is based on 2x2 MIMO-OFDM, using up to 256-QAM for the modulation. Advanced FEC coding using modern LDPC code is proposed with signal-space diversity coding to significantly improve the performance. The proposal set 3x3 MIMO and 4x4 MIMO as optional.

Contributors:

Kim Wu (kimwu@inprocomm.com.tw)

Chao-Yu Chen (d919609@oz.nthu.edu.tw )

Tsung-Yu Wu (d919610@oz.nthu.edu.tw )

Racy Cheng (racy@inprocomm.com.tw )

Chi-chao Chao (ccc@inprocomm.com.tw )

Mao-Ching Chiu (mcchiu@inprocomm.com.tw )

# Features of the proposal

This document represents the technical specification of Inprocomm’s partial proposal in response to the Call for Proposals by the 802.11 TGn on May 17. The proposal is targeted at the PHY layer using bandwidth of 20 MHz based on the assumption that the MAC efficiency is enhanced to be more than 60%. Under such an assumption, the MAC throughput of the PHY proposal could be reached up to 100 Mbps in Mandatory mode and up to 200 Mbps in Optional mode. In other words, the PHY data rate could be reached up to 167 Mbps and 333 Mbps in Mandatory and optional mode respectively.

The PHY approach of the proposal: Multiple-Antenna-Signal-Space-DIversity-Coded OFDM (MASSDIC-OFDM) is extended from the OFDM technology specified in 802.11a standard with the following features:

- Transmission using multiple antennas (MA)

- The constellation size of modulation is extended from 64-QAM to 256-QAM.

- The number of subcarriers in an OFDM symbol is increased from 64 to 128.

- Only PHY data rates more than 53 Mbps are newly defined.

- Orthogonal space-frequency block coding is used to enhance system performance.

- The guard intervals are variable with respect to different data rates.

- A well known signal space diversity coding (SSDIC) technique called linear constellation precoding (LCP) is introduced as an option.

To enhance the PHY efficiency especially in MIMO case, the following features are also included in the proposal:

- No more overhead is needed for the PHY header.

- New preamble structures are designed.

- The maximum data length is extended to 65536 bytes.

A more powerful error control coding scheme using low parity check density code (LDPCC) is also proposed with the following properties:

- Coding rates of 1/2, 2/3, 3/4 are separately designed.

- Codeword shortening with hybrid code rate combination is proposed.

A new scheme to make use of the pad bits is also proposed:

The pad bits are put in front of the payload. Some pad bits are replaced with repeated information of the header in some conditions.

Table of Contents

1.	Features of the proposal	3

2.	References	6

3.	Definitions	6

4.	Abbreviations and acronyms	7

5.	MASSDIC-OFDM high throughput PHY specification	8

5.1	Overview	8

5.1.1	Introduction	8

5.1.2	Frequency Bands of Operation	9

5.1.3	Scope	9

5.1.4	MASSDIC-OFDM PHY functions	9

5.2	MASSDIC-OFDM PLCP sublayer	10

5.2.1	Introduction	10

5.2.2	PLCP frame format	10

5.2.2.1	Rate-dependent parameters and timing related parameters	11

5.2.3	MASSDIC-OFDM TX architecture	13

5.2.4	PLCP preamble (SYNC)	14

5.2.4.1	N mode Preamble	14

5.2.4.2	LN mode Preamble	18

5.2.5	PLCP header	20

5.2.5.1	Data Rate (RATE)	20

5.2.5.2	Reserved (R), Interleaver (I) and Header Tail (TAIL)	21

5.2.5.3	Service field (SERVICE)	21

5.2.5.4	PLCP length field	21

5.2.5.5	CCITT CRC-16 for the HCS	21

5.2.6	Data field	21

5.2.6.1	Pad bits (PAD)	21

5.2.6.2	Repeated header	22

5.2.6.3	Frame check sequence	22

5.2.6.4	PLCP DATA scrambler and descrambler	22

5.2.6.5	Low density parity check codes (LDPCC)	23

5.2.6.6	Space-frequency interleaving	25

5.2.6.7	Subcarrier modulation mapping	26

5.2.6.8	Pilot subcarrier	30

5.2.6.9	MIMO-OFDM modulation	30

5.2.6.10	Linear constellation precoding (optional)	31

5.2.7	Space Time Block Code (STBC)	33

5.2.7.1	STBC for NT=2 and 3	33

5.2.7.2	STBC for NT=4	34

List of Figures

Figure 1 ─ PPDU Frame Format for N mode	10

Figure 2 ─ PPDU Frame Format for LN mode	11

Figure 3 ─ MASSDIC-OFDM TX architecture	14

Figure 4 ─ Preamble structure in N mode for NT=2	15

Figure 5 ─ Preamble structure in N mode for NT=3	17

Figure 6 ─ Preamble structure in N mode for NT=4	18

Figure 7 ─ Preamble structure in LN mode for NT=2	19

Figure 8 ─ Preamble structure in LN mode for NT=3	19

Figure 9 ─ Preamble structure in LN mode for NT=4	20

Figure 10 ─ PLCP header bit assignment	20

Figure 11 ─ CCITT CRC-16 structure	21

Figure 12 ─ Data scrambler	23

Figure 13 ─ FEC encode structure	24

Figure 14 ─ Short information block	24

Figure 15 ─ OFDM-symbol-level multiplexing scheme	26

Figure 16 ─ BPSK, QPSK, 16-QAM, 64-QAM, and 256-QAM constellation bit encoding	28

Figure 17 ─ Subcarrier frequency allocation	31

Figure 18 ─ Subcarrier grouping for LCP	33

List of Tables

Table 1 ─ Rate-dependent and timing related parameters for NT=2	12

Table 2 ─ Rate-dependent and timing related parameters for NT=3	12

Table 3 ─ Rate-dependent and timing related parameters for NT=4	13

Table 4 ─ Provided codeword rates	24

Table 5 ─ The threshold of rate switch for codeword shortening	25

Table 6 ─ Modulation-dependent normalization factor KMOD	26

Table 7 ─ BPSK encoding table	28

Table 8 ─ QPSK encoding table	29

Table 9 ─ 16-QAM encoding table	29

Table 10 ─ 64-QAM encoding table	29

Table 11 ─ 256-QAM encoding table	29

# References

- IEEE Std 802.11®-1999 (Reaff 2003)
Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- Project Authorization Request  for IEEE 802.11n.
11-02-798r7-HT-SG-Draft-PAR.doc

- Zhiqiang Liu, Yan Xin, and Georgios B. Giannakis,“Linear Constellation Precoding for OFDM With Maximum Multipath Diversity and Coding Gains,” IEEE Trans. Commun., vol. 51, pp. 416–427, Mar. 2003.

- J. Boutros and E. Viterbo, “Signal space diversity: a power- and bandwidth-efficient diversity technique for the Rayleigh fading channel,” IEEE Trans. Inform. Theory, vol. 44, pp. 1453–1467, July 1998.

- M. Yang, W. E. Ryan, and Y. Li, "Design of efficiently encodable moderate-length high-rate irregular LDPC codes," IEEE Trans. Comm., Vol. 52, No. 4, pp. 564-571, April 2004.

- Kee-Bong Song and Syed Aon Mujtaba, “A low complexity space-frequency BICM MIMO-OFDM system for next-generation WLANs,” in Proc. IEEE Globecom, Dec. 2003, pp. 1059-1063.

# Definitions

Header: A piece of information that defines all that the receiver needs to decode the received signal.

L mode: An access mode that is intended for a scenario that 802.11n devices communicate with legacy devices

LN mode: An access mode that is intended for a scenario that some of the devices are legacy and others are 802.11n.

MIMO: Multiple input multiple output

N mode: An access mode that is intended for a scenario that all devices are 802.11n.

OFDM: Orthogonal frequency division multiplexing

OFDM Symbol: An OFDM-modulated signal occupying an interval of time which consists of a lot of orthogonal subcarriers in the frequency domain.

Preamble: A part of the 802.11n PPDU dedicated for packet detection, synchronization, and channel estimation.

# Abbreviations and acronyms

| Term | Description |
| --- | --- |
| CRC | Cyclic Redundancy Check |
| FCS | Frame Check Sequence |
| HCS | Header Check Sequence |
| HT | High Throughput |
| IFFT | Inverse Fast Fourier Transform |
| LCP | Linear Constellation Precoding |
| L SIG | Legacy Signal |
| L-STS | Legacy Short Training Sequence |
| LTS | Long Training Sequence |
| LDPCC | Low Density Parity Check Code |
| MAC | Medium Access Controller |
| MASSDIC | Multiple Antenna Signal Space Diversity Coded |
| MIMO | Multiple Input Multiple Output |
| MSDU | MAC Service Data Unit |
| NT | Total Number of Transmit Antennas |
| OFDM | Orthogonal Frequency Division Multiplexing |
| STBC | Space Time Block Code |
| PDM | Physical Medium Dependent |
| PHY | Physical Layer |
| PLCP | PHY Layer Convergence Protocol |
| PPDU | PHY Protocol Data Unit |
| PSDU | PHY Service Data Unit |
| RX | Receiver |
| STS | Short Training Sequence |
| TGn | 802.11 Task Group n - Enhancements for Higher Throughput |
| TX | Transmitter |

# MASSDIC-OFDM high throughput PHY specification

## Overview

This clause describes the PHY specification for the multiple antenna signal space diversity coded OFDM (MASSDIC-OFDM) system. The specification includes definitions of preamble, header, scrambler, modulations and error control coding for various data rates.

### Introduction

The PHY data rate supports up to 167 Mbps in mandatory mode and up to 333 Mbps in optional mode. The MASSDIC-OFDM extended from the OFDM technology specified in 802.11a standard includes the following features:

1. Transmission using multiple antennas (MA), also known as Multiple Input Multiple Output (MIMO), is adopted to significantly boost data rates without expanding the available 20 MHz bandwidth. Transmission using 2 independent data streams is mandatory while 3 and 4 are optional.

2. The constellation size of modulation is extended to 256-QAM to increase the bandwidth efficiency.

3. The number of subcarriers in an OFDM symbol is increased from 64 to 128 to gain guard interval efficiency.

4. Only PHY data rates more than 53 Mbps are newly defined while other data rates less than 54 Mbps are chosen to be identical to those defined in 802.11a.

5. For data rates identical to those defined in 802.11a, space-time block coding (STBC) scheme is used to enhance system performance.

6. Based on the fact that lower data rate operations are more likely to have longer channel delay spread due to possibly longer operation range, the guard intervals are variable with respect to different data rates to optimize the guard interval efficiency in different channel environments.

7. To enhance the performance of the OFDM technique in multipath fading channels without sacrificing data rates, a well known signal space diversity coding (SSDIC) technique suitable for OFDM called linear constellation precoding (LCP) is introduced as an option.

Due to the fact that the PHY overhead including preamble and header would degrade the overall PHY efficiency especially in higher data rate modes, e.g., 4x4 MIMO case, the following features are also adopted in the proposal:

8. No more overhead is needed for the PHY header, which has been embedded to the pilot tones of antenna 1 and 2 in the first OFDM symbol that conveys data.

9. New preamble structures are designed to minimize the overhead without sacrificing performance.

10. The maximum data length that can be addressed is extended from 4096 bytes as defined in 802.11a standard to 65536 bytes.

To maintain comparable operation range relative to that in 802.11a, a more powerful error control coding scheme using low density parity check code (LDPCC) is also proposed.  The proposed LDPCC has the following features:

11. Coding rates of 1/2, 2/3, 3/4 are separately designed with codeword length 2667 to optimize performance.

12. A new scheme to shorten codewords with hybrid code rate combination is proposed.

Because, it is very likely that the number of pad bits added to make the total OFDM symbols integral be so large to occupy more than 32 subcarriers, the approach to define the pad bits are modified to have the following feature:

13. The pad bits are put before the payload. If the number of pad bits is so large that more than 32 subcarriers are needed to convey them, 32 subcarriers in the first OFDM symbol are replaced with a repeated header. Then the new number of pad bits is recalculated to make the number of OFDM symbols in a packet integral. This feature enhances the robustness of the header with average probability 0.88 without adding any PHY overhead.

### Frequency Bands of Operation

The radio frequency is operated on the 5 GHz bands as specified in subclause 17.3.8.3 in IEEE Std 802.11a, 1999 Edition.

### Scope

This subclause describes the PHY services provided to the IEEE 802.11 wireless LAN MAC by the MASSDIC-OFDM system. The MASSDIC-OFDM PHY layer consists of the following protocol functions:

- A PHY convergence function, which adapts the capabilities of the physical medium dependent (PMD) system to the PHY service. This function is supported by the physical layer convergence procedure (PLCP), which defines a method of mapping the IEEE 802.11 PHY sublayer service data units (PSDU) into a framing format suitable for sending and receiving user data and management information between two or more stations using the associated PMD system.

- A PMD system whose function defines the characteristics and method of transmitting and receiving data through a wireless medium between two or more stations, each using the MASSDIC -OFDM PHY.

### MASSDIC-OFDM PHY functions

The MASSDIC-OFDM PHY architecture, operating in bands specified in 5.1.2, is depicted in the reference model shown in Figure 11 of IEEE Std 802.11, 1999 Edition (5.8). The PHY contains three functional entities: the PMD function, the PHY convergence function, and the layer management function. Each of these functions is described in 17.1.2.1 through 17.1.2.4 in IEEE Std 802.11a, 1999 Edition.

## MASSDIC-OFDM PLCP sublayer

### Introduction

This subclause provides a convergence procedure for PSDUs to be converted to and from PPDUs during transmission and reception. During transmission, the PSDU shall follow a PLCP preamble and header to form the PPDU. During reception, the preamble and header are processed to aid in the demodulation and delivery of the PSDU.

It is assumed that the MAC layer has successfully dealt with the backward compatibility issue in LN mode scenario. In this case, an N mode PLCP frame format is used during communication. Alternatively, an LN mode PLCP frame format is specifically designed to make legacy devices recognize the HT signal as if it were a legal legacy signal for medium reservation.

### PLCP frame format

The PPDU format in L mode is identical to that shown in Figure 107 in IEEE Std 802.11a, 1999 Edition and the preamble is specified in 17.3.3 in IEEE Std 802.11a, 1999 Edition except that all transmission antennas transmit the same preamble.

The PPDU format in N mode is shown in Figure 1 consisting of a PLCP preamble, a PLCP header, possibly pad bits with a repeated header, and PSDU.

The PLCP preamble is composed of 10 short training sequences and 2 long training sequences. The format of the PLCP preamble is defined in subclause 5.2.4 for each mode of operation.

The PLCP header is composed of 64 bits of information in which 8 subparts are defined, including configurations, and length-related parameters for PPDU transmission. The PLCP header is coded with code rate R=1/2 defined in subclause 17.3.5.5 in IEEE Std 802.11a, 1999 Edition. The details of the PLCP header are specified in 5.2.5.

The PSDU is composed of a payload ranging from 1 to 65536 bytes, an FCS using CRC-16, and possibly pad bits. The pad bits are appended if necessary to make total transmission OFDM symbols integral. A repeated header is substituted for the front-most pad bits if the number of pad bits is large enough to occupy more than 32 subcarriers.

Figure 1 ─ PPDU Frame Format for N mode

The PPDU format in LN mode is shown in Figure 2 consisting of a PLCP preamble, a legacy signal symbol (denoted by L SIGNAL), a PLCP header, possibly pad bits with a repeated header, and PSDU.

In N mode access, the PLCP preamble is composed of 10 short training sequences and 2 long training sequences. The format of the PLCP preamble is defined in subclause 5.2.4.1 for each mode of operation.

In LN mode access, a legacy signal symbol, L SIGNAL, identical to the SIGNAL field defined in subclause 17.3.4 of IEEE Std 802.11a, 1999 Edition except that the reserved bit in the SIGNAL field shall be set to 1 in order to tell a MASSDIC-OFDM transmission from a legacy OFDM transmission. The rule for transmitting L SIGNAL for each antenna is specified in 5.2.4.2. The time interval when the medium will be occupied by the MASSDIC-OFDM PPDU transmission is signaled to legacy stations via the LENGTH and RATE fields of the L Signal symbol.

Figure 2 ─ PPDU Frame Format for LN mode

#### Rate-dependent parameters and timing related parameters

There are totally 29 data rates defined in this proposal. Among those data rates, 8 data rates, including 6, 9, 12, 18, 24, 36, 48, 54 Mbps are identical to those defined in IEEE Std 802.11a, 1999 Edition; 7 data rates, including 53, 70, 79, 105, 125, 148, and 167 Mbps are mandatory HT data rates spatial-multiplexed transmitted via 2 antennas; Other 14 data rates, including 80, 105, 118, 158, 188, 222, 250 Mbps are optional HT data rates spatial-multiplexed transmitted via 3 antennas, while 106, 140, 158, 211, 250, 296, and 333 Mbps are optional HT data rates spatial-multiplexed transmitted via 4 antennas. The details of the HT and legacy parameters are specified in subclauses 5.2.2.1.1 and 5.2.2.1.2 respectively.

##### HT parameters

Only data rates more than 53 Mbps are defined in HT access. The rate-dependent and timing related parameters for NT=2, 3, and 4 are listed in Table 1, 2, and 3 respectively. In addition, the subcarrier spacing is F=20/128=0.1563 MHz.

Table 1 ─ Rate-dependent and timing related parameters for NT=2

| Rate Code | 011111 | 010101 | 010111 | 011001 | 011011 | 010001 | 010011 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Info.Data Rate Mbps | 53 | 70 | 79 | 105 | 125 | 148 | 167 |
| QAM Constellation | 16 | 16 | 16 | 64 | 64 | 256 | 256 |
| Coded bits per subcarrier per antenna (NBPSC) | 4 | 4 | 4 | 6 | 6 | 8 | 8 |
| Coding Rate (R) | 1/2 | 2/3 | 3/4 | 2/3 | 3/4 | 2/3 | 3/4 |
| Number of Pilot Tones (NSP) | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Number of Data Tones (NSD) | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Coded bits per MA-OFDM symbol (NCBPS) | 800 | 800 | 800 | 1200 | 1200 | 1600 | 1600 |
| Data bits per MA-OFDM symbol (NDBPS) | 400 | 533 | 600 | 800 | 900 | 1067 | 1200 |
| Info. Length s (TFFT) | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |
| Cyclic Prefix s (TGI) | 1200 | 1200 | 1200 | 1200 | 800 | 800 | 800 |
| Number of Null Tones | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Symbol Length s (TSYM) | 7.6 | 7.6 | 7.6 | 7.6 | 7.2 | 7.2 | 7.2 |

Table 2 ─ Rate-dependent and timing related parameters for NT=3

| Rate Code | 101111 | 100101 | 100111 | 101001 | 101011 | 100001 | 100011 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Info.Data Rate Mbps | 80 | 105 | 118 | 158 | 188 | 222 | 250 |
| QAM Constellation | 16 | 16 | 16 | 64 | 64 | 256 | 256 |
| Coded bits per subcarrier per antenna (NBPSC) | 4 | 4 | 4 | 6 | 6 | 8 | 8 |
| Coding Rate (R) | 1/2 | 2/3 | 3/4 | 2/3 | 3/4 | 2/3 | 3/4 |
| Number of Pilot Tones (NSP) | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Number of Data Tones (NSD) | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Coded bits per MA-OFDM symbol (NCBPS) | 1200 | 1200 | 1200 | 1800 | 1800 | 2400 | 2400 |
| Data bits per MA-OFDM symbol (NDBPS) | 600 | 800 | 900 | 1200 | 1350 | 1600 | 1800 |
| Info. Length s (TFFT) | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |
| Cyclic Prefix s (TGI) | 1200 | 1200 | 1200 | 1200 | 800 | 800 | 800 |
| Number of Null Tones | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Symbol Length s (TSYM) | 7.6 | 7.6 | 7.6 | 7.6 | 7.2 | 7.2 | 7.2 |

Table 3 ─ Rate-dependent and timing related parameters for NT=4

| Rate Code | 111111 | 110101 | 110111 | 111001 | 111011 | 110001 | 110011 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Info.Data Rate Mbps | 106 | 140 | 158 | 211 | 250 | 296 | 333 |
| QAM Constellation | 16 | 16 | 16 | 64 | 64 | 256 | 256 |
| Coded bits per subcarrier per antenna (NBPSC) | 4 | 4 | 4 | 6 | 6 | 8 | 8 |
| Coding Rate (R) | 1/2 | 2/3 | 3/4 | 2/3 | 3/4 | 2/3 | 3/4 |
| Number of Pilot Tones (NSP) | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Number of Data Tones (NSD) | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Coded bits per MA-OFDM symbol (NCBPS) | 1600 | 1600 | 1600 | 2400 | 2400 | 3200 | 3200 |
| Data bits per MA-OFDM symbol (NDBPS) | 800 | 1067 | 1200 | 1600 | 1800 | 2133 | 2400 |
| Info. Length s (TFFT) | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |
| Cyclic Prefix s (TGI) | 1200 | 1200 | 1200 | 1200 | 800 | 800 | 800 |
| Number of Null Tones | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Symbol Length s (TSYM) | 7.6 | 7.6 | 7.6 | 7.6 | 7.2 | 7.2 | 7.2 |

##### Legacy parameters

Data rates for 802.11a Legacy access, including 6, 9, 12, 18, 24, 32, 48, and 54 Mbps, are specified in subclauses 17.3.2.2 and 17.3.2.3 in IEEE Std 802.11a, 1999 Edition. Legacy data rates can be operated in any access modes as specified in 5.2.2.1.2.1 and 5.2.2.1.2.2.

###### L mode

In L mode access, all transmission antennas transmit the same data streams.

###### N and LN mode

In N and LN mode access, the data stream is coded by STBC specified in subclause 5.2.7.

### MASSDIC-OFDM TX architecture

The TX architecture for minatory mode of the MASSDIC-OFDM is illustrated in Figure 3. First, the PSDU plus pad bits are input to the scrambler described in 5.2.6.4. Then, the scrambled bit streams are encoded by LDPCC specified in 5.2.6.5. The encoded data is then interleaved according to 5.2.6.6 if the option of the interleaver is on. After that, the bit stream is QAM mapped to different constellation according to the data rate. The modulated signals are then coded with LCP and subcarrier grouping as specified in 5.2.6.10. After that, the data stream is spatially multiplexed to 2 data substreams, each of which is passed to a 128 IFFT block followed by cyclic prefix extension with respect to different guard intervals for different data rates. Then the base band signals are input to RF parts for transmission by 2 antennas.

LDPCC

1

QAM mapping

Linear constellation Precoding 

Subcarrier grouping

128-IFFT

128-IFFT

CP

CP

RF

RF

Scrambler

Figure 3 ─ MASSDIC-OFDM TX architecture

### PLCP preamble (SYNC)

The PLCP preamble filed is used for synchronization, including signal detection, auto gain controlling, frame boundary estimation, frequency and timing offset estimation, and channel estimation. Two different types of preambles are specified in subclauses, 5.2.4.1 and 5.2.4.2 for N mode and LN mode operations respectively.

#### N mode Preamble

In N mode access, preambles are designed to optimize the PHY efficiency without sacrificing performance. The preamble structure in N mode for NT=2, 3, 4 are depicted in Figure 4, Figure 5, and Figure 6 respectively.

##### N mode Preamble for NT=2

Figure 4 shows the preamble structure in N mode for NT=2. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. STS1 and STS2 are generated as follows:

- Generate time domain signals s1 and s2 from frequency domain signals S1 and S2 given by  and  respectively via 128-IFFT.

- Take the first 16 samples of s1 and s2, namely u1 and u2.

- Get unity x1 and x2 via Gram-Schmidt procedure from u1 and u2.

- Set  and  .

- The STS1 and STS2 in Figure 4 are derived respectively by repeating six v1 and v2 followed by four repeated -v1 and -v2

Figure 4 ─ Preamble structure in N mode for NT=2

For long training sequences, LTS1 and LTS2 are generated as follows:

- Define a basic signal L from .

- L1 and L2 are derived from L by  and  respectively, where the notation (a:b:c) means (starting index: step value: ending index).

- Generate time domain signals LTS1 and LTS2 from frequency domain signals L1 and L2 respectively via 128-IFFT.

##### N mode Preamble for NT=3

Figure 5 shows the preamble structure in N mode for NT=3. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. STS1, ST2 and STS3 are generated as follows:

- Generate time domain signals s1, s2, and s3 from frequency domain signals S1, S2, and S3 given by , , and  respectively via 128-IFFT.

- Take the first 16 samples of s1 , s2, and s3, namely u1, u2 , and u3.

- Get unity x1, x2, and x3 via Gram-Schmidt procedure from u1, u2, and u3.

- Set  ,, and .

- The STS1 to STS3 in Figure 5 are derived respectively by repeating six v1, v2, and v3 followed by four repeated -v1, -v2, and –v3.

For long training sequences, LTS1 to LTS3 are generated as follows:

- Define a basic signal L from

- L1 to L3 are derived from L by , , and  respectively, where the notation (a:b:c) means (starting index: step value: ending index).

- Generate time domain signals LTS1 to LTS3 from frequency domain signals L1 to L3 respectively via 128-IFFT.

Figure 5 ─ Preamble structure in N mode for NT=3

##### N mode Preamble for NT=4

Figure 6 shows the preamble structure in N mode for NT=4. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. STS1 to STS4 are generated as follows:

- Generate time domain signals s1, s2, s3, and s4 from frequency domain signals S1, S2, S3, and S4 given by , , , and  respectively via 128-IFFT.

- Take the first 16 samples of s1 to s4, namely u1 to u4.

- Get unity x1 to x4 via Gram-Schmidt procedure from u1 to u4.

- Set  for each i.

- The STS1 to STS4 in Figure 6 are derived respectively by repeating six v1, v2, v3, and v4 followed by four repeated -v1, -v2, –v3 , and –v4.

For long training sequences, LTS1 to LTS4 are generated as follows:

- Define a basic signal L from .

- L1 to L4 are derived from L by  to  respectively, where the notation (a:b:c) means (starting index: step value: ending index).

- Generate time domain signals LTS1 to LTS4 from frequency domain signals L1 to L4 respectively via 128-IFFT.

Figure 6 ─ Preamble structure in N mode for NT=4

#### LN mode Preamble

The preamble structure in LN mode for NT=2, 3, 4 are respectively depicted Figure 7, Figure 8, and Figure 9. In LN mode access, preambles are designed to be recognized by legacy devices. Therefore, a Legacy SIGNAL field is appended after the LTS field.

##### LN mode Preamble for NT=2

Figure 7 shows the preamble structure in N mode for NT=2. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. The STSs are generated as follows:

- The L-STS in antenna 1 is identical to the legacy STS field specified in subclause 17.3.3 in IEEE Std 802.11a, 1999 Edition.

- Right circular shift the L-STS in antenna 1 by 400 ns to form the STS in antenna 2.

For long training sequences, LTS1 and LTS2 are identical to those defined in subclause 5.2.4.1.1.

For the Legacy SIGNAL field, the L SIG in antenna 1 is identical to that defined in 17.3.4 in IEEE Std 802.11a, 1999 Edition. The L SIG in antenna 2 is obtained by right circular shifting the L SIG in antenna 1 by 800 ns.

Figure 7 ─ Preamble structure in LN mode for NT=2

##### LN mode Preamble for NT=3

Figure 8 shows the preamble structure in N mode for NT=3. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. The STSs are generated as follows:

- The L-STSs in antenna 1 and 2 are identical to those defined in 5.2.4.2.1.

- Right circular shift the L-STS in antenna 1 by 600 ns to form the L-STS in antenna 3.

For long training sequences, LTS1 to LTS3 are generated as follows:

- Generate time domain signal LTS1 from , where the frequency domain signal L1 is defined in  via 128-IFFT.

- Right circular shift the LTS1 in antenna 1 by 800 ns and 1200ns respectively to form LTS2 and LTS3.

For the Legacy SIGNAL field, the L SIG in antenna 1 and 2 are identical to those defined in 5.2.4.1.1. The L SIG in antenna 3 is obtained by right circular shifting the L SIG in antenna 1 by 600 ns.

Figure 8 ─ Preamble structure in LN mode for NT=3

##### LN mode Preamble for NT=4

Figure 9 shows the preamble structure in N mode for NT=4. The preamble is composed of 10 short training sequences (STS) and 1 long training sequence (LTS) indexed according to the antenna number in the figure. The STSs are generated as follows:

- The L-STS in antenna 1 and 2 is identical to those defined in 5.2.4.2.1.

- Right circular shift the L-STS in antenna 1 by 600 ns and 200 ns respectively to form the L-STSs in antenna 3 and 4.

For long training sequences, LTS1 to LTS4 are identical to those defined in subclause 5.2.4.1.3.

For the Legacy SIGNAL field, the L SIG in antenna 1 and 2 are identical to those defined in 5.2.4.1.1. The L SIGs in antenna 3 and 4 are obtained by right circular shifting the L SIG in antenna 1 by 400 ns and 600ns respectively.

Figure 9 ─ Preamble structure in LN mode for NT=4

### PLCP header

The PLCP header is encoded by R=1/2 convolutional code as specified in subclause 17.3.5.5 in IEEE Std 802.11a 1999 Edition and then is 16-QAM modulated and substituted for the pilot tones of the first OFDM symbol in the first 2 antennas. The PLCP header is not scrambled. The PLCP header is composed of 64 bits of information in which 8 subparts are defined, including configurations, and length-related parameters for PPDU transmission. The bit assignment for the PLCP header is defined in Figure 10.

Figure 10 ─ PLCP header bit assignment

#### Data Rate (RATE)

As shown in Figure 10, b0 to b5 in RATE field shall encode the PHY data rate defined in 5.2.2.1.

#### Reserved (R), Interleaver (I) and Header Tail (TAIL)

In Figure 10, b6 and b7 in R field shall be reserved for further applications and b9 shall indicate the use of interleaver with 1 indicating the interleaver is on. The last 6 bits b58 to b63 in the TAIL field are set to zero.

#### Service field (SERVICE)

The b26 through b41 shown in Figure 10 define the service field. The specification for the service field is identical to that specified in subclause 17.3.5.1 in IEEE Std 802.11a, 1999 Edition.

#### PLCP length field

The PLCP length field shall be an unsigned 16-bit integer indicating the number of octets in the payload in Figure 1 that the MAC is currently requesting the PHY to transmit. In Figure 10, b10 to b25 in the LENGTH field shall encode the length of the payload in PSDU, with the least significant bit (LSB) being transmitted first.

#### CCITT CRC-16 for the HCS

In Figure 10, b42 to b57 in the HCS field is the header check sequence for b0 to b41. The PLCP header shall be protected with HCS via the CCITT V.41 standard, CRC-16. The HCS shall be the ones complement of the remainder by operating the modulo-2 division of the PLCP header with the polynomial: . The structure for implementing the CCITT CRC-16 is shown in Figure 11, where all the registers are preset to all ones and  is outputted first.

Figure 11 ─ CCITT CRC-16 structure

### Data field

The data field contains PSDU, possibly repeated header, FCS, and the pad bits, as described below.

#### Pad bits (PAD)

The additional pad bits are added such that the number of OFDM symbol, NSYM, is an integer. The pad bits are computed in terms of the length of the PSDU (LENGTH) as the following steps.

First, we have to compute NPADI, the number of pad bits such that the number of codeword NC is an integer. The pad bits are computed as follows:

where NB is the codeword block size and  is the changed code rate of the last codeword as described in Table 5. Note that the NPADI bits are not transmitted.

Next, we need to calculate NPAD, the number of pad bits such that the number of coded bits becomes a multiple of NCBPS.

.

The pad bits are randomly generated with equal probability of ones and zeros.

#### Repeated header

If the number of pad bits , then a 16-QAM modulated repeated header is sequentially inserted into the 16 data subcarriers {d53, d59, d65, d71, d78, d84, d90, d96, d3, d9, d15, d21, d28, d34, d40, d46} of the 2nd  antenna and the 16 data subcarriers {d53, d59, d65, d71, d78, d84, d90, d96, d3, d9, d15, d21, d28, d34, d40, d46} of the 1st antenna. Then the number of pad bits is recalculated by the following equation:

.

#### Frame check sequence

The payload is protected by the frame check sequence via the same CRC-16 structure as described in 5.2.5.5.

#### PLCP DATA scrambler and descrambler

The data field composed of PSDU and the pad bits shall be scrambled with a length-127 frame-synchronous scrambler. The scrambler and descrambler use the following generator polynomial:

which is the same as that in IEEE Std 802.11a, 1999 Edition, as illustrated in Figure 12. The sequences generated by this scrambler are (00001110 11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100 00101010 11111010 01010001 10111000 1111111) with period 127 and the initial state is “all ones”.

Data input

Scrambled data output

Figure 12 ─ Data scrambler

#### Low density parity check codes (LDPCC)

##### LDPCC overview

An LDPCC is an  block code with a sparse parity check matrix of size , where  is the codeword block size,  is the length of parity check block, and  is the number of information bits, .

Here, we choose systematic encoding. Thus the codeword can be written as

where  =  is the vector of information bits and  =  is the parity check bits. And we can separate  into two parts as follows:

where  is an  matrix corresponding to the information bits of the codeword. And  is an  square matrix corresponding to the parity check bits of the codeword. Hence the codeword is satisfied with the following equation:

.

Under binary operation, we have

.

The generator matrix is

.

For extended irregular repeat-accumulate (eIRA) codes,  is randomly generated to ensure low BER through the density evolution algorithm and  is a “twin diagonal” matrix designed for efficient encoding. The structure is

.

Then, the inverse of  is a full lower-triangular matrix. The inverse operation of  can be implemented as a differential encoder as shown in Figure 13.

Figure 13 ─ FEC encode structure

For detail of  please refer to 802.11-04/1018.

##### Code rates and shortened code

The data field composed of PSDU and the pad bits shall be coded with an LDPC encoder of the desired coding rate. For various protections of data or different requirements of transmission throughput, there are three rates provided: 3/4, 2/3 and 1/2 in Table 4 .

Table 4 ─ Provided codeword rates

|  | N | M | K |
| --- | --- | --- | --- |
| R=1/2 | 2676 | 1338 | 1338 |
| R=2/3 | 2676 | 892 | 1784 |
| R=3/4 | 2676 | 692 | 2007 |

When encoding the information data, we divide the data filed into several blocks with the desired size , and encode each block with the LDPC encoder. However, the size of data is not always a multiple of . Hence, we need a method to encode the remainder information data. Furthermore, for shorter information bits, we encode them by padding zeros. Here, a short information block with size  is encoded by padding first  information bits with zeros. Then the new information block is  =  and the codeword is  = . We only transmit  =  with the code rate .

Figure 14 ─ Short information block

In addition, for shorter information block sizes, it may not be efficient to encode them in the original code rate. Hence, we support a simple switch rate algorithm to enhance the transmission efficiency. We decide a threshold of information block size  by simulation. If the block size  is smaller than , we replace the original rate with a higher one to reduce the number of parity check bits.

Table 5 ─ The threshold of rate switch for codeword shortening

| The original code rate | The threshold of information bit () | The changed code rate |
| --- | --- | --- |
| R=1/2 | 850 | R=2/3 |
|  | 480 | R=3/4 |
| R=2/3 | 1000 | R=3/4 |

#### Space-frequency interleaving

For multiple TX antennas, space-frequency interleaver is considered. First, the data bits are interleaved by a block interleaver with block size corresponding to NCBPS and NT such that the data bits are permuted across all bit positions of the constellation and across subcarriers. Next, we use the OFDM-symbol-level multiplexing method to permute the data bits across all TX antennas.

##### Bit interleaver

The block interleaver can be obtained by the following two permutations. Denote by k the index before the first permutation; i the index after the first permutation and before the second permutation; and j the index after the second permutation.

The first permutation can be described as

.

The second permutation can be described as

,

where

.

The deinterleaver can be also obtained by the following two permutations. Denote by i the index before the first permutation; j the index after the first permutation and before the second permutation; and k the index after the second permutation.

The first permutation can be described as

,

where s is defined in Equation .

The second permutation can be described as

.

##### OFDM-symbol-level multiplexing

Let  be the permutation such that the th input bit becomes the ith bit after bit interleaving. Denote by  the index of the input bit which is placed at the ith bit position in the jth TX antenna after space-frequency interleaving. The relationship between  and  can be illustrated as the following equation:

,

where  and .

The OFDM-symbol-level multiplexing scheme is shown in Figure 15.

OFDM-symbol-level multiplexing

1

2

N=NT

L=NCBPS

NL

Figure 15 ─ OFDM-symbol-level multiplexing scheme

#### Subcarrier modulation mapping

The subcarriers are modulated by using BPSK, QPSK, 16-QAM, 64-QAM, or 256-QAM modulation. The mappings are Gray-coded, as shown in Figure 16. To normalize the constellations to an average power of 1, each constellation point is multiplied by the normalization factor, KMOD listed in Table 6. The output value d is described as the following equation:

.

For BPSK, b0 determines the I value and Q is zero. For QPSK, b0 determines the I value and b1 determines the Q value. For 16-QAM, b0b1 determines the I value and b2b3 determines the Q value. For 64-QAM, b0b1b2 determines the I value and b3b4b5 determines the Q value. For 256-QAM, b0b1b2b3 determines the I value and b4b5b6b7 determines the Q value. The corresponding mappings are illustrated in Table 7 to Table 11.

Table 6 ─ Modulation-dependent normalization factor KMOD

| Modulation | KMOD |
| --- | --- |
| BPSK | 1 |
| QPSK |  |
| 16-QAM |  |
| 64-QAM |  |
| 256-QAM |  |

I

Q

BPSK

-1

1

0

1

b0

I

Q

-1

1

-1

1

01

11

10

00

b0b1

QPSK

1

I

Q

-1

1

-1

0111

1111

1101

0101

16-QAM

0011

1011

1010

1110

0110

0010

1000

0100

0001

0000

1100

1001

-3

3

3

-3

b0b1b2b3

010100

110011

64-QAM

-1

-3

-5

-7

1

3

7

5

1

7

3

5

-1

-7

-5

-3

Q

I

000000

001000

000001

001001

011000

000011

010000

011001

000010

001010

001011

011010

011011

010011

010001

010010

110001

110010

111010

111011

111001

110000

111000

101010

101011

101001

101000

100010

100011

100001

100000

b0b1b2b3b4b5

000110

001110

000111

001111

011110

000101

010110

011111

000100

001100

001101

011100

011101

010101

010111

110110

111110

110111

111111

101110

110101

100110

101111

110100

111100

111101

101100

101101

100101

100111

100100

Q

00000000

00010000

00110000

00100000

01100000

01110000

01010000

01000000

00000001

00010001

00110001

00100001

01100001

01110001

01010001

01000001

00000011

00010011

00110011

00100011

01100011

01110011

01010011

01000011

00000010

00010010

00110010

00100010

01100010

01110010

01010010

01000010

00000110

00010110

00110110

00100110

01100110

01110110

01010110

01000110

00000111

00010111

00110111

00100111

01100111

01110111

01010111

01000111

00000101

00010101

00110101

00100101

01100101

01110101

01010101

01000101

00000100

00010100

00110100

00100100

01100100

01110100

01010100

01000100

00001100

00011100

00111100

00101100

01101100

01111100

01011100

01001100

00001101

00011101

00111101

00101101

01101101

01111101

01011101

01001101

00001111

00011111

00111111

00101111

01101111

01111111

01011111

01001111

00001110

00011110

00111110

00101110

01101110

01111110

01011110

01001110

00001010

00011010

00111010

00101010

01101010

01111010

01011010

01001010

00001011

00011011

00111011

00101011

01101011

01111011

01011011

01001011

00001001

00011001

00111001

00101001

01101001

01111001

01011001

01001001

00001000

00011000

00111000

00101000

01101000

01111000

01011000

01001000

11001100

11011100

11111100

11101100

10101100

10111100

10011100

10001100

11001101

11011101

11111101

11101101

10101101

10111101

10011101

10001101

11001111

11011111

11111111

11101111

10101111

10111111

10011111

10001111

11001110

11011110

11111110

11101110

10101110

10111110

10011110

10001110

11001010

11011010

11111010

11101010

10101010

10111010

10011010

10001010

11001011

11011011

11111011

11101011

10101011

10111011

10011011

10001011

11001001

11011001

11111001

11101001

10101001

10111001

10011001

10001001

11001000

11011000

11111000

11101000

10101000

10111000

10011000

10001000

11000000

11010000

11110000

10100000

10010000

11100000

10110000

10000000

11000001

11010001

11110001

10100001

10010001

11100001

10110001

10000001

11000011

11010011

11110011

10100011

10010011

11100011

10110011

10000011

11000010

11010010

11110010

10100010

10010010

11100010

10110010

10000010

11000110

11010110

11110110

10100110

10010110

11100110

10110110

10000110

11000111

11010111

11110111

10100111

10010111

11100111

10110111

10000111

11000101

11010101

11110101

10100101

10010101

11100101

10110101

10000101

11000100

11010100

11100100

10110100

10000100

11110100

10010100

10100100

I

256-QAM

b0b1b2b3b4b5b6b7

15

-15

-15

15

Figure 16 ─ BPSK, QPSK, 16-QAM, 64-QAM, and 256-QAM constellation bit encoding

Table 7 ─ BPSK encoding table

| Input bits (b0) | I-out | Q-out |
| --- | --- | --- |
| 0 | -1 | 0 |
| 1 | 1 | 0 |

Table 8 ─ QPSK encoding table

| Input bits (b0) | I-out |  | Input bits (b1) | Q-out |
| --- | --- | --- | --- | --- |
| 0 | -7 |  | 0 | -7 |
| 1 | -5 |  | 1 | -5 |

Table 9 ─ 16-QAM encoding table

| Input bits (b0b1) | I-out |  | Input bits (b2b3) | Q-out |
| --- | --- | --- | --- | --- |
| 00 | -3 |  | 00 | -3 |
| 01 | -1 |  | 01 | -1 |
| 11 | 1 |  | 11 | 1 |
| 10 | 3 |  | 10 | 3 |

Table 10 ─ 64-QAM encoding table

| Input bits (b0b1b2) | I-out |  | Input bits (b3b4b5) | Q-out |
| --- | --- | --- | --- | --- |
| 000 | -7 |  | 000 | -7 |
| 001 | -5 |  | 001 | -5 |
| 011 | -3 |  | 011 | -3 |
| 010 | -1 |  | 010 | -1 |
| 110 | 1 |  | 110 | 1 |
| 111 | 3 |  | 111 | 3 |
| 101 | 5 |  | 101 | 5 |
| 100 | 7 |  | 100 | 7 |

Table 11 ─ 256-QAM encoding table

| Input bits (b0b1b2b3) | I-out |  | Input bits (b4b5b6b7) | Q-out |
| --- | --- | --- | --- | --- |
| 0000 | -15 |  | 0000 | -15 |
| 0001 | -13 |  | 0001 | -13 |
| 0011 | -11 |  | 0011 | -11 |
| 0010 | -9 |  | 0010 | -9 |
| 0110 | -7 |  | 0110 | -7 |
| 0111 | -5 |  | 0111 | -5 |
| 0101 | -3 |  | 0101 | -3 |
| 0100 | -1 |  | 0100 | -1 |
| 1100 | 1 |  | 1100 | 1 |
| 1101 | 3 |  | 1101 | 3 |
| 1111 | 5 |  | 1111 | 5 |
| 1110 | 7 |  | 1110 | 7 |
| 1010 | 9 |  | 1010 | 9 |
| 1011 | 11 |  | 1011 | 11 |
| 1001 | 13 |  | 1001 | 13 |
| 1000 | 15 |  | 1000 | 15 |

#### Pilot subcarrier

In each OFDM symbol, 16 of the subcarriers are dedicated to pilot signals in order to make coherent detection robust against frequency offsets and phase noise. These pilot signals shall be put in subcarriers –58, -57, –55, –45, –35, –25, –15 –5, 5, 15, 25, 35, 45, 55, 57 and 58. The contribution due to the pilot subcarriers for the nth OFDM symbol is produced by Fourier transform of sequence P, given by

.

The polarity of the pilot subcarriers is controlled by the sequence, pn, which is a cyclic extension of the 127 elements sequence and is given by

.

One element of this sequence is used for one OFDM symbol.

#### MIMO-OFDM modulation

The nth OFDM symbol for the ith antenna can be written as

,

where NSD is the number of data subcarriers, NST is the number of total subcarriers, and the  mapping function is defined as

.

The subcarrier frequency allocation is shown in Figure 17. To avoid difficulties in D/A and A/D converter offsets and carrier feedthrough in the RF system, the subcarrier falling at DC (0th subcarrier) is not used.

Subcarrier Numbers

-55

-45

-35

-15

-25

-5

0

5

15

45

25

35

55

57 58

P-58

-58 -57

P-55

P-57

P-45

P-35

P-25

P-15

P-5

P5

P15

P25

P35

P45

P55

P57

P58

d0

d1

d9

d18

d10

d19

d27

d28

d36

d37

d45

d46

d49

d50

d53

d54

d62

d63

d71

d72

d80

d81

d89

d90

d98

d99

DC

Figure 17 ─ Subcarrier frequency allocation

The concatenation of NSYM OFDM symbols can now be written as

.

#### Linear constellation precoding (optional)

##### Linear constellation precoding construction

Linear constellation precoding (LCP) is described by the  matrix  with entries over the complex field satisfying the transmit power constraint. In this proposal,  is adopted and the precoding matrix is given by

.

##### Subcarrier grouping

Denote by d0~d99 the data subcarriers of the 1st antenna, d100~d199 the data subcarriers of the 2nd antenna, d200~d299 the data subcarriers of the 3rd antenna, and d300~d399 the data subcarriers of the 4th antenna. The total number of data subcarriers for  MIMO system is . We further define the subcarrier selector matrix as

,

where  is an  identity matrix, and

,

, and

.

The linear precoded signal can be expressed as

,

where .

The subcarrier grouping is illustrated in Figure 18.

Group 1

Group 50

…

…

…

…

…

Antenna 1

Antenna 2

d0

d49

d50

d99

d100

d124

d125

d174

NT=2

d175

d199

…

Group 1

Group 75

…

…

Antenna 1

Antenna 2

d0

d74

d75

d99

d100

d150

d149

d199

d200

d224

d225

Antenna 3

…

…

…

…

…

NT=3

d299

…

Group 2

Group 100

…

…

…

…

Antenna 2

Antenna 4

d149

d150

d199

d300

d324

d325

d374

d375

d399

…

…

NT=4

Group 1

Group 99

…

…

…

…

Antenna 1

Antenna 3

d49

d50

d99

d200

d224

d225

d274

d275

d299

…

d0

d100

Figure 18 ─ Subcarrier grouping for LCP

### Space Time Block Code (STBC)

When HT devices are communicating using legacy data rates, Space Time Block Code is used for performance enhancement.

#### STBC for NT=2 and 3

For NT=2, there exists an orthogonal STBC with rate 1. And the square transmission matrix  is shown in :

where  and   are two OFDM symbols,  means the OFDM symbol whose each subcarrier symbol is the conjugate of the one in , and  means the OFDM symbol whose each subcarrier symbol is the minus of the one in . Assume there are two OFDM symbols  and  transmitted. At the first time slot, we transmit  and  in the first and second antennas. At the second time slot, we transmit  and  in the first and second antennas.

For NT=3, the transmission matrix is in . However, if we want to keep the rate 1, all possible transmission matrices will lose the orthogonal relation between each row. And we use the similar method in NT=2 to transmit the OFDM symbols.

#### STBC for NT=4

For NT=4, there does not exist any orthogonal matrix for rate 1. Hence, we construct the matrix  by repeating  format. The transmission matrix  is shown in .

(END)