IEEE 802.11k
Wireless LANs

Proposal and Normative Text for a Scheduled Autonomous Probe Response Generation Function

Date: September 9, 2004

Author:

Steve Emeott

Steve.Emeott@motorola.com

Walter Johnson

Walter.Johnson@motorola.com

Floyd Simpson

Floyd.Simpson@motorola.com

Stephen Wang

Stephen.Wang@motorola.com

Tim Wilson

Tim.Wilson@motorola.com

Motorola, Inc.

Abstract

#### This document contains normative text for the proposed addition of a Scheduled Autonomous Probe Response (SAPR) generation function, which allows an AP to autonomously transmit Probe Response frames at periodic intervals.  The SAPR interval is indicated by a SAPR subfield included in a SAPR information element within Beacon and Probe Response frames.   The presence of scheduled, autonomous Probe Response frames with the SAPR information element allows stations to rapidly take measurements and find neighbor APs using passive scanning.

# Normative Text for the Proposed Enhancements

7 Frame formats

7.2 Format of individual frame types

7.2.3 Management frames

7.2.3.1 Beacon frame format

Insert the following row at the appropriate location in Table 5 in 7.2.3.1 as shown:

| Order | Information | Notes |
| --- | --- | --- |
| TBD | Scheduled Autonomous Probe Response | The Scheduled Autonomous Probe Response element shall be present if dot11ScheduledAutonomousProbeResponseFunctionEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Insert the following row at the appropriate location in Table 12 in 7.2.3.9 as shown:

Table 12 – Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| TBD | Scheduled Autonomous Probe Response | The Scheduled Autonomous Probe Response element shall be present if dot11ScheduledAutonomousProbeResponseFunctionEnabled is true. |

### 7.3.2 Information elements

Insert the element IDs shown in the following table into Table 20 replacing previously reserved values:

| Information Element | Element ID |
| --- | --- |
| Scheduled Autonomous Probe Response | TBD |

Insert the following subclauses with the figures included therein, after subclause 7.3.2.27:

#### 7.3.2.28 Scheduled Autonomous Probe Response information element

The Scheduled Autonomous Probe Response information element contains a Scheduled Autonomous Probe Response Inverval field as shown in Figure kXX.

|  | Element ID | Length | Scheduled Autonomous Probe Response Interval |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure kXX—Scheduled Autonomous Probe Response information element format

The Length field shall be set to 2.

The Scheduled Autonomous Probe Response Interval field shall contain the number of time units (TU) between two consecutive scheduled, autonmous Probe Response frame transmissions. The Scheduled Autonomous Probe Response interval shall be smaller than regular beacon intervals.

11 MAC sublayer management entities

Insert the following subclauses with the figures included therein, after subclause 11.8:

11.9 Usage of Scheduled Autonomous Probe Response element

The presence of the Scheduled Autonomous Probe Response (SAPR) information element in a Probe Response or Beacon frame indicates that an AP supports an autonomous probe response generation function and generates Probe Response frames at the interval specified in the element.

APs supporting the Scheduled Autonomous Probe Response (SAPR) generation function shall automatically generate Probe Response frames including the SAPR information element at the interval specified by the Scheduled Autonomous Probe Response Interval field, as illustrated in Figure kZZ.  The SAPR generation function shall transmit the Probe Response frame including the SAPR information element when the value of the TSF Timer modulo the SAPR interval (converted to microseconds) equals 0.   The Probe Response frame generated by the SAPR generation function shall be addressed to the broadcast destination address.

Beacon

Beacon

SAPR

SAPR

SAPR Interval

Beacon Interval

Figure kZZ - Scheduled Autonomous Probe Response generation (informative)

# Annex A – PICS

# A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9

### A.4.3 IUT configuration

Append this entry to the end of this table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF11 | Is Scheduled Autonomous Probe Responses implemented? | 7.2.3.1, 7.3.2.9, 7.3.2.28, 11.9 | O | Yes, No |

### A.4.4.1 MAC protocol capabilities

Insert this new item after PC11.10 in A.4.4.1:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| PC11.11 | Scheduled Autonomous Probe Response Function | 11.9 | CF11:M | Yes, No. N/A |

Annex D

In the dot11StationConfig table of Annex D, insert the following entries after the “dot11RadioMeasurementEnabled” entry:

dot11ScheduledAutonomousProbeResponseFunctionCapable		TruthValue,

dot11ScheduledAutonomousProbeResponseFunctionEnabled		TruthValue,

dot11ScheduledAutonomousProbeResponseInterval			INTEGER,

Insert the following elements to the dot11StationConfigTable element definitions after dot11RadioMeasurementEnabled and renumber dot11RadioMeasuremtnProbeDelay to 42:

dot11ScheduledAutonomousProbeResponseFunctionCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the AP is capable of supporting the Scheduled Autonomous Probe Response Generation Function. The capability is disabled otherwise. The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 39 }

dot11ScheduledAutonomousProbeResponseFunctionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"An AP may use the defined Scheduled Autonomous Probe Response Generation Function procedures if this attribute is TRUE. The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 40 }

dot11ScheduledAutonomousProbeResponseInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that an AP shall use for scheduling scheduled, autonomous Probe Response transmissions. The value is transmitted in Beacon and Probe Response frames if enabled.”

::= { dot11StationConfigEntry 41 }