IEEE P802.11
Wireless LANs

RCPI in Active Scan Procedure

Date:	September 14, 2004

Author:	Simon Black (Nokia)
	email: simon.black@intalk2k.com

Abstract

This submission proposed changes related to the use of the RCPI element to address the following comments in the IEEE802.11k/D1.0 letter ballot comment resolution: 942, 336, 337, 339, 775, 777, 25, 27, 28, 31, 25

The change proposed makes the RCPI element in active scan a requested element.

[Replace 7.2.3.9 as follows:]

#### 7.2.3.9 Probe Response frame format

Change the order 10, 13 and 17 information fields, insert the order 21 information field and change the starting order of the Requested information elements field in Table 12 as follows:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | The Country information element shall be presentIncluded if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementRequired or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | The Power Constraint information element Shall be included shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | The TPC Report information element Shall be included shall be present if dot11SpectrumManagementRequired or dot11RadioMeasurementEnabled is true. |
| 21 | AP Channel Report | The AP Channel Report information element shall only be present within Probe Response frames generated by a Radio Resource Measurement capable AP when dot11RadioMeasurementEnabled is true. The AP Channel Report Element may be omitted if there are no channels to report. |
| 2122-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

[Replace 7.3.2.27 with the following:]

#### 7.3.2.27 RCPI information element

The RCPI information element contains a RCPI value.

|  | Element ID | Length | RCPI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure k1 — RCPI element format

The length field shall be set to 1.

The RCPI field contains an RCPI value for the PHY type used to exchange the RCPI element. The RCPI element is used in the active scan procedure as described in 11.1.3.2.2.

[Replace 10.3.2.2.2 with the following:]

#### 10.3.2.2 MLME-SCAN.confirm

##### 10.3.2.2.2 Semantics of the Service Primitive

Add the following row to the BSSDescription following BSSBasicRateSet:

| RCPIMeasurement | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI of the Probe Request frame measured during the active scan procedure. This parmeter shall be present within a BSSDescription returned in an MLME-SCAN.confirm primitive when an RCPI element was present in the received Probe Response. |
| --- | --- | --- | --- |

[Remove 11.1.3 from the 11k draft due to no changes being required and replace 11.1.3.2.2 as follows:]

##### 11.1.3.2.2 Active scanning procedure

Change the last paragraph in this subsection as shown below:

When all channels in the ChannelList have been scanned, the MLME shall issue an MLME-Scan.confirm with the BSSDescriptionSet containing all of the information gathered during the scan.

If dot11RadioMeasurementEnabled is true, a STA may request RCPI information in the active scan procedure by including the RCPI element ID within the Request information element in a Probe Request frame. If an RCPI information element is received in a Probe Response frame, the RCPI value shall be returned in the RCPIMeasurement parameter of the BSSDescription.

If dot11RadioMeasurementEnabled is true, an AP shall include an RCPI information element in the Probe Response if requested in the Request information element of the Probe Request. An AP may measure RCPI on received Probe Request frames and include the result in the RCPI element of the Probe Response. If no measurement result is available the RCPI value shall be set to indicate measurement not available.

[Include new PICS item RRM16 in A.4.13 with the following and renumber all following PICS items:]

### A.4.13 Radio Resource Measurement extensions

| RRM16 RRM16.1 RR16.2 | RCPI Measurement during Active Scanning Respond with RCPI information element when requested Measurement of RCPI on Probe Request frames | 11.1.3.2.2 11.1.3.2.2 | (CF12 AND CF1):M (CF12 AND CF1):O | Yes No N/A Yes No N/A |
| --- | --- | --- | --- | --- |