IEEE P802.11
Wireless LANs

| TPC Related Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2004-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Black | Nokia |  | +44 1353 648567 | Simon.black@ intalk2k.com |

Abstract

This submission proposes changes related to the use of TPC in 11k to address the following comments in the IEEE802.11k/D1.0 letter ballot comment resolution: 13, 19, 24, 37, 40, 273, 278, 786, 787, 821, 825, 826, 1027, and 1034. See document 04/1508.

The net effect of these changes is to remove adaption of transmit power from the 11k draft (was optional as according to PICS entry RRM23) and clarify that the TPC elements from IEEE802.11h are only being used for link measurement.

[Italic text in square brackets] is instruction to the editor as to how the proposed normative text changes should be applied and is not a part of the text itself.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[Delete 5.4.4 since we are not now reusing theTransmit Power Control spectrum management service.]

[Delete 7.4.1 since spectrum management category action frames are now not reused by 11k. Replace the rows for Action field values 2 and 3 in Table k12 with the following:]

## 7.4 Action Frame Format Details

### 7.4.2 Radio Measurement Action Details

| 2 | Link Measurement Request |
| --- | --- |
| 3 | Link Measurement Report |

[Insert new sections 7.4.2.3 and 7.4.2.4 to make frame format definitions appear in the order in Table 12. Neighbor Report Request frame format becomes 7.4.2.5 and Neighbor Report Response 7.4.2.6. Delete existing 7.4.2.5 (TPC Request) and 7.4.2.6 (TPC Report). Figures k25 and k26 inserted in this new text. Renumber figures following insertion.]

#### 7.4.2.3 Link Measurement Request Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA to request another STA to respond with a Link Measurement Report frame to enable measurement of link path loss and estimation of link margin. The format of the frame is shown in Figure k25.

|  | Category | Action | Dialog Token | TPC Request element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 |

Figure k25 - Link Measurement Request frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a.

The Action field shall be set to indicate a Link Measurement Request according to Table k12.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the request to identify the transaction.

The TPC Request element shall be set as described in 7.3.2.15.

#### 7.4.2.4 Link Measurement Report Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame. The format of the frame is shown in Figure k26.

|  | Category | Action | Dialog Token | TPC Report element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 |

Figure k26 – Link Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a.

The Action field shall be set to indicate a Link Measurement Report according to Table k12.

The Dialog Token field shall be set equal to the Dialog Token value in the corresponding Link Measurement Request frame.

The TPC Report element shall be set as described 7.3.2.16.

[Replace the additions to the BSSDescription in 10.3.2.2.2 with the following:]

#### 10.3.2.2 MLME-SCAN.confirm

##### 10.3.2.2.2 Semantics of the Service Primitive

Add the following rows to the BSSDescription following BSSBasicRateSet:

| RCPIMeasurement | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI of the Probe Request frame measured during the active scan procedure. This parameter shall be present within a BSSDescription returned in an MLME-SCAN.confirm primitive when an RCPI element was present in the received Probe Response. |
| --- | --- | --- | --- |
| PowerConstraint | As defined in frame format | As defined in frame format | The contents of the Power Constraint element when received. Present only when the MIB attribute dot11SpectrumManagementRequired is true and when or dot11RadioMeasurementEnabled is true. |
| TPCReport | As defined in frame format | As defined in frame format | The contents of the TPC Report element when received. Present only when either the MIB attribute dot11SpectrumManagementRequired or dot11RadioMeasurementEnabled is true. |
| APChannelReportSet | Set of AP Channel Reports each as defined in the Channel Report element format | Set of AP Channel Reports each as defined in the Channel Report element format | The contents of any AP Channel Report elements received. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |

[Delete 10.3.16 from the draft and add the following MLME primitives:]

### 10.3.25 Link Measure Request

The following primitives support the measurement of link path loss, and the estimation of link margin between peer entities for Radio Resource Measurement.

#### 10.3.25.1 MLME-LINKMEASURE.request

##### 10.3.25.1.1 Function

This primitive supports the measurement of link path loss, the estimation of link margin between peer entities.

##### 10.3.25.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMEASURE.request	(
PeerMACAddress,
DialogToken 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Link Measure Request shall be set. |
| DialogToken | Integer | 1 – 255 | The dialog token to identify the Link Measure transaction. |

#### 10.3.24.1.3 When Generated

This primitive is generated by the SME to request that a Link Measurement Request frame be sent to the peer entity to request that entity to report transmit power and link margin information.

#### 10.3.24.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Link Measurement Request frame. The STA then attempts to transmit this to the STA indicated in the peerMACAddress parameter.

#### 10.3.24.2 MLME-LINKMEASURE.confirm

##### 10.3.24.2.1 Function

This primitive reports the result of a Link Measurement request.

##### 10.3.24.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMEASURE.confirm	(
ResultCode, 
DialogToken, 
TransmitPower, 
LinkMargin, 
RCPI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, TRANSMISSION_FAILURE UNSPECIFIED FAILURE | Indicates the result of the corresponding MLME-LINKMEASURE.request. |
| DialogToken | Integer | As defined in the corresponding MLME-LINKMEASURE.request | The Dialog Token to identify the link measurement transaction. |
| TransmitPower | As defined in the TPC Report element | As defined in the TPC Report element | The contents of the Transmit Power field of the received Link Measure Report frame. Only present if ResultCode = SUCCESS. |
| LinkMargin | As defined in the TPC Report element | As defined in the TPC Report element | The contents of the Link Margin field of the received Link Measure Report frame. Only present if ResultCode = SUCCESS. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI level of the received Link Measurement Report frame. Only present if ResultCode = SUCCESS. |

#### 10.3.24.1.3 When Generated

This primitive is generated by the MLME as a result of an MLME-LINKMEASURE.request and indicates the results of the request.

This primitive is generated when the MLME-LINKMEASURE.request contains invalid parameters, or when the STA receives a Link Measurement Response frame from the requested STA.

#### 10.3.24.1.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

[Delete 11.5 and add new section 11.9 Link Measurement]

## 11.9 Link Measurement

A STA may use a Link Measurement Request frame to request another STA to respond with a Link Measurement Report frame containing link margin and transmit power information. A STA receiving a Link Measurement Request frame shall respond with a Link Measurement Report frame containing the power used to transmit the response in the Transmit Power field and the estimated link margin in a Link Margin field.

An AP in a BSS or a STA in an IBSS shall autonomously include a TPC Report element with the Link Margin field set equal to zero and containing transmit power information in the Transmit Power field in any Beacon or Probe Response it transmits.

[Amend PICS items RRM2, RRM 12 (reference renumbering only), RRM17 and remove RRM23 in A.4.13 as follows:]

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM2 RRM2.1 RRM2.2 RRM2.3 RRM2.4 | Action frame protocol for measurements Measurement Request frame Measurement Report frame TPC Link Measurement Request frame TPC Link Measurement Report frame | 7.4 7.4.2.1, 7.3.2.21 7.4.2.2 7.4.2.3 7.4.2.4 | CFk:M CFk:M CFk:M CFk:M CFk:M | Yes, No Yes, No Yes, No Yes, No Yes, No |
| RRM12 | Action Frame Protocol for Neighbor Reports Neighbor Report Request Neighbor Report Response | 7.4 7.4.2.35 7.4.2.46, 7.3.2.26 | CFk:M CFk:M CFk:M | Yes, No Yes, No Yes, No |
| RRM17 | TPC Information in Beacon and Probe Response frames |  |  |  |
| RRM17.1 | Country and Transmit Power Control (TPC) Report elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.9, 7.3.2.18, 11.9 | CFk:M | Yes, No |
| RRM17.2 | Power Constraint element included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.15 | CFk:O | Yes, No, N/A |
| RRM23 RRM23.1 RRM23.2 | Adaptation of transmit power TPC Report in Beacon and Probe Response frames Dynamic transmit power adaptation | 11.5.4 11.5.4 | CFk:CF1:M CFk:CF2:O | Yes, No Yes, No |