IEEE P802.11
Wireless LANs

| IEEE 802.11 TGs Comparison Categories and Informative Checklists |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| W. Steven Conner | Intel Corp. | M/S JF3-206 2111 NE 25th Ave, Hillsboro OR 97124 | +1-503-264-8036 | w.steven.conner@intel.com |

Abstract

This document specifies comparison criteria that must be addressed by any proposal in response to the IEEE 802.11 TGs call for proposals.  This document is intended to provide categories for comparison of proposals and a recommendation for data to include with proposals.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Document Version History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | - First strawman document version | 13 October 2004 |
| R1 | - Added additional informative text and initial category teamplates | 27 October 2004 |
| R2 | - Incorporated updates based on comments during Oct. 27 TGs teleconference | 28 October 2004 |
| R3 | - Updated to align with the latest version of the functional requirements and scope document 11-04/1174r4. | 15 November 2004 |
| R4 | - Incorporated updates based on comments discussion in Tuesday evening TGs Ad-Hoc session in San Antonio, Nov. 16. | 16 November 2004 |
| R5 | - Updated to align with the latest version of the functional requirements and scope document 11-04/1174r7. | 17 November 2004 |
| R6 | - Incorporated proposed updates from informal ad-hoc discussions | 7 December 2004 |
| R7 | - Minor editorial changes to clean up document formatting. | 4 January 2005 |
| R8 | - Incorporated updates based on comments during Jan. 7 TGs teleconference. Updated Section 3 and Section 4 to use identical text from document 11-04/1174r11. | 10 January 2005 |
| R9 | - Updated with new document template. Updated Section 3 and Section 4 to use identical text from document 11-04/1174r12. | 16 January 2005 |
| R10 | - Replaced all instances of “ESS Mesh” with “WLAN Mesh”. Updated Section 4 to use identical text from document 11-04/1174r13. | 18 January 2005 |

Table of Contents

1	Introduction	2

1.1	Purpose of document	2

1.2	Relationship to Functional Requirements and Scope	3

2	Additional Supporting Material	3

3	Coverage of Minimum Functional Requirements	3

4	Coverage of In-Scope Functions (Informative)	4

5	Applicability to Usage Scenarios	8

6	Quantitative Comparison Criteria	9

7	References	10

# Introduction

## Purpose of document

This document provides categories for comparison of proposals and a recommendation for data to include with proposals that are submitted to 802.11 TGs.

This document is intended to provide a recommended structure for convenient comparison of features and characteristics of different proposals.  It provides recommended guidelines to developers of proposals on the types of characteristics that TGs members will evaluate when selecting one or more winning proposals.

This document does not specify an exhaustive collection of functions, detailed simulation scenarios, and measurements for comparison.  It is the responsibility of those submitting proposals to TGs to convince the task group of the technical merits of their proposal.  Proposers are welcome to provide additional quantitative and/or qualitative data to that specified in this document.

Section 2 through Section 6 of this document provide informative templates to be filled out by those submitting proposals to 802.11 TGs, including references on where to find relevant materials for each category.  Section 2 is an important template that includes references to additional supporting materials relating to a proposal.  Section 3 is an important template that indicates coverage of minimum functional requirements for TGs.  Sections 4 and 5 are checklists of some possible functions that are in scope for TGs.  Section 6 defines quantitative metrics for comparing and evaluating proposals and provides a template for listing references for relevant quantitative descriptions and data.

In the case of partial proposals, only the relevant template sections need be filled in.  Sections 2 and 3 must be filled in for all proposals.

## Relationship to Functional Requirements and Scope

The main purpose of the comparison criteria is to define categories and metrics to enable comparison of TGs proposals, while the functional requirements and scope document [5] specifies the minimum functional requirements that must be addressed by IEEE 802.11s and clarifies the scope of efforts undertaken by TGs.

# Additional Supporting Material

This section contains requirements for additional documentation that must be submitted with a proposal.  This is a template that must be filled in and included with a proposal submission.

| Number | Name | Definition | Coverage (Yes/No) |  | Notes | References |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of IEEE 802 submissions related to the proposal, both documents and presentations. |  |  |  |  |
| AD2 | Simulation and/or experimental methodology | Any proposal submission that includes simulation results must include a description of the simulation methodology used for mesh simulations. The simulation methodology documentation should provide enough information to, in principle, reproduce the simulation (e.g., including node positions, traffic and propagation model (including PHY assumptions), packet sizes, etc.). |  |  |  |  |

# Coverage of Minimum Functional Requirements

This section contains a template for disclosure of coverage of minimum functional requirements with a proposal.  See [5] for detailed definitions of functional requirements.  This template must be filled in and included with a proposal submission.

[Editor Note: This template should be synchronized with Section 3 in 11-04/1174 when the CFP is issued.]

| Number | Category | Name | Coverage (Complete /Partial/ None) | Notes | References |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery |  |  |  |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol |  |  |  |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture |  |  |  |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery |  |  |  |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery |  |  |  |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios |  |  |  |
| FR7 | TOPO_RT_FWD | Mesh Network Size |  |  |  |
| FR8 | SECURITY | Mesh Security |  |  |  |
| FR9 | MEAS | Radio-Aware Routing Metrics |  |  |  |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA |  |  |  |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension |  |  |  |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh |  |  |  |
| FR13 | MMAC | Amendment to MAC with no PHY changes required |  |  |  |
| FR14 | INTRWRK | Compatibility with higher-layer protocols |  |  |  |

# Coverage of In-Scope Functions (Informative)

This section contains a template for informative disclosure of coverage of in-scope functions with a proposal.  See [6] for detailed description of in-scope functions considered by TGs.  This template may be filled in and included with a proposal submission.

This purpose of this template is primarily to aid the reader in identifying which in-scope features are covered by a proposal, and which sections of the proposal are relevant to each feature.   The in-scope functions listed below are NOT mandatory for inclusion in a proposal and are NOT functional requirements.  Proposers are welcome to provide additional functions that are in-scope.

[Editor Note: This template should be synchronized with Section 4 in 11-04/1174 when the CFP is issued.]

| Number | Name | Coverage Yes / No / N/A | Notes | References |  |
| --- | --- | --- | --- | --- | --- |
| Mesh Topology Learning, Routing, and Forwarding (TOPO_RT_FWD) |  |  |  |  |  |
| TOPO_RT_FWD_SCP1 | - Mesh topology discovery, including Mesh Point neighbor discovery within a WLAN Mesh |  |  |  |  |
| TOPO_RT_FWD_SCP2 | - MAC address-based mesh routing protocols and algorithms |  |  |  |  |
| TOPO_RT_FWD_SCP3 | - MAC-layer mesh broadcast/multicast and unicast data delivery |  |  |  |  |
| TOPO_RT_FWD_SCP4 | - Architecture to support alternative routing protocols and metrics |  |  |  |  |
| TOPO_RT_FWD_SCP5 | - Mesh routing with single-radio devices |  |  |  |  |
| TOPO_RT_FWD_SCP6 | - Mesh routing with multiple-radio devices |  |  |  |  |
| TOPO_RT_FWD_SCP7 | - Use of radio-aware route selection metrics |  |  |  |  |
| TOPO_RT_FWD_SCP8 | - QoS-based route selection |  |  |  |  |
| TOPO_RT_FWD_SCP9 | - Proactive routing |  |  |  |  |
| TOPO_RT_FWD_SCP10 | - On-demand routing |  |  |  |  |
| TOPO_RT_FWD_SCP11 | - Hybrid routing |  |  |  |  |
| TOPO_RT_FWD_SCP12 | - Mesh Point neighbor discovery within a WLAN Mesh via passive scanning |  |  |  |  |
| TOPO_RT_FWD_SCP13 | - Mesh Point neighbor discovery within a WLAN Mesh via active scanning |  |  |  |  |
| TOPO_RT_FWD_SCP14 | - Mesh routing in the presence of low-power (e.g. battery-powered) Mesh Points |  |  |  |  |
| TOPO_RT_FWD_SCP15 | - Support for WLAN Mesh network configurations with more than 32 Mesh Points. |  |  |  |  |
| TOPO_RT_FWD_SCP16 | - Ability to recognize changes in the topology within a bounded time |  |  |  |  |
| TOPO_RT_FWD_SCP17 | - Ability to reconfigure the routing scheme within a bounded time in response to detected changes |  |  |  |  |
| TOPO_RT_FWD_SCP18 | - SAP interfaces and management primitives to provide support to enable higher-layer (e.g., Layer 2.5 and Layer 3) routing protocols |  |  |  |  |
| TOPO_RT_FWD_SCP_Other |  |  |  |  |  |
| Mesh Security (SECURITY) |  |  |  |  |  |
| SECURITY_SCP1 | - Secure association of Mesh Points to a WLAN Mesh |  |  |  |  |
| SECURITY_SCP2 | - Securing a WLAN Mesh in which all of the Mesh Points are controlled by a single logical administrative entity for security. |  |  |  |  |
| SECURITY_SCP3 | - Secure data message exchange between Mesh Points over mesh links |  |  |  |  |
| SECURITY_SCP4 | - Secure management message exchange between Mesh Points over mesh links |  |  |  |  |
| SECURITY_SCP5 | - Secure topology and routing information exchange between Mesh Points over mesh links |  |  |  |  |
| SECURITY_SCP6 | - Centralized authentication and key management |  |  |  |  |
| SECURITY_SCP7 | - Distributed authentication and key management |  |  |  |  |
| SECURITY_SCP8 | - Hybrid authentication and key management |  |  |  |  |
| SECURITY_SCP9 | - Static key support |  |  |  |  |
| SECURITY_SCP10 | - Dynamic key support |  |  |  |  |
| SECURITY_SCP11 | - Extension of IEEE 802.11i security mechanisms for mesh |  |  |  |  |
| SECURITY_SCP_Other |  |  |  |  |  |
| Mesh Measurement (MEAS) |  |  |  |  |  |
| MEAS_SCP1 | - Specification of radio-aware metrics for use by mesh routing protocols |  |  |  |  |
| MEAS_SCP2 | - Specification of radio-aware metrics for use by mesh medium access coordination |  |  |  |  |
| MEAS_SCP3 | - Mesh link quality measurements |  |  |  |  |
| MEAS_SCP4 | - Mesh path quality measurements |  |  |  |  |
| MEAS_SCP5 | - Measurements to support the use of various types of antennas in a mesh network |  |  |  |  |
| MEAS_SCP6 | - Mesh related measurements to aid STAs in making roaming decisions, e.g., WDS capacity currently available for traffic forwarding. |  |  |  |  |
| MEAS_SCP_Other |  |  |  |  |  |
| Mesh Discovery and Association (DISC_ASSOC) |  |  |  |  |  |
| DISC_ASSOC_SCP1 | - Protocols to allow Mesh Points to discover WLAN Mesh networks |  |  |  |  |
| DISC_ASSOC_SCP2 | - Protocols to allow Mesh Points to associate and disassociate with a WLAN Mesh network |  |  |  |  |
| DISC_ASSOC_SCP3 | - Protocols to allow Mesh Points to associate and disassociate with other Mesh Points within a WLAN Mesh |  |  |  |  |
| DISC_ASSOC_SCP_Other |  |  |  |  |  |
| Mesh Medium Access Coordination (MMAC) |  |  |  |  |  |
| MMAC_SCP1 | - Mitigate performance degradation caused by hidden nodes |  |  |  |  |
| MMAC_SCP2 | - Mitigate performance degradation caused by exposed nodes |  |  |  |  |
| MMAC_SCP3 | - Flow control over multi-hop paths to avoid performance degradation and/or meet QoS goals. |  |  |  |  |
| MMAC_SCP4 | - Coordinating channel access across multiple nodes to avoid performance degradation and/or meet QoS goals in the multi-hop network. |  |  |  |  |
| MMAC_SCP5 | - Traffic prioritization within a WLAN Mesh |  |  |  |  |
| MMAC_SCP6 | - Enhancements to make the MAC work well across a range of different network sizes, usage models, etc. |  |  |  |  |
| MMAC_SCP7 | - Mesh link communication coordination |  |  |  |  |
| MMAC_SCP8 | - Support for admission control to determine if a particular flow can be admitted into the mesh network based on the availability of resources and existing usages. |  |  |  |  |
| MMAC_SCP9 | - Mangement of multiple classes of traffic, for example when both BSS traffic and mesh forwarding traffic are present in one device (e.g. in a Mesh AP). |  |  |  |  |
| MMAC_SCP10 | - Rate control enhancements for efficient multicasting and broadcasting in a mesh network. |  |  |  |  |
| MMAC_SCP11 | - Improving spatial reuse in a mesh network. |  |  |  |  |
| MMAC_SCP_Other |  |  |  |  |  |
| Compatibility to 802.11 Services (SERV_CMP) |  |  |  |  |  |
| SERV_CMP_SCP1 | - Mesh Point DS Services (DSS) Integration |  |  |  |  |
| SERV_CMP_SCP2 | - WLAN Mesh compatibility with STA mobility/roaming |  |  |  |  |
| SERV_CMP_SCP3 | - Techniques to allow WLAN Mesh to meet 802.11r system requirements |  |  |  |  |
| SERV_CMP_SCP4 | - Dissemination of STA-to- destination MeshAP or STA-to-destination Mesh Portal routing information in the WLAN Mesh. |  |  |  |  |
| SERV_CMP_Other |  |  |  |  |  |
| Mesh Interworking (INTRWRK) |  |  |  |  |  |
| INTRWRK_SCP1 | - Allow a WLAN Mesh to interface with higher layer protocols |  |  |  |  |
| INTRWRK_SCP2 | - Interfacing a WLAN Mesh with other IEEE 802 LANs |  |  |  |  |
| INTRWRK_SCP3 | - Support for interfacing a WLAN Mesh with other IEEE 802 LANs using 802.1D |  |  |  |  |
| INTRWRK_SCP4 | - Support for efficient utilization of multiple Mesh Portals in a single WLAN Mesh. |  |  |  |  |
| INTRWRK_SCP_Other |  |  |  |  |  |
| Mesh Configuration and Management (CFG_MGMT) |  |  |  |  |  |
| CFG_MGMT_SCP1 | - Protocol extensions to support self-configuring formation of a WLAN Mesh network |  |  |  |  |
| CFG_MGMT_SCP2 | - Interfaces to support 802.11h DFS compliancy |  |  |  |  |
| CFG_MGMT_SCP3 | - Interfaces and parameter exchange to enable RF auto configuration support |  |  |  |  |
| CFG_MGMT_SCP4 | - Support for managed network management model |  |  |  |  |
| CFG_MGMT_SCP5 | - Support for unmanaged network management model |  |  |  |  |
| CFG_MGMT_SCP6 | - Interfaces and parameter exchange to exchange information about the capabilities of Mesh Point devices |  |  |  |  |
| CFG_MGMT_SCP7 | - Mesh network channel selection |  |  |  |  |
| CFG_MGMT_SCP8 | - Mesh network Tx power control |  |  |  |  |
| CFG_MGMT_SCP9 | - QoS policy and management |  |  |  |  |
| CFG_MGMT_SCP10 | - Support for time synchronization of Mesh Points if required by mesh services |  |  |  |  |
| CFG_MGMT_SCP_Other |  |  |  |  |  |

# Applicability to Usage Scenarios

This section contains a template for reporting usage scenarios for which the proposer believes the solution is relevant.  It is desirable for proposals to provide evidence of meeting the soft requirements from the Usage Models document [3].  This template may be filled in and included with a proposal submission.

It is not mandatory to prove the applicability of a proposal to any of the following usage scenarios.  If simulation results are included with a proposal, simulation scenarios relating to the following usage models are preferred.  If simulation results are included with the proposal, a detailed simulation methodology must also be included (See AD2: Simulation and/or experimental methodology in Section 2).

| Number | Name | Definition | Data Included? Yes/No |  | Notes |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UM1 | Residential | Include a description of how the proposal is applicable to residential usage scenarios, as described in [3]. |  |  |  |  |  |
| UM2 | Office | Include a description of how the proposal is applicable to office usage scenarios, as described in [3]. |  |  |  |  |  |
| UM3 | Campus/Community/Public Access Networks | Include a description of how the proposal is applicable to campus/community/public access usage scenarios, as described in [3]. |  |  |  |  |  |
| UM4 | Public Safety | Include a description of how the proposal is applicable to public safety usage scenarios, as described in [3]. |  |  |  |  |  |
| UM5 | Military | Include a description of how the proposal is applicable to military usage scenarios, as described in [3]. |  |  |  |  |  |

# Quantitative Comparison Criteria

This section contains a template for reporting quantitative results relating to a proposal.  This list provides a list of quantitative metrics that are considered useful for comparing proposals to TGs.   It is expected that the comparison process for proposals will be iterative, and more detailed comparison methods may be added later as necessary.

This template may be filled in and included with a proposal submission.  Proposers are welcome to provide additional quantitative data to that specified in this section.

If simulation or experimental results are included with the proposal, a detailed simulation methodology must also be included or referenced (See AD2: Simulation and/or experimental methodology in Section 2).  In addition to simulations, a proposal may also include analytical results with clearly documented scenarios and assumptions.

| Number | Name | Definition | Data Included? Yes/No | Notes | References |
| --- | --- | --- | --- | --- | --- |
| QC1 | Routing complexity | Memory, computation, communication complexity as a function of number of Mesh Points, number of STAs, number of active Mesh Paths, diameter and degree of the topology graph |  |  |  |
| QC2 | Routing convergence and recovery | Time to discover a route, detect a link or node failure or performance metric change, repair a route due to failure or metric change as a function of number of Mesh Points, number of STAs, number of active Mesh Paths, diameter and degree of the topology graph, rate of change of the topology |  |  |  |
| QC3 | Data delivery | Performance factors such as end-to-end throughput, latency, robustness for traffic flows across the mesh as a function of the number of Mesh Points, number of active Mesh Paths, number of simultaneous traffic flows, diameter and degree of the topology graph, rate of change of the topology |  |  |  |
| QC_Other |  |  |  |  |  |

# References

[1] IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh

[2] IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh

[3] IEEE 802 11-04/662r13, TGs Usage Models

[4] IEEE 802.11-04/1477r1, Terms and Definitions for 802.11s

[5] IEEE 802.11-04/1174r13, 802.11 TGs Functional Requirements and Scope