IEEE P802.11
Wireless LANs

Structured LDPC code design

Date:	November 4, 2004

Author:                           Victor Stolpman, Nico vanWaes and Jian Zhong (Charlie) Zhang
	Nokia
	6000 Connection Dr, Irving, TX
	Phone: 972-894-5669
	e-Mail: {Victor.Stolpman, Nico.vanWaes, Charlie.Zhang}@Nokia.com

This document contains a partial PHY proposal for TGn. It addresses a structured approach to irregular LDPC code construction based on “seed” matrices that are expanded using permutation matrices for purposes of error correction control. These codes have small storage requirements with good block error rate performances over a wide and flexible range of block sizes.

1.	Explanation of design	1

2.	LDPC parity check matrix construction	2

3.	Simulation results	4

4.	References:	9

# Explanation of design

Some random irregular LDPC constructions based on edge ensemble designs have error correcting capabilities measured in Bit Error Rate (BER) that are within 0.05dB of the rate distorted Shannon limit for the AWGN channel [3].

Unfortunately, these random LPDC code constructions require long codeword constructions (on the order of 106 to 107 bits) in order to achieve these error rates, and despite good BER performance, these random code constructions often have poor Block Error Rate (BLER) performances. Hence, these random constructions do not lend themselves well to packet-based communication systems. Yet another disadvantage of random constructions based on edge distribution ensembles is for each codeword length another random construction is needed. Thus, communication systems employing variable block sizes would require multiple code definitions that could consume a significant about of non-volatile memory storage for a large combination of codeword lengths and code rates.

An alternative to random LDPC construction is structured LDPC constructions that rely on a general algorithmic approach to constructing LDPC matrices and require much less non-volatile memory than random constructions. Thus, the problem is to design irregular structured LDPC codes that have good overall error performance (both BER and BLER) for a wide range of code rates and block sizes with attractive storage requirements. The result of such LDPC codes is a better performing communication system with lower cost terminals. These factors make such a FEC attractive for application over a wide range of products including but not limited to IEEE802.16 and IEEE802.11n compliant products.

Thus, this exact description below for LDPC code construction succeeds at solving the above said problem while providing excellent performance for all block sizes without the non-volatile memory requirements of randomly constructed codes.

The LDPC codeset design below fits within the LDPC framework as proposed by TGn Sync and provides an optimisation of consumed power per bit while meeting the specified datarate with a negligible variation in SNR for constant PER performance for different packet lengths. It is designed such that it allows efficient near-linear encoding and various decoding algorithms including layered belief propagation, which is a preferred implementation due to its convergence rate.

The codeset below is designed for Lcword_inc = 48 (or any multiple thereof by omission) and  z = Nspread  per the TGn Sync definition.

# LDPC parity check matrix construction

The parity check matrices H are derived from binary seed matrices Hb of size mb×nb by expanding each ‘1’ in Hb with P(i×(j+1−i))modp for all rows i, i∈1,...,mb, and columns j, i∈1,...,nb, for which i≤j+1 holds, where P1 is the singular cyclicy left shifted identity matrix of size z  and by expanding every other element with the all-zeros matrix of the same size as P1. The modulo factor p is the smallest positive primitive for which the relations p≥nb+2 and p≥z hold.The codeword length is therefore z×nb.

A parity check matrix H is hence entirely defined by the selection of Hb, the code rate R and the value z.

The parity check matrices below are provisioned in hexadecimal format.

Hb1=0x8000010203850xC000004020390x60000040203C0x30000048B0100x1800001840320x0C00002328200x06000000232C0x0300000720080x0180004842400x00C0008400260x00600040022C0x0030014040480x001800A024800x000C004403010x0006000482140x0003004290040x000180120A080x0000C002021C0x00006002A4100x0000304221020x0000188028140x00000C0204B00x000006431020, Hb2=0x8000010109510xC000000083980x6000003010260x300000C020940x18000014440A0x0C00000300CA0x0600005008A10x0300000022680x0180000090700x00C0006400050x00600100401A0x0030000004560x0018000400A60x000C000010330x0006008201840x0003000104890x0001808800620x0000C002A0810x00006008400D0x0000300004550x0000180003430x00000C0002A90x000006000836, Hb3=0x80000140B0420xC0000020806A0x6000009041140x300000002E410x1800002811420x0C000008121A0x0600001A12200x0300006202020x0180004007020x00C0000858040x0060000150410x0030000288900x0018000410C80x000C00C044800x0006006940000x0003004801420x0001804800230x0000C06850000x0000602842400x0000303460000x0000180412240x00000C6002060x000007284040,

Hb4=0x80010050127B0xC0002050301F0x60000022067D0x3000480208DD0x18001308006F0x0C001848031E0x06001006C1530x03004084406F0x0180030088BB0x00C0A40104330x0061243000650x0030000464EC0x00180100296E0x000C86A110140x000608090077Hb5=0x8001814A085C0xC000501010D70x60000E06005D0x30008400503F0x1800005000FF0x0C00220D80690x0600018827250x03010284045E0x01809C0042630x00C02112882C0x00601080A03E0x00300800087F0x00184020107E0x000C0021207B0x000600610783Hb6=0x80010450407B0xC0002480007F0x6000002C026F0x3000414211AA0x18000126A4260x0C001210016F0x06000204847B0x03011061085C0x0180C100403F0x00C00A0105530x0060000268CD0x0030208020770x0018C00808370x000C188002790x00062C0810D4 Hb7=0x80010A0054E50xC0005040C25C0x6000200C403F0x30008040027F0x1800102004FB0x0C000110217E0x06002AC1002D0x0300000328DB0x01802488017A0x00C1040C00760x0060003085590x0030818230520x00184020085F0x000C040682270x00069300002F

Hb8=0x8013065040EF0xC00F2141C0A40x600806D641680x30152AC0EC000x18031AF050280x0C030A1B48B00x060051D2D3100x0303C2BD10200x018B61B044220x00C792C825020x006306D40A25Hb9=0x80129DB0E0810xC001215434A30x600051146E310x300CA60620CA0x1812835E00A40x0C01919528A10x0601805D31E00x030CC11611290x018E881460A50x00C0C729A2810x0060A900A0F7Hb10=0x80130ABC59010xC0087A4443810x6006160769020x30088806EC1C0x18060A821F240x0C068A44491A0x06065A3C48800x03070E00D1980x018F091C05240x00C62100EB610x0074A3250942 Hb11=0x80101A2662E90xC004314002F70x6000A4502CF20x30038141823B0x1819340208CD0x0C0E402811C70x060001A8447F0x0303008E107D0x018C4A01219E0x00C0081184FF0x0060069411BE

Hb12=0x850E6CB37BFE0xC40E8ACF8FFF0x63A1B3F57CB30x30D9F118FF5F0x1AF25F0437FF Hb13=0x8665C2EC76FF0xC2D43592FBFB0x6229AE4D9FBF0x35E3CC31BDED0x199A139F6F77 Hb14=0x843395FEF78D0xC1B75258BDF70x6708ADD63BFB0x32F976A10FFE0x1B4CCA23FE7F Hb15=0x86F1F4293EEF0xC3AE0FBDB5960x6199694C7F7F0x35449AC65FFF0x18368553FBFF

In Table 1, the seed matrices and their associated values z for the construction of all codewords lengths for all rates are shown.

Table 1 LDPC seed matrices selection per code rate and expansion factors

| Code rate (nb-mb) / nb | Seed matrices | Expansion factors z |
| --- | --- | --- |
| 1/2 | Hb1 | 13, 15, 16, 20, 21, 23, 24, 26, 30, 32, 34, 36, 38, 39, 40, 41, 45, 46, 48, 49, 53 |
|  | Hb2 | 12, 14, 19, 22, 25, 28, 31, 35, 37, 42, 43, 44, 47, 52 |
|  | Hb3 | 17, 18, 27, 29, 33, 50, 51, 54 |
| 2/3 | Hb4 | 17, 21, 22, 27, 28, 30, 31, 32, 35, 36, 38, 39, 40, 41, 42, 44, 46, 47, 48, 53 |
|  | Hb5 | 12, 16, 20, 26, 34, 49 |
|  | Hb6 | 13, 54 |
|  | Hb7 | 14, 15, 18, 19, 23, 24,25, 29, 33,37, 43, 45, 50, 51, 52 |
| 3/4 | Hb8 | 13, 23, 24, 26, 27, 29, 30, 31, 33, 34, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 49, 50, 51, 52, 53 |
|  | Hb9 | 15, 17, 19, 20, 21, 25, 32, 44, 45 |
|  | Hb10 | 12, 18, 28 |
|  | Hb11 | 14, 16, 22, 35, 54 |
| 7/8 | Hb12 | 13, 14, 16, 17, 18, 21, 25, 28, 31, 32, 33, 34, 35, 38, 39, 40, 42, 44, 45, 47, 48, 49, 50, 51, 52, 53, 54 |
|  | Hb13 | 15, 22, 27, 36, 37, 41, 43,46 |
|  | Hb14 | 19, 20, 24, 26 |
|  | Hb15 | 12, 23, 29, 30 |

# Simulation results

Figure 3-1 Rate 1/2 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-2 Rate 1/2 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-3 Rate 2/3 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-4 Rate 2/3 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-5 Rate 3/4 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-6 Rate 3/4 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-7 Rate 7/8 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-8 Rate 7/8 BPSK results in AWGN using 50 iterations of conventional BP (SPA)

Figure 3-9 BLER comparision of 50 conventional BP (SPA) iterations and 15 layered BP for Rate 1/2 BPSK in AWGN of codeword length of N=1152

Figure 3-10 BPSK-AWGN results of 12 iterations of layered BP for coderates 1/2, 2/3 and 3/4 of codeword lengths N=1920.

Figure 3-11 BPSK AWGN comparisions of various BP decoding approaches.

# References:

- R. G. Gallager, “Low-density parity-check codes,” IRE Trans. Inform. Theory, vol. IT-8, pp.21-28, Jan. 1962.

- T. J. Richardson, M. A. Shokrollahi, and R. L. Urbanke, “Design of Capacity Approaching Irregular Low-Density Parity-Check Codes,” IEEE Transactions on Information Theory, vol. 47, pp. 619-637, Feb. 2001.

- Sae-Young Chung, On the Construction of Some Capacity-Approaching Coding Schemes, PhD Dissertation, MIT, 2000.

- T. J. Richardson and R. L. Urbanke, “Efficient Encoding of Low-Density Parity-Check Codes,” IEEE Transactions on Information Theory, vol. 47, pp. 638-656, Feb. 2001.

- M. M. Masour and N. R. Shanbhag, “Turbo decoder architectures for low-density parity check codes,” IEEE Global Comm. Conf. (GLOBECOM), Nov. 2002, pp. 1383-1388.

- M. M. Mansour and N. R. Shanbhag, “Low power VLSI architectures for LDPC codes,” in 2002 International Low Power Electronics and Design, 2002, pp. 284-289.

- D. E. Hocevar, “LDPC code construction with flexible hardware implementation,” Proc.: IEEE Int’l Conf. On Comm. (ICC), Anchorage, AK, May 2003.

- M. M. Mansour and N. R. Shanbhag, “High-Throughput LDPC Decoders,” IEEE Trans. On VLSI Systems, vol. 11, No. 6, pp. 976-996, December 2003.