IEEE P802.11
Wireless LANs

Nokia Partial Proposal for TGn

Date:	Nov 4, 2004

Authors:  {Yousuf.Saifullah, Naveen.Kakani, Srinivas.Sreemanthula, Nico.vanWaes, Jari.Jokela, Victor.Stolpman}@Nokia.com

Power efficiency is a critical aspect of making 802.11n suitable for the handset market.

The proposal has the following MAC features:

- Multi-rate Frame Aggregation - for increasing data throughput along with Power Efficiency. The partial proposals of Nokia, Philips and Samsung regarding Multi-rate FA have been merged.

- Power Efficiency in Single-rate Aggregation - by indicating STA address and its frame length in the aggregation control header

- MAC Header Compression – Robust compression from the very first MPDU. Compression is applicable to both aggregation, and non-aggregation case.

- Aggregate ACK – an enhancement over existing BA for aggregation. Implicit A-ACK without BAR. Compresses BA format by having BA bitmap size equal to the received frames. One A-ACK across multiple TIDs.

The proposal has the following PHY feature

- Structured LDPC code design having small storage and low complexity requirements with good block error rate performances over a wide and flexible range of block sizes

References:

- 11-04-0946-02-000n-mac-enhancements-tgn.ppt

- 11-04-0947-02-000n-mac-enhancements-tgn.doc

- 11-04-1362-00-000n- structured-ldpc-code-design