IEEE P802.11
Wireless LANs

Structured LDPC Codes as an Advanced Coding Scheme for 802.11n – Summary of the Key Points

Date:	November 04, 2004

Authors: 	Aleksandar Purkovic 
	Nortel Networks
	20410 Century Blvd., Germantown, MD 20874, USA
	Phone: 1-301-528-3230
	Fax: 1-301-528-3298 
	e-Mail: apurkovi@nortelnetworks.com

Abstract

This document summarizes the key points of the partial proposal on structured LDPC codes described in submission documents IEEE 802.11-04/884r0 and IEEE 802.11-04/885r0.

# Key Points

General points on structured LDPC codes

- LDPC codes have matured and become a strong advanced coding candidate due to the significant activity in the coding community, especially during the last several years

- LDPC codes are proved to achieve coding gains comparable to turbo codes

- LDPC codes based on a structured parity check matrix greatly reduce encoding and decoding hardware bottlenecks with minimal performance penalty

Points on structured LDPC codes described in this proposal

- LDPC parity check matrix construction is based on a small “base” matrix and simple expansions of it (to fit various block sizes), which offers required flexibility in terms of block sizes and code rates

- Data portion of the base LDPC parity check matrix is made up of permutation matrices, which further reduce complexity and enable very low latency “layered” Belief Propagation decoding

- Parity portion of the base LDPC parity check matrix is in the “dual-diagonal” form, which enables efficient encoding

- The approach taken by this partial proposal was to achieve a good trade off between performance and complexity/latency