IEEE P802.11
Wireless LANs

Mitsubishi MERL/ITC 802.11n Proposal Summary

Date:                                                                  Nov. 4, 2004

Author:                                                          Andreas F. Molisch

Mitsubishi Electric Research Laboratories (MERL) 
201 Broadway, Cambridge, MA 02139, U.S.A

Phone: 617-621-7558   Fax: 617-621-7550
Email: molisch@merl.com

Abstract

This document summarizes the main features of Mitsubishi Electric MERL/ITC’s PHYand MAC partial proposal for 802.11n.

- Reference documents: 11-04/0995r1 (proposal specifications), 11-04/0996r2 (PPT presentation), 11-04/0997r0 (response to FRCC)

Main Features of PHY

- Techniques for performance enhancement of generic baseline system

- Features of the baseline:

  - Basic MIMO-OFDM system with layered structure

  - 2x2 antenna modes with 20 MHz channelization as mandatory, 3x3 and 4x4 as optional

  - Convolutional codes for backward compatibility; other coding methods possible

- Statistical data rate allocation

  - Determine the optimal data rates for different layers based on average CSI

- Joint radio frequency (RF)-baseband processing for antenna selection

  - Perform processing in RF domain, i.e., before selection is done

  - Reduce implementation cost by using only phase-shifter and adder in RF processing

  - Maintain diversity gain and mean-SNR gain

  - Method works also for non-layered structures with linear (MMSE) or ML detection

- Quasi-block diagonal low-density parity-check (QBD-LDPC) codes as advanced coding scheme

  - The code can be decoded partially with the help of other layers by introducing the correlation between different layers.

Main Features of MAC

- Enhancements to an 802.11e baseline:

- Two-level frame aggregations

  - MSDU-level frame aggregation: Frames with identical TID and the same (source, destination) pare can be aggreagated into a MPDU to improve efficeincy.

    - BlockACK is used for reception acknowlegement

  - PSDU-level frame aggregation:  PPDUs from the same transmitter can be transmitted consecutively to reduce transmission overhead.

    - Aggregated blockACK with scheded transmission order is used for reception acknowledgement

- Adaptive distributed channel access (ADCA) is used in CP- an enhancement to EDCA

  - Allow adaptive batch (multiple frames) transmission in one channel access

  - Opportunistically select high rate STAs for transmission

  - Provide long-term temperal fairness

- Sequential Controlled Channel Access (SCCA) in CFP – an enhancement to HCCA

  - Scheduled transmission order by assigning STAs with incrementally different backoff times

  - Eliminate polling overhead

- Efficeient BockACK

  - Reduced bitmap fied size by using relative sequence numbering secheme