IEEE P802.11
Wireless LANs

Duo-Binary Turbo Codes Summary

Date: 	November 4th, 2004

Author: 	Marie-Helene Hamon (France Telecom), John Benko (France Telecom), Claude Berrou (ENST Bretagne), Jacky Tousch (TurboConcept), Brian Edmonston (iCoding)
	France Telecom
	4, rue du Clos Courtel – BP91226 – 35512 Cesson-Sevigne- France
	Phone: 33 299 12 48 73 
	Fax: 33 299 12 40 98
	e-Mail:  mhelene.hamon@francetelecom.com

Abstract

This document summarizes the partial proposal on Duo-Binary Turbo Codes (specified in IEEE802.11-04/904).  Information has been taken from the following documents:  IEEE802.11-04/902 (CC disclosure), IEEE802.11-04/903r1 (presentation), and IEEE802.11-04/1382 (complexity estimates).

Introduction to Turbo Codes

Introducing a more powerful error-correcting code is one of the possible enhancements considered for 802.11n standard and represents a significant step towards the target throughput enhancements. Turbo Codes are one of the candidate schemes. Since their discovery, they have been improved on several aspects: simpler decoding algorithms (Max-Log-MAP,…), performance improvements (enhanced permutations, circular encoding,…), increased flexibility and throughput (parallelism)… All these improvements have led to their introduction in several standards (UMTS, IEEE 802.16, DVB-RCS, DVB-RCT,…) and are enhanced with the latest of convolutional Turbo Codes, the Duo-Binary Turbo Codes, which are proposed as advanced forward error-correcting schemes for the future standard IEEE 802.11n.

Advantages

The Duo-Binary Turbo Codes differ from previous classical convolutional Turbo Codes (used in UMTS) by the simultaneous encoding of two data bits instead of only one.  Duo-Binary Turbo Codes enjoy several advantages:

- Reduction in latency and complexity per decoded bit, which is estimated 35 % lower than binary Turbo Codes.

- Better convergence in the iterative decoding process

- Thanks to their improved permutation, the asymptotic performance has been improved

- High flexibility: all coding rates are possible through puncturing, and the Duo-Binary Turbo Code can be adjusted to any frame size by modifying only four values which parametrize the internal interleaver.

- Peformance vs complexity trade-off flexibility by adjusting for example the number of iterations.  Turbo Codes do not require large numbers of iterations to achieve significant performance gains.

- Turbo Codes are a mature technology. The hardware implementation is well controlled.

- Turbo Codes will perform well independent of the configuration: with MIMO techniques, increased bandwidth, etc.

Performance

- Gains up to 4 dB (AWGN) and 3 dB(channel D) over .11a convolutional code (SISO configuration).

- TC outperforms ST-Micro LDPC code by 0.9 dB (8 iterations) and 0.7 dB (6 iterations), at FER=10-2., see below.

~1 dB gain over LDPC, w/ 8 iterations FER=10-2

(LPDC Results taken from 11-04-0900-04-000n)

Complexity

Complexity estimates, based on post synthesis results, with 0.13 μm, 8192 bit frames, and a clock rate of 200 MHz.

| Decoder complexity (mm²) | Decoded bitrate @ 8 iterations | Decoded bitrate @ 5 iterations |
| --- | --- | --- |
| 2.35 | 100 Mbps | 160 Mbps |
| 2.93 | 150 Mbps | 240 Mbps |
| 3.14 | 200 Mbps | 320 Mbps |
| 4.28 | 300 Mbps | 480 Mbps |