IEEE P802.11
Wireless LANs

- Turbo Codes: Complexity Estimates

Date: 	November 12th , 2004

Author(s): 	Jacky Tousch
	TurboConcept
	email:  jacky.tousch@turboconcepts.com

Marie-Helene Hamon, John Benko
	France Telecom
	4, rue du Clos Courtel – BP91226 – 35512 Cesson-Sevigne- France
	Phone: 33 299 12 48 73 
	Fax: 33 299 12 40 98
	e-Mail:  mhelene.hamon@francetelecom.com

- Abstract

This document discloses complexity estimates for the Turbo Code proposed for 802.11n, as described in document IEEE 802.11-03/904.

# Content

Content	2

Scope	3

References	3

Document releases	3

1	Description of the turbo code	3

1.1	Turbo Encoding	3

1.2	Turbo Code Permutation	4

1.3	Rates & Puncturing Map	4

2	Decoding Algorithm	6

2.1	Parameters	6

2.2	The SISO decoder	6

2.3	Decoder architecture	7

3	Complexity estimate – Max Frame Size = 8192 bits	8

3.1	Number of operators and memory bits	8

3.1.1	Number of operators	8

3.1.2	Memory requirements	9

3.2	ASIC oriented complexity	10

3.2.1	Parallelism degree	10

3.2.2	Complexity figures	10

3.3	Latency	10

4	Complexity estimate – Max frame size = 1536 bits	11

4.1	Number of operators and memory bits	11

4.1.1	Number of operators	11

4.1.2	Memory requirements	11

4.2	ASIC oriented complexity	12

4.2.1	Parallelism degree	12

4.2.2	Complexity figures	12

4.3	Latency	12

# Scope

This report describes the complexity of a decoder implementation for the France Telecom partial proposal to IEEE 802.11n standardization process.

# References

[1] IEEE802.11-04/904 “Partial Proposal for 802.11n: Turbo Codes Technical Specification” , Marie-Helene Hamon, John Benko, Olivier Seller,	France Telecom, Sept. 04

# Document releases

| Revision | Date | Description |
| --- | --- | --- |
| 1 |  | initial release |
| 2 |  | Latency figures included. |
| 3 | 12/11/04 | Additional block size requirements. Section 4 |
| 4 | 12/11/04 | Additional latency figures |

# Description of the turbo code

In this part, the turbo code is described. The description is extracted from [1].

## Turbo Encoding

The turbo encoder is an 8-state turbo encoder, as depicted on Figure 1. It uses a double binary Circular Recursive Systematic Convolutional (CRSC) code. The most significant bit of the first byte of the useful payload is assigned to A, the next bit to B, and so on for the remaining of the data burst content. The encoder is fed by blocks of k bits or N couples (k=2*N bits). N is a multiple of 4 (k is a multiple of 8).

Figure 1 : Encoder block diagram

The polynomials, which shall be used for the connections, are described in octal and symbolic notations as follows:

- for the feedback branch: 15 (in octal), equivalently 1+D+D3 (in symbolic notation);

- for the Y1 and Y2 parity bits, 13, equivalently 1+D2+D3;

The input A shall be connected to tap “1” of the shift register and the input “B” shall be connected to the input taps “1”, D and D2.

After initialisation by the circulation state Sc1, the encoder shall be fed by the sequence in the natural order with incremental address i = 0,…,N-1. This first encoding is called C1 encoding.

After initialisation by the circulation state Sc2, the encoder shall be fed by the interleaved sequence with incremental address j = 0,… N-1. This second encoding is called C2 encoding.

The function (j) that gives the natural address i of the considered couple, when reading it at place j for the second encoding, is given in clause 1.2.

## Turbo Code Permutation

The permutation shall be done on two levels:

- the first one inside the couples (level 1),

- the second one between couples (level 2),

The permutation is expressed in the following algorithm.

- Set the permutation parameters P0, P1, P2 and P3.

For MPEG2-TS packet size (188 bytes): P0 = 19, P1 = 376, P2 = 224 and P3 = 600.

- j = 0,… N-1.

- level 1

if j mod. 2 = 0, let (A,B) = (B,A) (invert the couple)

- level 2

- if j mod. 4 = 0, then P = 0;

- if j mod. 4 = 1, then P = N/2 + P1;

- if j mod. 4 = 2, then P = P2;

- if j mod. 4 = 3, then P = N/2 + P3.

    - i = P0*j + P + 1 mod. N

The interleaving relations shall satisfy the odd/even rule (i.e. when j is even, i is odd and vice-versa) that enables the puncturing patterns to be identical for the two puncturing levels.

## Rates & Puncturing Map

Three code rates are defined  : R=1/2, 2/3, ¾.

These rates shall be achieved through selectively deleting the parity bits (puncturing). The puncturing pattern defined in Table 3 shall be applied.

| Code Rate | Puncturing vector |
| --- | --- |
| 1/2 | Y = [1 1 1 1 1 1] |
| 2/3 | Y = [1 0 1 0 1 0] |
| 3/4 | Y = [1 0 0 1 0 0] |

Table 1 : Puncturing patterns for turbo codes (“1”=keep, “0”=delete)

In Table 1, the value “1” means that the bit shall be kept, while the value “0” means that the bit shall be deleted.  The puncturing patterns shall be identical for both codes C1 and C2.

# Decoding Algorithm

This section gives an overview of the decoding algorithm and architecture.

## Parameters

The notations adopted for the main parameters of the encoder and decoder are given here. Their default value is also given. If no mention of the contrary, the default value stated here is applicable:

| Parameter | Name | Value |
| --- | --- | --- |
| Maximum payload size (bits) | Kmax | 8192 |
| Minimum code rate | Rmin | 1/2 |
| Quantization input bit LLRs | wd | 5 |
| Quantization extrinsic data | wz | 5 |
| Quantization metrics | wm | 9 |
| Sliding window size | L | 32 |
| Number of states | Ns | 8 |
| Number of iterations | ITmax | 8 |
| Decoder clock frequency | Fck | 200 |
| Number of SISO modules (parallelism) | P |  |

Table 2 : Parameters and notations

## The SISO decoder

The basic component decoder is called a Soft-Input Soft Output module (SISO). A symbol SISO module is required for optimal performance. A symbol is defined as a couple of bits S = (A,B). The purpose of the SISO module is to compute a set of 4 symbol extrinsic information for each symbol : The extrinsic corresponding to A=i and B=j is denoted Leij.

The SISO module implements a Max-Log MAP algorithm using forward metrics  and backward metrics .

For each k = 0..N-1, and each state s =0 to 7, the metrics are computed recursively using :

| ak(s)=max(s,s')ak−1(s')+ck(s',s) | (2.2-) |
| --- | --- |
| bk(s)=max(s,s')bk+1(s')+ck+1(s,s') | (2.2-) |

The branch metrics ck are computed with the channel observations Ak, Bk,Yk, their respective binary value A, B and Y on the branch, and the extrinsic value produced by the other SISO module used as a priori information Lk(i,j):

| ck(s',s)=Ak2A−1+Bk2B−1+Yk2Y−1+Lk(i,j) | (2.2-) |
| --- | --- |

The extrinsic value is then obtained

| Leij=max(s',s)Ak=i,Bk=jak−1(s')+ck(s',s)+bk(s)−ckp(s',s) | (2.2-) |
| --- | --- |

where ckp is the following (a priori) part of the branch metric :

| ckp(s',s)=Ak2A−1+Bk2B−1+Lk(i,j) | (2.2-) |
| --- | --- |

## Decoder architecture

The architecture of the decoder is shown in Figure 2. Alternative decoder architectures can be adopted, leading to different complexity results.

The main components are :

- two memories storing input bit LLR samples, each one containing 1 frame ;

- an extrinsic memory containing the extrinsic data for 1 frame.

- P SISO modules working in parallel ;

- a memory containing the decision bits for 1 frame.

Figure 2: architecture of the decoder

The turbo decoder permutation allows for such a parallel implementation. Each of the shared memory of size w words by b bits (input samples and extrinsic memories) is split into P sub-memories, each of size w/P words of b bits.

Each SISO implements a sliding window algorithm with a window size of L trellis steps. A memory of L backward metrics is contained into each SISO module. A SISO module contains a backward recursion unit, a forward recursion unit and a symbol extrinsic computation unit.

A decoding iteration consists in two successive half iterations. The same P SISOs are used successively to perform each one of the two half-iterations, as illustrated in Figure 3 for the case P=2.

Figure 3 : sliding window processing (case P=2)

Each SISO decodes 2 bits per clock cycle per half-iteration, resulting in the following overall decoder throughput (in decoded bits/sec):

D=PITmax⋅Fck

Notes :

- the impact on throughput due to pipe-lining and frame overhead processing has not been taken into account.

- the interleaving / deinterleaving units are not represented. However the complexity estimate take these units into account.

# Complexity estimate – Max Frame Size = 8192 bits

This section gives a complexity estimate, in terms of :

- number of operators and memory

- ASIC surface

## Number of operators and memory bits

This section gives the number of operators needed per decoded bit as a function of the code and decoder parameters listed in Table 2. The memory requirements are also addressed.

### Number of operators

This section details the computation requirements for each SISO trellis step, and derives the computational requirements needed for each turbo decoded bit.

A backward or forward trellis steps require each:

- the computation of the set of branch metrics : 14 adders

- for each of the 22.Ns branches:

  - addition of a branch metric to a state metric : 4 adders

- for each of the Ns states :

  - selection between 4 concurrent metrics, i.e. 3 max( )

The extrinsic information computation requires for each trellis step :

- for each of the 22.Ns branches:

  - addition of forward and backward metrics

- for each of the 22 symbols:

  - a max over Ns values : Ns-1 max( )

  - a subtraction of the a priori metric.

- selection of the min. value and substation from all 4 values : 3 max() and 4 adders

The hard decision is needed only for the last half iteration, and is the result of a comparison between 4 symbol a posteriori values, which are intermediate results of the extrinsic computation. The additional complexity required is then3 max() operations.

The following table gives the summary of the complexity in terms of logic operators, as well as the total number of operators per decoded bit per iteration, taking into account that :

- 2 bits are decoded per trellis step,

- 2 SISOs are needed for 1 iteration

|  | Sum | Max |
| --- | --- | --- |
| Backward | 22.Ns+14 = 46 | 3.Ns = 24 |
| Forward | 22.Ns+14= 46 | 3.Ns= 24 |
| Extrinsic | 22.Ns+4 +4= 40 | 4.(Ns-1) +3= 31 |
| Hard decision | 0 | 3 |
| Total per trellis step | 132 | 79 |
| Total per decoded bit per iteration | 132 | 79 |
| Total per decoded bit (8 iteration) | 1056 | 632 |
| Total per decoded bit (5 iteration) | 660 | 395 |

Table 3 : Number of operators

### Memory requirements

The different memory blocks needed are listed in Table 4. The resulting memory need for a turbo decoder is given in Table 5, for different parallelism values.

| Name | heigth (words) | width (bits) | size (bits) | instances needed |
| --- | --- | --- | --- | --- |
| input samples | 8192 | 10 | 81920 | 2 |
| extrinsic | 4096 | 17 | 69632 | 1 |
| decision bits | 4096 | 2 | 8192 | 1 |
| sliding window | 32 | 72 | 2304 | P |

Table 4 : Memory blocks

| P | total memory (bits) | memory bits per decoded bit |
| --- | --- | --- |
| 4 | 250880 | 30.6 |
| 6 | 255488 | 31.2 |
| 8 | 260096 | 31.8 |
| 12 | 269312 | 32.9 |

Table 5 : Decoder memory requirements

## ASIC oriented complexity

### Parallelism degree

An assumption of 200 MHz has been taken as clock frequency, which is conservative for technologies of 0.13um and beyond.

Using the architecture described above, the required parallelism degree needed to achieve 150 and 300 Mbits/s respectively is given in Table 6, under two different assumptions on the maximum number of iterations:

| P | ITmax | Fck | Decoder bitrate (decoded Mbits/s) |
| --- | --- | --- | --- |
| 6 | 8 | 200 | 150 |
| 12 | 8 | 200 | 300 |
| 4 | 5 | 200 | 160 |
| 8 | 5 | 200 | 320 |

Table 6 : Parallelism degree required

A parallelism degree of 6 is required to achieve 150 Mbits/s with 8 iterations. If only 5 iterations are used, a parallelism degree of 4 is sufficient.

A parallelism degree of 12 is required to achieve 300 Mbits/s with 8 iterations. If only 5 iterations are used, a parallelism degree of 8 is sufficient.

### Complexity figures

The complexity of the turbo decoder corresponding to the above parallelism degrees is given in the following table. These results are based on post-synthesis results, using a 0.13um technology of average density of 222 kgates / mm2. The respective proportion associated to logic and memory is also given

| parallelism degree P | decoder complexity (mm2) | logic (%) | memory (%) | decoded bitrate @ 8 it. | decoded bitrate @ 5 it. |
| --- | --- | --- | --- | --- | --- |
| 4 | 2.35 | 15 | 85 | 100 | 160 |
| 6 | 2.93 | 18 | 82 | 150 | 240 |
| 8 | 3.14 | 22 | 78 | 200 | 320 |
| 12 | 4.28 | 28 | 72 | 300 | 480 |

Table 7 : complexity figures, 0.13um technology

It can be observed that a significant part of the decoder complexity is composed by memory.

## Latency

The decoding latency T depends on the frame size k (in bits) , the number of decoding iterations Nit , the decoder clock frequency Fck (Hz) and the parallelism degree P as follows :

T=k⋅NitP⋅Fck seconds

Note that the decoding time is not dependant on the code rate.

The following table gives the decoding time associated to a frame of 8192 payload bit, for different parallelism degree and number of iterations. A clock frequency of 200 MHz has been used :

| parallelism degree P | Iterations Nit | decoding time T (us) |
| --- | --- | --- |
| 4 | 5 | 50 |
| 6 | 8 | 54 |
| 8 | 5 | 25 |
| 12 | 8 | 27 |

Table 8 : decoding time

Note that the frame transmission time, being not dependant on the decoder, has not been taken into account.

# Complexity estimate – Max frame size = 1536 bits

This section is identical to the previous section, but takes into account different block size and code rate constraints :

| max. payload size Kmax (bits) | min. code rate R | max. coded block size (bits) |
| --- | --- | --- |
| 1536 | 3/4 | 2048 |
| 1365 | 2/3 | 2048 |
| 1024 | 1/2 | 2048 |

Table 9 : Second set of block size and code rate constraints

## Number of operators and memory bits

This section gives the number of operators needed per decoded bit as a function of the code and decoder parameters listed in Table 2, amended by the constraints stated in . The memory requirements are also addressed.

### Number of operators

This section is unchanged.

### Memory requirements

The different memory blocks needed are listed in Table 4. The resulting memory need for a turbo decoder is given in Table 5, for different parallelism values.

| Name | heigth (words) | width (bits) | size (bits) | instances needed |
| --- | --- | --- | --- | --- |
| input samples - systematic | 768 | 10 | 7680 | 2 |
| input samples - redundancy | 512 | 10 | 5120 | 2 |
| extrinsic | 768 | 17 | 13056 | 1 |
| sliding window | 32 | 72 | 2304 | 6 |
| decision bits | 768 | 2 | 1536 | 1 |

Table 10 : Memory blocks

| P | total memory (bits) | memory bits per decoded bit (case 1536bits) |
| --- | --- | --- |
| 4 | 49408 | 32,1 |
| 6 | 54016 | 35,1 |
| 8 | 58624 | 38,1 |
| 12 | 67840 | 44,1 |

Table 11 : Decoder memory requirements

## ASIC oriented complexity

### Parallelism degree

Same text applies.

### Complexity figures

The complexity of the turbo decoder corresponding to the above parallelism degrees is given in the following table. These results are based on post-synthesis results, using a 0.13um technology of average density of 222 kgates / mm2. The respective proportion associated to logic and memory is also given

| parallelism degree P | decoder complexity (mm2) | logic (%) | memory (%) | decoded bitrate @ 8 it. | decoded bitrate @ 5 it. |
| --- | --- | --- | --- | --- | --- |
| 4 | 0.836 | 43 | 57 | 100 | 160 |
| 6 | 1.18 | 44 | 56 | 150 | 240 |
| 8 | 1.40 | 48 | 52 | 200 | 320 |
| 12 | 2.00 | 50 | 50 | 300 | 480 |

Table 12 : complexity figures, 0.13um technology

## Latency

The decoding latency T depends on the frame size k (in bits) , the number of decoding iterations Nit , the decoder clock frequency Fck (Hz) and the parallelism degree P as follows :

T=k⋅NitP⋅Fck seconds

Note that the decoding time is not dependant on the code rate.

The following tables give the decoding time associated to different block sizes(1024, 1365, 1536 payload bits), for different parallelism degree and number of iterations. A clock frequency of 200 MHz has been used :

| parallelism degree P | Iterations Nit | Decoding Time T (us) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1024 bits | 1365 bits | 1536 bits |
| 4 | 5 | 6,4 | 8,5 | 9,6 |
| 6 | 8 | 6,8 | 9,1 | 10,24 |
| 8 | 5 | 3,2 | 4,2 | 4,8 |
| 12 | 8 | 3,4 | 4,5 | 5,12 |

Table 13 : decoding time, 1024, 1365, and 1536 payload bits (all code rates)

Note that the frame transmission time, being not dependant on the decoder, has not been taken into account.