IEEE P802.11
Wireless LANs

Proposed Normative Text for QoS Metrics

Date:	November 2004

Author: 	Emily H. Qi
	Intel Corporation

2111 N.E 25th Avenue

Hillsboro, OR 97124

e-Mail: emily.h.qi@intel.com

Abstract

This document contains editorial instructions to add normative text to the latest TGk draft for incorporating QoS metrics for the specified traffic category or traffic stream.  This contribution provides comment resolution for LB71comment #928.

Modify the table 20b as indicated below:

Table 20b—Measurement Type definitions for measurement requests

| Name | Measurement Type |
| --- | --- |
| Basic Request | 0 |
| Clear Channel Assessment (CCA) request | 1 |
| Receive power indication (RPI) histogram request | 2 |
| Traffic Stream request | 3 |
| Noise histogram request | 4 |
| Beacon request | 5 |
| Frame request | 6 |
| Hidden node request | 7 |
| Medium sensing time histogram request | 8 |
| STA statistics request | 9 |
| Traffic Stream request | 10 |
| Reserved | 11-255 |

Add the following subsection after Clause 7.3.2.21.10 in 802.11k draft 1.0 as indicated below:

##### 7.3.2.21.11 Traffic Stream Request

The Measurement Request field corresponding to a Traffic Stream Request is shown in Figure +1. A response to a Traffic Stream Request is a Traffic Stream Report.

|  | Randomization Interval | Measurement Duration | Traffic Identifier (TID) |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure +1 – Measurement Request field format for a Traffic Stream Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.7.3.

The Measurement Duration field shall be set equal to the duration of the requested measurement, expressed in TUs.

Traffic Identifier (TID) field indicates the requested measuring traffic category or traffic stream. The definition of TID is described in 7.1.3.5.1 “TID field” of IEEE 802.11e/D11.0.

Modify the table 20c as indicated below:

Table 20c—Measurement Type definitions for measurement reports

| Name | Measurement Type |
| --- | --- |
| Basic report | 0 |
| Clear Channel Assessment (CCA) report | 1 |
| Receive power indication (RPI) histogram report | 2 |
| Traffic Stream report | 3 |
| Noise histogram report | 4 |
| Beacon report | 5 |
| Frame report | 6 |
| Hidden node report | 7 |
| Medium sensing time histogram report | 8 |
| STA statistics report | 9 |
| Traffic Stream report | 10 |
| Reserved | 311-255 |

Add the following subsection after Clause 7.3.2.22.10 in 802.11k draft 1.0 as indicated below:

##### 7.3.2.22.11 Traffic Stream Report

The format of the Measurement Report field corresponding to a Traffic Stream Report is shown in Figure

k+2.

|  | Traffic Identifier (TID) | Actual Measurement Start Time | Measurement Duration | Transmitted Frame Count | Frame Lose Count | Accumulate Transmit Frame Delay |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 8 | 2 | 4 | 4 | 4 |

Figure k+2 – Measurement Report field format for a Traffic Stream Report

Traffic Identifier (TID) field indicates the measuring traffic category or traffic stream. The definition of TID is described in 7.1.3.5.1 “TID field” of IEEE 802.11e/D11.0. TID

Actual Measurement Start Time shall be set equal to the TSF at the time at which the measurement started.

Measurement Duration shall be set equal to the duration over which the Beacon Report was measured, expressed in TUs.

Transmitted Frame Count indicates the total number of frames that are successfully transmitted after 0 or n reties during the measurement duration for the measured traffic TID.

Frame Loss Count indicates the total number of frames that are discarded after given numbers of retries during the measurement duration for the measured traffic TID.

Accumulate Transmit Frame Delay indicates the total of delay for all frames that are successfully transmitted during the measurement duration for the measured traffic TID. Delay is measured from the time a packet arrived to MAC layer to the time acknowledgement received.

### ANNEX D

Add the following text after dot11STAStatisticsReport table in TGk draft Annex D:

-- ********************************************************************

-- * dot11TrafficStreamReport	TABLE

-- ********************************************************************

dot11TrafficStreamReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF dot11TrafficStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Traffic Stream reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11RRMReport 1 }

dot11TrafficStreamReportEntry OBJECT-TYPE

SYNTAX dot11TrafficStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11TrafficStreamReportTable Indexed by dot11TrafficStreamRprtIndex."

INDEX { dot11TrafficStreamRprtIndex }

::= { dot11TrafficStreamReportTable 1 }

Dot11TrafficStreamReportEntry ::=

SEQUENCE {

dot11TrafficStreamRprtIndex			Unsigned32,

dot11TrafficStreamRprtRqstToken 		OCTET STRING,

dot11TrafficStreamRprtIfIndex		InterfaceIndex,

dot11TrafficStreamMeasuringSTAAddr		MacAddress,

dot11TrafficStreamRprtTID			INTEGER,

dot11TrafficStreamRprtActualStartTime		TSFType,

dot11TrafficStreamRprtMeasurementDuration	Unsigned32,

dot11TrafficStreamRprtTranmittedFrameCount	Integer32,

dot11TrafficStreamRprtFrameLoseCount		Integer32,

dot11TrafficStreamRprtAccumulateTransmitFrameDelay Integer32

}

dot11TrafficStreamRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Traffic Stream Report elements in dot11TrafficStreamReportTable, greater than 0."

::= { dot11TrafficStreamReportEntry 1 }

dot11TrafficStreamRprtRqstToken OBJECT-TYPE

SYNTAX OctetString

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the measurement request that generated this measurement report.  This should be an exact match to the original dot11RRMRqstToken attribute. Note that there may be multiple entries in the table that match this value since a single request may generate multiple measurement reports."

::= { dot11TrafficStreamReportEntry 2 }

dot11TrafficStreamRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of TrafficStream Report has been received on."

::= { dot11TrafficStreamReportEntry 3 }

dot11TrafficStreamMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Traffic Stream report."

::= { dot11TrafficStreamReportEntry 4 }

dot11TrafficStreamRprtTID OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute identifies the requested measuring traffic category or traffic stream. The definition of TID is described in 7.1.3.5.1 “TID field” of IEEE 802.11e/D11.0.”

::= { dot11TrafficStreamReportEntry 5 }

dot11TrafficStreamRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the measurement started."

::= { dot11TrafficStreamReportEntry 6 }

dot11TrafficStreamRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Traffic Stream Report was measured, expressed in TUs."

::= { dot11TrafficStreamReportEntry 7 }

dot11TrafficStreamRprtTransmittedFrameCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the total number of frames that are successfully transmitted after 0 or n reties during the measurement duration for the measured Traffic Identifier (TID)”

::= { dot11TrafficStreamReportEntry 8 }

dot11TrafficStreamRprtFrameLoseCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the total number of frames that are discarded after given numbers of retries during the measurement duration for the measured traffic Identifier (TID)”

::= { dot11TrafficStreamReportEntry 8 }

dot11TrafficStreamRprtAccumulateTransmitFrameDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the total of delay for all frames that are successfully transmitted during the measurement duration for the measured traffic Identifier (TID). Delay is measured from the time a packet arrived to MAC layer to the time acknowledgement received".

::= { dot11TrafficStreamReportEntry 8 }

-- ********************************************************************

-- * End of dot11TrafficStreamReport	TABLE

-- ********************************************************************