IEEE 802.11k
Wireless LANs

Normative Text for the Proposed Simplifications to TBTT Offset Format and Calculation

Date: November 12, 2004

Author:

Stephen Wang

Stephen.Wang@motorola.com

Tim Wilson

Tim.Wilson@motorola.com

Steve Emeott

Steve.Emeott@motorola.com

Walter Johnson

Walter.Johnson@motorola.com

Floyd Simpson

Floyd.Simpson@motorola.com

Motorola, Inc.

Abstract

#### This proposal presents normative text for the proposed TBTT Offset simplifications outlined in document 11-04-1213-00-000k. Major additions and updates are highlighted in red color. Deleted text is not shown in the document.

7.3.2.26 Neighbor Report element

Update the clause from Figure k18 and below as shown:

|  | Element ID | Length |
| --- | --- | --- |
| Octets: | 1 | 2 |

|  | BSSID | BSSID Information | Channel Number | Channel Band | PHY Options | Neighbor TSF Information (Optional) | The element contains zero or more neighbor list entries |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 1 | 1 | 1 | 4 |  |

Figure k18 – Neighbor Report element format

The Element ID field shall be equal to the Neighbor Report value in Table 20.

The Length field is dependent on the number of Neighbor List Entries representing the neighboring AP’s being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Current Channel, Channel Band, PHY Options, and optionally may include the Neighbor TSF Information fields. The minimum value of the Length field is 0 (i.e. with no neighbor APs in the Neighbor Report element).

The BSSID is the address of the STA contained in the AP. The subsequent fields: BSSID Information, Channel Number, Channel Band, PHY Options, and optionally Neighbor TBTT Offset and Beacon Interval are presumed to be for the corresponding BSSID.

The BSSID Information field may be used to help determine neighbor AP roaming candidates. It shall be two octets in length and shall contain the subfields as shown in Figure k19.

|  | B0 B1 | B2 | B3 | B4 B8 | B9 B15 |
| --- | --- | --- | --- | --- | --- |
|  | Reachability | RSN | Key Scope | Capabilities | Reserved |
| Bits | 2 | 1 | 1 | 5 | 7 |

Figure k19 – BSSID Information field format

The Reachability field indicates whether the AP represented by this BSSID is reachable by the STA for the purposes of pre-authentication. The values are as follows:

| Reachability | Value |
| --- | --- |
| Reserved | 0 |
| Not Reaciable | 1 |
| Unknown | 2 |
| Reachable | 3 |

The value "Reachable" implies that the AP represented by this BSSID is reachable by the STA for the purposes of pre-authentication. Reachability may be achieved by a variety of mechanisms, including VLANs, tunnels, etc. In the case where VLANs are supported, an AP is “Reachable” if it is a member of the same VLAN as the STA. For example, in the case where APs are only members of the default VLAN, an AP will be "Reachable" if the STA is also assigned to the default VLAN, but will otherwise be "Not Reachable". Where the AP may be a member of multiple VLANs, but the VLAN membership of either the AP or the STA is not known, the value Unknown should be returned.

	The RSN bit, if set, indicates that the AP represented by this BSSID matches the current AP’s RSN IE capabilities. If the bit is not set, it indicates that either the capabilities do not match or the information is not available to the AP at this time.

	Bit 3 indicates whether the AP represented by this BSSID has the same authenticator identity as the AP sending this report.

	The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent by the AP being reported. The format of the Capabilities subfield is as follows:

|  | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Block Ack |
| Bits | 1 | 1 | 1 | 1 | 1 |

Figure k20 – Capabilities Subfield

	Bits 9, 10, and 11 are reserved due to use by existing implementations (e.g. WPA, WMX/EDCA and WMX/HCCA).

	Bits 12 – 15 are reserved and shall be set to 0 on transmission and ignored on reception.

Channel Number indicates the current operating channel of the AP represented by this BSSID.

Channel Band shall contain an enumerated value from Table k1 specifying the frequency band in which the Current Channel is valid.

The PHY Options field represents the Condensed PHY type and Neighbor TSF Information Flag as shown in the Figure k21.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | Neighbor TSF Information Flag |
| Bits: | 7 | 1 |

Figure k21 – PHY Options field

- Condensed PHY Type is the lower 7 bits of the PHY Type of the AP represented by this BSSID. The PHY type will be encoded as defined by dot11PHYType.

- Neighbor TSF Information Flag is a one bit field. When set to 1, it indicates that the Neighbor List Entry contains the Neighbor TSF Information field. The Neighbor Report may have the Neighbor TSF Information Flag set in some Neighbor List Entries and not set in others.

The Neighbor TSF Information is 4 octets long and contains Neighbor TSF Offset and Neighbor Beacon Interval subfields.

|  | Neighbor TSF Offset | Neighbor Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Neighbor TSF Information field

Figure  k22 – Neighbor Report TSF Information field format

The Neighbor TSF Offset subfield is 2 octets long and contains TSF timer offset, in units of TUs(1.024ms), between the serving AP and a neighbor AP, modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Neighbor Beacon Interval subfield is the beacon interval of the Neighbor AP being reported. This subfield is 2 octets long and defined in clause 7.3.1.3.

11.8.2 Neighbor Report Response

Replace the entire clause with text shown below:

An AP accepting a Neighbor Report Request shall respond with a Neighbor Report Response frame containing one or more Neighbor Report elements. If the SSID information element is specified in the corresponding Neighbor Report Request frame, the Neighbor Report elements shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the SSID. Otherwise, the Neighbor Report elements shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the current SSID with which the requesting STA is associated.

A serving AP shall include a neighbor AP’s Neighbor TSF Information field in the Neighbor Report Response frame only if it is able to guarantee an accumulated error of 1.5 TU or less on the Neighbor TSF Offset subfield. The error budget (1.5 TU) can be broke down as the followings:

- Delays by the measuring STA in transmitting the first bit of the Beacon Report after receiving the last bit of a neighbor AP’s Beacon or Probe Response (±0.5 TU).

- Error caused by rounding to the nearest TU boundary when converting Neighbor TSF Offset from microseconds to TUs (0.5 TU).

- Delays by the serving AP between reception of the last bit of the Beacon Report and transmission of the first bit of the Neighbor Report (0.5 TU).

An AP may also send Neighbor Report Response frames autonomously. For example, the AP may autonomously send a Neighbor Report Response frame to a recently associated STA without receiving a Neighbor Report Request from that STA, or the AP may autonomously broadcast a Neighbor Report Response frame to all STAs in the BSS.