| Project | IEEE P802.11k Radio Resource Measurement Task Group <http://grouper.ieee.org/groups/802/11/> |  |
| --- | --- | --- |
| Title | A Proposed Resolution to TGk LB71 comment on Neighbor Report in Association Response |  |
| Date Submitted | 12 November 2004 |  |
| Source(s) | Motorola, Inc. 1. Floyd Simpson 2. Walter Johnson 3. Stephen Wang 4. Steve Emeott | Email: floyd.simpson@motorola.com Email: walter.johnson@motorola.com Email: Stephen.wang@motorola.com Email: steve.emeott@motorola.com |
| Re: | A Proposed Resolution to TGk Letter Ballot #71, Comment #167, 743 |  |
| Abstract | Instruction to the TGk technical editor containing Normative text for allowing a Neighbor Report to be returned in an Association Response frame as requested by accepted TGk comments. |  |
| Purpose | Provide a definitive resolution to TGk LB#71 comments #167,743 to allow Neighbor Report information element to be sent in an Association Response frame as suggested by the accepted comment in doc 11-04-1210-00-000k-lb71-comment-spreadsheet-neighbor-report-resolution.xls. |  |
| Notice | This document has been prepared to assist IEEE 802.11k. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.11 may make this contribution public. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.11 Patent Policy http://standards.ieee.org/guides/bylaws/sect6-7.html#6, and Procedures http://standards.ieee.org/guides/opman/sect6.html#6.3, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the 802.11 Working Group Chair mailto:stuart.kerry@philips.com as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. The Chair will disclose this notification at Working Group meetings and via the IEEE website http://standards.ieee.org/db/patents/pat802_11.html. |  |

### A Proposed Resolution to TGk LB71 comment on Neighbor Report in Association Response

Abstract: Instruction to the TGk technical editor containing normative text for allowing a requested Neighbor Report to be returned in an Association Response frame.

7.3.2 Information Elements

Insert Element ID TBD-TBD into Table 20 and change the reserved row accordingly:

Insert ‘Neighbor Report w/ Neighbor TSF Information’ element to Table 20 as shown below:

#### Table 20 – Element IDs

| #### Information Element | #### Element ID |
| --- | --- |
| AP Channel Report | TBD |
| Neighbor Report | TBD |
| Neighbor Report w/ Neighbor TSF Information | TBD |
| RCPI | TBD |
| Reserved | TBD-255 |

7.2.3.4 Association Request frame format

Insert order 10 Request element to Table 7 as shown below:

Table 7 – Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Request Information | Optional information element that shall only contain the Neighbor Report element ID or the Neighbor Report w/ Neighbor TSF Information element ID. |

7.2.3.5 Association Response frame format

Insert order 6 Requested information elements to Table 8 as shown below:

Table 8 – Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Requested Information element | Neighbor Report information element requested in the Request information element of the Association Request frame. |

7.3.2.12 Request information element

This element is placed in a Probe Request or Association Request frame to request that the responding STA include the requested information in the Probe Response or Association Response frame.  The format of the information element shall be as shown in Figure 46.

The Element ID of this information element shall be 10.  The information element is variable in length.  The length of the information element is indicated in the Length field.

The Requested Element IDs are the list of elements that are to be included in the responding STA’s Probe Response or Association Response frame.  The Requested Element IDs shall be listed in order of increasing element ID.

A STA shall return only those information elements that it supports.  In an improperly formed Request information element, a STA may ignore the first information element requested that is not ordered properly and all subsequent information elements requested.  In the probe or association response frame, the STA shall return the requested information elements in the same order requested in the Request information element of the probe or association request frame.

Edit clause 11.8.1 and 11.8.2 as shown below:

11.8.1 Requesting a Neighbor Report Request

An associated STA requesting a Neighbor Report shall send a Neighbor Report Request frame to its associated AP. If the STA is interested in information concerning neighbor APs matching a specific SSID, it shall specify the SSID element in the Neighbor Report Request frame.A STA that is not yet associated may send an Association Request frame with a Request information element containing the element ID for the Neighbor Report or the Neighbor Report w/ Neighbor TSF Information in order to request a Neighbor Report in the Association Response frame. If the Request element in the Association Request contains the element ID for the Neighbor Report, then the Neighbor Report in the Association Response frame shall not contain the Neighbor TSF Information in the Neighbor Report. If the Request element in the Association Request frame contains the element ID for the Neighbor Report w/ Neighbor TSF Information, then the Neighbor Report returned in the Association Response frame shall also contain the Neighbor TSF Information, if available from the AP.

11.8.2 Receiving a Neighbor Report Response

An AP accepting a Neighbor Report Request shall respond with a Neighbor Report Response frame containing one or more Neighbor Report elements. If the Neighbor Report is requested using the Request element in an Association Request frame, an AP accepting the request shall respond with the Neighbor Report in the corresponding Association Response frame. If the SSID information element is specified in the corresponding Neighbor Report Request frame, the Neighbor Report elements shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the SSID. Otherwise, the Neighbor Report elements shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the current SSID with which the requesting STA is associated.

A Neighbor Report Response frame may also be sent autonomously by an AP. For example, the AP may autonomously send a Neighbor Report Response frame to a recently associated STA without receiving a Neighbor Report Request from that STA, or the AP may autonomously broadcast a Neighbor Report Response frame to all STAs in the BSS.