IEEE P802.11
Wireless LANs

Proposed Text for Adding Neighbor Report to Association Response

Date:	15 November 2004

Author: 	Joe Kwak
	InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

This paper provides normative text to provide a Neighbor Report in an Association Response frame as indicated by accepted TGk comments from Seattle AdHoc.  Same change is also made in Reassociation Response frame. These are the draft resolutions to TGk LB71 comments #167, #743, which were assigned to Joe Kwak.

.

7.2.3.5 Association Response frame format

Insert new row into Table 8 as shown below:

Table 8 – Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| kTBD | Neighbor Report | The Neighbor Report element shall be present if dot11RadioMeasurementEnabled is true in both the AP and in the associating STA and if there are neighbors to report. The Neighbor Report shall include any optional elements which are available. |

7.2.3.7 Reassociation Response frame format

Insert new row into Table 10 as shown below:

Table 10 – Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| kTBD | Neighbor Report | The Neighbor Report element shall be present if dot11RadioMeasurementEnabled is true in both the AP and in the associating STA and if there are neighbors to report. The Neighbor Report shall include any optional elements which are available. |