IEEE P802.11
Wireless LANs

| Interpretation Response 01 11/04 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: November 18, 2004 |  |  |  |  |
| Author(s) |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bob O’Hara | Airespace | 110 Nortech Pkwy, San Jose, CA 95134 | 408 635 2025 | bob@airespace.com |

Abstract

An interpretation request was received on the content of Annex G and its accuracy.  This document is the response to that interpretation request.  The result of analysis of the material in Annex G is that some of the material is not accurate.  The corrected material is presented here.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at < patcom@ieee.org>

Interpretation Number: 1-11/04 (Annex G acccuracy)

Topic: Accuracy of material in Annex G

Relevant Clauses: Annex G

Classification: Conflicts with clause 17

Interpretation Request

I wrote an email to your colleague Terry Cole related to a possible mistake

I found out while working with IEEE 802.11a standard. He told me to ask for

a request for an interpretation, so I am writing this mail to you. I will

try to follow the steps given in the web for it.

1.Designation of the standard, including the year of publication.

IEEE Std 802.11a-1999

Part 11: Wireless LAN Medium Access Control (MAC) and Pjysical Layer

(PHY) specifications

High-speed Physical Layer in the 5 GHz Band

2.The specific subsection being questioned.

Annex G.

An example of encoding a frame for OFDM PHY. Table G.17-Last 144 bits

scrambling.

3. The applicable conditions for the case in question.

In order to check if our design of the PHY is correct, we have check

it by means of the example given in Annex G "An example of encoding a

frame for OFDM PHY".

All our results match properly with those given in all the tables

except for 'Table G.17-Last 144 bits after scrambling'. In this case,

the bits number 818 and 820 which are in the standar are '0'.

However, we obtain two '1'.

In order to check if the scrambler module works properly we have try

to obtaining the 127-bit repeatitive sequence given in page 16 in

chapter '17.3.5.4. PLCP DATA scrambler and descrambler' and we do it

correctly. As it can be saw, the values of the 7-bit shift register

of the scrambler depends only on the seed. In the example given, the

seed is '1011101' and we have checked as well that we match the

'Table G.15- Scrambling sequence for seed 1011101'. Therefore, in our

opinion the mismatch in 'Table G.17' might be caused by the input of

the scrambler, that is, the DATA bits. However, we are obtaining the

same values than those given in 'Table G.14-Last 144 DATA bits'.

After all this explanation, i would be really grateful if you could confirm

to us that the data showed at 'Table G.17' are completely correct or if

there is any mistakes on it.

Interpretation for IEEE STD 802.11a-1999 (reaffirmed 2003)

In document 11-04-1198-00-00m, Inoue-san, et al, provide an analysis of the material in question in Annex G.  This analysis is reproduced here.

- The two bits (bit#818 and bit#820) in the Table G.17 should be corrected.

  - Both of bit #818 and bit #820 in the Table G.17 are “0” in current standard.

  - Both bit #818 and bit #820 should be modified to be “1”.

- Table G.1:

  - Table G.1 consists of following items.

    - The MAC header (24 octets)

    - The first 72 characters of the original message converted to ASCII code

    - The  CRC32 (4 octets)

  - A PSDU of length 100 octets (= 800 bits)

    - {04 02 00 2e 00 60 08 cd … 74 72 65 61 da 57 99 cd}

- Generation of DATA field in PPDU frame:

  - The DATA field in a PPDU frame consists of SERVICE field, PSDU, tail (PPDU TAIL) and Pad bits.

  - Table G.13 shows the first 144 bits of the DATA field.

    - The SERVICE field (sixteen “0” bits) are added before PSDU.

  - Table G.14 shows the last 144 bits of the DATA field.

    - Assuming the modulation mode of 36M bps (16QAM, r=3/4), 42 Pad bits are appended in this case.

- Table G.15 – Scrambling sequence for seed 1011101

  - This is the output sequence of the scrambler when the initial value of the shift register is 1011101.

  - We have confirmed that the Table G.15 was correct.

- We have confirmed that:

  - Table G.16 (First 144 bits after scrambling) was correct.

  - Table G.17 (Last 144 bits after scrambling) was NOT correct as the requester pointed out.

Table G.17

| ## | Bit | ## | Bit | ## | Bit | ## | Bit | ## | Bit | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 720 | 0 | 744 | 0 | 768 | 1 | 792 | 0 | 816 | 0 | 840 | 0 |
| 721 | 1 | 745 | 0 | 769 | 0 | 793 | 0 | 817 | 0 | 841 | 0 |
| 722 | 1 | 746 | 0 | 770 | 1 | 794 | 1 | 818 | 1 | 842 | 0 |
| 723 | 1 | 747 | 1 | 771 | 0 | 795 | 1 | 819 | 0 | 843 | 0 |
| 724 | 1 | 748 | 0 | 772 | 0 | 796 | 0 | 820 | 1 | 844 | 1 |
| 725 | 1 | 749 | 1 | 773 | 0 | 797 | 0 | 821 | 0 | 845 | 1 |
| 726 | 0 | 750 | 1 | 774 | 0 | 798 | 0 | 822 | 0 | 846 | 1 |
| 727 | 1 | 751 | 1 | 775 | 0 | 799 | 0 | 823 | 0 | 847 | 0 |
| 728 | 1 | 752 | 0 | 776 | 1 | 800 | 1 | 824 | 1 | 848 | 1 |
| 729 | 0 | 753 | 0 | 777 | 1 | 801 | 0 | 825 | 1 | 849 | 1 |
| 730 | 0 | 754 | 1 | 778 | 1 | 802 | 0 | 826 | 0 | 850 | 1 |
| 731 | 0 | 755 | 1 | 779 | 0 | 803 | 0 | 827 | 1 | 851 | 1 |
| 732 | 1 | 756 | 1 | 780 | 1 | 804 | 1 | 828 | 1 | 852 | 0 |
| 733 | 0 | 757 | 0 | 781 | 1 | 805 | 1 | 829 | 1 | 853 | 0 |
| 734 | 1 | 758 | 0 | 782 | 0 | 806 | 0 | 830 | 0 | 854 | 1 |
| 735 | 0 | 759 | 1 | 783 | 0 | 807 | 0 | 831 | 0 | 855 | 0 |
| 736 | 0 | 760 | 0 | 784 | 0 | 808 | 1 | 832 | 0 | 856 | 1 |
| 737 | 0 | 761 | 0 | 785 | 0 | 809 | 1 | 833 | 1 | 857 | 1 |
| 738 | 1 | 762 | 0 | 786 | 1 | 810 | 0 | 834 | 1 | 858 | 0 |
| 739 | 0 | 763 | 1 | 787 | 0 | 811 | 0 | 835 | 1 | 859 | 0 |
| 740 | 0 | 764 | 0 | 788 | 1 | 812 | 1 | 836 | 1 | 860 | 1 |
| 741 | 1 | 765 | 1 | 789 | 0 | 813 | 0 | 837 | 1 | 861 | 0 |
| 742 | 1 | 766 | 0 | 790 | 0 | 814 | 1 | 838 | 1 | 862 | 0 |
| 743 | 1 | 767 | 1 | 791 | 0 | 815 | 0 | 839 | 1 | 863 | 1 |

| Data | Tail | Pad |
| --- | --- | --- |

- The bit #818 and bit #820 in the Table G.17 correspond to the PPDU TAIL bits.

- The PPDU TAIL bits and PAD bits are all “0”

  - There will be exact sequence of the scrambler output after bit #816 in the Table G.17.

  - The bits #818 and #820 in the Table G.17 correspond the bits #56 and #58 in the Table G.15 which have value of “1”.

The conclusion of this analysis is that two bits in table G.17 are in error.  This error will be brought to the attention of the 802.11 working group to be addressed in the next revision of the standard.