IEEE P802.11
Wireless LANs

Partial Proposal for 802.11n: ITRI Preamble Summary

Date:	November 16, 2004

Authors:	Yung-Yih Jian, Pangan Ting, Wei-Ping Chuang,
	Chun-Chun Kuo, Chang-Lung Hsiao
	Industrial Technology Research Institute (ITRI)
	Computer and Communication Research Laboratories (CCL)
	Bldg.14, 195 Sec. 4, Chung Hsing Rd. Chutung, Hsinchu, Taiwan 310, R.O.C.
	Phone: +886-3-5914752
	Fax: +886-3-5820371
	e-Mail: {yyjian, pating, joeChuang, godeman, clhsiao}@itri.org.tw

Wei-De Wu
	National Tsing Hua University
	Institute of Communications Engineering
	welder@micro.ee.nthu.edu.tw

Chih-Kai Chen
	Stanford University
	Department of Electrical Engineering
	chihkai@stanford.edu

Abstract

In this summary, we summarized the proposed novel preamble structure for IEEE 802.11n standard. The proposed preamble structure is compliant with the FRCC documents and has good properties for frame detection and channel estimation.

### Short Training Symbols

The new STSs for IEEE 802.11n have following three properties:

- Periodicity with period 32.

- Zero cross-correlation among different STSs.

- Low detection probability for IEEE 802.11a receivers.

### Long Training Symbols

The proposed training block is shown, where W4k=exp(−jk2π4), cyclic prefix (CP) is of 16 samples, and L is the basis of the proposed LTS for IEEE 802.11n, which is the same with the LTS of IEEE 802.11a. With the structure of the proposed LTS for IEEE 802.11n, the receiver can estimate channel responses with low complexity. Specifically, an efficient 4 x 4 FFT matrix can be used.