IEEE P802.11
Wireless LANs

| JTC1/SC6 doc ChinaComment-B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2004-12-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jesse Walker | Intel Corporation | JF3-206, 2111 NE 25th Avenue, Hillsboro, OR USA 97124 | 503-712-1849 | jesse.walker@intel.com |

Abstract

This document contains the text of document “ChinaCommentB” forwarded by Mr. Robin Tasker, the Liaison from ISO/IEC JTC1/SC6 to IEEE 802, to Mr. Jesse Walker, the Liaison from IEEE 802 to ISO/IEC JTC1/SC6, as information.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Chinese Comment "B" on DIS 8802-11 / Agreed Disposition of Comments

From the fast track ballot of  ISO/IEC DIS 8802.1, China voted NO against two comments.Comment A relates to their security model (dealt with separately), and their Comment B which stated that the DIS text was devoid of Chinese regulatory information. In the agreed Disposition of Comments Report the following was produced (in coordination with IEEE 802.11) and agreed as a part of publication process to full International Standard,

Chinese Comment B, ISO/IEC DIS 8802.11 is devoid of China regulation

The PE thanks the Chinese NB for their Comment B and acknowledge that this specific regulatory information is currently not included in ISO/IEC 8802-11 (IEEE 802.11). The IEEE 802.11 committee has currently open a maintenance item of work and following appropriate liaison would welcome the Chinese input to it. Once this IEEE 802.11 work is complete (and this is scheduled for end-2004) it is proposed to forward it through the ISO/IEC fast track procedures to ensure its incorporation within the International Standard.

Below I have cut and pasted the Chinese Comment B; please can you regard this as the liaison to ensure that the comment is included within the Maintenance release for WG ballot etc etc

COMMENT B

14.6.2 Regulatory requirements

Change the text in 14.6.2 and Table 39 as shown:

Wireless LANs implemented in accordance with this standard are subject to equipment certification and operating requirements established by regional and national regulatory administrations. The PMD specification establishes minimum technical requirements for interoperability, based upon established regulations for China, Europe, Japan, and North America at the time this standard was issued. These regulations are subject to revision, or may be superseded. Requirements that are subject to local geographic regulations are annotated within the PMD specification. Regulatory requirements that do not affect interoperability are not addressed within this standard. Implementers are referred to the following regulatory sources for further information.

Operation in countries within Europe, or other regions outside Japan or North America, may be subject to additional or alternative national regulations.

The documents listed below specify the current regulatory requirements for various geographic areas at the time this standard was developed. They are provided for information only, and are subject to change or revision at any time.

Table 39—Regulatory requirements for various geographic areas

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| China | The Radio Administration of Information Industry Ministry of P.R.China | The Radio Administration of Information Industry Ministry NO.353[2002] | The Radio Administration of Information Industry Ministry of P.R.China |
| Europe | EuropeanTelecommunications Standards Institute(ETSI) | ETS300-328 ETS300-339 | National type approval authorities |
| France | Règle technique applicable aux équipments radioélectriques de transmission de données à large bande fonctionnant dans la bande de fréquences à 2.4GHz et utilisant la technique de étalenment de spectre(Édition févier 1995) | SP/DGPT/ATAS/23 ETS 300-328 ETS 300-339 | Direction Générale des Postes et Télécommunications (DGPT) |
| Japan | Association of Radio Industries and Business(ARIB) | RCR STD-33A | Ministry of Telecommunications (MKK) |
| North America Canada United States | Industry Cannada(IC) Federal Communications Commision(FCC) | GL-36 [B7] CFR47 [B6], Part 15, Sections 15.205, 15.209, 15.247 | IC FCC |
| Spain | Supplement Del Numbero164 Del Boletin Oficial Del Estado(Published 10 July 16， 1991，Revised 25 June 1993) | ETS 300-328 ETS 300-339 | Cuadro Nacional De Atribucion De Frecuesias |

14.6.3 Operating frequency range

Change Table 40 as shown:

Table 40—Operating frequency range

| Lower Limit | Upper limit | Regulatory range | Geography |
| --- | --- | --- | --- |
| 2.402 GHz | 2.480 GHz | 2.400-2.4835 GHz | China |
| 2.402 GHz | 2.480 GHz | 2.400-2.4835 GHz | North America |
| 2.402 GHz | 2.480 GHz | 2.400-2.4835 GHz | Europea |
| 2.473 GHz | 2.495 GHz | 2.471-2.497 GHz | Japan |
| 2.447 GHz | 2.473 GHz | 2.445-2.475 GHz | Spain |
| 2.448 GHz | 2.482 GHz | 2.4465-2.4835 GHz | France |
| NOTE—The frequency ranges in this table are subject to the geographic-specific regulatory authorities. |  |  |  |

aExcluding Spain and France.

14.6.4  Number of operating channels

Change the text in 14.6.4 and Table 41 as shown:

The number of transmit and receive frequency channels used for operating the PMD entity is 79 for China , the United States and Europe, and 23 for Japan. Table 41 summarizes these frequencies for a number of geographic locations. This is more fully defined in Table 42 through Table 45.

Table 41—Number of operating channels

| Minimum | Hopping set | Geography |
| --- | --- | --- |
| 75 | 79 | China |
| 75 | 79 | North America |
| 20 | 79 | Europea |
| N/A | 23 | Japan |
| 20 | 27 | Spain |
| 20 | 35 | France |
| NOTE—The number of required hopping channels is subject to the geographic-specific regulatory authorities. |  |  |

aExcluding Spain and France.

14.6.5  Operating channel center frequency

Change the text in 14.6.5 and Table 42 as shown:

The channel center frequency is defined in sequential 1.0 MHz steps beginning with the first channel, channel 2.402 GHz for China, the USA and Europe excluding Spain and France, as listed in Table 42. The channel centers for Japan, starting at 2.473 GHz with 1 MHz increments, are listed in Table 43. The channel centers for Spain and France are listed in Table 44 and Table 45, respectively.

Table 42—Requirements in China, North America and Europe

(excluding Spain and France; values specified in GHz)

| Channel # | Value | Channel # | Value | Channel # | Value |
| --- | --- | --- | --- | --- | --- |
| 2 | 2.402 | 28 | 2.428 | 54 | 2.454 |
| 3 | 2.403 | 29 | 2.429 | 55 | 2.455 |
| 4 | 2.404 | 30 | 2.430 | 56 | 2.456 |
| 5 | 2.405 | 31 | 2.431 | 57 | 2.457 |
| 6 | 2.406 | 32 | 2.432 | 58 | 2.458 |
| 7 | 2.407 | 33 | 2.433 | 59 | 2.459 |
| 8 | 2.408 | 34 | 2.434 | 60 | 2.460 |
| 9 | 2.409 | 35 | 2.435 | 61 | 2.461 |
| 10 | 2.410 | 36 | 2.436 | 62 | 2.462 |
| 11 | 2.411 | 37 | 2.437 | 63 | 2.463 |
| 12 | 2.412 | 38 | 2.438 | 64 | 2.464 |
| 13 | 2.413 | 39 | 2.439 | 65 | 2.465 |
| 14 | 2.414 | 40 | 2.440 | 66 | 2.466 |
| 15 | 2.415 | 41 | 2.441 | 67 | 2.467 |
| 16 | 2.416 | 42 | 2.442 | 68 | 2.468 |
| 17 | 2.417 | 43 | 2.443 | 69 | 2.469 |
| 18 | 2.418 | 44 | 2.444 | 70 | 2.470 |
| 19 | 2.419 | 45 | 2.445 | 71 | 2.471 |
| 20 | 2.420 | 46 | 2.446 | 72 | 2.472 |
| 21 | 2.421 | 47 | 2.447 | 73 | 2.473 |
| 22 | 2.422 | 48 | 2.448 | 74 | 2.474 |
| 23 | 2.423 | 49 | 2.449 | 75 | 2.475 |
| 24 | 2.424 | 50 | 2.450 | 76 | 2.476 |
| 25 | 2.425 | 51 | 2.451 | 77 | 2.477 |
| 26 | 2.426 | 52 | 2.452 | 78 | 2.478 |
| 27 | 2.427 | 53 | 2.453 | 79 | 2.479 |
| — | — | — | — | 80 | 2.480 |

14.6.8 Hop sequences

Change the text in 14.6.8 and Table 46 as shown:

The hopping sequence of an individual PMD entity is used to create a pseudorandom hopping pattern utilizing uniformly the designated frequency band. Sets of hopping sequences are used to co-locate multiple PMD entities in similar networks in the same geographic area and to enhance the overall efficiency and throughput capacity of each individual network.

An FH pattern, Fx, consists of a permutation of all frequency channels defined in Table 42 and Table 43.For a given pattern number, x, the hopping sequence can be written as follows:

FX={fx(1)， fx(2)，… fX(p)}

Where:

fx(i)  is the channel number (as defined in 14.6.4) for i th frequency in xth hopping pattern;

p    is the number of frequency channels in hopping pattern (79 for China, North America and most of Europe,23 for Japan, 35 for France, 27 for Spain).

Given the hopping pattern number, x, and the index for the next frequency, i (in the range 1 to p), the channel number shall be defined to be as follows:

fx(i) =[b(i)+ x] mod(79)+2，   in China, North America and most of Europe, with b(i) defined in Table 46.

=[(i-1)×x] mod(23)+73，  in Japan.

=[b(i)+ x] mod(27)+47，  in Spain with b(i) defined in Table 47.

=[b(i)+ x] mod(35)+48    in France with b(i) defined in Table 48.

Table 46—Base-Hopping sequence b(i) for China, North America and most of Europe

| i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 11 | 76 | 21 | 18 | 31 | 34 | 41 | 14 | 51 | 20 | 61 | 48 | 71 | 55 |
| 2 | 23 | 12 | 29 | 22 | 11 | 32 | 66 | 42 | 57 | 52 | 73 | 62 | 15 | 72 | 35 |
| 3 | 62 | 13 | 59 | 23 | 36 | 33 | 7 | 43 | 41 | 53 | 64 | 63 | 5 | 73 | 53 |
| 4 | 8 | 14 | 22 | 24 | 71 | 34 | 68 | 44 | 74 | 54 | 39 | 64 | 17 | 74 | 24 |
| 5 | 43 | 15 | 52 | 25 | 54 | 35 | 75 | 45 | 32 | 55 | 13 | 65 | 6 | 75 | 44 |
| 6 | 16 | 16 | 63 | 26 | 69 | 36 | 4 | 46 | 70 | 56 | 33 | 66 | 67 | 76 | 51 |
| 7 | 71 | 17 | 26 | 27 | 21 | 37 | 60 | 47 | 9 | 57 | 65 | 67 | 49 | 77 | 38 |
| 8 | 47 | 18 | 77 | 28 | 3 | 38 | 27 | 48 | 58 | 58 | 50 | 68 | 40 | 78 | 30 |
| 9 | 19 | 19 | 31 | 29 | 37 | 39 | 12 | 49 | 78 | 59 | 56 | 69 | 1 | 79 | 46 |
| 10 | 61 | 20 | 2 | 30 | 10 | 40 | 25 | 50 | 45 | 60 | 42 | 70 | 28 | — | — |

The sequences are designed to ensure some minimum distance in frequency between contiguous hops. The minimum hop size is 6 MHz for China, North America and Europe, including Spain and France, and 5 MHz for Japan.

The hopping pattern numbers x are divided into three sets. The sets are designed to avoid prolonged collision periods between different hopping sequences in a set. Hopping sequence sets contain 26 sequences for China, North America and Europe, and 4 sequences per set for Japan:

For China, North America and most of Europe:

x1={0,3,6,9,12,15,18,21,24,27,30,33,36,39,42,45,48,51,54,57,60,63,66,69,72,75} Set 1

x2={1,4,7,10,13,16,19,22,25,28,31,34,37,40,43,46,49,52,55,58,61,64,67,70,73,76} Set 2    x3={2,5,8,11,14,17,20,23,26,29,32,35,38,41,44,47,50,53,56,59,62,65,68,71,74,77} Set 3

For Japan:

x1={6,9,12,15}                          Set 1

x2={7,10,13,16}                         Set 2

x3={8,11,14,17}                         Set 3

For Spain:

x1={0,3,6,9,12,15,18,21,24}               Set 1

x2={1,4,7,10,13,16,19,22,25}              Set 2

x3={2,5,8,11,14,17,20,23,26}              Set 3

For France:

x1={0,3,6,9,12,15,18,21,24,27,30}          Set 1

x2={1,4,7,10,13,16,19,22,25,28,31}         Set 2

x3={2,5,8,11,14,17,20,23,26,29,32}         Set 3

The three sets of hopping sequences for China, North America and most of Europe, of 26 patterns each, are listed in Tables B.1, B.2, and B.3 in Annex B. Similarly, there are three sets for Japan of four patterns each. The three sets for Spain have nine patterns each. The three sets for France have 11 patterns each. The channel numbers listed under each pattern refer to the actual frequency values listed in Table 42 and Table 43.

14.8.1 FH PHY attributes

Change theTable 53 as shown:

Table 53—FHSS PHY attributes

| Attribute | Default value | Operational semantics | Operational behavior |
| --- | --- | --- | --- |
| dot11PHYType | FHSS=X‘01’ | Static | Identical for all FH PHYs |
| dot11RegDomainsSupported | FCC=X‘10’ IC=X‘20’ ETSI=X‘30’ Spain=X’31’ France=X’32’ MKK=X’40’ China=X’50’ | Static | Implementation dependent |
| dot11CurrentRegDomain | X‘00’ | Dynamic LME | Implementation dependent |
| dot11TempType | Type1=X‘01’ Type2=X‘02’ Type3=X‘03’ | Static | Implementation dependent |
| dot11SupportedDataRatesTX | 1Mbit/s=X‘02’ mandatory 2Mbit/s=X‘04’ optional | Static | Identical for all FH PHYs |
| dot11SupportedDataRatesRX | 1Mbit/s=X‘02’ mandatory 2Mbit/s=X‘04’ optional | Static | Identical for all FH PHYs |
| dot11SupportedTxAntennas | Ant1=X‘01’ Ant2=X‘02’ Ant3=X‘03’ Antn=n | Static | Implementation dependent |
| dot11CurrentTxAnttenna | Ant1= default | Dynamic LME | Implementation dependent |
| dot11SupportedRxAntennas | Ant1=X‘01’ Ant2=X‘02’ Ant3=X‘03’ Antn=n | Static | Implementation dependent |
| dot11DiversitySupport | Available= X‘01’ Not avail.= X‘02’ Control avail.= X‘03’ | Static | Implementation dependent |
| dot11DiversitySelectionRx | Ant1= X‘01’ Ant2= X‘02’ Ant3= X‘03’ Ant4= X‘04’ Ant5= X‘05’ Ant6= X‘06’ Ant7= X‘07’ Ant8= X‘08’ | Dynamic LME | Implementation dependent |
| Dot11NumberSupportedPower-Levels | Lvl1= X‘01’ Lvl2= X‘02’ Lvl3= X‘03’ Lvl4= X‘04’ Lvl5= X‘05’ Lvl6= X‘06’ Lvl7= X‘07’ Lvl8= X‘08’ | Static | Implementation dependent |
| dot11TxPowerLevel1 | Factory default | Static | Implementation dependent |

Table 53

| Attribute | Default value | Operational semantics | Operational behavior |
| --- | --- | --- | --- |
| dot11TxPowerLevel2 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel3 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel4 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel5 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel6 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel7 | Factory default | Static | Implementation dependent |
| dot11TxPowerLevel8 | Factory default | Static | Implementation dependent |
| dot11CurrentTxPowerLevel | TxPowerLevel1 | Dynamic LME | Implementation dependent |
| dot11HopTime | 224 µs | Static | Identical for all FH PHYs |
| dot11CurrentChannelNumber | X‘00’ | Dynamic LME |  |
| dot11MaxDwellTime | 390 TU | Static | Regulatory domain dependent |
| dot11CurentSet | X‘00’ | Dynamic LME |  |
| dot11CurrentPattern | X‘00’ | Dynamic LME |  |
| dot11CurrentIndex | X‘00’ | Dynamic LME |  |
| dot11CurrentPowerState | X‘01’ off X‘02’ on | Dynamic LME |  |
| NOTE—The column titled “Operational semantics” contains two types: static and dynamic. Static MIB attributes are fixed and cannot be modified for a given PHY implementation. MIB attributes defined as dynamic can be modified by some management entity. Whenever an attribute is defined as dynamic, the column also shows which entity has control over the attribute. LME refers to the MAC sublayer management entity (MLME), while PHY refers to the physical layer management entity (PLME). |  |  |  |

14.8.2.2  dot11CurrentRegDomain

Change theTable 54 as shown:

Table 54—Regulatory domain codes

| Code point | Regulatory agency | Region |
| --- | --- | --- |
| X‘50’ | the Radio Administration of Information Industry Ministry | China |
| X‘10’ | FCC | United States |
| X‘20’ | IC | Canada |
| X‘30’ | ETSI | Most of Europe |
| X‘31’ | Spain | Spain |
| X‘32’ | France | France |
| X‘40’ | MKK | Japan |

      - Operating frequency range

Change the text in 15.4.6.1 as shown:

The DSSS PHY shall operate in the frequency range of 2.4 GHz to 2.4835 GHz as allocated by regulatory bodies in China, Europe and the United States or in the 2.471 GHz to 2.497 GHz frequency band as allocated by regulatory authority in Japan.

15.4.6.2 Number of operating channels.

Change the first paragraph in 15.4.6.2 and Table 69 as shown:

The channel center frequencies and CHNL_ID numbers shall be as shown in Table 69. In China, the regulation made by the Radio Administration of Information Industry Ministry specifies operation from 2.4 GHz to 2.4835 GHz. The FCC (United States), IC (Canada), and ETSI (Europe) specify operation from 2.4 GHz to 2.4835 GHz. For MKK (Japan), operation is specified as 2.471 GHz to 2.497 GHz. France allows operation from 2.4465 GHz to 2.4835 GHz, and Spain allows operation from 2.445 GHz to 2.475 GHz. For each supported regulatory domain, all channels in Table 63 marked with .X. shall be supported.

Table 69—DSSS PHY frequency channel plan

| CHNL_ID | Frequency | Regulatory domains |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X‘10’ FCC | X‘20’ IC | X‘30’ ETSI | X‘31’ Spain | X‘32’ France | X‘40’ MKK | X’50’ China |
| 1 | 2412 MHz | X | X | X | — | — | — | X |
| 2 | 2417 MHz | X | X | X | — | — | — | X |
| 3 | 2422 MHz | X | X | X | — | — | — | X |
| 4 | 2427 MHz | X | X | X | — | — | — | X |
| 5 | 2432 MHz | X | X | X | — | — | — | X |
| 6 | 2437 MHz | X | X | X | — | — | — | X |
| 7 | 2442 MHz | X | X | X | — | — | — | X |
| 8 | 2447 MHz | X | X | X | — | — | — | X |
| 9 | 2452 MHz | X | X | X | — | — | — | X |
| 10 | 2457 MHz | X | X | X | X | X | — | X |
| 11 | 2462 MHz | X | X | X | X | X | — | X |
| 12 | 2467 MHz | — | — | X | — | X | — | X |
| 13 | 2472 MHz | — | — | X | — | X | — | X |
| 14 | 2484 MHz | — | — | — | — | — | X | － |

15.4.6.5 Transmit and receive in-band and out-of-band spurious emissions

Change the text in 15.4.6.5 as shown:

The DSSS PHY shall conform with in-band and out-of-band spurious emissions as set by regulatory bodies. For China, refer to the Radio Administration of Information Industry Ministry NO.353[2002]. For the United States, refer to Sections 15.205, 15.209, and 15.247 of FCC CFR47 [B6]. For Europe, refer to ETS 300-328 [B4].

15.4.7.1 Transmit power levels

Change the text in 15.4.7.1 as shown:

The maximum allowable output power as measured in accordance with practices specified by the regulatory bodies is shown in Table 72.

In China, maximum transmitted power shall be no more than 100mW(EIRP) and the RF part and antenna part of radio equipments must be designed or produced in integration. It is not allowed that other antenna is used or the transmit power amplifier is added additionally while setting and using the equipments.

In the United States, the radiated emissions should also conform with the IEEE uncontrolled radiation emission standard (IEEE Std C95.1-1991).

Table 72—Transmit power levels

| Maximum output power | Geographic location | Compliance document |
| --- | --- | --- |
| 100mW(EIRP) | China | the Radio Administration of Information Industry Ministry NO.353[2002] |
| 1000mW | United States | Section 15.247 of FCC CFR47 [B6] |
| 100mW(EIRP) | Europe | ETS300-328 |
| 10mW/MHz | Japan | MPT ordinance for Regulating Radio Equipment, Article 49-20 |

18.4.6.1 Operating frequency range

Change the text in 18.4.6.1 as shown:

The High Rate PHY shall operate in the 2.4-2.4835 GHz frequency range, as allocated by regulatory bodies in China, the United States, Europe, and Japan, or in the 2.471-2.497 GHz frequency range, as allocated by regulatory authority in Japan.

18.4.6.2 Number of operating channels

Change the first paragraph in 18.4.6.2 and Table 112 as shown:

The channel center frequencies and CHNL_ID numbers shall be as shown in Table 112. The FCC (United States), IC (Canada), MPHPT (Japan), ETSI (Europe) and China specify operation from 2.4 GHZ to 2.4835 GHz. For Japan, operation is additionally specified as 2.471 GHz to 2.497 GHz. France allows operation from 2.4465 GHz to 2.4835 GHz, and Spain allows operation from 2.445 GHz to 2.475 GHz. For each supported regulatory domain, all channels in Table 112 marked with .X. shall be supported.

Table 112—High Rate PHY frequency channel plan

| CHNL_ID | Frequency | Regulatory domains |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X‘10’ FCC | X‘20’ IC | X‘30’ ETSI | X‘31’ Spain | X‘32’ France | X‘40’ MKK | X’50’ China |
| 1 | 2412 MHz | X | X | X | — | — | — | X |
| 2 | 2417 MHz | X | X | X | — | — | — | X |
| 3 | 2422 MHz | X | X | X | — | — | — | X |
| 4 | 2427 MHz | X | X | X | — | — | — | X |
| 5 | 2432 MHz | X | X | X | — | — | — | X |
| 6 | 2437 MHz | X | X | X | — | — | — | X |
| 7 | 2442 MHz | X | X | X | — | — | — | X |
| 8 | 2447 MHz | X | X | X | — | — | — | X |
| 9 | 2452 MHz | X | X | X | — | — | — | X |
| 10 | 2457 MHz | X | X | X | X | X | — | X |
| 11 | 2462 MHz | X | X | X | X | X | — | X |
| 12 | 2467 MHz | — | — | X | — | X | — | X |
| 13 | 2472 MHz | — | — | X | — | X | — | X |
| 14 | 2484 MHz | — | — | — | — | — | X | － |

18.4.6.7.2 Operating channels

Change the Table 118 as shown:

Table 118—China, North American operating channels

| Set | Number of channels | HR/DSSS channel numbers |
| --- | --- | --- |
| 1 | 3 | 1,6,11 |
| 2 | 6 | 1,3,5,7,9,11 |

Change the Figure 145 as shown:

Figure 145—China, North American channel selection—non overlapping

Change the Figure 146 as shown:

Figure 146—China, North American channel selection—overlapping

18.4.6.7.3 Hop patterns

Change the text in 18.4.6.7.3 and Table 120 as shown:

A FH pattern, Fx, consists of a permutation of all frequency channels defined in Table 118 and Table 119.For a given pattern number x, the hopping sequence can be written as

Fx = {fx (1), fx (2), ..., fx (p)}

where

fx (i)   is the channel number (as defined in 14.6.4) for ith frequency in the xth hopping pattern;

p    is the number of hops in pseudo-random hopping pattern before repeating sequence (79 for China, North America and most of Europe).

The FH patterns for Set 1 of each geographic area are based on the hop patterns in Table 120 and Table 121.

The FH patterns for Set 2 of each geographic area are defined by the 1/2 Mbit/s FH PHY hop sequences, as described in the FH PHY (14.6.8). Given the hopping pattern number x, and the index for the next frequency, i (in the range 1 to p), the DS channel number (as defined in 18.4.6.2) shall be selected with the following algorithm:

China, North America

f’x (i) = f’x (i) for 1 ≤ f’x (i) ≤ 11;

f’x (i) = null for f’x (i) < 1 and f’x (i) > 11;

f’x (i) = 2 × Int [({[b(i) + x] mod (79) +2} . 6) / 10] . 1;

with b(i) defined in Table 42 (in 14.6.8).

Most of Europe

f’x (i) = f’x (i) for 1 ≤ f’x (i) ≤ 13;

f’x (i) = null for f’x (i) < 1 and f’x (i) > 13;

f’x (i) = 2 × Int [({[b(i) + x] mod (79) +2} . 6) / 10] . 1;

with b(i) defined in Table 42 (in 14.6.8).

Table 120—China, North American Set 1 hop patterns

| Index | Pattern 1 | Pattern 2 |
| --- | --- | --- |
| 1 | 1 | 1 |
| 2 | 6 | 11 |
| 3 | 11 | 6 |

18.4.6.8 Transmit and receive in-band and out-of-band spurious emissions

Change the text in 18.4.6.8 as shown:

The High Rate PHY conforms with in-band and out-of-band spurious emissions as set by regulatory bodies. For China, refer to the Radio Administration of Information Industry Ministry NO.353[2002].For the United States, refer to Sections 15.205, 15.209, and 15.247 of FCC CFR47 [B6]. For Europe, refer to ETS 300.328 [B4]. For Japan, refer to MPT ordinance for Regulating Radio Equipment, Article 7.

18.4.7.1 Transmit power levels

Change the text in 18.4.7.1 and Table 122 as shown:

The maximum allowable output power, as measured in accordance with practices specified by the appropriate regulatory bodies, is shown in Table 122.

In China, maximum transmitted power shall be no more than 100mW(EIRP) and the RF part and antenna part of radio equipments must be designed or produced in integration. It is not allowed that other antenna is used or the transmit power amplifier is added additionally while setting and using the equipments.

In the United States, the radiated emissions should also conform with the ANSI uncontrolled radiation emission standards (IEEE Std C95.1-1999).

Table 122—Transmit power levels

| Maximum output power | Geographic location | Compliance document |
| --- | --- | --- |
| 100 mW（EIRP） | China | the Radio Administration of Information Industry Ministry NO.353[2002] |
| 1000 mW | United States | FCC15.247 |
| 100 mW(EIRP) | Europe | ETS300-328 |
| See Table 123 | Japan | MPT ordinance for Regulating Radio Equipment,Article 49-20 |

A.4.5 Frequency hopping (FH) PHY functions

Change the FH7.1 item as shown:

| Item | Protocol feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| * FH7.1 FH7.1.1 FH7.1.2 FH7.1.2 FH7.1.3 FH7.1.2 FH7.1.4 FH7.1.3 FH7.1.5 FH7.1.4 FH7.1.6 FH7.1.5 | Geographic areas China North America Most of Europe Japan Spain France | 14.6.2 14.6.2 14.6.2 14.6.2 14.6.2 14.6.2 | M O.1 O.1 O.1 O.1 O.1 | Yes□ No□ Yes□ No□ Yes□ No□ Yes□ No□ Yes□ No□ Yes□ No□ |

A.4.6 Direct sequence PHY functions

Add the following item as shown:

| Item | PHY feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| *DS5.7 DS5.7.1 DS5.7.2 DS5.7.3 DS5.7.4 DS5.7.5 DS5.7.6 DS5.7.7 DS5.7.8 DS5.7.9 DS5.7.10 DS5.7.11 DS5.7.12 DS5.7.13 | China Channel 1 Channel 2 Channel 3 Channel 4 Channel 5 Channel 6 Channel 7 Channel 8 Channel 9 Channel 10 Channel 11 Channel 12 Channel 13 | 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 15.2.6,15.4.6.2 | M M M M M M M M M M M M M | Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ Yes□ No□ N/A□ |

A.4.9 High Rate, direct sequence PHY functions

Insert the item as shown:

| Item | PHY feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| HRDS6.7 | the Radio Administration The Radio Administration of P.R.China | 18.4.6.2 | HRDS7:O.3 | Yes□ No□ N/A口 |
| HRDS6.7.1 HRDS6.7.2 HRDS6.7.3 HRDS6.7.4 HRDS6.7.5 HRDS6.7.6 HRDS6.7.7 HRDS6.7.8 HRDS6.7.9 HRDS6.7.10 | Channel 1 Channel 2 Channel 3 Channel 4 Channel 5 Channel 6 Channel 7 Channel 8 Channel 9 Channel 10 | 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 18.4.6.2 | M M M M M M M M M M | Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 |
| HRDS6.7.11 HRDS6.7.12 HRDS6.7.13 | Channel 11 Channel 12 Channel 13 | 18.4.6.2 18.4.6.2 18.4.6.2 | M M M | Yes□ No□ N/A口 Yes□ No□ N/A口 Yes□ No□ N/A口 |

Annex B

Change the text in Annex B as shown:

The following tables pertain to the hopping sequences for China, North America and ETSI.

References: