IEEE P802.11
Wireless LANs

| Item 13-a – TIM Virtual Bit Map Example |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tom Tsoulogiannis | Neesus | 107 Statesman Sq. Scarborough, Ontario M1S 4H7 Canada | +1 (905) 475-5111 | tomt@neesus.com |
| Jon Rosdahl | Samsung Electronics | 10871 North 5750 West Highland, UT 84003 | 801-376-6435 | jrosdahl@ieee.org |

Abstract

TGm is resolving comments.  I was asked to produce an example of TIM Virutal Bit Map and how an example might be done.  Tom T. provided an excellent example that is documented here.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following Examples and code snipet can be used for helping clearify the useage of TIM values.  See item 13-a in the TGm status work item file.

Example 1:

B0

B7

Bitmap Control

AID 0

Bitmap Offset

AID 1

0

0

0

0

0

0

0

0

0

1

0

0

0

0

1

0

Partial Virtual Bitmap

- No Broadcasts/Multicasts buffered in AP

- Nodes with AID 2 and AID 7 have data buffered in AP

Example 2:

B0

B7

Bitmap Control

AID 0

Bitmap Offset

AID 1

1

AID 8

0

0

0

0

0

0

0

1

1

0

0

0

0

1

0

0

0

0

0

0

0

0

0

0

0

0

0

0

1

0

0

Partial Virtual Bitmap

AID 16

1

0

0

0

0

0

0

0

AID 24

- Broadcasts/Multicasts are buffered in AP

- Nodes with AID 2, AID 7, AID 22 and AID 24 have data buffered in AP

Example 3:

B0

B7

Bitmap Control

AID 0

Bitmap Offset

1

AID 24

0

0

0

0

0

0

1

1

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

Partial Virtual Bitmap

AID 16

- Broadcasts/Multicasts are buffered in AP

- Bitmap Offset of 1 means Partial Virtual Bitmap starts on third byte

- Node AID 24 has data buffered in AP

The following Code snipet will provide the pictures above.  No warrenty or efficiency is implied.

struct  _tim

{

BYTE	Element_id;

BYTE	Length;

BYTE	DtimCount;

BYTE	DtimPeriod;

BYTE	BitMapControl;

BYTE	PartialVBM[1];

}

BYTE	vbm[251];

BYTE	mcast_pending;

void

Build_TIM(struct _tim * Tim)

{

BYTE i;

BYTE offset = 0;

BYTE length = 0;

/* Find first non-zero byte in the virtual bit map */

for (i = 0; ((vbm[i] == 0) && (i < 251)); i++);

if (i < 251)

/* Clear the lsb as it is reserved for AID 0 */

offset = i & 0xFE;

/* Find last non-zero byte in the virtual bit map */

for (i = 250; ((vbm[i] == 0) && (i > 0)); i--);

length = i - offset + 1;

Tim->BitMapControl = offset;

/* Update broadcast/multicast Indication bit if necessary */

if (mcast_pending)

Tim->BitMapControl |= 1;

/* Copy the virtual bit map bytes that are non-zero */

/* Note: A NULL vbm will still add a single byte of zero  */

for (i = 0; i < length; i++)

Tim->PartialVBM[i] = vbm[offset + i];

}

void

Update_VBM ( WORD station_id, BYTE Action)

{

WORD	aid;

BYTE	aid_byte;

BYTE	aid_bit;

BYTE	i;

aid = station_id;

/*

*	get aid position in VBM.

*/

aid_byte = (BYTE)(aid >> 3);

aid_bit	 = (BYTE)(0x01 << (aid & 0x07));

if (Action == REMOVE_TIM_BIT)

{

vbm[aid_byte] &= ~aid_bit;

}

else

{

vbm[aid_byte] |= aid_bit;

}

}