IEEE P802.11
Wireless LANs

| Item 13-a – TIM Virtual Bit Map Example |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jon Rosdahl | Samsung Electronics | 10871 North 5750 West Highland, UT 84003 | 801-376-6435 | jrosdahl@ieee.org |
| Tom Tsoulogiannis | Neesus | 107 Statesman Sq. Scarborough, Ontario M1S 4H7 Canada | +1 (905) 475-5111 | tomt@neesus.com |
| Darwin Engwer | Nortel Networks | 4655 Great America Pkwy, Santa Clara, CA 95054 | 408-495-7099 | dengwer@nortelnetworks.com |

Abstract

TGm is resolving comments.  This submission provides draft text for an informative annex describing some examples of the TIM Virutal Bit Map along with some sample source code that illustrates how to construct the Virutal Bit Map.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following examples and C source code can be helpful for helping clarify the usage of TIM values.  See item 13-a in the TGm status work item file.

In the process of preparing this submission a few inconsistencies were noted in clause 11.  Resolution instructions for those inconsistencies are also provided.

Editor: Change the text in 11.2.1.2 as follows: change “SIDs” to “AIDs”.

Editor: Change the text in 11.2.1.4 as follows:

Change the last sentence of bullet c) from:

“The bit for AID 0 (zero) shall be set whenever broadcast or multicast traffic is buffered.”

to:

“The bit for AID 0 (zero) in the bit map control field of the TIM IE shall be set whenever broadcast or multicast traffic is buffered, per 7.3.2.6.”

Editor: Change the text in 11.2.1.4 as follows:

Change the last sentence of bullet e) from:

“… by setting the bit for AID 0 (zero) of the TIM element of every Beacon frame …:

to:

“… by setting the bit for AID 0 (zero) in the bit map control field of the TIM element of every Beacon frame …:

Editor: Change the text in 11.2.1.5 as follows:

Change the last sentence of bullet c) from:

“The bit for AID 0 (zero) shall be set whenever broadcast or multicast traffic is buffered.”

to:

“The bit for AID 0 (zero) in the bit map control field of the TIM IE shall be set whenever broadcast or multicast traffic is buffered, per 7.3.2.6.”

Editor: Change the text in 11.2.1.5 as follows:

Change the last sentence of bullet e) from:

“… by setting the bit for AID 0 (zero) of the TIM element of every Beacon frame …:

to:

“… by setting the bit for AID 0 (zero) in the bit map control field of the TIM element of every Beacon frame …:

Editor: Add the following text as a new informative annex:

# TIM Examples

The following examples and source code demonstrate how to construct the TIM Virtual Bit Map.

Example 1:

- No Broadcasts/Multicasts buffered in AP

- STAs with AID 2 and AID 7 have data buffered in AP

B0

B7

Bitmap Control

AID 0

Bitmap Offset

AID 1

0

0

0

0

0

0

0

0

0

1

0

0

0

0

1

0

Partial Virtual Bitmap

Example 2:

- Broadcasts/Multicasts are buffered in AP and the DTIM Count field in the TIM IE equals zero

- STAs with AID 2, AID 7, AID 22 and AID 24 have data buffered in AP

B0

B7

Bitmap Control

AID 0

Bitmap Offset

AID 1

1

AID 8

0

0

0

0

0

0

0

0

1

0

0

0

0

1

0

0

0

0

0

0

0

0

0

0

0

0

0

0

1

0

0

Partial Virtual Bitmap

AID 16

1

0

0

0

0

0

0

0

AID 24

Example 3:

- Broadcasts/Multicasts are buffered in AP and the DTIM Count field in the TIM IE equals zero

- Bitmap Offset of 1 means Partial Virtual Bitmap starts on third octet

- STA AID 24 has data buffered in AP

B0

B7

Bitmap Control

AID 0

Bitmap Offset

1

AID 24

0

0

0

0

0

0

1

1

0

0

0

0

0

0

0

0

0

0

0

0

0

0

0

Partial Virtual Bitmap

AID 16

The following C source code illustrates how to construct the TIM Virutal Bit Map.  No warranty or efficiency is implied.

#include <stdio.h>

#define ADD_TIM_BIT     0

#define REMOVE_TIM_BIT  1

#define TIM_ELEMENT_ID  5

#define TIM_BASE_SIZE   3       /* size of TIM fixed fields */

#define AID_SIZE        2008    /* valid AIDs are 1 thru 2007 */

#define VBM_SIZE        251     /* size of VBM array = 2008/8 = 251 */

typedef unsigned char UINT8;

typedef unsigned short int UINT16;

struct  _tim

{

UINT8   Element_id;

UINT8   IELength;

UINT8   DtimCount;

UINT8   DtimPeriod;

UINT8   BitMapControl;

UINT8   PartialVirtualBitMap [VBM_SIZE];

};

UINT8   virtualBitMap [VBM_SIZE];

UINT8   mcast_pending = 0;

UINT8   dtimCount = 0;

UINT8   dtimPeriod = 5;

void

Build_TIM (struct _tim * Tim)

{

UINT8 octetIndex;

UINT8 offset = 0;

UINT8 lengthOfPartialVirtualBitMap = 0;

/* Find first non-zero octet in the virtual bit map */

for (octetIndex = 0; ((virtualBitMap [octetIndex] == 0) && (octetIndex < VBM_SIZE)); octetIndex++)

/* empty */;

if (octetIndex < VBM_SIZE)

/* Clear the lsb as it is reserved for the broadcast/ multicast indication bit */

offset = octetIndex & 0xFE;

/* Find last non-zero octet in the virtual bit map */

for (octetIndex = (VBM_SIZE-1); ((virtualBitMap [octetIndex] == 0) && (octetIndex > 0)); octetIndex--)

/* empty */;

lengthOfPartialVirtualBitMap = octetIndex - offset + 1;

Tim->Element_id = TIM_ELEMENT_ID;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->DtimCount = dtimCount;

Tim->DtimPeriod = dtimPeriod;

Tim->BitMapControl = offset;

/* Update broadcast/ multicast indication bit if necessary */

if ((Tim->DtimCount == 0) && mcast_pending)

Tim->BitMapControl |= 0x01;

/* Copy the virtual bit map octets that are non-zero */

/* Note: A NULL virtualBitMap will still add a single octet of zero */

for (octetIndex = 0;  octetIndex < lengthOfPartialVirtualBitMap;  octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [offset + octetIndex];

}

void

Update_VirtualBitMap (UINT16 station_id, UINT8 Action)

{

UINT16  aid = station_id;

UINT8   aid_octet;

UINT8   aid_bit;

if ((aid > 0) && (aid < AID_SIZE))

{

/* Get aid position in Virtual Bit Map. */

aid_octet = (UINT8)(aid >> 3);

aid_bit   = (UINT8)(0x01 << (aid & 0x07));

if (Action == REMOVE_TIM_BIT)

virtualBitMap [aid_octet] &= ~aid_bit;

else

virtualBitMap [aid_octet] |= aid_bit;

}

}

void main (void)

{

struct _tim    Tim;

UINT8          ExampleCase;

ExampleCase = 1;

switch (ExampleCase)

{

case 1:

mcast_pending = 0;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

break;

case 2:

mcast_pending = 1;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

Update_VirtualBitMap (22, ADD_TIM_BIT);

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 3:

mcast_pending = 1;

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 4:

mcast_pending = 0;

Update_VirtualBitMap (3, ADD_TIM_BIT);

Update_VirtualBitMap (37, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 5:

mcast_pending = 0;

Update_VirtualBitMap (35, ADD_TIM_BIT);

break;

case 6:

mcast_pending = 0;

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 7:

mcast_pending = 0;

Update_VirtualBitMap (35, ADD_TIM_BIT);

Update_VirtualBitMap (35, REMOVE_TIM_BIT);

break;

case 8:

mcast_pending = 1;

Update_VirtualBitMap (13, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

Update_VirtualBitMap (63, ADD_TIM_BIT);

Update_VirtualBitMap (73, ADD_TIM_BIT);

break;

case 9:

mcast_pending = 1;

Update_VirtualBitMap (2007, ADD_TIM_BIT);

break;

default:

break;

}

Build_TIM (&Tim);

printf ("Element_id = %d.\n", Tim.Element_id);

printf ("IELength = %d.\n", Tim.IELength);

printf ("DtimCount = %d.\n", Tim.DtimCount);

printf ("DtimPeriod = %d.\n", Tim.DtimPeriod);

printf ("BitMapControl = 0x%02X\n", Tim.BitMapControl);

if (Tim.IELength - TIM_BASE_SIZE > 0)

{

int octetIndex;

for (octetIndex = 0;  octetIndex < Tim.IELength - TIM_BASE_SIZE;  octetIndex++)

printf ("PartialVirtualBitMap [%d] = 0x%02X\n", octetIndex, Tim.PartialVirtualBitMap [octetIndex]);

}

}

/* The End. */