IEEE P802.11
Wireless LANs

| Proposed Integration Function Informative Annex |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Johnny Zweig |  |  |  | Johnny@zweig.org |

Abstract

Since a large number of vendors manufacture 802.11 products that implement the Integration Function in a particular manner, I propose that a new Informative Annex be added to the IEEE 802.11 standard.

Add the following text as a new Informative Annex:

It is recommended that any WLAN System that logically incorporates

a Portal integrating the WLAN System with an Ethernet V2.0/IEEE 802.3 LAN

use the procedures defined in IEEE Std. 802.1H-1997 (ISO/IEC TR11802-5:1997), with the

following 2-entry Selective Translation Table, to perform the Integration Function.

| Protocol use | Ethernet Type value | Encoding in Type field -First octet | Encoding in Type field -Second octet |
| --- | --- | --- | --- |
| AppleTalk Address Resolution Protocol | 0x80F3 | 80 | F3 |
| Novell NetWare Internetwork Packet exchange (IPX) | 0x8137 | 81 | 37 |

Table 1 – NTI STT

References:

- Document: IEEE 802.11-05/0159r0 Integration Function Detailed Description

- IEEE Std. 802.1H-1997 (redesignated ISO/IEC TR11802-5:1997(E))

The 2-entry STT was originally developed by Netwave Technologies, Inc. and was called the NTI STT for short.