IEEE P802.11
Wireless LANs

| Proposed Integration Function Informative Annex |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Johnny Zweig |  |  |  | Johnny@zweig.org |

Abstract

Since a large number of vendors manufacture 802.11 products that implement the Integration Function in a particular manner, I propose that a new Informative Annex be added to the IEEE 802.11 standard.

Add the following text as a new Informative Annex:

# Recommended Practice for the Integration Function

It is recommended that any WLAN System that logically incorporates

a Portal integrating the WLAN System with an Ethernet V2.0/IEEE 802.3 LAN

use the procedures defined in IEEE Std. 802.1H-1997 (ISO/IEC TR11802-5:1997), with the

following 2-entry Selective Translation Table, to perform the Integration Function.

| Protocol use | Ethernet Type value | Encoding in Type field -First octet | Encoding in Type field -Second octet |
| --- | --- | --- | --- |
| AppleTalk Address Resolution Protocol | 0x80F3 | 80 | F3 |
| Novell NetWare Internetwork Packet exchange (IPX) | 0x8137 | 81 | 37 |

Table 1 – NTI Selective Translation Table

# Examples

In order to illustrate the behavior of the Integration Function operating using the encapsulation/decapsulation procedures defined in IEEE Std. 802.1H-1997 (ISO/IEC TR11802-5:1997), the following tables show how the octets in an 802.11 MSDU correspond to the octets in the Ethernet/802.3 MSDU that represents the same LLC SDU on the integrated LAN.

## Ethernet/802.3 LAN to 802.11 LAN Encapsulation

| Protocol | Type/Length | LLC Header |  | 802.11 LLC Header |
| --- | --- | --- | --- | --- |
| IP | 08-00 | -- |  | AA-AA-03-00-00-00-08-00 |
| IP 802.3 | length | AA-AA-03-00-00-00-08-00 |  | AA-AA-03-00-00-00-08-00 |
| IP ARP | 08-06 | -- |  | AA-AA-03-00-00-00-08-06 |
| AppleTalk (1) | 80-9B | -- |  | AA-AA-03-00-00-00-80-9B |
| AppleTalk (2) | length | AA-AA-03-08-00-07-80-9B |  | AA-AA-03-08-00-07-80-9B |
| AppleTalk AARP (1) | 80-F3 | -- |  | AA-AA-03-00-00-F8-80-F3 |
| AppleTalk AARP (2) | length | AA-AA-03-00-00-00-80-F3 |  | AA-AA-03-00-00-00-80-F3 |
| IPX Ethernet II | 81-37 | -- |  | AA-AA-03-00-00-F8-81-37 |
| IPX SNAP | length | AA-AA-03-00-00-00-81-37 |  | AA-AA-03-00-00-00-81-37 |
| IPX 802.2 | length | E0-E0-03 |  | E0-E0-03 |
| IPX 802.3 | length | FF-FF |  | FF-FF |

Table 2 – Ethernet/802.3 to 802.11 Translation

## 802.11 LAN to Ethernet/802.3 LAN Decapsulation

| Protocol | 802.11 LLC Header |  | Type/Length | 802.3 LLC Header |
| --- | --- | --- | --- | --- |
| IP | AA-AA-03-00-00-00-08-00 |  | 08-00 | -- |
| IP 802.3 | AA-AA-03-00-00-00-08-00 |  | 08-00 | -- |
| IP ARP | AA-AA-03-00-00-00-08-06 |  | 08-06 | -- |
| AppleTalk (1) | AA-AA-03-00-00-00-80-9B |  | 80-9B | -- |
| AppleTalk (2) | AA-AA-03-08-00-07-80-9B |  | length | AA-AA-03-08-00-07-80-9B |
| AppleTalk AARP (1) | AA-AA-03-00-00-F8-80-F3 |  | 80-F3 | -- |
| AppleTalk AARP (2) | AA-AA-03-00-00-00-80-F3 |  | length | AA-AA-03-00-00-00-80-F3 |
| IPX Ethernet II | AA-AA-03-00-00-F8-81-37 |  | 81-37 | -- |
| IPX SNAP | AA-AA-03-00-00-00-81-37 |  | length | AA-AA-03-00-00-00-81-37 |
| IPX 802.2 | E0-E0-03 |  | length | E0-E0-03 |
| IPX 802.3 | FF-FF |  | length | FF-FF |

Table 3 – 802.11 to Ethernet/802.3 Translation

References:

- Document: IEEE 802.11-05/0159r0 Integration Function Detailed Description

- IEEE Std. 802.1H-1997 (redesignated ISO/IEC TR11802-5:1997(E))

The 2-entry STT was originally developed by Netwave Technologies, Inc. and was called the NTI STT for short.

This format of IP packet over 802.3 is denigrated, and the change (see next table) to the canonical Ethernet IP format is not considered harmful.

The use of this nonstandard format happens to work with these rules, even though the FF-FF is not actually a valid LLC header value. (The broadcast LSAP is not valid as a source SAP in LLC. See 802.2.)

Note that this format of IP packet does not survive the trip across the non-802.3 LAN intact.