IEEE P802.11
Wireless LANs

| Suggested TGu Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mike Moreton | STMicroelectronics |  |  | mm2004@mailsnare.net |

Abstract

This submission contains some possible functional requirements for TGu.  The aim is that these suggested requirements would be considered by the TG, and classified as “In Scope”, “Out of Scope”, and “To Be Forwarded”.

Major Contributions from:

| Eleanor Hepworth | Siemens Roke Manor |
| --- | --- |
| Sabine Demel | T-Mobile |
| Stephen McCann | Siemens Roke Manor |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Definition of Requirement Classes

Each of the requirements will eventually be allocated to one of the following requirement classes by the TG:

| Requirement Class | Definition |
| --- | --- |
| In Scope | A requirement that the TG expects to be addressed by a proposal or proposals. |
| Out of Scope | A requirement that the TG expects not to be addressed by a proposal or proposals |
| To Be Forwarded | A requirement that the TG believes to be out of scope for the TG, but that must be addressed in another place if the work of the TG is to succeed. |
| Complete | A requirement that is believed to be addressed by existing technology. A note will be made of how it is addressed. |

# Source of Requirements

In developing this list of requirements, TGu members investigated the requirements for WLAN interworking that had been published by other organisations.  This table lists those organisations whose requirements were investigated.

Note that formal liaisons were not requested from these organisations.

| Organisation | Web Site |
| --- | --- |
| Wi-Fi Alliance | http://www.wi-fi.org |
| 3GPP | http://www.3GPP.org |
| 3GPP2 | http://www.3GPP2.org |
| IRAP | http://www.irap.nl |

# Requirements

|  | Requirement | Notes (informative) | Requirement Class |
| --- | --- | --- | --- |
|  | Define functionality by which the user is able to determine what online enrolment (also called online subscription) methods are supported by the network . | Some networks allow users to enrol “over the air” – for example, the Wi-Fi alliance has defined such functionality (called UAM) based on browser capture. The idea is to allow a STA to determine whether a network supports such functionality (and if so which one). If the network does not support enrolment, then the user must already be in possession of security credentials (e.g. as determined by the EAP method in use) unless the network provides open access. Need to identify what methods are actually used here, or how they would be identified in the future. |  |
|  | Define functionality for online user enrolment. | The only current widely adopted online enrolment mechanism is the Wi-Fi Alliance’s UAM mechanism and this has many problems. What are these problems. Need to send this list to Wi-Fi Alliance. A standardised mechanism is desirable. |  |
|  | Define functionality by which a STA can determine whether its subscription to an SSPN would allow it to access a particular 802.11AN before actually joining a BSS within that 802.11 AN. This mechanism shall take into account the possibility of hierarchical authentication arrangements including roaming agreements between the SSPN or Proxy Network and the 802.11 AN. The mechanism must be scalable. | It’s not acceptable for a STA to be required to attempt 802.1X authentication with all available networks until it finds one that works. Equally it’s not practical for a solution that requires every possible credential supplier to be listed in a beacon due to scalability problems. Is this mechanism uni or bi-directional. What is meant by scalable. |  |
|  | The mechanism described in Error: Reference source not found must allow a STA that has multiple SSPN subscriptions to select the correct security credentials when authenticating with a Local Network. | It’s difficult to see how this would not be the case, but it’s perhaps worth stating explicitly… |  |
|  | Define functionality by which Authorisation Information can be transferred from the SSPN to the Local Network. | This requirement does not define what’s actually in the Authentication Information – that’s inferred from later requirements that explicitly assume that certain information is part of the AI. |  |
|  | Define STA behaviour when it is in possession of suitable credentials to use an 802.11 AN, and where the current AP does not require these credentials. | This is a very difficult area. If a STA with credentials happily associates with an AP that doesn’t support the appropriate authentication then we have a potential security hole. But then if we don’t allow this, it makes it much more difficult to upgrade existing networks. This appear to be a potential problem for only basic credentials such as username and password. This appears to be covered by REQ1 already. |  |
|  | Define functionality by which admission control and QoS mapping decisions made locally in the 802.11 AN can be influenced by the contents of the Authorisation Information. | Certain SSPNs may want access to different classes of service to be dependent on different subscription options. Depending on the roaming agreement, the 802.11 AN may wish to honour these limitations. |  |
|  | Define functionality to support authentication with multiple SSPNs through a single AP. | It’s not acceptable to require a separate “virtual” AP for each SSPN. Note that this is not a requirement that a STA be able to use multiple SSPNs simultaneously – that comes later. This is just saying that a single AP can have a population of STAs where some are using one SSPN, and some another one. |  |
|  | Define functionality by which a STA can determine which interworking services are available before joining a BSS. | A classic example is whether internet access is provided (some open networks might exist only to give access to a local server) but the style of interworking may also be significant – is tight or loose coupling provided? Is access to IMS in the SSPN provided? See 11-05/1595r0 for more on this subject. |  |
|  | All mechanisms shall be defined with minimising battery consumption for mobile devices in mind. | While this isn’t a directly measurable requirement, it’s important that proposals take this into account, and describe what impact it has on their proposals. This is actually an objective and should be removed as a requirement. |  |
|  | Define accounting information for transfer to the SSPN. This shall include information about accepted TSPECs, their duration, and about actual traffic flows. | It’s expected that there would be some sort of byte and packet count, together with time, cross-referenced against TSPEC, or QoS class. |  |
|  | Define functionality to prevent hijack of MAC addresses. | The current standard does not prevent an authorised STA connecting to the DS with a MAC address that is already a duplicate of an existing address, and hence hijacking traffic that was meant for it. IEEE 802.1AL will not satisfy this requirement, but may have a significant impact on this in the future. Hence IEEE 802.1AL should be monitored. |  |
|  | Provide functionality for MAC Address Anonymity | See 05/170r0. Many cellular networks currently provide mechanisms by which the identity of a cellular terminal is not disclosed over the air, in order to prevent tracking. Recall that 802.11p and 802.11w also have this issue. |  |
|  | Provide functionality to certify beacons so that rogue APs can not masquerade as real ones. | This may be difficult to do without prohibitive overhead due to the problem of rogue APs copying beacons from valid APs elsewhere in the building. However, the network selection mechanism should probably include functionality for distinguishing accidental SSID clashes. Possibly an issue for TGw. Additionally, the term SSID should be avoided and replaced with information blob. Clarify the definition of rogue AP. |  |
|  | Need an information procedure by which a STA can determine which APs are connected to the same DS as the current AP, and hence may be preferred for roaming. |  |  |
|  | A STA shall be able (subscription permitting) to access multiple Corresponding Networks at the same time. | For example a STA may access subscribed services in its Subscription Service Network while using the Local Network for internet access. The assumption here is that there is only one SSPN ,and one set of authorisation information, but that multiple CNs are being accessed. See REQ17 for multiple SSPNs. This appears to be out of scope, as this is a higher layer issue. |  |
|  | A STA shall be able to use different SSPNs simultaneously, in order to gain simultaneous access to multiple Corresponding Networks. | Note that certain Corresponding Networks (such as the Internet) may be accessible based on Authorisation from more than one SSPN at a time. Same issue as previous requirement about appearing to be out of scope. |  |
|  | Functionality shall be provided by which traffic destined for a particular Corresponding Network can be segregated from traffic destined for other Corresponding Networks. | There seem to be two possible ways of doing this – either by segregating at layer 2 (i.e. a VLAN based solution) or by layer 3 routing restrictions. If the latter then there is the problem of how the mapping from user identity to IP address is transmitted to the devices that restrict the routing. There may also be solutions that combine aspects from layer 2 and layer 3. Clearly an AP network issue, which requires some more justification to be worked on. |  |
|  | Functionality shall be provided by which APs can advertise the charges that will be made for use of the network. | There are some thoughts on sizing of the information in 11-04-0691r0. TGu perhaps should just define the mechanism to exchange the information and not the details of the content itself. This should be left to other organisations or indeed industry. |  |
|  | Functionality shall be provided by which during the association/authentication process a STA can be informed of the actual charges to be applied to this session. | It’s possible that there will be user specific costs – for example if the SSPN has different service plans. If this is the case, you probably don’t want the AP advertising every service plan for every SSPN! Should this be part of the SSPN selection process (see Error: Reference source not found). Again, this is essentially a requirement for the definition of a information element, rather than the actual contents/details of that information. In this respect it is identical to REQ19. |  |
|  | Provide a mechanism (for APs) to indicate Media Independent Handoff capability | This could be just a bit/flag to inform STAs that an access point is .21 MIH (Media Independent Handover) enabled. Thus the STAs can query for further MIH information. Again this is a request for content transfer. |  |
|  | Provide a transport mechanism for MIH related information elements in state 1: unauthenticated, unassociated (class 1 frames) | This transport could be both, broadcast or point to point communication. Data to be transferred is mainly a basic set of MIH information elements, but also remote events and commands are possible. Possibly a very important requirement, as this implies that TGu may have to ensure that 802.11 is capable of carrying .21 information elements. |  |
|  | Define SAPs, primitives and event/state information to support .21 MIH functionalities. | .21 MIH proposes MAC layer Service Access Points and associated primitives. These primitives help in collecting link information and controlling link behaviours. Link state information has to be provided to MIH functions. This includes mechanisms and functions to generate the state information based on e.g. thresholds (also set from higher layer). Again this is a requirement for tight integration between .11 and .21, so again is very important. Refer to 802 A SC. |  |
|  | Provide functionality to map external QoS classes to 802.11 specific parameters | Perhaps the requirement should read that some sort of mapping mechanism is available, not it’s intended functionality. IEEE 802.21 would then try to normalise it. This should then conform to the .21 model. Indication of end to end QoS service may be desired here. It may be that the network also requires QoS information from the 802.11 AN itself. Possibly 2 requirements here? |  |
|  | Define functionality by which the Authorisation Information described in REQ5 can be modified by the SSPN. | Allows the SSPN to revoke, or modify the Authorisation Information during a session. See TS23.234v6.40 (section 5.1.1). Related to RADIUS/Diameter issues in the AP. |  |
|  | Information is required to enable a STA to determine whether selection of a different AP implies a change of L3 routing and to allow the STA to obtain suffient information to speed up the L3 handoff. | See document 402r0 |  |
|  | Allow an indication of “OPEN” (anyone can use the local network without prior credentials). | Define OPEN Really about network selection? Possibly combine with early requirement. |  |
|  | It should be possible to inform an STA about un broadcasted SSID without letting the STA probe for each preferred SSID. | Multiple SSIDs are not a scaleable solution. 3GPP already provides this using EAP methods. It is the intention of this requirement to provide similar information at IEEE 802.11 MAC layer. See 11-05-383r0. |  |

The following requirement is believed to be satisfied by requirement 3.

|  | Requirement | Derived From | Notes (informative) | Requirement Class |
| --- | --- | --- | --- | --- |
|  | Find functionality for discovering subscription service providers supported by the IEEE 802.11 access network, prior to association and/or authentication, as early as possible. 3GPP will have list on the handset of preferred local networks (SSIDs), it will attempt to connect via each of them in turn. If connection succeeds, it will stop, if not EAP returns a list of proxy networks which can be displayed to the user if no connection to any local network worked. We need to do better than this. | 3GPP | Is this different from the Wi-Fi requirement to be able to access different networks? That is, is a service a network, or an application running over the network? Sabine agrees is a duplicate. |  |

# Comparison Criteria

This section describes criteria which will help to evaluate the different proposals.

|  | Requirement | Evidence Required |
| --- | --- | --- |
|  | Proposals should minimise the reduction in network throughput due to increases in overhead that may be required by the proposal. | Increase in length (octets) of beacon and probe response. Description of other overheads included in the proposal. |