IEEE P802.11
Wireless LANs

| Normative Text Changes for Advanced Antennas |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-MAY-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4327 | joekwak@mindspring.com |

Abstract

This document contains normative text changes to TGk measurement reports to accommodate the use of advanced antennas.

This contribution addresses 9 LB73 comments:  #465, 868, 1013, 1452, 1502, 1558, 1594, 1601, and 1602.

.

Modify clauses of TGk draftD2.0 as indicated below:

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the order 11, 14 and 18 information fields, and insert the order 22 field as follows:

Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | The Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true |
| 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 22 | AP Channel Report | The AP Channel Report element shall be present only if dot11RadioMeasurementEnabled is true. |
| 23 | Antenna Information | The Antenna Information element shall be present only if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Change the order 10, 13 and 17 information fields, insert the order 22 information field and change the starting order of the Requested information elements field in Table 12 as follows:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | Shall be included if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | Shall be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 22 | AP Channel Report | Shall be included if dot11RadioMeasurementEnabled is true. |
| 23 | Antenna Information | The Antenna Information element shall be present only if dot11RadioMeasurementEnabled is true. |
| 2224-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

### 7.3.2 Information Elements

Insert Element ID 51-53 into Table 20 and change the Reserved row accordingly:

Table 20—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| Neighbor Report w/ TSF Information | TBD |
| Antenna Information | TBD |
| Reserved | 5154-255 |

##### 7.3.2.22.5 Noise Histogram Report

The format of the Measurement Report field of a Noise Histogram Report is shown in Figure k1.

|  | Channel Number |  |  | Regulatory Class |  | Actual Measurement Start Time |  |  | Measurement Duration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  |  | 1 |  | 8 |  |  | 2 |  |  |  |  |
|  |  | RPI 0 Density | RPI 1 Density |  | RPI 2 Density |  | RPI 3 Density | RPI 4 Density |  | RPI 5 Density | RPI 6 Density |  | RPI 7 Density |
| Octets: |  | 1 | 1 |  | 1 |  | 1 | 1 |  | 1 | 1 |  | 1 |

Figure k1—Measurement Report field format for a Noise Histogram Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Noise Histogram Report was measured, expressed in TUs.

The Noise Histogram Report shall contain the RPI densities observed in the channel for the eight RPI levels defined in Table k1. To compute the RPI densities, the STA shall measure the CPI in the specified channel as a function of time over the measurement duration when NAV is equal to 0 (when virtual CS mechanism indicates idle channel). The time resolution of the CPI measurements shall be in microseconds. The CPI measurements are normalized to values representing the received power measured with a 0 dBm peak gain antenna.  For example, if an RPI of  -64 dBm is measured using an antenna with a +6dBm peak gain, -70 dBm is the normalized RPI measurement. The normalized RPI measurements are converted to a sequence of RPI values by quantizing the measurements according to Table k8. The RPI densities are then computed for each of the eight possible RPI values using Ceiling (255 * [Duration receiving at RPI value (microseconds) / (1024 * Measurement Duration)]). The sum of the RPI densities will be approximately 255, but could be up to 262 beacuse of rounding effects.

Table k1—RPI Definitions for a Noise Histogram Report

| RPI | RPI Measured Power (dBm) |
| --- | --- |
| 0 | RPI  92 |
| 1 | RPI -87 |
| 2 | -87  RPI -82 |
| 3 | -82  RPI -77 |
| 4 | -77 RPI -72 |
| 5 | -72 RPI -67 |
| 6 | -67 RPI -62 |
| 7 | -62 RPI -57 |
| 8 | -57 RPI |

##### 7.3.2.22.6 Beacon Report

The format of the Measurement Report field corresponding to a Beacon Report is shown in Figure k2.

|  |  | Channel Number | Regulatory Class |  | Actual Measurement Start Time |  |  | Measurement Duration |  |  | PHY Type |  |  | RCPI | BSSID |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 |  | 8 |  |  | 2 |  |  | 1 |  |  | 1 | 6 |  |  |  |  |  |  |  |
|  | Antenna ID |  |  | Antenna Count |  | Parent TSF |  |  | Target TSF |  |  | Beacon Interval | Capability Information |  |  | Received Elements |  |  |  |  |  |  |
| Octets | 1 |  |  | 1: |  | 4 |  |  | 8 |  |  | 2 | 2 |  |  | variable |  |  |  |  |  |  |

Figure k2—Measurement Report field format for a Beacon Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Beacon Report was measured, expressed in TUs.

PHY Type indicates the physical medium type of the beacon or probe response frame being reported. Valid entries are coded according to the value of dot11PHYType.

RCPI indicates the received channel power of the beacon or probe response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type. The indicated RCPI value is normalized to a value representing the received power for an antenna with a 0 dBm peak gain.  For example, if an RCPI of  -94 dBm is measured using an antenna with a +6dBm peak gain, an RCPI value of -100 dBm is reported.

The BSSID field contains the BSSID from the Beacon or Probe Response frame being reported.

The Antenna ID field contains the identifying number for the antenna used for this measurement.  The valid range for the Antenna ID is 1 through 254. The value 0 indicates that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas.

The Antenna Count field contains the number of antennas available at this STA and may indicate from 1 to 254 antennas. The value 0 indicates that the Antenna Count is unknown. The value 255 is reserved. Each available antenna is unique for the reporting STA and is characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

The Parent TSF field shall contain the lower 4-octets of the measuring STA's TSF timer value at the time the Beacon or Probe Response frame being reported was received.

The Target TSF field shall contain the Timestamp field from the Beacon or Probe Response frame being reported.

The Beacon Interval field shall contain the Beacon Interval field from the Beacon or Probe Response frame being reported.

The Capability Information field shall contain the Capability Information field from the Beacon or Probe Response frame being reported.

The Received Elements portion of the Beacon report contains a number of information elements from the received Beacon, or Probe Response. All information elements that are present in the reported frame shall be included. TIM elements shall be truncated such that only the first 4 octets of the element are reported.

##### 7.3.2.22.7 Frame Report

The format of the Measurement Report field corresponding to a Frame Report is shown in Figure k3—Measurement Report field format for a Frame Report.

|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Frame Report Entry |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 16 |

Figure k3—Measurement Report field format for a Frame Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Frame Report was measured, expressed in TUs.

|  | Transmit Address | BSSID | RCPI | Antenna ID | Antenna Count | Number of Frames |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 1: | 1 |

Figure k4—Frame Report Entry format

Transmit Address contains the 6-octet MAC address of the transmitter STA of the measured frame.

The BSSID field contains the BSSID from the frames being reported.

RCPI indicates the received channel power of the received frame in dBm, as defined in the RCPI measurement clause for the PHY Type. This field shall be the RCPI value for the most recently received frame. The indicated RCPI value is normalized to a value representing the received power for an antenna with a 0 dBm peak gain.  For example, if an RCPI of  -94 dBm is measured using an antenna with a +6dBm peak gain, an RCPI value of -100 dBm is reported.

The Antenna ID field contains the identifying number for the antenna used for the RCPI measurement included in this report.  The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this measurement was made with multiple antennas.

The Antenna Count field contains the number of antennas available at this STA and may indicate from 1 to 254 antennas. The value 0 indicates that the Antenna Count is unknown. The value 255 is reserved. Each available antenna is unique for the reporting STA and is characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

Number of Frames is a count of the individual frames received with the indicated Transmit Address and BSSID during the measurement duration.

##### 7.3.2.22.8 Hidden Station Report

The format of the Measurement Report field of a Hidden Station Report is shown in Figure k5.

|  |  |  |  |  |  |  | Zero or more entries |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Antenna ID | Antenna Count | Hidden Station Entry |
| Octets: | 1 | 1 | 8 | 2 | 1 | 1: | 7 |

Figure k5—Measurement Report field format for a Hidden Station Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Hidden Station Report was measured, expressed in TUs.

The Antenna ID field contains the identifying number for the antenna used for this measurement.  The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this measurement was made with multiple antennas.

The Antenna Count field contains the number of antennas available at this STA and may indicate from 1 to 254 antennas. The value 0 indicates that the Antenna Count is unknown. The value 255 is reserved. Each available antenna is unique for the reporting STA and is characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

The format for a Hidden Station Entry is shown in Figure k31.

|  | Hidden Station Address | Number of Frames |
| --- | --- | --- |
| Octets: | 6 | 1 |

Figure k6—Hidden Station Entry

Hidden Station Address contains the MAC address of the hidden station being reported.

Number of frames is a count of the unique unicast frames, for which an immediate ACK is required, received (retransmissions not counted) by the measuring STA which were addressed to a STA in any BSS from a transmitting STA and for which the measuring STA did not receive an ACK addressed to the transmitting STA. A count of 255 is used to indicate frame counts of 255 or greater. A count of 0 shall not be reported.

##### 7.3.2.22.9 Medium Sensing Time Histogram Report

The format of the Measurement Report field of a Medium Sensing Time Histogram Report is shown in Figure k7.

|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Medium Sensing Measurement Subtype | Received Power Threshold | Bin Offset | Bin Duration | Number of Bins |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | Total Number of Medium Sensing Intervals | Bin 0 Density | Bin 1 Density | … | Bin N-1 Density |  |  |  |  |
| Octets: | 4 | 1 | 1 |  | 1 |  |  |  |  |

Figure k7—Measurement Report field format for a Medium Sensing Time Histogram Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

The Measurement Duration shall be set equal to the duration over which the Medium Sensing Time Histogram Report was measured, expressed in TUs.

The Medium Sensing Measurement Subtype indicates the subtype of Medium Sensing Time Histogram Report, as in Table k4.

The Received PowerI Threshold identifies a measured power threshold as defined in Table k5. The Received Power Threshold is used to determine when a Medium Sensing Interval begins and ends, while collecting information for the Received Power Time Histogram.

The Bin Offset, expressed in microseconds, specifies the minimum medium sensing interval; any medium sensing interval smaller than the Bin Offset is ignored and not to be included in the measurement report.

The Bin Duration indicates the time interval during which Medium Sensing Events are counted to be in this bin, expressed in slot times. Medium Sensing Events are defined in Table k2.

The Number of Bins specifies the total number of bins in the medium sensing time histogram.

The Total Number of Medium Sensing Intervals indicates how many medium sensing intervals have been counted during the measurement. The Medium Sensing Intervals are defined in Table k2.

Here symbols are defined as:

i0 	= bin offset

i 	= bin duration

i 	= bin index

N 	= number of bins

Table k2—Definition of Medium Sensing Interval

| Medium Sensing Measurement Subtype | Medium Sensing Measurement Name | Medium Sensing Interval |
| --- | --- | --- |
| 0 | Received Power Time Histogram | Time interval during which the normalized receive power (RCPI or RPI) is above the specified Received Power Threshold. Measured RCPI and RPI are normalized to values representing the received power for an antenna with a 0 dBm peak gain. For example, if an RCPI of -94 dBm is measured using an antenna with a +6dBm peak gain, a received power value of -100 dBm is used to determine the Medium Sensing Interval. |
| 1 | CCA Idle Time Histogram | CCA Idle Time interval |
| 2 | CCA Busy Time Histogram | CCA Busy Time interval |
| 3 | NAV Busy Time Histogram | Initial NAV time value when set |
| 4-255 | reserved | reserved |

To compute Bin i density, Bi, 0 ≤ i < N, the STA initializes all Bin values to zero.  During the measurement duration the STA monitors the measurement channel for the medium sensing intervals of the requested subtype. The length (duration) of the detected interval determines which bin count is to be incremented.  If the detected interval has a duration time t with:

i0+(i* i)   t < i0+((i+1)*  i)      for i < N-1

i0+(i* i)   t       for i = N-1

then the interval count in Bin i is increased by one, except when the bin contains a count of 255. During the Medium Sensing Measurement, a histogram is generated that represents the probability distribution of Medium Sensing Time Intervals.

#### 7.3.2.28 RCPI information element

The RCPI information element contains a RCPI value.

|  | Element ID | Length | RCPI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure k8 — RCPI element format

The length field shall be set to 1.

The RCPI field shall contain the RCPI value as defined in the RCPI measurement clause for the PHY type at the measuring STA. The RCPI element is used in the active scan procedure as described in 11.1.3.2.2.

#### 7.3.2.29 Antenna Information element

The Antenna Information element contains the Antenna ID and Antenna Count fields as shown in Figure k9.

|  | Element ID | Length | Antenna ID | Antenna Count |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1: |

Figure k9 — Antenna Information element format

The length field shall be set to 2.

The Antenna ID field contains the identifying number for the antenna used to transmit the frame containing this Information element. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this frame was transmitted using multiple antennas.

The Antenna Count field contains the number of antennas available at this STA and may indicate from 1 to 254 antennas. The value 0 indicates that the Antenna Count is unknown. The value 255 is reserved. Each available antenna is unique for the reporting STA and is characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

# Annex A - PICS

Add new row to PICs table inAnnex A as shown below:

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRMTBD | Antenna Information elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.29, | CFk:M | Yes, No |

### ANNEX D

Modify dot11BeaconReportTable as follows:

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon report that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex Unsigned32,

dot11BeaconRprtRqstToken OCTET STRING,

dot11BeaconRprtIfIndex InterfaceIndex,

dot11BeaconMeasuringSTAAddr MacAddress,

dot11BeaconRprtChanNumber INTEGER,

dot11BeaconRprtRegulatoryClass INTEGER,

dot11BeaconRprtActualStartTime TSFType,

dot11BeaconRprtMeasurementDuration Unsigned32,

dot11BeaconRprtPhyType INTEGER,

dot11BeaconRprtRCPI INTEGER,

dot11BeaconRprtBSSID MacAddress,

dot11BeaconRprtAntennaID INTEGER,

dot11BeaconRprtAntennaCount INTEGER,

dot11BeaconRprtParentTSF TSFType,

dot11BeaconRprtTargetTSF TSFType,

dot11BeaconRprtBeaconInterval Unsigned32,

dot11BeaconRprtCFStatus INTEGER,

dot11BeaconRprtPrivacy TruthValue,

dot11BeaconRprtBSSMode INTEGER,

dot11BeaconRprtReceivedElements OCTET STRING }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable, greater than

0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Beacon Report has been received on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected beacons

and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band, in which this row

of beacon report has been conducted."

REFERENCE

"IEEE 802.11k, Error: Reference source not found"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report was

measured."

UNITS

"TUs"

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the PHY Type for this row of

Beacon Report."

UNITS

"dot11PHYType"

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or probe response

frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

The indicated RCPI value is normalized to a value representing the received power for

an antenna with a 0 dBm peak gain.  For example, if an RCPI of  -94 dBm is measured

using an antenna with a +6dBm peak gain, an RCPI value of -100 dBm is reported.

"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID in the measurement

Report Frame for this row of the Beacon Report.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas."

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtAntennaCount OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute corresponds to the Antenna Count in the measurement

Report Frame for this row of the Beacon Report.

The Antenna Count field contains the number of antennas available at

this STA and may indicate from 1 to 254 antennas. The value 0 indicates

that the Antenna Count is unknown. The value 255 is reserved. Each available

antenna is unique for the reporting STA and is characterized by a fixed

relative position, a fixed relative direction and a peak gain for that

position and direction."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving measuring STA’s

TSF value at the time the measuring STA received the beacon or probe

response frame."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtTargetTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value contained in the beacon or

probe response received by the measuring STA."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the beacon interval value contained in the

beacon or probe response received by the measuring STA."

UNITS

"TUs"

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtCFStatus OBJECT-TYPE

SYNTAX INTEGER {

noPc(0),

pcDeliveryOnly(1),

pcDeliveryAndPolling(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the combination of CF pollable, and CF-Poll

Request bits in the capability information contained in this row of Beacon

report, thus indicating the contention-free capabilities of

the STA that this row of beacon report corresponds to."

::= { dot11BeaconReportEntry 17 }

dot11BeaconRprtPrivacy OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Privacy subfield of the capability

information contained in the beacon report element corresponding to this

row."

::= { dot11BeaconReportEntry 18 }

dot11BeaconRprtBSSMode OBJECT-TYPE

SYNTAX INTEGER {

unknown(0),

infrastructure(1),

adhoc(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the BSS in the beacon report element

corresponding to this row is in the Ad-Hoc mode or not, based on the

information available from the beacon report corresponding to this row."

::= { dot11BeaconReportEntry 19 }

dot11BeaconRprtReceivedElements OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains a variable number of information elements. All

fields, except Timestamp, Beacon interval and Capability Information, that

are present in the measured beacon or probe response must be included if

the measured BSSID is different from the STA’s current BSS. The Beacon Report

will hereby enclose information about the 802.11 enhancements supported by the

STA that transmitted the beacon or probe response. All reported TIM elements

shall be truncated to 4 octets."

::= { dot11BeaconReportEntry 20 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex Unsigned32,

dot11FrameRprtIfIndex InterfaceIndex,

dot11FrameRprtRqstToken Unsigned32,

dot11FrameRprtChanNumber INTEGER,

dot11FrameRprtRegulatoryClass INTEGER,

dot11FrameRprtActualMsmtStart TSFType,

dot11FrameRprtMeasurementDuration Unsigned32,

dot11FrameRprtMeasuringSTAAddr MacAddress,

dot11FrameRprtBSSID MacAddress,

dot11FrameRprtRCPI INTEGER,

dot11FrameRprtAntennaID INTEGER,

dot11FrameRprtAntennaCount INTEGER,

dot11FrameRprtNumberFrames INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameReportTable, greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Frame Report has been received on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that corresponds

to this row of frame report. Since a single frame request can generate

multiple rows in the frame report table, one per BSSID, this

dot11FrameRprtRqstToken indicates which request this particular row

corresponds to. If this row of report is received without a particular

request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected frame request

responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band, taken from Error: Reference source not found, TGk

Spec, in which this row of frame report has been conducted."

REFERENCE

" "

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualMsmtStart OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when measurement

started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report

was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has been

received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that transmitted this

frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute indicates the received channel power of the beacon or probe response

frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

The indicated RCPI value is normalized to a value representing the received power for

an antenna with a 0 dBm peak gain.  For example, if an RCPI of  -94 dBm is measured

using an antenna with a +6dBm peak gain, an RCPI value of -100 dBm is reported.

"

::= { dot11FrameReportEntry 10 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID in the measurement

Report Frame for this row of the Beacon Report.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas."

::= { dot11BeaconReportEntry 11 }

dot11FrameRprtAntennaCount OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute corresponds to the Antenna Count in the measurement

Report Frame for this row of the Beacon Report.

The Antenna Count field contains the number of antennas available at

this STA and may indicate from 1 to 254 antennas. The value 0 indicates

that the Antenna Count is unknown. The value 255 is reserved. Each available

antenna is unique for the reporting STA and is characterized by a fixed

relative position, a fixed relative direction and a peak gain for that

position and direction."

::= { dot11BeaconReportEntry 12 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report."

::= { dot11FrameReportEntry 13 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11HiddenStationReport TABLE

-- ********************************************************************

dot11HiddenStationReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HiddenStationReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Hidden Station Reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 5 }

dot11HiddenStationReportEntry OBJECT-TYPE

SYNTAX Dot11HiddenStationReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11HiddenStationReportTable Indexed by

dot11HiddenStationRprtIndex."

INDEX { dot11HiddenStationRprtIndex }

::= { dot11HiddenStationReportTable 1 }

Dot11HiddenStationReportEntry ::=

SEQUENCE {

dot11HiddenStationRprtIndex Unsigned32,

dot11HiddenStationRprtRqstToken OCTET STRING,

dot11HiddenStationRprtIfIndex InterfaceIndex,

dot11HiddenStationMeasuringSTAAddr MacAddress,

dot11HiddenStationRprtChanNumber INTEGER,

dot11HiddenStationRprtRegulatoryClass INTEGER,

dot11HiddenStationRprtActualStartTime TSFType,

dot11HiddenStationRprtMeasurementDuration Unsigned32,

dot1HiddenStationRprtAntennaID INTEGER,

dot11HiddenStationRprtAntennaCount INTEGER,

dot11HiddenStationRprtHiddenSTAAddr MacAddress,

dot11HiddenStationRprtNumFrames INTEGER }

dot11HiddenStationRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Request elements in dot11HiddenStationReportTable, greater

than 0."

::= { dot11HiddenStationReportEntry 1 }

dot11HiddenStationRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11HiddenStationReportEntry 2 }

dot11HiddenStationRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Hidden Station Report has been received on."

::= { dot11HiddenStationReportEntry 3 }

dot11HiddenStationMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Hidden Station report."

::= { dot11HiddenStationReportEntry 4 }

dot11HiddenStationRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The channel number on which the this measurement report was made."

::= { dot11HiddenStationReportEntry 5 }

dot11HiddenStationRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

Hidden Station report has been conducted."

REFERENCE

"IEEE 802.11k, Error: Reference source not found"

::= { dot11HiddenStationReportEntry 6 }

dot11HiddenStationRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11HiddenStationReportEntry 7 }

dot11HiddenStationRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Hidden Station

Report was measured."

UNITS

"TUs"

::= { dot11HiddenStationReportEntry 8 }

dot11HiddenStationRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID in the measurement

Report Frame for this row of the Beacon Report.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas."

::= { dot11BeaconReportEntry 9 }

dot11HiddenStationRprtAntennaCount OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute corresponds to the Antenna Count in the measurement

Report Frame for this row of the Beacon Report.

The Antenna Count field contains the number of antennas available at

this STA and may indicate from 1 to 254 antennas. The value 0 indicates

that the Antenna Count is unknown. The value 255 is reserved. Each available

antenna is unique for the reporting STA and is characterized by a fixed

relative position, a fixed relative direction and a peak gain for that

position and direction."

::= { dot11BeaconReportEntry 10 }

dot11HiddenStationRprtHiddenSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the destination of the frame received by the

measuring STA without detecting the subsequent acknowledgement."

::= { dot11HiddenStationReportEntry 11 }

dot11HiddenStationRprtNumFrames OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of frames is a count of the frames received from the hidden station

address."

::= { dot11HiddenStationReportEntry 12 }

-- ********************************************************************

-- * End of dot11HiddenStationReport TABLE

-- ********************************************************************