IEEE P802.11
Wireless LANs

| TG 11k Conference Call Minutes Mar-May |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-15 |  |  |  |  |
| Author(s): Paul Gray |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Gray | AirWave | 1700 S. El Camino Real | 650-678-5633 | paul@airwave.com |

Abstract

This document contains minutes for TGk conference call between March and May 2005.

.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 03/23 Minutes 08:30 AM PST

- Attendance – Paine, Gray, Black, Kwak

- Comment Resolution

Comment #6 – Clause 7.3.2.22.9 - Emeott

Problem - The units and format of the Bin i Density are never properly defined.  However symbols and equations and normative text for calculating the density are buried in this clause.  The normative text would be more visible in clause 11.7.8.6

Remedy - Relocate the text describing how Bin Densities are calculated to clause 11.7.8.6.

Resolution – deferred – Medsense (reassign) to get a complete text submission

We don’t want sorting of bins in clause 11

Comment #7 – Clause 7 - Emeott

Problem - There are bits and pieces of normative text embedded throughout this section defining how to calculate different values used when taking measurements or defining how to take the measurements themselves.  Scattering the text around in this manner makes it difficult to find.  Move it all to the appropriate place in clause 11.

Remedy - Move normative text defining how to calculate different values used when taking measurements or defining how to take the measurements themselves from clause 7 to clause 11.

Resolution – declined – comment redundant (like 2, 5, 6) which are instances of the same general comment

11e has similar definition wording in clause 7

Comment #13 – Clause 11.8.1 – Emeott

Problem - The reference to 802.11i in this clause could be replaced by the clause number defining the 4-way handshake.

Remedy - Change the text to read "(for example, the 4-way handshake defined in clause 8.5.3)"

Resolution – accept – change P56 L5 to reference “4 way handshake”.

Comment #14 – Clause 11.8.1 - Emeott

Problem – The note on utilizing the beacon report in order to determine which APs can be heard is not entirely correct.  The note would be more accurate if the word "only" in the phrase "an AP may only utilize information corroborated by Beacon Reports from multiple STAs" were deleted.  Other means are available to an AP to corroborate information placed in a neighbor report, such as communicating with another AP over the DS.

Remedy - Delete the word "only" in the sentence quoted in the comment.

Comment - P56 L10

Resolution – accept -

Comment – not sure if this to restrictive or not restrictive enough from the comment

Comment #15 – Clause 11.8.2 – Emeott

Problem - The clause contains several abbreviations in the form "w/".

Remedy - Replace abbreviation with the word "with"

Resolution – accept – replace “w/” with “with on P56 L23 and L28

Comment #17 – Clause 7.4.2.3 - Emeott

Problem - The last sentence of the clause reads "The TPC Request element shall be set as described in 7.3.2.15."  However, the TPC Request element is defined in 7.3.2.17.

Remedy - Correct the reference to refer to 7.3.2.17.

Resolution – accept – correct reference P35 L20

Comment #18 – Clause 7.4.2.4 – Emeott

Problem - The last sentence of the clause reads "The TPC Report element shall be set as described 7.3.2.16.".  However, the TPC Report element is defined in 7.3.2.18

Remedy - Correct the reference to refer to 7.3.2.18.

Resolution – accept - correct the reference P36 L7

Comment #29 – Clause 11.7.8.6 – Bilstad

Problem - page 55 line 3 contains the word "CCA";  this is not the name of the measurement

Remedy - replace "CCA" with "medium sensing time"

Resolution – counter – change P55 L3 the name is “CCA Idle Time Histogram”

Comment #30 – Clause 11.7.8.6 – Bilstad

Problem - it's a little strange that this particular section has an informative note about usage, but yet the other measurements do not

Remedy - create a new section to capture this and other informative notes on usage

Resolution – deferred – re-categorize to medsense

Comment #33 – Clause 11.7.8.5 – Lin

Problem - It is not very clear how station detects potential hidden station.

Remedy - Need more text to describe how this works. It says STA receives an frame but not the acknowledge frame. I think it means STA receives an frame between two other STAs but not the acknowledgement frame.

Comment – it is already defined in section 7

Resolution – deferred – Hidden

Comment #34 – Clause 11.7.85 – Lin

Problem - Detecting of hidden station can also be done by receiving acknowledge frame between a STA and a hidden station but not the data/management frame.

Remedy - I think this will be the most common case sine the data frame is sent out at higher speed but ACK is sent out using BasicRate. If the detecting station is far away, it can probably only hear lower-rate ACK(e.g. 6Mbps), but not be able to receive the higher rate(e.g. 54M bps) data frame correctly

Resolution – deferred – reclassified as a “technical” comment in the “Hidden” Category

Comment #81 – Clause 7.2.3.1 - Estrada

Problem - The table lists the conditions that require the Country element to be present. But the paragraph in the original spec is now incomplete in its description of these conditions.

Remedy - Add the following: Change the first paragraph as follows: The frame body of a management frame of subtype Beacon contains the information shown in Table 5. If thedot11MultiDomainCapabilityEnabled attribute is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true, a STA shall include a Country information element in the transmission of Beacon frames. Optionally, the Beacon frame format may also include the information described in either or both of orders 12 and 13. If the information in both orders 12 and 13 are sent, they shall describe the same hopping pattern. Note that the information described in orders 12 and 13 also may be contained in the Probe Response frame.

Comment – this got through 11h group

Resolution – counter - In the recommended change is 7.2.3.1 delete all sentences of the first paragraph except the first sentence

Comment #82 – Clause 7.3.2.12 – Estrada

Problem - Line 16 should clearly identify the probe response frame.

Remedy - Change the phrase "probe or association response frame" to "probe response or association response frame".

Resolution – counter – P8 L18, change “probe” to “Probe Request”. P8 L16 change “probe” to “Probe Response”.

Comment #84 – Clause 7.3.2.12 – Estrada

Problem - Line 18 should clearly identify the probe request frame.

Remedy - Change the phrase "probe or association request frame" to "probe request frame or association request frame"

Resolution – accept – see comment #82

Comment #90 – Clause Annex D – Estrada

Problem - Page 83, line 49 contains an unresolved reference

Remedy - Correct the unresolved reference

Resolution – deferred – assign to Joe Kwak

Comment #91 – Clause Annex D – Estrada

Problem - Page 84, line 13 contains an unresolved reference

Remedy - Correct the unresolved reference

Resolution – deferred – same as 90

Comment #95 – Clause 11.8.1 – Johnson

Problem - Is the NOTE on p56, L7 indicating informational text? If it is should indicate where the note ends.

Remedy - Replace "NOTE" with "Informational text" and determine a way to make it clear where the informational text ends.

Resolution – deferred – Editor should determine how to flag text between L7 and L19 as editorial

Comment #101 – General - Quinn

Problem - It is very difficult to understand the general protocol and sequences of operations in the protocol.

Remedy - The specification would benefit from a "synopsis" section that provides an overview of the purpose and general operation of the protocol. To that end, use-oriented descriptions of an implementation of the protocol would serve well. Also, protocol timing diagrams (or some such visual aid) would serve to capture the protocol sequence without the clouding (or choking) detail. These changes would help orient a new reader of the document and provide a guiding context for the detailed technical material that would follow.

Comment – we agreed that we needed an external document with use cases.  Nobody is assigned to do this.

Resolution – deferred – re-categorize as general

Comments #102, 103, 104, 105, 106 are all the same as 101

Comment #107 – Clause 5.2.5 – Jokela

Problem - "The resulting information is then available for both the station and upper layers for any purpose, such as radio resource management".  Station is defined to be any device that contains IEEE 802.11 MAC and PHY and it can (and most likely it does) contains also upper layers so it might be good to rewrite this sentence.

Remedy - Remove words "and upper layers"

Resolution – declined – “Upper Layers” is part of 11k scope

Comment #120 – Clause 7.3.2.22.9 – Jokela

Problem - It is not very easy to understand Medium Sensing Time Histogram measurements and usage of Bins. Additional text could help understand it better.

Remedy - Add more explanatory text (&figures).

Resolution – defer – reassign to Medsense

Comment #128 - Jokela

Resolution – accept – same as comment 17

Comment #129 - Jokela

Resolution – accept – same as comment 17

Comment #174 – Clause Annex A – Faccin

Problem - Reference numbering in the PICS table is incorrect from RRM4 through RRM10

Remedy - Update the chapter numbers in references column.

Resolution – accept – re-categorize as PICs (Black)

- Call ends 10:30 AM PST

# 03/30 Minutes 08:30 AM PST

- Attendance – Paine, Gray, Black, Kwak, Roy

- Comment Resolution

Comment  #272

Resolution – Decline

Comment #276

Resolution – Defer – assign Kwak

Comment #277

Resolution – accept – same as 272

Comment #278

Resolution – Defer – same #273

Comment #281

Resolution – defer – same as 276

Comment #282

Resolution – Counter – we fixed this Atlanta 05/136r0

Comment #283

Resolution – Decline – text is appropriate tgk

Comment #284

Resolution – accept

Comment #285

Resolution – accept

Comment #289

Resolution – counter - Change to "The radio measurement service provides the following:"

Comment #296

Resolution – accept

Comment #297

Resolution – accept

Comment #305

Resolution – accept

Comment #311

Resolution – counter - Add "and elsewhere" to the end of the sentence (after 11.1.3.2.2).

Comment #314

Resolution – accept

Comment #315

Resolution – decline - RFC 2579 is not normative

Comment #316

Resolution – accept

Comment #317

Resolution – accept – P3L7 Replace last sentence with "One of the radio services is used for radio measurement."

Comment #320

Resolution – accept

Comment #321

Resolution – accept

Comment #329

Resolution – accept

Comment #351

Resolution – accept

Comment #354

Resolution – accept – same as 90

Comment #356

Resolution – accept – P95L30

Comment #357

Resolution – accept

Comment #371

Resolution – accept – same 296

Comment #380

Resolution – decline – k12 is correctly numbered

Comment #381

Resolution – counter – P33L11 remove lines 11-25

Comment #391

Resolution - accept

Comment #416

Resolution – accept

Comment #417

Resolution – accept – same as 15

Comment #418

Resolution – accept

Comment – 422

Resolution – accept

Comment #427

Resolution – counter - Should be "TSF Information Request" bit.  P36L21, figure k34, change "Neighbor TBTT Offsett Type" to "TSF Information Request".   P36L23, beginning of the line, change "Neighbor TBTT Offset" to "TSF Information".   P36L23, end of line, replace "TBTT Offset" and "Beacon Interval Fields" with "TSF Information".

Comment #428

Resolution – counter - Change "The Dialog Token field shall be set equal to the value in *any* corresponding Neighbor Report Request frame." to "The Dialog Token field shall be set equal to the value in the corresponding Neighbor Report Request.

Comment #431

Resolution – accept

Comment #473

Resolution – accept – same 316

Comment #483

Resolution – counter - Replace P25 L2 with "Transmit Address contains the Transmit Address (TA) from the frame(s) received during the measurement duration."

- Call ends 10:30 AM PST

# 04/06 Minutes 08:30 AM PST

- Attendance – Paine, Gray, Black, Kwak, Roy

- Comment Resolution

Comment #484 – Clause 7.3.2.22.9 – Qi

Problem - P26, L1. Since 11k draft 2.0 doesn’t have Hidden Station defined in clause 3, the definition of Hidden Station Address should be more specific.

Remedy - Replace L1 on P26 with the following statement: “Hidden Station Address contains the MAC address of the hidden station that the measuring STA detects.”

Resolution - defer – reassigned to hidden

Comment #494 – Clause Annex D - Gray

Problem - “UNITS” declaration should be immediately under “SYNTAX” declaration

Resolution - Move “UNITS” declaration immediately under “SYNTAX” declaration

Remedy - Accept – assigned to Paul Gray

Comment #495 – Clause Annex D - Gray

Problem - “UNITS” declaration should be immediately under “SYNTAX” declaration

Resolution - Move “UNITS” declaration immediately under “SYNTAX” declaration

Remedy - Accept – assigned to Paul Gray

Comment #496 – Clause Annex D – Gray

Problem - P108 L66 ":=" should be "::="

Remedy - Change ":=" to "::="

Resolution – accept

Comment #497 – Clause Annex D – Gray

Problem - P108 L75 ":=" should be "::="

Remedy - Change ":=" to "::="

Resolution – accept

Comment #498 – Clause Annex D – Gray

Problem - P109 L45 ":=" should be "::="

Remedy - Change ":=" to "::="

Resolution – accept

Comment #499 – Clause Annex D – Gray

Problem - P80 L23 "det11LCIRqstOctet" is incorrect

Remedy - Change "det11LCIRqstOctet" to dot11LCIRqstOctet

Resolution - accept

503 – Clause Annex D – Gray

Problem - P115 L7 dot11ChannelLoadRprtChannelLoad is missing "," in compliance statement

Remedy - Add "," to dot11ChannelLoadRprtChannelLoad in compliance statement

Resolution – accept

505 – Clause 10.3.24.1.2 – Heubaum

Problem - There's a period missing in the fourth column of the third row of the table in clause 10.3.24.1.2: "Indicates the results of the corresponding MLME-NEIGHBORREP.indication".

Remedy - Add a period at the end.

Comment – it is actually on P46L8

Resolution – accept – correct the result code row

506 – 10.3.25.1.2 – Heubaum

Problem - On page 47, line 16, the first letter of the word "PeerMACAddress" should be capitalized (the sentence ending, "…to the STA indicated in the peerMACAddress parameter.").

Remedy - Capitalize "PeerMACAddress".

Resolution – accept

507 – 11.7.8.9 – Heubaum

Problem - On page 55, line 28, the word "Measurement" is misspelled in the sentence ending, "…than the Pause delay specified in the Maeasurement Pause Request.".

Remedy – correct spelling

Resolution – accept – same as 305

578 – Clause 4 – Kolze

Problem - Terms with defined meaning should be capitalized. E.g. 3.53 and clause 4.

Remedy – Capitalize

Resolution – decline – the clause following the standard (check with editor)

595 – Clause 3.5.1 – Myles

Problem - Text states, "Any AP to which one or more STAs in the current BSS could successfully associate from its current location." "its" should be "their"

Remedy - Change "its" to Their"

Resolution – accept –

597 – Clause 5.2.5 – Myles

Problem - The text uses "stations" and "STAs" interchangablely

Remedy - Replace "stations" with  "STAs"

Resolution – accept

598 – Clause 5.2.5 – Myles

Problem - "… for both the station and upper layers …" does not add anything

Remedy - Remove "… for both the station and upper layers …"

Resolution – decline – same as 107

Comment #599 – Clause 5.2.5 – Myles

Problem - "Wireless LAN Radio Measurements enable the stations of the BSS and the ESS …" is too wordy

Remedy - Change to "Wireless LAN Radio Measurements enable the STAs …"

Resolution – accept

Comment #616 – Cause 7.3.2.21.9 – Myles

Problem - The text describes something related to the RPI Time Histogram but not the other histograms. It makes more sense to describe the various histograms in one location

Remedy - Move the RPI Time Histogram description to same location as other histogram descriptions

Resolution – decline – see spreadsheet for reasons

Comment #617 – Clause 7.3.2.21.9 – Myles

Problem - The language is not clear that the value 255 shall be used when the sub-type is not 0

Remedy – Make the language clearer

Resolution – accept – Add new sentence after P17L14 “The RPI Threshold shall be set to 255 when Medium Sensing Subtype does not equal 0.”

Comment #618 – Clause 7.3.2.21.4 – Myles

Problem - The first line gives no hint as to what the request is used for.  This makes it definition somewhat abstract to the casual reader

Remedy - Include a small amount of informative text in the first paragraph describing the function of the request, e.g. like in 802.11h 7.3.2.17

Resolution – decline – unable to determine intent, author is invited to suggest specific change.

Comment #655 – Clause 7.3.2.27 – Balachander

Problem - Line 14 is referring to the wrong figure

Remedy - Refer to figure k22 instead of k23

Resolution – accept – change P30L14

Comment #664 – Clause 5.5 – Cole

Problem - Editing instruction says to change item a)2)vii but this is an addition.

Remedy - Please reword as "change item x and insert item y".

Resolution – accept – P3L18 change “and add a.2.vii” to “insert a.2.vii”

Comment #665 – Clause 5.5 – Cole

Problem - "Add" used for new material but "Insert" used previously

Remedy - Please use insert consistently in editing instructions.

Resolution – accept

Comment #666 – Clause 7.3.2.7 – Cole

Problem - "Add" used for new material but "Insert" used previously.

Remedy - Please use insert consistently in editing instructions.

Resolution – accept

Comment #667 – Clause 7.3.2.8 – Cole

Problem – "Add" or "Insert" information underlined.

Remedy - Please remove the underlines.

Resolution – accept

Comment #668 – Clause 7.3.2.8 – Cole

Problem - "Add" used for new material but "Insert" used previously

Remedy - Please use insert consistently in editing instructions.

Resolution – accept

Comment #670 – Clause 7.3.2 – Cole

Problem - TBD value in element identifier needs assignment.

Remedy - Please assign number as provided by ANA.

Resolution – defer – reclassified as ANA

Comments 83, 205, 524, 602, 640, 697, 741, 878, 1043, 1346

Comment #671 – Clause 7.3.2 – Cole

Problem - Editing instructions don't cover the new TBD element ID.

Remedy - Please modify editing instructions to include all the new values.

Resolution – defer – reclassified as ANA

Comment #680 – Clause Annex D – Palm

Problem - References printed as "Error! Reference not Found."

Remedy - Repair cross references.

Resolution – defer –same as #88, reclassify as periodic

Comment #682 – Clause 4 – Palm

Problem - Terms with defined meaning should be capitalized. E.g. 3.53 and clause 4.

Remedy - Capitalize

Resolution – decline – same as 578

Comment #765 – Clause 11.7.3 – Ecclesine

Problem - The last sentence of the second paragraph is not normative and is not complete.

Remedy - Change to be a note like this: NOTE-It is important that designers recognize the need for statistical independence among the pseudo random number streams among STAs.

Resolution – accept

Comment #769 – Clause 11.8.1 – Ecclesine

Problem - The first sentence notes use of Neighbor Report in BSS transition, but ignores ESS aspect of operation.

Resolution - change sentence to end "to optimize aspects of neighbor BSS transition and ESS operation."

Resolution – accept

Comment #770 – Clause 11.8.3 – Ecclesine

Problem - Second paragraph specifies accumulated error in TSF of 1.5 TUs, then provides an error budget that is not normative.

Remedy - Make the error budget sentence an informative NOTE

Resolution – accept – P56L41 change “. The error budget” with “. New Paragraph Note – The error budget …”

Comment #782 – Clause 7.3.2.21.7 – Wright

Problem - I repeat my comment about the need for a one line description.  Also, I don't think the name is descriptive enough: "Frame" doesn't tell me that this request is really about frame reception statistics

Remedy - How about "Frame Reception Statistics"?  Also insert some text at the start of 7.3.2.21.4 to the effect of "The Frame Reception Statistics Request is used by a STA when it wants to (fill in the blank)"

Remedy – Defer – reclassify to general

Comment #784 – Clause 7.3.2.22.7 – Wright

Problem - I don't think the name is descriptive enough: "Frame" doesn't tell me that this request is really about frame reception statistics

Remedy - How about "Frame Reception Statistics"?

Resolution – decline – changing the name would require complication of changing every measurement accept statistics.

Comment #796 – Clause 11.8.2 – Wright

Problem - Suggested name change for this report: "Frame Reception Statistics Report"

Remedy - Change to suggested name

Resolution – decline – same as 784

Comment #797 – Clause 11.7.8.9 – Wright

Problem - Line 28: Spelling error

Remedy - "Maeasurement" -> "Measurement"

Resolution – accept – same as 305

- Call ends 10:30 AM PST

# 04/13 Minutes 08:30 AM PST

- Attendance – Paine, Gray, Black

- Comment Resolution

Comment #800 – Clause 7.2.3.4 and 7.2.3.5 – Chaplin

Problem - Inconsistent titles in the tables.  Either they should be "Request Information" and "Requested Information", or "Request Information Element" and "Requested Information Element"

Remedy - See Comment

Resolution – decline – it is correct the way it is, in association request, request information refers to the name of an IE where as in association response, requested IE refers to the requested information.

Comment #801 – Clause 7.2.3.7 - Chaplin

Problem - "Add new row to table 10 as follows:"

Remedy -  Add a space to say "Add new row to table 10 as follows:"

Resolution – accept – note: issue is with editing instructions P6 L2

Comment #802 – Clause 7.2.3.9 – Chaplin

Problem - Table 12 - Probe Response frame body:  inconsistent terminology.

Remedy - Change Country entry to be "Shall be included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true."

Resolution – decline – raise as editorial on 11m ballot

Comment #807 – Clause 7.2.21.x - Chaplin

Problem - What was confusing to this voter was that the request specifications are presented before the results specifications, and only the results specifications have a description of the measurements.  This leads to confusion when reading the request specifications which is not cleared up until the results specification are read.

Remedy - At a minimum, some sort of note in the request specifications pointing out that explanations of the measurements exists in the results specification would be nice.

Comment – Richard is working on overview document which we reviewed on t

Resolution – defer – reclassify as general

Comment #811 – Clause 7.4.2.4 – Chaplin

Problem - There is a cut and past error in this section: "The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame."

Remedy - Should be: "The Link Measurement Report frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame."

Resolution – accept – Change P35L21 to start “Link Measurement Report frame”

Comment #831 – 10.3.2.2.2 – Hayes

Problem – "is true or when and"

Remedy – "is true or when"

Resolution – accept – same as 321

Comment #909 – Clause 3.53 – O’Hara

Problem – "indicator" is the wrong form of the word to match the definition. The definition describes an "indication", the thing indicated by an indicator.

Remedy – use "indication" to match the definition or change the definition to match "indicator"

Resolution – accept – also change indicator to indication 15.4.8.5, 17.3.10.6, 18.4.8.5, and Clause 4.

Comment #910 – Clause 4 – O’Hara

Problem - "indicator" does not match the definition in 3.53.

Remedy - replace "indicator with "indication"

Resolution – accept – same as 909

Comment #986 – Clause 7.3.2.27 – Van Nee

Problem – Line 14 is referring to the wrong figure

Remedy - Refer to figure k22 instead of k23

Resolution – accept – same as 655

Comment #990 – Clause 3.5.2 – Klein

Problem - P1:L28 - change "in which" to "of which"

Remedy - This matches the text in 3.54 and makes more sense

Resolution – accept

Comment #991 – Clause 4 – Klein

Problem - P2:L8 - Add LCI acronym for Occasioning

Remedy - "Location Configuration Indication"

Resolution – accept – same as 316

Comment #993 – Clause 5.4.5 – Klein

Problem - P3:L11 - Klunky use of language.

Remedy - change "measurement support offered" to "measurements offered".

Problem – accept – same 289

Comment #994 – Clause 7.2.3.4 – Klein

Problem - P5:Table7:Row2 - Misspelled word "Neighbor".

Remedy – Fix

Resolution - accept

Comment #995 – Clause 7.3.4.1 - Klein

Problem - P7:L8 - Elements out of order in the description with respect to the actual field format.

Remedy - Swap "Radio Measurement" and "Short Slot Time" to match the format of the field.

Resolution – accept

Comment #997 – Clause 7.3.2.21.6 – Klein

Problem - P12:Figure K3 - "Randomization" is wrapping the last "n" to the next line in the figure

Remedy -Fix

Resolution – accept

Comment #1007 – Clause 7.4.2.1 – Klein

Problem - P34:L15 - 17 - Table / Figure References are not found.

Remedy - Fix

Resolution – accept – same as 88

Comments 319, 680, 352, 383, 764, 1007, 1058, and 1163 are all the same as 88

Comment #1008 – Clause 7.4.2.2 – Klein

Resolution – P35:L8 - The elements described on this line are not found in 7.4.1.2 as far as I can tell

Remedy – Find correct clause reference and change.

Resolution – counter – P35 L8 change “Measurement Request Element” to “Measurement Report Element”

Comment #1009 – Clause 7.4.2.2 – Klein

Problem - P35:L5 - The elements described on this line are not found in 7.4.1.2 as far as I can tell

Remedy - Find correct clause reference and change.

Resolution - decline – no elements are described her

Comment #1010 – Clause 7.4.2.6

Problem - P37:L13 - Should reference 7.3.2.27 not 7.23.2.26.

Remedy – Fix

Resolution – accept – same as 320

Comment #1011 – Clause 7.4.2.6 – Klein

Problem - P37:L15 - Change "omitted" to "empty"

Remedy - There are several places in the document where variable length items do not get returned when there is no data to report.  We have used "empty" on Page 11:L7 and should use it here too.

Resolution – defer – reclassify rqst-rpt-el

Comment #1016 – 11.7.5 – Klein

Problem - P51:L5 - Change "in a BSS" to "to a BSS" – Grammar

Remedy – Fix

Resolution – accept

Comment #1023 – 11.7.8.1 – Klein

Problem - P53:L9 - Sentence is not clear.

Remedy – Change "received in the" to "received for the measured BSS in the"…

Resolution – counter – P53L9 change “from a BSS is received” to “is received for the measured BSS”.

Comment #1026 – Clause 11.7.8.1 – Klein

Problem - P54:L8 – Typo

Remedy - Change the first word from "no" to "not".

Comment – P54L8 is not a valid reference

Resolution – decline – unable to find reference

Comment #1029 – Clause 11.8.1 - Klein

Problem - P56:L2 - add clarification.

Remedy - Change "obtained through" to "obtained by the report recipient through"

Resolution – accept

Comment #1030 – Clause 11.8.1 - Klein

Problem - P56:L3: Language Usage

Comment – he is referring to L4

Remedy – Change "obtained through" to "obtained by the report recipient through"

Resolution – accept – note line reference is incorrect, should be L4

Comment #1031- Clause 11.8.1 – Klein

Problem - P56:L7: Add clarification

Remedy - Change "Including:" to "Including, but not limited to:"

Resolution – decline – we don’t need to be so formal for informative note

Comment #1032 – 11.8.1 – Klein

Problem - P56:L8 - Add clarification

Remedy - Change "a.  Configuring" to "a.  Manually Configuring"

Resolution – decline

Comment #1033 – Clause 11.8.1 – Klein

Problem - P56:L7: Add clarification

Remedy - Change "Including:" to "Including, but not limited to:"

Resolution – decline

Comment #1034 – Clause 11.8.1 – Klein

Problem - P56:L16 - Add clarification

Remedy - Change "a.  Configuring" to "a.  Manually Configuring"

Resolution – decline

Comment #1035 – Clause 11.8.1 - Klein

Problem - P56:L18 - Add IAPP to acronyms section at beginning of document if it's not already there.

Remedy - Fix

Resolution – accept

Comment #1046 – Clause 7.3.2.21 - Amman

Problem - Table 20a - Several of the "Meaning bits" descriptions are incorrectly marked as "Not allowed".

Remedy - Change all descriptions that state "Not allowed" to "Reserved".

Resolution – defer – reclassify rqst-rpt-el

Comment #1047 – Clause 7.3.2.21 – Amman

Problem - Table 20a, Entry "1 1 0" - Grammar.

Remedy - Replace the text "it is not be sent" with "it not be sent".

Resolution – accept – note to editor: this is an error in 11h text

Comment #1048 – Clause 7.2.2.21 – Amman

Problem - Table 20a, Entry "1 0 0" - The usage of English in this statement is inconsistent with the next entry in terms of how the "negative" is represented.

Remedy - Make the grammar consistent within the table.

Resolution – defer – assign to rqst-rprt

Comment #1051 – Clause 7.3.2.21.5 – Amman

Problem - Page 12, line 2: Grammar.

Remedy - Replace the word "an" following the phrase "…corresponding to" with "a".

Resolution – accept

Comment #1056 – Clause 7.4.1.1 – Amman

Problem - Grammar.

Remedy - Remove the first occurrence of the word "of" in the editing instructions.

Comment – we have decided to remove all or some of this section

Resolution – defer – reclassify rqst-rpt

Comment #1057 – Clause 7.4.2.1 – Amman

Problem - Page 34, line 10: Spelling

Remedy – Replace the word "Repetions" with "Repetitions".

Resolution – same as 788, 809

Comment #1062 – Clause 11.7.4 – Amman

Problem - Page 50, line 31: Grammar.

Remedy - Remove the duplicate occurrence of the word "report" at the beginning of the line.

Resolution – accept

Comment #1066 – Clause 11.7.6 – Amman

Problem - Page 52, line 23: Grammar.

Remedy - Remove the first occurrence of the word "the" following the phrase "…Request with an indication that".

Resolution – accept

Comment #1067 – Clause 11.7.7 – Amman

Problem - Page 52, line 42: Grammar.

Remedy - Insert the word "of" between the phrases "…completes the processing" and "all the Measurement…".

Resolution – accept

Comment #1070 – Clause 11.7.8.9

Problem - Page 55, line 28: Spelling.

Remedy - Replace the word "Maeasurement" with "Measurement".

Resolution – accept – same as 305

Comment #1077 – Clause Annex D – Amman

Problem - Page 83, line 49: The text contains the statement "Error! Reference source not found".

Remedy -  Correct the unrecognized reference.

Resolution – defer – same 680

Comment #1078 – Clause Annex D – Amman

Problem - Page 83, line 60: Grammar.

Remedy - Replace the word "Periods" with "Period".

Resolution – accept – assign to Gray reclassify MIB

Comment #1079 – Clause Annex D – Amman

Problem - Page 84, line 13: The text contains the statement "Error! Reference source not found".

Remedy - Correct the unrecognized reference.

Resolution – defer – same 680

Comment #1081 – Clause Annex D – Amman

Problem - Page 89, line 29: Grammar.

Remedy - Remove the word "the" following the phrase "…number on which".  This particular issue occurs several times within Annex D, and should be corrected in all cases (cut and paste error!).

Resolution – accept – assign to Gray reclassify MIB

Comment #1082 – Clause Annex D Amman

Problem - Page 89, line 40: Sentence structure.

Remedy - Remove the comma (",") following the phrase "…the frequency band".  This particular issue occurs several times within Annex D, and should be corrected in all cases (cut and paste error!).

Resolution – accept – assign to Gray reclassify MIB

Comments #1257, 1423 – accept – same as 316

- Call end 10:30 AM PST

# 04/20 Minutes 08:30 AM PST

- Attendance – Gray, Kwak

- Gray acting Editor

- Comment Resolution

Comment #1085

Resolution – accept – P12L2

Comment #1086

Resolution – accept

Comment #1089

Resolution – accept

Comment #1098

Resolution – accept – same as #149

Comment #1099

Resolution – accept

Comment #1100

Resolution – defer

Comment #1101

Resolution – accept – same 1424

Comment #1102

Resolution – accept

Comment #1108

Resolution – accept

Comment #1109

Resolution – accept

Comment #1115

Resolution – accept – Delete P13 L23-27

Comment #1127

Resolution – accept –

Comment #1129

Resolution – counter – see 483

Comment #1135

Resolution – counter - Replace P26 L29 with "Number of Bins indicates the total number of time interval ranges, or Bins that are included in this measurement report."

Comment #1145

Resolution – accept

Comment #1148

Resolution – counter - Replace Paragrah at P31 L3 with "The BSSID field contains the BSSID of a neighbor AP BSS being reported. Subsequent fields in this Neighbor List Entry relate to this neighbor AP BSS."

Comment #1150

Resolution – counter - P32 L7 change: "shall be" to "is" and "shall contain" to "contains".

Comment #1153

Resolution – counter - P32, L2 Replace sentence with "{Bullet} 10-15 are reservered.  Reserved bits shall be set to 0 on transmission and ignored on reception."

Comment #1155

Resolution – accept - P32, L7 Replace "represents" with "contains"

Comment #1157

Resolution – counter - Replace P32 L17 with "The TSF Offset subfield is 2 octets long and contains the neighbor AP TSF timer offset.  The neighbor AP TSF timer offset is the time difference in units of TU between the TSFs of the serving AP and the neighbor AP (TSFneighborAP - TSFservingAP).  This neighbor AP timer offset is represented modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary."

Comment #1159

Resolution – accept – same as 381

Comment #1160

Resolution – accept - Copy field defintions for Dialog Token and Measurement Request Elements from 7.4.1.1 to this section

- Call end 10:30 AM PST

# 04/27 Minutes 08:30 AM PST

- Attendance – Paine, Kwak, Gray

- Comment Resolution

Comment #1165 – Clause 7.4.2.3 – Black

Problem - P35, L19: TPC request element is in 7.3.2.17 not 7.3.2.15.

Remedy – correct reference.

Resolution - accept – same as 17

Comment #1166 – Clause 7.4.2.4 –Black

Problem - P36, L7: TPC report element is in 7.3.2.18 not 7.3.2.16

Remedy - Correct reference.

Resolution – accept – same as (18, 128, 129)

Comment #1167 – Clause 7.4.2.5 – Black

Problem - P36, L21, Figure k34: 'Neighbor TBTT Offset Type' needs to catch up with the terminology. This should be renamed 'Neighbor TSF Offset' to be consistent with 7.3.2.26.

Remedy - none

Resolution - accept – counter – same as 427

Comment #1169 – Clause 7.4.2.5 – Black

Problem - P36, L25: This reserved text is poor. Suggest 'All other bits are reserved and shall be set to 0 on transmission and ignored on reception.'

Remedy - Consider editorial improvement suggested.

Resolution - Accept

Comment #1171 – Clause 10.3.2.2.2 - Black

Problem - P38: The format of this table needs attention. Also the text 'and when' needs to be removed from the PowerConstraint notes column.

Remedy - Consider editorial improvement suggested.

Accept – counter see comment #321

Comment #1172 – Clause 10.3.11 – Black

Problem - P38, L13: This text is in editorial note style and has been merged with the change note for the following sections.

Remedy - Fix formatting.

Resolution – counter P38L18: insert paragraph break before “Change”. P38L13-18 should not be bold.

Same as comments 675, 812, 944, 1172

Comment #1174 – Clause 10.3.24.1.2 – Black

Problem - P42, L14: Change TBTT_OFFSET to be TSF_OFFSET to be consistent with 7.3.2.26.

Remedy - P42, L14: Change TBTT_OFFSET to be TSF_OFFSET to be consistent with 7.3.2.26.

Resolution – counter same as 1520

Comment #1175 – Clause 10.3.24.2.2 – Black

Problem - P43, L17: Remove status REFUSED - the draft supports no concept of refusing a neighbor report.

Remedy - Remove REFUSED status.

Resolution – counter same as 1521

Comment #1178 – Clause 10.3.24.1.4 – Black

Problem - P46, L16: Editorial instruction after 10.3.24 not required.

Remedy - Remove

Resolution – accept

Comment #1191 – Clause General – Black

Problem - 11.7.8.1 uses STA in the first sentence, others use station. Be consistent

Remedy - Be consistent - STA, or station (here it is mostly STA).

Resolution - Accept

Comment #1206 – Clause 11.9 – Black

Problem - P57, L9: 'the' link margin field rather than 'a' link margin field

Remedy - Editorial.

Resolution - accept

Comment #1207 – Clause 15.4.4.2 – Black

Problem - P61, L5, Table 66: This is table 61 in IEEE 802.11-1999 R.2003 but Table 66 in the unofficial roll-up. This occurs elsewhere in the PHY clause changes. Over to you Mr. Editor!

Remedy – Editorial

Problem - defer – assigned to Joe Kwak

Comment #1208 – Clause 15.4.4.2 – Black

Problem - P61, L5, Table 66: Value should be 0-255 and not 8-bits of RCPI.

Remedy - Editorial

Resolution - defer – assigned to Joe Kwak

Comment #1211 – Clause 18.3.5 – Black

Problem - P68, L1, Table 108: This table is not the same as in my copy of IEEE802.11 1999 R.2003.

Remedy - Editorial

Resolution - defer – assigned to Joe Kwak

Comment #1213 – Clause Annex A – Black

Problem - There is a general editing problem here. Where already conditional PICS items have been made conditional on CFk a boolean expression needs to be constructed.

Remedy - Add necessary boolean equations.

Resolution - defer – Change to PICs and assigned to Simon Black

Comment #1226 – Clause Annex D – Black

Problem - P83, L21: Should be measurement mode, not scan mode.

Remedy – Fix

Resolution - accept – assigned Paul Gray MIB

Comment #1237 – Clause 7.2.3.7 – Olson

Problem - In the notes section of the RCPI entry of table 10 the word "only" in the sentence adds no clarity and made me think twice about what the actual requirement really is.

Remedy - Strike the word only from the sentence.

Resolution - Accept

Comment #1242 – Clause 7.3.2.21 p11 l3 – Olson

Problem - With the addition of Measurement Pause Request the text should be, "…as described in 7.3.2.21.1 through 7.3.2.21.12".

Remedy - Update as indicated.

Resolution - accept – P11,L3: change “7.3.2.21.11” to “7.3.2.21.12”

Comment #1244 – Clause 7.3.2.21.6 p13 l23 – Olson

Problem - This sentence refers to "scan mode" when it is really Measurement Mode.

Remedy - change "scan mode" to "Measurement Mode"

Resolution - accept – same as 1115

- Call end 10:30 AM PST

# 05/04 Minutes 08:30 AM PST

- Attendance – Paine, Kwak, Gray

- Comment Resolution

Comment #1245 – Clause 7.3.2.22 p21 l6 – Olson

Problem - Reference to where measurements are described is not correct.

Remedy - Update to be "..as described in 7.3.2.22.1 through 8.3.2.22.11"

Resolution - accept – corrected wording should be 7.3.2.22.1 through 7.3.2.22

Comment #1253 – Clause 7.4.2.4 p35 l21 – Olson

Problem - The descriptions says "Link Measurement Request" when it should be "Link Measurement Report".

Remedy - Change description to be "Link Measurement Report".

Resolution - accept

Comment #1255 – Clause 7.4.2.6 p37 l10 – Olson

Problem - This line indicates that the Dialog Token field can be set to the value in "any" corresponding Neighbor Report Request.  This use of any is confusing since it should match just a single request.

Remedy - Change the word "any" to "the".

Resolution - accept – same as 455

Comment #1258 – Clause 10.3.24.2.3 p43 l21 – Olson

Problem - The paragraph is confusing, what primitive is this in reference to?  It is describing the error case, but seems awkward.

Remedy – Remove Paragraph

Resolution - decline – P45L18 is correct – the incorrect reference was addressed in 431

Comment #1260 – Clause 10.3.25.1 – Olson

Problem - The neighbor list and link measurement are out of order with respect to their definition in table k13.

Remedy - Switch the order of the two so they match the order in table k13.

Resolution - Decline

Comment #1261 – Clause 10.3.25.1.2 – Olson

Problem - In the description of DialogToken there is a type with the word "set" when it should be "sent".

Remedy - Replace "set" with "sent".

Resolution – accept – same as 1089

Comment #1262 – Clause 10.3.25.1.3 p48 l4 – Olson

Problem - The paragraph is confusing, what primitive is this in reference to?  It is describing the error case, but seems awkward.

Remedy - 	Remove paragraph.

Resolution - Counter – P48L4 – see spreadsheet for complete information

Comment #1265 – Clause 11.7.4 p50 l31 – Olson

Problem - An extra "report" was included in the sentence.

Remedy - Remove on of the "report" words.

Resolution - Accept – same as 1062

Comment #1267 – Clause 11.7.6 p52 l37 – Olson

Problem - Since the parameters used for periodic measurements now uses the term repeat, the descriptive text should be consistent with that.

Remedy - In this sentence replace "periodic" with "repeated".

Resolution - counter – same as 1527 and 1188

Comment #1268 – Clause 11.7.7 p53 l2  - Olson

Problem - This sentence here would seem to indicate that some measurements are periodic and others are not.  This is not the case.

Remedy - Strike the word "periodic" so the sentence reads, "After completing the processing of all the Measurement Request elements in the…."

Resolution - defer – assigned to periodic

Comment #391 – Clause 11.7.8.1 – Lefkowitz

Problem - Spelling -- line 2 pg 54 "returing to the lowest channel number"

Remedy - change to ""returning to the lowest channel number"

Resolution - accept – reclassify as Editorial

Comment #1269 - accept – same as 391

Comment #1272 - #1279

Accept – same as 174 reclassify as PICs (Black)

Comment #1281 – Clause Annex D – Olson

Problem - In dot11RRMRequestTable we should either preface all items in the table with RRM or none of the items.

Remedy - Either remove RRM from each item in the table or update the medium sense elements to include RRM.

Resolution - Counter – replace with RRMRqst

Comment #1284, 1220

Accept – same as 499

Comment #1286 – Clause Annex D - Olson

Problem - dot11RRMRqstType is missing type for the LCI Request and the Measurement Pause.

Remedy - Add the following entries: lciRequest(10) and measurementPause(11).

Accept – reclassify as MIB

Comment #1287 – Clause Annex D – Olson

Problem - dot11RRMRqstChanNumber description should indicate to also ignore the channel number for the LCI Request and the Measurement Pause.

Remedy - Add required text.

Resolution - defer – reclassify to Location

Comment #1288 - defer – reclassify Location

Comment #1289 - defer – reclassify Location

Comment #1290 – Clause Annex D – Olson

Problem - In dot11RRMBeaconRqstMode the description text should use Measurement Mode instead of Scan Mode.

Remedy - Replace "Scan" with "Measurement".

Resolution - accept – assigned to MIB

Comment #1296 – Clause Annex D p85 – Olson

Problem - Change references to RCPI to RPI to be consistent with the text that describes this measurement.

Remedy - Make the change in dot11MedSenseType and dot11MedSenseRCPIThreshold

Resolution - defer – assigned to RPI

Comment #1299 -  Clause Annex D p88 l22 – Olson

Problem - The word report is missing an "s".

Remedy - Add the "s".

Resolution - accept – assigned to MIB

Comment #1297 – Clause Annex D – Olson

Problem - After the entry for dot11LCIRqstOctet there needs to be entries for dot11RRMPauseTimeUnits and dot11RRMPauseTime.

Remedy - Add entries.

Resolution - defer – assigned to Kwak

Comment #1300 – Clause Annex D p88 l65 – Olson

Problem - The text should read "Noise Histogram" instead of Beacon Request.

Remedy - Replace with Noise Histogram.

Resolution - accept – assigned to MIB

Comment #1301 – Clause Annex D – Olson

Problem - The word report is missing an "s".

Remedy – Add the “s”

Resolution - accept – assigned to MIB

Comment #1302 – Clause Annex D – Olson

Problem - The text should read "Frame Report" instead of Frame Request.

Remedy - Replace with Frame Report.

Accept – assigned to MIB

Comment #1303 – Clause Annex D – Olson

Problem - The text should read "Frame Report" instead of Beacon Report.

Remedy - Replace with Frame Report.

Resolution - accept – assigned to MIB

Comment #1304 – Clause Annex D – Olson

Problem - The text should read "Hidden Station Report" instead of Beacon Request.

Remedy - Replace with Hidden Station Report.

Resolution - accept – assigned to MIB

Comment #1305 – Clause Annex D – Olson

Problem - dot11RRMNeighborReportRSN should be named to match the corresponding field.

Remedy - Change to dot11RRMNeighborReportSecurity.

Resolution - accept – assigned to MIB

Comment #1341 – Clause 4 – Duran

Problem - No acronym for LCI

Remedy - Add acronym for LCI

Accept –same as 316

Comment #1497

Accept – same as 316

Comment #1347

Accept

Comment #1349

Accept – underline the last sentence of 5.4 P3:L7

Comment #1351 – Clause 5.4.5 – Moreton

Problem - "The following are the wireless LAN radio measurement support". "Support" is singular, not plural.

Remedy - Change it to singular.

Resolution - counter – P3:11 change “are” to “items comprise”

Comment #1424 – Clause 5.4 – Hillman

Problem - Which two services are used to support radio measurements? I suspect DFS and RM. In fact since there are 13 uses of services defined and only 12 logical services identified it would be informative to enumerate which 6, 3, 2 and 2 services in each case.

Remedy - enumerate for informative purposes

Resolution – accept (same as #1101)

3.    Call Ends 10:30

# 05/11 Minutes 08:30 AM PST

- Attendance – Paine, Barber, Gray

- Comment Resolution

Comment #1426 – Clause 7.3.2.21.12  - Hillman

Problem - Local and remote are not defined here, the first place they are used I believe.

Remedy - define them in this clause

Resolution - Deferred – Peter reclassify to location

Comment #1434 – Clause 7.3.2.17 - Kandala

Problem - "hysteresis is unused" is awkward

Remedy - Rephrase

Resolution - Counter – “Change to “hysterosis is not used”

Comment #1435 – Clause General – Kandala

Problem - It would be nice to have a difference draft between D1.0 and D2.0 as I have difficulty in finding the actions that have been taken.

Remedy - Please provide the diff document in the future.

Deferred – because D1.0 and D2.0 are beginning, they need to be clean draft.

Comment #1453 – Clause 7.2.3.4 – Rudolf

Problem - Request Information in Table 7 is currently labeled #10. Shouldn't this be #9 ? 11i RSN is #8.

Remedy - Change order of Request Information from #10 to #9.

Resolution - Deferred assigned to Simon Barber

Comment #1460 – Clause 7.3.2.22.9 – Bahr

Problem - P27 L3-8: These lines belong to lines 10-18

Remedy - Move lines 3-8 down behind line 9

Resolution - Accept

Comment #1461 – Clause 7.3.2.22.9 – Bahr

Problem - P27 L3-8, 10-18: The explanation of Bin density is not introduced.

Remedy - Add introductory paragraph/sentence before line 3 behind line 9 (see other comment). Sentence could be something like: Bin i density indicates …

Resolution - deferred – assigned to Histogram

Comment #1462 – Clause 7.3.2.22.8 – Bahr

Problem - P 25 L19: Introductory paragraph/sentence about hidden station entry is missing.

Remedy - Add introductory paragraph/sentence after line 19. Sentence could be something like: Hidden Station Entry contains the fields Hidden Station Address and Number of Frames (see figure k17).

Resolution - Accepted

Comment #1463 – Clause 7.3.2.22.7 – Bahr

Problem - P25 L1: Introductory paragraph/sentence about frame report entry is missing.

Remedy - Add introductory paragraph/sentence before line 1/figure k15. Sentence could be something like: The format of the Frame Report Entry is shown in figure k15.

Resolution - Accepted

Comment #1465 – Clause 7.3.2.22.4 – Bahr

Problem - P21 L17: include directive for the editor is missing

Remedy - add include directive before line 17

Resolution - Accepted

Comment #1466 – Clause 7.3.2.21.6 – Bahr

Problem - P12 L16: line break in word "Randomization" in figure k3

Remedy - make field for Randomization Interval larger so that line break in Randomization disappears

Resolution - accepted – same as 997

Comment #1471 – Clause Appendix A – Ye

Problem - There are references to non-existing clauses and undefined capabilities. E.g., 7.3.2.21.1 and Periodic Measurement.

Remedy - Correct references.

Resolution - Counter – with 7.4.2 and “repeated measurements”

Comment #1479 – Clause 5.5 – Kwak

Problem - P3L31: Delete duplicate "item".

Remedy - Delete duplicate "item".

Resolution – Accept

Comment #1480 – Clause 7.2.3.8 – Kwak

Problem - P6L5: Table 11, order "4" is already used in the latest 802.11ma rollup.

Remedy – Editor to correct

Decline – 802.11ma is only a draft

Comment #1477, 1478, 1481, 1482 – Clause

Problem - This comment unintentionally left blank.

Remedy - None

Resolution - Decline

Comment #1522 – Clause 10.3.25.2.2 – Kwak

Problem –Typo

Remedy - P48 Table: Insert comma after TRANSMISSION_FAILURE in Col3Row2.

Resolution - Accept

Comment #1531 – Clause 11.7.8.5 – Kwak

Problem - Use consistent wording.

Remedy - P54L27:  Change sentence to "If a station accepts a Hidden Station Request it shall respond with a Radio Measurement Report frame including one or more Hidden Station Report elements."

Resolution - accept

Comment #1534 – Clause 11.7.8.6 – Kwak

Problem - Use consistent wording.

Remedy - P54L36:  Change sentence to "If a station accepts a Medium Sensing Time Histogram Request it shall respond with a Radio Measurement Report frame including one Medium Sensing Time Histogram Report element containing the histogram for the requested Medium Sensing Measurement Subtype."

Resolution - accept

Comment #1536 – Clause 11.7.8.6 – Kwak

Problem - Missing preposition.

Remedy - P55L9: Change "interfere the" to " interfere with the".

Resolution - accept

Comment #1538 – Clause 11.7.8.8 – Kwak

Problem - Use consistent wording.

Remedy - P55L16:  Change sentence to "A STA receiving an LCI request shall respond with a Radio Measurement Report frame including one LCI Report element."

Resolution - Accept

Comment #1577 – Clause 7.3.2.21 – Jalfon

Problem - enable bit determines validity of request and report bits, and not the other way around

Remedy - replace "The Enable bit shall be set to 1 when the Request bit and Report bit are valid. The Enable bit shall be set to 0 when the Request bit and Report bit are invalid" with "When the Enable bit is set to 0, the Requested bit and Report bit are invalid; they should be set to 0 by the STA transmitting the request and ignored by the STA receiving the receiving the request

Resolution - accept

Comment #1582 – Clause 7.3.2.27 – Jalfon

Problem - neighbor report element contains a variable length *list* of neighbors

Remedy - remove the word "Entry" in figure k22

Resolution - Accept

Comment #1588 – Clause 11.7.8.5 – Jalfon

Problem - replace "doublet" with "pair"

Remedy - replace "doublet" with "pair"

Resolution - counter – see comment 1195

- Call end 10:30 AM PST