IEEE P802.11
Wireless LANs

| Normative Text for Removing BSSID Info Field From Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Wang | Motorola, Inc. | 8000 W Sunrise Boulevard, Plantation, FL 33322 | 954-723-8124 | Stephen.Wang@motorola.com |

Abstract

This document presents supplemental normative text for document 05/1599r2, which was presented and accepted by TGk in the IEEE 802.11 March 2005 plenary meeting in Atlanta, GA. The document is based on document 05/1599r2 and incorporated normative text necessary to address comments made by TGk members during the Atlanta meeting.

This document tracks only the delta changes between 05/1599r2 and 05/1599r3.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 7. Frame formats

## 7.1 MAC frame formats

### 7.3.2 Information elements

Change the following subclause with tables and figures included herein as follows:

7.3.2.27 Neighbor Report element

The format of the Neighbor Report element is shown in Figure k23.

|  |  |  | Zero or more, one for each neighbor |
| --- | --- | --- | --- |
|  | Element ID | Length | Neighbor List Entry |
| Octets: | 1 | 1 | variable |

Figure k22—Neighbor Report element format

|  | BSSID |  | Channel Number | Channel Band | PHY Options | Neighbor TSF Information (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 |  | 1 | 1 | 1 | 4 |

Figure k23—Neighbor list entry format

The Element ID field shall be equal to the Neighbor Report value in Table 20.

The Length field is dependent on the number of Neighbor List Entries representing the neighboring APs being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Channel Number, Regulatory Class, PHY Options, and optionally may include the TSF Information fields. The minimum value of the Length field is 0 (i.e., with no neighbor APs in the Neighbor Report element).

The BSSID is the BSSID of the BSS being reported. The subsequent fields: BSSID Information, Channel Number, Regulatory Class, PHY Options, and optionally TSF Information fields are presumed to be for the corresponding BSSID.

|  | B0 B1 | B2 | B3 | B4 B8 | B9 B15 |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |

| Value | Reachability | Usage |
| --- | --- | --- |

•

•

•

|  | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |

Channel Number indicates the current operating channel of the AP represented by the BSSID in this neighbor list entry.

Regulatory Class shall contain an enumerated value from Table k1 specifying the frequency band in which the Current Channel is valid.

The PHY Options field represents the Condensed PHY type and TSF Information Flag as shown in the Figure k26.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | Neighbor TSF Information Flag |
| Bits: | 7 | 1 |

Figure k26—PHY Options field

• 	Condensed PHY Type is the lower 7 bits of the PHY Type of the AP represented by this BSSID. The PHY type shall be encoded as defined by dot11PHYType.

• 	TSF Information Flag is a one bit field. When set it indicates that a TSF Information field is present for this Neighbor List Entry. The Neighbor Report may have the TSF Information Flag set in some Neighbor List Entries and not set in others.

The TSF Information is 4 octets long and contains TSF Offset and Beacon Interval subfields.

|  | Neighbor TSF Offset | Neighbor Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k27—TSF Information field format

• 	The TSF Offset subfield is 2 octets long and contains the TSF timer offset, in terms of TUs between the serving AP and a neighbor AP, modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

• 	The Beacon Interval field is the beacon interval of the Neighbor AP being reported. This field is defined in clause 7.3.1.3 and illustrated in Figure 26.