IEEE P802.11
Wireless LANs

| Triggered QoS Measurements Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-04-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jarkko Kneckt | Nokia |  | +358 50 482 1550 | Jarkko.kneckt@nokia.com |
| Simon Black | Nokia |  | +44 1353 648567 | EXT-simon.3.black@nokia.com |
| Jari Jokela | Nokia |  | +358 50 486 0445 | Jari.jokela@nokia.com |

Abstract

Contains proposed normative text additions and changes to support the Triggered QoS Measurements proposal outlined in document IEEE802.11-05/0294r1. Proposed text is relative to IEEE802.11k/D2.0 modified by document 05/1637r4 (Transmit QoS metrics).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change the paragraph after Table 20b in 7.3.2.21 as follows:

The Measurement Request field shall be empty when the Enable bit is set to 1., except when requesting a triggered QoS Metrics measurement. When the Enable bit is set to 0, the Measurement Request field shall contain the specification of the measurement request, as described in 7.3.2.21.1 through 7.3.2.21.113.

Change Clause 7.3.2.21.12 as follows:

##### 7.3.2.21.13 Transmit QoS Metrics Request

The Measurement Request field corresponding to a Transmit QoS Metrics Request is shown in Figure k(unknown1). A response to a Transmit QoS Metrics Request is a Transmit QoS Metrics Report.

|  | Randomization Interval | Measurement Duration | Peer QSTA Address | Traffic Identifier | Bin 0 Range | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 1 | 1 | 5 |

Figure k(unknown1)—Measurement Request field format for a Transmit QoS Metrics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.7.3. Randomization Interval is not used and shall be set to 0 when requesting a triggered QoS metrics measurement.

The Measurement Duration shall be set equal to the duration of the requested measurement, expressed in TUs. When setting up a triggered QoS measurement, Measurement Duration contains the duration of the averaging period used for the Average Error Threshold and reported metrics. See 11.7.8.10.

The Peer QSTA Address shall contain the 6-byte MAC address in the Address 1 field of the measured Data frames.

Traffic Identifier shall indicate the TC or TS for which traffic is to be measured. Values 0 through 15 are defined. Values 16-255 are reserved.

Bin 0 Range shall indicate the delay range of the first bin (Bin 0) of the Transmit Delay Histogram, expressed in TUs. It is also used to calculate the delay ranges of the other 5 bins making up the histogram. The delay range for each bin shall increase in a binary exponential fashion as described in 7.3.2.22.13.

The Triggered Reporting field is used to specify measurement trigger thresholds. It is only present if setting up triggered QoS metrics reporting – see 11.7.8.10. The Triggered Reporting field is as shown in Figure k(tbd1).

|  | Trigger Condition | Average Error Threshold | Consecutive Error Threshold | Delay Error Threshold | Trigger Timeout |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure k(tbd1)—Triggered Reporting Field

Trigger Condition is a bit-field that specifies reporting triggers when requesting a triggered QoS metrics measurement. The format of Trigger Condition is shown in Figure k(tbd2).

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average | Consecutive | Delay | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure k(tbd2)—Trigger Condition field

- Average is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the Traffic Identifier that are discarded over the moving average period specified in Measurement Duration is equal to the value given in Average Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit, or due to the MSDU lifetime having been reached shall be counted.

- Consecutive is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the Traffic Identifier that are discarded in succession is equal to the value given in Consecutive Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding the appropriate retry limit, or due to the MSDU lifetime having been reached shall be counted.

- Delay is set to 1 to request that a QoS Metrics Report be generated when the number of consecutive MSDUs for the TC, or TS given by the Traffic Identifier that experience a transmit delay greater than or equal to the lower bound of the bin of the Transmit Delay Histogram specified by the value in Delayed MSDU Range equals the value given in Delayed MSDU Count.

The Average Error Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the Average trigger condition.

The Consecutive Error Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the Consecutive trigger condition.

The Delay Error Threshold field contains two subfields as shown in Figure k(tbd3).

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Delayed MSDU Range |  | Delayed MSDU Count |  |
| Bits: | 2 |  | 6 |  |

Figure k(tbd3)—Delay Error Threshold subfield

Delayed MSDU Range contains a value representing the MSDU transmit delay at or above which an MSDU will be counted towards the Delayed MSDU Count threshold. Delayed MSDU Range is encoded as a value representing the lower bound of a bin in the Transmit Delay Histogram as shown in Table k(tbd1).

Table k(tbd1)—Delayed MSDU Range Definitions

| Delayed MSDU Range | Condition |
| --- | --- |
| 0 | Transmit Delay ≥ Lower Bound of Bin 2 |
| 1 | Transmit Delay ≥ Lower Bound of Bin 3 |
| 2 | Transmit Delay ≥ Lower Bound of Bin 4 |
| 3 | Transmit Delay ≥ Lower Bound of Bin 5 |

Delayed MSDU Count contains a value representing the number of MSDUs to be used as the threshold value for the Delay trigger condition.

The Trigger Timeout field contains a value in units of 100TU during which a measuring STA shall not generate further triggered QoS metrics reports after a trigger condition has been met. See 11.7.8.10.

Change Clause 7.3.2.22.13 as follows:

##### 7.3.2.22.13 Transmit QoS Metrics Report

The format of the Measurement Report field corresponding to a Transmit QoS Metrics Report is shown in Figure k(unknown2).

|  | Actual Measurement Start Time | Measurement Duration | Peer QSTA Address | Traffic Identifier | Reporting Reason |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 2 | 6 | 1 | 1 |

|  | Transmitted MSDU Count | MSDU Discarded Count | MSDU Failed Count | MSDU Multiple Retry Count | QoS CFPolls Lost Count | Average Queue Delay | Average Transmit Delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

|  | Bin 0 Range | Bin 0 Count | Bin 1 Count | Bin 2 Count | Bin 3 Count | Bin 4 Count | Bin 5 Count |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 4 | 4 | 4 | 4 | 4 | 4 |

Figure k(unknown2)—Measurement Report field 
format for Transmit DelayQoS Metric Report

Actual Measurement Start Time shall be set equal to the TSF at the time at which the measurement started, or for a triggered QoS metrics report the TSF value at the reporting QSTA when the trigger condition was met.

Measurement Duration shall be set equal to the duration over which the Transmit Frame Delay QoS Metric Report was measured, expressed in TUs.

The Peer QSTA Address shall contain the 6 byte MAC address in the Address 1 field of the measured Data frames.

Traffic Identifier shall indicate the TC or TS for which traffic is to be measured. Values 0 through 15 are defined. Values 16-255 are reserved.

The Reporting Reason field is a bit field indicating the reason that the measuring QSTA sent the Transmit QoS metrics report. The Reporting Reason field is shown in Figure k(tbd4).

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average Trigger | Consecutive Trigger | Delay Trigger | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure k(tbd4)—Reporting Reason field

- The Average Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Average Error trigger.

- The Consecutive Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Consecutive Error trigger.

- The Delay Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Delay Error trigger.

In a requested Transmit QoS Metrics Report, all bit fields in the Reporting Reason field set to 0. More than one bit field in the Reporting Reason field may be set if more than one trigger condition was met.

Change 11.7.8.10 as follows:

##### 11.7.8.10 Transmit QoS Metrics Report

A QSTA receiving a Transmit QoS Metrics Request shall respond with a Radio Measurement Report frame containing one Measurement (Transmit QoS Metrics) Report element. If the traffic stream (TS) that is corresponding to the Traffic Identifier is deleted, either by a DELTS Action Frame or by disassociation, the STA shall cease sending Radio Measurement Reports.

The Transmit QoS Metrics measurement shall be made on traffic that is transmitted from the measuring QSTA to the peer QSTA and TC or TS indicated in the request. The Peer QSTA Address may be the MAC address of the QSTA from which the Measurement Request was sent, or the MAC address of another QSTA within the QBSS. This enables a QAP to query Transmit QoS metrics for DLS links. A QAP shall refuse measurement requests for traffic to other QSTAs in the BSS.

The requesting and reporting STAs must be QSTAs. A non-QSTA receiving a Transmit QoS Metrics Measurement Request shall reject the request with indication of “incapable”.

A QSTA may request that a QoS metrics report be sent when MSDU discard, or delay metrics for a specified TC, or TS at a measuring QSTA reach a defined threshold. This is termed a triggered QoS metrics measurement and shall be requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the QoS Metrics Measurement Type. The Measurement Request field shall contain a QoS Metrics Request with the trigger conditions specified in the Triggered Reporting field. One or more trigger conditions may be set with specified thresholds. A triggered QoS metrics request shall not be sent to a QAP. A QAP that receives a triggered QoS metrics request shall not respond.

A non-AP QSTA accepting a triggered QoS measurement shall measure the requested TC, or TS. If a trigger condition occurs, the measuring non-AP QSTA shall send a QoS metrics measurement report to the requesting QSTA. The measuring non AP-QSTA shall not send further triggered QoS reports until the Trigger Timeout period specified in the request has expired, or new trigger conditions have been requested. Measurement of QoS Metrics shall continue during the reporting timeout period.

QoS metrics reported in a triggered QoS metrics report shall be measured over the averaging period given in the Measurement Duration in the QoS metrics measurement request that established the trigger condition. This period shall always be used regardless of the value of Duration Mandatory. The averaging period shall be included in the Measurement Duration field of a triggered QoS metrics measurement report.

The Measurement Start Time field of a triggered QoS metrics report shall contain the value of the QSTA TSF timer at the time the trigger condition occurred to an accuracy of ±1TU.

Once accepted by a measuring non-AP QSTA, a triggered QoS measurement continues to be active until:

- The relevant TS is deleted

- The measuring non-AP QSTA disassociates or successfully reassociates, or

- The measurement is terminated by the requesting QSTA.

All triggered QoS measurements shall be terminated at a measuring non-AP QSTA by receiving a triggered QoS metrics measurement request with the Enable bit set to 1 and the Report bit set to 0. A QoS metrics measurement request with no trigger conditions shall terminate a triggered QoS measurement for the TC, or TS specified in the request. A QSTA requesting a triggered QoS measurement may update the trigger conditions by sending a triggered QoS metrics measurement request specifying the new trigger conditions.

### ANNEX A

In A.4.13 add new entry RRM12.3 as follows:

| RRM12 RRM12.1 RRM12.2 RRM12.3 | Transmit QoS Metrics Type Transmit QoS Metrics Request Transmit QoS Metrics Report Triggered QoS Metrics Report | 11.7, 11.7.8.10 7.3.2.21.13 7.3.2.22.13 7.3.2.21.13 | CFk AND CF12:O CFk AND CF12:O CFk AND CF12:O CFk AND CF12:O | Yes, No Yes, No Yes, No, Yes, No, N/A |
| --- | --- | --- | --- | --- |

References: