IEEE P802.11
Wireless LANs

| Requirements for Management Frame Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jon Edney | Nokia | Cambridge, UK |  | EXT-jon.1.edney@nokia.com |
| Fabrice Stevens | France Telecom |  |  | Fabrice.stevens@francetelecom.com |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Emily H Qi | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-7799 | Emily.H.Qi@intel.com |
| Jesse Walker | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-1849 | Jesse.Walker@intel.com |
| Nancy Cam-Winget | Cisco Systems | 3625 Cisco Way, San Jose, CA 95134 | +1-408-853-0532 | ncamwing@cisco.com |

Abstract

This document captures requirements for Management Frame Protection. It proposes what types of threat are to be protected against, what security protections are to be implemented and which 802.11 management frames are to be protected.

# Requirements Summary

This section provides a point by point summary of the main requirements. The rationale behind these requirements along with secondary requirements and desirable features is given in subsequent sections

## Req 100: Forgery Protection

This protocol shall provide protection against forgery and message modification.

## Req 110: Confidentiality Protection

This protocol shall provide confidentiality protection.

## Req 120: Backwards compatibility

This protocol shall be compatible with the operation of 802.11 stations that do not support 802.11w.

## Req 130: Negotiating Security Protection

This protocol shall negotiate the security properties to be enforced for the protection of management frames.

## Req 131: Protect the Negotiation

There shall be a mechanism to protect negotiation of the security properties to be enforced for the protection of management frames.

## Req 140: Compatibility with 802.11i/r Key Hierarchy

Management frame protection mechanism shall be compatible both with the 802.11i and TGr (Fast Transitioning) key hierarchies and key management schemes. The mechanism may:

- Extend the 802.11i/r key hierarchies

- Derive additional keys from 802.11i/r key hierarchies

This mechanism shall operate alongside 802.11i and TGr protocols.

This protocol shall handle fragmented management frames.

## Req 150: Unicast-Broadcast-Multicast Protection

This protocol shall provide protection for unicast, multicast, and broadcast management frames.

## Req 151: Different schemes for Unicast and Broadcast

The mechanism may be different for protecting unicast and multicast/broadcast management frames.

## Req 160: Selected Deployment of categories of Management Frames protection

This protocol shall provide for selected deployment of different categories of management frames protection but shall selection shall not depend on interpretation of the payload of the management frame.

## Req 170: Protection only after Key establishment

This protocol shall protect management frames only after the establishment of transient session keys for protection of management frames.

## Req 180: Regulatory Requirements

Solutions from the standard shall be capable of FIPS certification

# Background

This document defines requirements for solutions to protect management frames in IEEE802.11. The requirements relate to the work of task group ‘W’ which has the following scope and purpose:

Scope: Enhancements to the IEEE 802.11 Medium Access Control layer to provide, as appropriate, mechanisms that enable data integrity, data origin authenticity, replay protection, and data confidentiality for selected IEEE 802.11 management frames including but not limited to: action management frames, deauthentication and disassociation frames.

Purpose: To improve the security of some or all IEEE 802.11 management frames by defining enhancements such as data integrity, data origin authenticity, replay protection, and data confidentiality.

This document defines requirements in the areas of:

- General requirements affecting all solutions

- Threats for which protection is required: Which types of attack are thwarted and known attacks to which systems remain vulnerable

- Required protection mechanisms.

- Specific requirements for individual management messages and which protections apply to each

- Negotiation of capabilities: Requirements for stations and APs to establish mutual expectations

In addition to the requirements, a set of design goals are presented. Goals are considered desireable aqttributes but are not in all cases completely specified and hance cannot be requirements.

This document may be used as the basis of a call for technical proposals for Task Group W.

# Overall Design

Solution shall have the following attributes:

- [RQ120] Backwards compatibility in the sense that it shall be possible for an access point or station to operate in a mixed environment of 802.11w and non-802.11w stations. In other words it shall be possible for stations to operate using 802.11w without disrupting the operation of stations that do not support 802.11w. This does not preclude polices driven operation in which an access point may refuse to associate with stations that do not support 802.11w

- [RQ150]Solutions shall be able to protect broadcast, multicast as well as unicast management frames. [151] Different mechanisms may be used for broadcast & unicast because unicast and multicast/broadcast requirements tend to have different security requirements. Multicast/Broadcast confidentiality key must be changed everytime someone leaves or joins the group.  For integrity protection for broadcast/multicast, the 802.11i broadcast key is applicable only to stateless/idempotent messages, and not for stateful messages like mesh routing.  For integrity protection solution, the frames must be delivered before the integrity keys are derived.

- [RQ160] Solutions should not require explicit knowledge of the payload of management frames. For example, information elements within a management frame should be opaque to the protection mechanism

# Threats

Solutions shall provide mechanisms to protect against the threats listed below.

- Denial of service through forged messages such as replayed disassociation or deauthentication

- Network disruption due to modification or forgery of information in action frames

- Modification of behaviour due to the delaying of message delivery

# Protection Mechanisms

Protection mechanisms are not necessarily used with all management frames.

Protection shall be provided against:

- [RQ100] Forgery of management frames (creating a frame using a source MAC address without authorisation). This is the ultimate rationale for doing 802.11w.  Protection against forgeries is needed by almost every amendment to 802.11 (e, h, k, r, s, v), with potentially others in the future.

- [RQ100] Unauthorised modification of management frames (changing the contents of a valid management frame undetectably)

- [RQ100] Unauthorised replay of a management frame

- [RQ110]Reading by an unauthorised party of data intended to be confidential in a management frame. There are selected subsets of management frames that require this property, including 802.11k Location Report Message. In addition, the work being proposed in 802.11v/k to access SNMP MIBs must support same or greater level of security policy implemented by higher level protocols, like SNMPv3

It should be noted that the concept of “authorised” varies depending on the type of message. For example, when messages are sent with multicast destination the “authorised” station may include all members of a group of stations. When messages are sent as unicast the authorised stations would normally only include the sender and receiver.

These mechanisms shall be applied only under the following conditions:

- A security association has been established between / among the parties

- The mechanism is specified for use with the particular management frame according to 802.11w

# Specific message protections

Currently 802.11 management frames include:

- (Re)Associate Request

- (Re)Associate Response

- Disassociate

- Probe Request

- Probe Response

- Beacon

- ATIM

- Authenticate

- De-authenticate

- Action

Requirements on protection are summarized in Table 1.

| Frame Class | Type | Confident-iality | Authenticity & Integrity |  | Replay |
| --- | --- | --- | --- | --- | --- |
| Class 1 | Beacon | Not protected* |  |  |  |
|  | Probe Req/Resp |  |  |  |  |
|  | ATIM |  |  |  |  |
|  | Action |  |  |  |  |
| Class 2 | (Re)Association | Not protected If PTKSA exists, it is addressed by TGr |  |  |  |
|  | Deauthentication |  |  |  |  |
| Class 3 | ATIM | n/a | Yes | Yes |  |
|  | Disassociation | n/a | Yes | Yes |  |
|  | Deauthentication | n/a | Yes | Yes |  |
|  | Action | Yes | Yes | Yes |  |

Table 1: Protection of specific management frames

IMPORTANT: A 802.11w STA shall not send (re)associate while in State 3.

*It is noted that 802.11u may introduce additional protections for service advertisement but this is considered out of scope for 802.11w if it is above the management frame layer.

Consideration is given to the work of TGr which is defining new methods to protect re-association. However, 802.11r will not protect initial association messages – only those in transition. Furthermore it may be that 802.11w protections will be needed also in systems that do not support 802.11r.

Forgery of disconnect messages is a major threat for denial of service attack that has been observed in practical systems. There is a requirement to protect these messages against forgery and modification. For disconnect messages the requirement to protect against replay is implicitly provided since the security association is broken down upon receipt of the first such message.

The use of ATIM (power management) is restricted to IBSS mode in which it coordinates the power save behaviour of stations. The consequence of forging or replaying ATIM is that station may stay awake longer than otherwise required to do. While a successful attack could be made the consequences are relatively benign.

The needs for protection of action frames differ according to application. Some applications need no protection since the action frame contents are informative only and false information is not damaging. Many applications need to protect against forgery or tampering of action frames and some applications need confidentiality for the information in action frames. Therefore protection for actions frames must be provided at a consistent level regardless of application.  It is therefore a requirement that stations sending action frames shall have the ability to protect with mechanisms for data integrity, data origin authenticity, replay protection, and data confidentiality (in cases where a suitable security context exists between the parties)

# Negotiation of capabilities

[RQ130] It is a requirement that a station be able to determine whether an access point supports 802.11w prior to attempting a connection. This allows operation with legacy STAs that do not implement management frame protection. The access point may advertise policy in addition to capability. For example it may advertise that it will not accept associations unless the station both supports and uses 802.11w.

Different levels of support shall be subject to negotiation.

[RQ131] It is a requirement in this case that a negotiation mechanism be available that is protected against attacks that might downgrade the negotiated security.

# Assumptions

It is assumed that all stations supporting TGw will also support 802.11i and that the provisions of 802.11i can be used as part of the 802.11w solution

It is assumed that station may or may not support the provisions of 802.11r

These requirements do not include protection against forged associate or reaasociate requests or responses. It is anticipated that such protections will be available through the use of 802.11r when that standard is completed.

# Goals (not requirements)

The following lists goals for the mechanism that should be achieved to whatever extent possible.

## Goal 100: Incremental Inclusion of new Management Frames

This protection mechanism should provide for incremental inclusion of management frames, as new management frames are developed by 802.11.

## Goal 110: Enabling upgrade of existing systems

Upgrading of existing systems to support 802.11w should generally be possible using firmware or software changes. It is recognized that not all implementations can be considered but solutions should avoid techniques that demand PHY changes, new frame formats and very high speed operations requiring hardware support.

## Goal 120: Protocol efficiency

The solutions should not introduce substantial additional packet overheads – by comparison TGi introduces 20 additional bytes and solutions are required to be comparable to this overhead.

## Goal 130: Re-use of technology and standards

When possible solutions shall be incremental and build upon existing standards and technology. Substantially new approaches must be justified by significant benefits. The rationale for this is:

- designing, managing, implementing, and deploying key hierarchies is costly and complex.

- constrained environments and their lower end computational and memory sizes make this approach costly to adopt and widely deploy.

- there will be deployments and implementations that will support one or both of 802.11i and 802.11r

## Goal 140: Computational requirements

Solutions should not place unreasonable burdens of memory, computation or power consumption on implementations

## Goal 150: Anonymity

It is desirable to hide the identity of stations from unauthorised parties

# References

11-05-0139-00-0ads-which-management-frames-need-protection.ppt

11-05-0237-00-0ads-requirements-management-frames-protection-schemes.ppt

11-05-0343-00-0ads-protectionmanagementframes-protocolrequirements.ppt

11-05-0350-00-0ads-strawman-requirements-tgw.doc

11-05-0427-00-0ads-PMF-Requirements.ppt