IEEE P802.11
Wireless LANs

| Introducing 11r-D0.00 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-06-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Initial draft: P802.11r-D0.00 (based on accepted contribution 0362-01)

Edited draft: P802.11r-D0.01 (with editorial changes, etc)

Changebar: P802.11r-D0.01-changes

See 11-05-0538-00-000r-introducing 11r-d0.00.ppt for further details

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Following is an attempt to generate a MSWord file from the FrameMaker source.  None of the section numbers came across, nor did the figures, nor tables.  (Section numbers and figures were quietly ignored by MSWord; attempt to copy/paste a table crashed.) Let me know if it is useful.

Draft Amendment to STANDARD FOR Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements -

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specification:

Amendment 8: Fast BSS Transition

[This amendment is based on IEEE Std 802.11TM, 2005 (based on 802.11ma D1.0), as amended by 802.11eTM-2005, 802.11k-????.]

The editing instructions contained in this amendment define how to merge the material contained herein into the existing base standard to form the new comprehensive standard.

The editing instructions are shown in bold italic. Three editing instructions are used: change, delete, and insert. Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instructions.

NOTE - Editorial notes (paragraphs starting with “NOTE”) are present throughout this document. These will be added and changed at the discretion of the Editor. These will be deleted at the discretion of the Editor when the Editor feels the issue identified in the note has been adequately addressed. Editorial notes will not be carried over into the final document.

References

Insert the following new normative references in alphabetical order:

NOTE: If there are no new references, delete this section

Definitions

Change the following definitions as indicated:

3.74 pairwise master key (PMK): The highest order key used within this standard. The PMK may be derived from an Extensible Authentication Protocol (EAP) method or may be obtained directly from a pre-shared key (PSK).

NOTE - need new wording above to make consistent with new PMK-R0 (which appears below).

3.76 pairwise transient key (PTK): A value that is derived form the pairwise master key (PMK), Authenticator address (AA), Supplicant address (SPA), AnthenticatorAuthenticator nonce (ANonce), and Supplicant nonce (SNonce) using the pseudo-random function (PRF) and that is split up into as many as five keys, i.e. temporal encryption key, two temporal message integrity code (MIC) keys, EAPOL-Key encryption key (KEK), EAPOL-Key confirmation key (KCK).

NOTE - need new wording above to make consistent with new PTK “Pairwise Transient Key shared between a STA and BSSID used to protect 802.1X EAPOL and 802.11 data frames”

3.108 Supplicant Address (SPA): The Supplicant’s medium access control (MAC) address. Typically this is the STA’s MAC address.

Insert the following new definitions in alphabetical order, renumbering as necessary:

3.121 AAA Key: Key information that is jointly negotiated between the Supplicant and Authentication Server as defined by RFC 3748. This key information is transported via a secure channel from the Authentication Server to the Authenticator. The PMK-R0 is derived from the AAA Key.

NOTE - need to confirm that “Supplicant” is proper word to replace “SPA” in the proposal’s text.

3.122 Authentication Server (AS): An entity that provides an authentication service to an Authenticator. This service determines, from the credentials provided by the Supplicant, whether the Supplicant is authorized to access the services provided by the Authenticator.

NOTE - need to confirm that “Supplicant” is proper word to replace “SPA” in the proposal’s text.

3.123 Authenticator: an entity at one end of a point-to-point LAN segment that facilitates authentication of the entity attached to the other end of that link.

3.124 Authenticator Address (AA): The Authenticator’s media access control (MAC) address.

3.125 Master Session Key (MSK): The Master Session Key is keying material that is derived between the EAP peer and server and exported by the EAP method to the NAS. The MSK is at least 64 octets in length.

3.126 Pairwise Master Key R0 (PMK-R0): The top level of the FBT key hierarchy; key is shared between a STA and the R0 key holder, and used to derive PMK-R1 keys.

3.127 Pairwise Master Key R0 Key Holder Identifier (R0KH-ID): the 16 octet identifier that is advertised as the holder of the PMK-R0.

3.128 Pairwise Master Key R1 (PMK-R1): The second level of the FBT key hierarchy; key is shared between a STA and the R1 key holder, and used to derive PMK-R2 keys.

NOTE - text in proposal section 7.2.3 says PMK-R1 is the second level of the hierarchy; other text is consistent with this, but definition in section 2.2 says first level. Confirmation of this change to be consistently “second level” is requested.

3.129 Pairwise Master Key R1 Key Holder Identifier (R1KH-ID): the 16 octet identifier that is advertised as the holder of the PMK-R1

3.130 Pairwise Master Key R2 (PMK-R2): The third level of the FBT key hierarchy; key is shared between a STA and the R2 key holder, and used to derive the PTK.

NOTE - text in proposal section 7.2.5 says (in one place) PMK-R2 is the third level of the hierarchy; other text is inconsistent; definition in section 2.2 says second level. Confirmation of this change to be consistently “third level” is requested.

3.131 Pairwise Master Key R2 Key Holder Identifier (R2KH-ID): the 16 octet identifier that is advertised as the holder of the PMK-R1.

NOTE: Proposal section 2.2 says that R2KH-ID is the holder of PMK-R1, which the Editor believes should be PMK-R2. Confirmation from the group requested.

3.132 Pairwise Transient Key Identifier (PTKID): the 16 octet identifier used to name and reference the PTK.

3.133 Resource Mobility Domain A unique identifier which specifies an AP's over-the-DS reachability. All APs in a Resource Mobility Domain have over-the-DS reachability to all other APs in that Resource Mobility Domain.

3.134 R0Name: the 16 octet globally unique identifier used to name and reference PMK-R0.

3.135 R1Name: the 16 octet globally unique identifier used to name and reference PMK-R1.

3.136 R2Name: the 16 octet globally unique identifier used to name and reference PMK-R2.

3.137 Security Domain (SD): A Security Domain consists of a single PMK-R0 Key Holder as a root, and all key holders of PMK-R1 and PMK-R2 keys derived from PMK-R0. Each Security Domain is contained within a Security Mobility Domain.

3.138 Security Mobility Domain (SMD): A set of associated Security Domains (SD) capable of exchanging PMK-R1s between them. A station may perform fast transitions among all APs in a Security Mobility Domain.

3.139 Security Mobility Domain Identifier (SMD-ID): the 16 octet globally unique identifier used to name and reference an SMD.

3.140 Transition Enabled Station (TSTA): The non-AP station capable of executing the fast BSS transition procedures, as defined in this standard.

3.141 Transition Resource Information Element (TRIE): An Information Element for enabling fast transitions between APs, which carries QoS and resource specific information.

3.142 Transition Security Information Element (TSIE): An Information Element for enabling fast transitions between APs, which carries security specific information.

Abbreviations and acronyms

Insert the following new abbreviations and acronyms in alphabetical order:

AA	Authenticator Address

FBT	Fast BSS Transition

FT	Fast Transition

MSK	Master Session Key

NAS	Network Access Server

PMK-R0	Pairwise Master Key, top level

PMK-R1	Pairwise Master Key, first level

PMK-R2	Pairwise Master Key, second level

PTKID	Pairwise Transient Key Identifier

R0KH	PMK-R0 Key Holder

R0KH-ID	PMK-R0 Key Holder Identifier

R1KH	PMK-R1 Key Holder

R1KH-ID	PMK-R1 Key Holder Identifier

R2KH	PMK-R2 Key Holder

R2KH-ID	PMK-R2 Key Holder Identifier

RDIE	RIC Data Information Element

RIC	Resource Information Container

RRIE	RIC Root Information Element

RRSAP	Resource Request Service Access Point

SD	Security Domain

SMD	Security Mobility Domain

SMD-ID	Security Mobility Domain Identifier

TRIE	Fast Transition Resource Information Element

TSIE	Fast Transition Security Information Element

TSTA	Transition-Enabled Station

TTAP	Transition Target Access Point

General Description

Logical Service Interfaces

Insert the following entry to the end of the list of 802.11 architectural services:

BSS-Transition Services

Overview of the Services

Insert the following new clause after 5.4.4.2, renumber figures and tables as appropriate.

BSS-Transition Services

As wireless networking becomes more prevalent, is deployed in a variety of architectures, and supports applications such as telephony, shortcomings that could previously be ignored now become apparent.

This section deals with the problem of intermittent connectivity loss due to transitioning between BSSs in an ESS. Each time the wireless station must abandon its connection with one AP and establish a new connection with another, there is a brief loss of connectivity, which may result in packet loss. In many interesting cases packet loss will have effects that are noticeable to the user, particularly when running real-time applications. With streaming TCP connections, loss of even a single packet will result in a transmission hiccup whose duration may be far in excess of the actual gap in connectivity. With real-time transmission of voice data, audible cracks and pops result. The more frequently the station must roam, the more annoying these packet losses will be.

This section defines an approach that minimizes the duration of connectivity loss during transition. This approach is scalable and secure, and accommodates a variety of 802.11 system architectures.

General Concepts

There are three state transitions involved when a STA transitions from its current AP to a new AP:

Discovery - the station must locate and decide to which AP it will transition;

Resource establishment - the station may establish that the new AP will provide connection resources it needs to maintain active sessions.

Transition - the station abandons the current AP and establishes a connection with a new AP.

Discovery

The STA must maintain a list of potential transition candidates so that it can transition to a new AP as quickly as possible when signal quality of the current association falls below some target level. The station will scan multiple channels for an AP with a better signal. This process typically takes hundreds of milliseconds. While scanning different channels, the station cannot receive on its current channel, and packets may be lost. In a QoS environment, the STA must maintain channel utilization metrics at the new AP; when it chooses to transition to a new AP, it can maintain adequate service levels for QoS streams.

BSS-transition services provide a mechanism for the STA to communicate and retrieve information on candidates prior to transition. The communications with the target AP can take place over-the-air or through the STA’s existing association with its current AP.

Resource Allocation

If the station depends on a level of resources to maintain its current connection, then it is the responsibility of the station to determine that such resources will be available before transitioning to the new access point. In a non-FBT QoS environment, a QSTA must associate with the new Access Point prior to requesting resource reservations. This significantly increases the transition latency, and furthermore, there is no guarantee that the target AP will be capable of delivering adequate services, causing the station to search for alternate Access Points.

BSS-transition services provide a mechanism for the STA to reserve resources at a candidate AP prior to transition or association time. The STA can communicate with the target AP to reserve resources prior to association by communicating directly with the target AP, or indirectly through its existing Association. The resource allocation mechanism can be used to allocate any type of resources, which could include TSPECs in a QoS environment, or keying material in a secure environment.

The Fast Transition Gap

Once a suitable transition candidate has been found, the station must break its association with the current AP and associate with the new AP. During this time, packet loss can occur. If the station fails to establish the new connection it may immediately resume communication with the old AP so that the transition gap is always terminated by resumption of communications (either to the current or new AP), subject to overlapping RF coverage.

BSS Transition services provide a mechanism for the STA to (re-)associate with the target AP which minimizes any latency introduced from protocol overhead. The STA can choose to reserve resources at the AP prior to (re-)association, thereby reducing the probability of losing packets on transition.

Fast Transition in a Robust Security Network

The transition process in a RSNA environment is defined as:

Setting up the radio for the new channel

Exchanging (re)association request/response with the new AP.

Performing authentication

Performing key management

Establishing other aspects of connection state, for example QoS

In a RSN, the latency introduced via 802.1X authentication is reduced via PMK caching and pre-authentication. When the station and the new AP have a cached security association, either due to a prior association or pre-authentication, the station does not need to re-authenticate to the new AP; however, it still must perform a 4-way handshake in order to establish session keys for data communication.

Even with this reduced pre-authentication exchange, latencies will be measured in the tens of milliseconds with well-powered stations and higher when handhelds are used.

A STA in a RSNA environment is able to pre-authenticate to a new AP prior to transition, using the connection to the current AP to communicate with the new AP via the DS. This allows the establishment of a PMKSA in advance of transition with an AP that has not yet been visited. Upon (re)association with an AP where pre-authentication has been accomplished, PMK caching can be utilized by presenting a PMKID in the (re)association request.

When an EAP authentication occurs to an AAA server, the AAA server informs the AP of the lifetime of the authentication; for example, RADIUS and Diameter servers return the Session-Timeout attribute. At some point in advance of expiration, the AP must initiate a re-authentication in order to allow the session to continue. Since a new PMK is generated as a result of authentication, the AAA lifetime represents an absolute maximum lifetime for a PMK.

In a RSNA, the AP does not communicate expiration information to the station. When the station attempts to re-associate with a cached PMKID, it may find that the AP does not accept it and a full authentication will ensue.

BSS-Transition services offer new Authentication and Key Management procedures which optimize key management to reduce transition latencies. The scope of the PMKSA is communicated to the station, allowing it to select the appropriate PMK for pre-keying or determine that pre-authentication is required. The lifetime of the PMKSA is communicated to the station, allowing it to judge whether it should attempt pre-keying or perform pre-authentication.

Continuity of network service quality is enabled by the use of resource allocation either prior to re-association, or at re-association time. It introduces a new protocol which allows PTK's to be derived prior to re-assocation, thereby minimizing transition latencies.

Fast BSS Transition Architecture Description

BSS-Transition services provides a mechanism to minimize the transition gap in a Robust Security Network between APs and QAPs. It enables a means to:

Minimize "over the air" latency by piggybacking resource allocation

Minimize PTK computation and plumbing latencies by enabling the STA to pre-compute prior to re-association

Enable an "Authenticated" re-association exchange

Removes the 4-way handshake race conditions

Enables allocation of QoS resources at reassociation time

Ensure compatibility with non-RSN and non-QoS STAs and APs

The BSS-Transition services include:

A new transition protocol which allows a STA and AP to allocate resources as part of the re-association; or pre-reserve resources prior to reassociation time;

A new key management framework for security which allows the STA and the AP to pre-compute the PMK;

A mechanism which uses the new transition protocol to perform PTK derivation at re-association time;

A mechanism for allocating QoS resources at reassociation time using the new transition protocol changes.

This addresses solutions to two classes of network infrastructures from a QoS perspective: one where the TTAP is capable of provisioning QoS resources at re-association time; and another where the TTAP needs to pre-reserve QoS resources prior to reassociation. Thus there are two mechanisms for BSS-Transition:

the basic BSS-transition mechanism, during which resources are allocated and committed during the re-association phase of the BSS-Transition. This solution addresses the case where the AP is lightly loaded and the STA can determine resources are available from Beacon/Probe exchanges.

the pre-reservation BSS-Transition mechanism, during which resources are allocated prior to re-association. This solution addresses the case where the infrastructure is under-provisioned; the DS infrastructure is slow; or the provider wants to offer a guarantee of service. This solution addresses networks with a security infrastructure that requires explicit messaging for resource reservation.

BSS-Transition services do not specifically address the solution of when or where a STA will roam. There are other tools which give the STA information that could be used in making this decision.

QoS mechanisms enable the AP to convey QBSS information in Probe responses and beacons. The QBSS IE has three fields which indicate the number of associated STAs, the channel utilization for the BSS, and the available admission capacity. The QBSS metrics give information on the AP's ability to accept new QoS streams.

BSS-Transition services define where a STA can roam. Fast BSS Transitions can only take place within an ESS. In a secure environment, the transition candidates for a STA are limited to APs that are members of the same Security Domain. In some infrastructures, resource policies may limit the STA to a subset of APs as transition candidates defined by the Mobility Domain. In the limiting case, the ESS, the Security Domain, and the Mobility Domain define the same set of APs. The Security Domain will always be a subset of the ESS, and the Mobility Domain will always be a subset of the Security Domain.

A representative BSS-Transition topology is given in Figure Error: Reference source not found. A simple representative topology consists of a number of FT-enabled APs, and an Authentication Server. Inter-AP communication over the DS is assumed to be protected and authenticated. The FT-enabled STA has an established connection with the DS through AP1 and may have one or more QoS streams active. In this representative topology, the STA has two potential transition candidates, AP2 and AP3. The STA decides through scanning, neighbor reports, and other means, that the best candidate for transition is AP2.

There are two mechanisms for the STA to issue resource requests to candidate APs. In one case, the STA goes off-channel from its current association and communicates over the air with the candidate AP. In the second case, the STA communicates with the candidate AP through its existing AP. In either case, the STA uses the Resource Request Service Access Point (RRSAP) to generate and respond to resource reservation.

The RRSAP handles Resource Requests. The RRSAP on the STA is used to generate queries or resource requests. The RRSAP at the AP will receive and process Resource Requests from STAs originating over the air or over the DS. The RRSAP can also forward resource requests on behalf of the STA over the DS to a potential transition candidate.

The STA may choose to reserve resources at the transition candidates prior to transition. Upon successful completion, resources are reserved at the target APs for a limited length of time. The STA must complete the transition within the time limit in order to access its reserved resources.

Reservation policies can be applied to limit the number of reservations that a STA can make or limit the number of APs where the STA can concurrently reserve resources.

Representative Fast Transition Topology

BSS-Transition service provides a generic mechanism to reserve resources at a candidate TTAP called the Resource Information Container (RIC). It provides a mechanism for the STA to build resource requests that could include mandatory and optional resources, or request for alternative resources.

In a Robust Security Network environment, the BSS-Transition service describes mechanisms to support pre-keying and resource pre-establishment between a station and an AP. It describes a key hierarchy to extend the scope of a PMK from a single AP to many APs in a complex network that may include both standalone APs and centralized architectures. It defines a new 802.11 authentication type, which allows the client to initiate a handshake to establish the PTK prior to (re)association.

Message information contents that support the services

Association

Change text of section 5.7.2 as follows:

For a STA to associate, the association service causes the following messages to occur:

Association request

Message type: Management

Message subtype: Association request

Information items:

IEEE address of the STA initiating the association

IEEE address of the AP with which the initiating STA will associate

SSID

Power capability

Supported channels

Fast Transition Security information

Fast Transition Resource information

Robust Security Network information

Direction of message: From STA to AP

NOTE - the information items in the above list are a bullet list in 802.11, but bullet lists are not allowed by IEEE style manual. Need to investigate this.

Association response

Message type: Management

Message subtype: Association response

Information items:

Result of the request association. This is an item with values “successful” and “unsuccessful”

If the association is successful, the response shall include the association identifier (AID)

Fast Transition Security information

Fast Transition Resource information

Robust Security Network information

Direction of message: From AP to STA

NOTE - Figure 121D, for first contact, shows three additional fields in the Association and Association response messages: TSIE, TRIE, and RSNIE. These need to be added to the above text, and kept consistent with 7.2.3.4 and 7.2.3.5.

Reassociation

Change text of section 5.7.3 as follows:

For a STA to reassociate, the reassociation service causes the follow message to occur:

Reassociation request

Message type: Management

Message subtype: Reassociation request

Information items:

IEEE address of the STA initiating the reassociation

IEEE address of the AP with which the initiating STA will reassociate

IEEE address of the AP with which the initiating STA is currently associated

SSID

Power capability

Support channels

Fast Transition Security information

Fast Transition Resource information

Robust Security Network information

Resource request information

Direction of message:

From STA to AP (The AP with which the STA is requesting reassociation)

The address of the current AP is included for efficiency. The inclusion of the current AP address facilitates MAC reassociation to be independent of the DS implementation.

Reassociate response

Message type: Management

Message subtype: Reassociation response

Information items:

Result of the request reassociation. This is an item with values “successful” and “unsuccessful”

If the reassociation is successful, the response shall include the AID

Fast Transition Security information

Fast Transition Resource information

Robust Security Network information

Resource request information

Direct of message: From AP to STA

Frame formats

Format of individual frame types

Management frames

Beacon frame format

Insert the additional rows as follows in the proper order in Table 5

NOTE - Text for this section still needed. Other text in proposal says TRIE and TSIE are included in Beacon frames.

Association Request frame format

Insert the following row into Table 7:

NOTE - Figure 121D for First Contact shows the term “(Re)Association” for two of the messages. That implies to editor that it could be Association or Reassociation. If so, then TSIE, TRIE, and RSNIE need to be added to this section.

Association Response frame format

Insert the following row into Table 8:

NOTE - Figure 121D for First Contact shows the term “(Re)Association” for two of the messages. That implies to editor that it could be Association or Reassociation. If so, then TSIE, TRIE, and RSNIE need to be added to this section.

Reassociation Request frame format

Delete order 9, and insert rows at the end of Table 9 as follows:

NOTE - Proposal section 4.2.1 does not include “RSN” in the list of IEs in the reassociation request. The text states that it is removed from Fast-Transition reassociations, since it is duplicated in the EAPKIE later. However, to support devices that do 11i but not 11r, the RSN IE will still be present and needs to be handled by the AP. Therefore, Editor suggests (1) RSN should be changed to be optional in Fast-Transition cases, and make the text “The RSN information element is only present within Reassociation Request frames generated by STAs that have dot11RSNAEnabled set to TRUE, and which do not have an EAPKIE later in the frame.”, or (2) keep the RSN IE separate and not in the Key-Info field of the EAPOL-Key message, but insert Count IE before the RSNIE and rest of additions to be immediately after the RSN IE.

NOTE - 11ma added “Vendor Specific” and 11e added “QoS Capability”, both as order 10; presumably one of them will use order 11. Numbers from proposal were adjusted.

NOTE - Text in 4.2.1 for the Count IE is “Specifies the number of IEs succeeding this IE and if present, protected by the EAPKIE” Above text is believed to match the intent. Editor requests confirmation.

NOTE - Need to further specify the conditions for inclusion of each of these IEs in the reassociation request frame - FBT/FBT&QoS/FBT&RSN

Insert the following text following Table 9:

For a Fast BSS Transition, the Reassociation request is used to request or confirm the availability of resources that the STA needs to complete its connection. For a Fast BSS Transition when RSNA is enabled, the TSTA must assert liveness of the PTK by including the ANonce provided by the TTAP, and authenticating the frame by including a valid MIC in the EAPKIE. The MIC shall protect the fields starting with the Count IE up to and including the EAPKIE. The RSN information element in the EAPOL-Key message contained in the EAPKIE must be bitwise identical to the RSN IE presented in the Fast Transition Authentication Request Frame.

Reassociation Response frame format

Insert rows at the end of Table 10 as follows:

NOTE - 11ma added a row for “Vendor Specific” for order 6; 11e added a row for “EDCA Parameter Set”, and called it order 5 (which was already taken in 802.11 standard for “Extended Supported Rates”); and 11k is adding a row for “RCPI” and also calling it order 5. Assume 11e will be changed to order 6, 11k to order 7, and Vendor Specific moved to order 8. Order numbers for our new entries adjusted accordingly.

NOTE - proposal noted several items (e.g. Listen Interval”, “Current AP address”, “SSID”, “Power Capability”, and “Supported Channels”) as “As defined by 802.11 standard” that are not in 11ma nor 11e nor 11k. These appear in the Reassociation Request, but not in the Response; assumed to be mis-placed in the proposal text. Also, the proposal noted a “RSN” in this frame, which does not appear anywhere.

NOTE - Text in 4.2.2 for the Count IE is “Specifies the number of IEs succeeding this IE and if present, protected by the EAPKIE” Above text is believed to match the intent. Editor requests confirmation.

NOTE - Text in 4.2.2 included the Time Interval IE for a re-association timeout value. But the Reassociation has just been done. Doesn’t seem reasonable that a timeout would be needed here.

NOTE - Need to further specify the conditions for inclusion of each of these IEs in the reassociation response frame - FBT/FBT&QoS/FBT&RSN

Insert the following text following Table 10:

For a Fast BSS Transition, the TTAP uses the Reassociation Response frame to respond to the TSTA resource and PTKSA requests. The TTAP will include TRIE, TSIE, RIC IE(s), and EAPKIE in response to the IEs included in the Reassociation request. If RSNA is enabled, the TTAP must assert liveness of the PTK by including the SNonce provided by the TSTA, and authenticate the frame by including a valid MIC in the EAPKIE. The MIC shall protect the fields starting with the Count IE up to and including the EAPKIE. The RSN information element in the EAPOL-Key message contained in the EAPKIE must be bitwise identical to the RSN IE presented in the TTAP Beacon Frame.

Probe Response frame format

Insert the order 23-TBD rows, and change the starting order in the final row (Request information elements) in Table 12 as follows:

NOTE - text needed for this section. Other sections in proposal identify TRIE and TSIE as being included in probe responses.

Authentication frame format

Change the first paragraph of section 7.2.3.10 as shown:

The frame body of a management frame of subtype Authentication contains the information shown in Table 13. Only Authentication frames with the authentication algorithm set to Open System authentication or Fast Transition authentication may be used within a RSNA. RSNA STAs shall not associate if shared authentication was invoked prior to RSN association. The Fast Transition Authentication exchange is only valid when the Fast Transition capability is enabled; more specifically, when the Fast Transition Information Element is advertised by the BSSID and employed by the TSTA.

Insert the following paragraph after the first and before Table 13:

The Fast Transition Authentication algorithm allows a TSTA to initiate a Fast Transition. If the FT Authentication algorithm is selected, the TSTA will include its TSIE and TRIE in the FT Authentication Request. If Robust Security Network Association (RSNA) is enabled, the TSTA shall also include its SNonce contribution and include the key holder identities used for generating the PTK.

Insert five additional entries, and change the order 5 table entry as shown:

NOTE - Is the TSIE to appear in all Authentication frames? Or only Fast Transition frames? Editor believes there is a missing reference to footnote 3 in the proposal text, and the text should read “Fast Transition Security Information Element. This field is only present in the Fast Transition Authentication frames as defined in Table 14.”.

Insert the following paragraph between Table 13 and Table 14:

The Fast Transition Authentication exchange may include a variable number of information elements to provision security and QoS resources. When RSNA is enabled, the QoS resource reservation mechanism is authenticated by the MIC specified in the EAPKIE. Further, the MIC specified in the EAPKIE shall protect all of the information commencing with the IE count to the (and including the) EAPKIE.

Change heading row of Table 14, change rows for “Shared Key 2” and “Shared Key 3” as shown, and Insert the following four rows at the end of the table.

NOTE - Editor doesn’t believe this table adequately covers the fast BSS transition case in a non-RSNA environment with QoS, nor a RSNA environment without QoS. If it doesn’t, contributions needed.

Management frame body components

Fixed fields

Authentication Algorithm Number field

Insert after “Authentication algorithm number = 1” the following line:

Authentication algorithm number = 2: Fast Transition

NOTE: Value of 2 for this authentication algorithm number needs to be cleared with the IEEE Assigned Number Authority.

Status Code field

Insert five rows into Table 20, and Change first “Reserved” row, as follows:

NOTE - 11ma modified table to make two reserved blocks, with vendor-specific values in the middle; 11e defined values up through 50 (in particular, values 32-35, 37-39, and 47-50); 11ma had left 27-39 and 47-50 reserved (leaving room for 11e), and defined value 51; values here adjusted accordingly. The values of these status codes need to be cleared with the IEEE Assigned Number Authority.

Action Field

Insert row into Table 21, and modify the “Reserved” row, as follows:

NOTE - 11k assigned value 5 to Radio measurement. Need to confirm our selection of value 6 with IEEE Assigned Number Authority.

Information Elements

Insert the following into Table 22, and change the final row as marked:

NOTE - 802.11 defines values up through 50; 11k defines 51-53. These were chosen as next available numbers. Need to check these assignments with IEEE Assigned Number Authority.

RSN Information element

AKM suites

Insert after third row of table 29, and change fourth row (suite type 3-255) as follows:

NOTE - proposal section 4.4.8 had suite type 4 both assigned for PSK and reserved. Request confirmation that above is the intention.

Insert the following new subsections after subsection 7.3.2.37, adjusting the figure numbers and table numbers as necessary

NOTE - 11ma ended with 7.3.2.26, Table 30, Fig 80; 11e added 7.3.2.13 through 7.3.2.20 (which will become 27-34), tables 20.1 through 20.8 (30A-30H), and Figures 46.1 through 46.15 (80A-80O); 11k added 7.3.2.26 through 7.3.2.28 (which will become 35-37), 1 table (30I), 8 figures (80P-80W). So we tentatively start with 7.3.2.38, Table 30J, Figure 80X. Need to monitor 11k for changes that will affect this numbering.

Count Information Element

The Count Information Element specifies the number of Information Elements that are integrity-protected by their inclusion in the MIC in a subsequent EAPKIE. The value includes the Count IE itself, and all subsequent IEs up through and including the EAPKIE. The format for this information element is defined in Figure Error: Reference source not found:

NOTE - Text in proposal was “The Count IE specifies the number of IEs that follow the Count IE. When security is enabled, all IEs including and following this Count IE will be integrity protected, including EAPKIE”. Editor believes the replacement text is clearer and technically equivalent. Confirmation requested.

The Length field must be set to 0x03.

NOTE - Proposal section 4.4.1 says length must be 0x03. But 11ma section 7.3.2 states that length indicates the length of the information field, not the total length of the element. Confirmation of this change requested.

The Information Element Count value gives the number of IEs, including itself, up through and including the EAPKIE. Its minimum value is 0x02.

Fast Transition Resource Information Element (TRIE)

The Fast Transition Resource Information Element is defined to enable the advertisement of network infrastructure policy and resource reservation information. This can assist a TSTA in deciding which transition mechanism(s) to use. A TTAP can advertise the TRIE through a Beacon frame and through a Probe Response frame. This information element is defined in Figure Error: Reference source not found.

NOTE - Should this be “A TTAP” or “An AP”? Is it somehow different from TSIE and KeyHolderIE (which can be advertised by “an AP”)?

NOTE - Beacon and Probe Response frames are not yet defined. Need to keep the above paragraph consistent with their eventual definition.

NOTE - Proposal section 4.4.3 showed the “Fast Transition Resource Mechanism” as 3 octets, but the detailed figure showed the Mobility Domain as bytes 1-16. Editor believes that the length was intended to be 1+16, for total of 17.

The Length field must be set to 19 (0x13).

The Fast Transition Resource Mechanism value is as defined in Table Error: Reference source not found.

The Resource Mobility Domain is set to the unique identifier which specifies the AP’s over-the-DS reachability. All APs in a RMD have over-the-DS reachability to all other APs in that RMD.

The STA can use information from the TRIE to determine the transition mechanisms supported and recommended by the AP and infrastructure. The choice of executing any specific transition mechanism is left as discretion of the STA.

Fast Transition Security Information Element (TSIE)

The Fast Transition Security Information Element is defined to enable the advertisement of network infrastructure security policy and information. This can assist a TSTA in deciding which security mechanism(s) to use. An AP advertises this IE only when security during fast transitions is enabled on the network, through a Beacon frame and a Probe Response frame. This information element is defined in Figure Error: Reference source not found.

NOTE - Proposal section 4.4.4 shows SMD-ID as bytes 1-15, with byte 0 unassigned. Editor believes that SMD-ID was intended to be 16 octets, bytes 0-15. This would change the above table to be 16 octets for SMD-ID, and total length to 64 (0x40).

The Length field must be set to 63 (0x3F).

Fast Transition Key Holder Information Element

The Fast Transition Key Holder Information Element is defined to enable the advertisement of network infrastructure security policy and information, which can assist a TSTA to decide which security mechanism to use. An AP advertises this IE only when security during fast transitions is enabled on the network, through a Probe Response frame. This information element is defined in Figure Error: Reference source not found.

NOTE - Not in a Beacon? Is this intentional?

NOTE - Proposal section 4.4.5 makes no sense to me. Figure above needs major editing. Editor needs help!

Time Interval Information Element

This generic Time Interval Information Element is defined to specify various types of time intervals and timeouts. This information element is defined in Figure Error: Reference source not found.

The length field must contain a value 0x03.

The Interval type field must contain one of the values from Table Error: Reference source not found.

The Interval Value must be an unsigned 16-bit integer representing the number of TUs (milliseconds).

Resource Information Container Information Elements

The Resource Information Container (RIC) refers to a collection of Information Elements that are used to express a resource request in a resource request message. RIC is also used to convey responses to the corresponding responses.

Each atomic resource is contained in one Information Element. For example, a TSPEC is contained in an Information Element as described in section 7.3.2.29.

NOTE - above cross reference to 7.3.2.29 is to the section entitled “Traffic Specification (TSPEC) element”, contained in amendment 802.11e, and given there as section 7.3.2.15. It is anticipated that this will become 7.3.2.29 when 11e is merged into the complete 802.11 specification.

Two new IE types are defined by this section to introduce structure to the RIC. These are:

RIC_ROOT (RRIE): used as the RIC header

RIC_DATA (RDIE): used to describe each Resource Request. A simple Resource Request is an RDIE followed by single TSPEC.

The basic format of the Resource Information Container has one RRIE and a list of Resource Requests, as shown in Figure Error: Reference source not found.

Each Resource Request comprises an RDIE followed by one or more TSPECs. If there are multiple TSPECs then these are treated as "choices" by the target AP. The AP shall attempt to allocate the first TSPEC; if this fails the AP shall attempt to allocate the next TSPEC, and so on until a successful allocation or the AP reaches the end of the TSPEC list. Thus, an "OR" relationship exists between TSPECs that follow an RDIE.

An example of a RIC with two resource requests, each with a single TSPEC, is given in Figure Error: Reference source not found.

An example of a RIC with one resource request, with a choice of two TSPECs, is given in Figure Error: Reference source not found. This indicates that the target AP can select one of the two TSPECs

RIC Root Information Element (RRIE)

The Resource Information Container Root Information Element (RRIE) is defined in Figure Error: Reference source not found.

The length field must contain a value 0x08.

The RRIE Identifier contains a unique value intended to match responses to requests.

The Count of RDIEs contains the number of RDIEs in this RIC.

The Target BSSID is the BSSID to which this RIC IE is to be sent.

RIC Data Information Element (RDIE)

The Resource Information Container Data Information Element (RDIE) is defined in Figure Error: Reference source not found.

The length field must contain a value 0x02.

The Sequence Number is a unique value within this RIC, and chosen by the resource requestor. It is used in the response to refer to a specific resource allocation request.

The Control Options value is as defined in Table Error: Reference source not found

The Mandatory bit set to 1 indicates that one of the TSPECs that follow the RDIE must be allocatable/allocated for the resource request to be considered successful. If the Mandatory bit is set to 0, the AP should keep processing the rest of the request in RIC even if none of the TSPEC IEs following the RDIE is allocatable/allocated.

The Confirm bit is used only in response messages to indicate success or failure of the allocation. It is set to zero and ignored in request messages.

The Resource count indicates the number of TSPEC IEs that follow this RDIE.

Fast Transition 802.1X EAPOL-Key Information Element (EAPKIE)

This element is defined to encapsulate the Fast Transition 802.1X EAPOL-Key message. This element must be present in the 3rd and 4th Fast Transition Authentication messages, or in the Fast Transition Confirm and Fast Transition Acknowledgement action frames, and association or reassociation request and response when Fast Transition and Security are enabled. The format for this information element is defined in Figure Error: Reference source not found:

The Length must be set to the length of the EAPOL-Key message.

Action Frame Format Details

Insert new section 7.4.3 at end of section 7.4

Fast BSS Transition Action Details

Four Action frame formats are defined to support Fast BSS Transitions initiated through the currently associated AP (e.g. over the DS). The action field values associated with each frame format within the Fast BSS Transition category are defined in Table Error: Reference source not found:

NOTE - values in above table are different than the values in the Authentication frames. Perhaps they should be the same to minimize confusion of methods.

NOTE - last table in 11ma section 7.4 is 32; 11k adds one table (which they numbered as k13), which will eventually become 32A. So this table is tentatively set to 32B, which may change if 11k adds more tables

Fast Transition Request

The FT Request Action frame is used by a TSTA to trigger a pre-reservation request through the current associated BSSID.

NOTE - Last figure in 11ma section 7.4 is 85. 11k is still using Word, and numbered the figures in 7.4.2 as k29 up through k35. These will eventually be renamed as 85A through 85G. So our starting point is tentatively set to 85H, which will change if 11k adds more figures.

NOTE - Proposal sections 4.3.1.1 through 4.3.1.4 all show the EAPKIE as fixed length of 95 octets. But the EAPKIE includes a RSN IE. While the Authentication request and response messages removed the separate RSN ID because it is included in the EAPKIE, this does not. So two questions here: (1) Is it intended that the RSN ID be separate rather than included in the EAPKIE (and if separate, what should the RSNIE within the EAPKIE contain), and (2) How is the length of the EAPKIE fixed at 95 octets when the length of the RSNIE is variable? Editor is confused and needs guidance.

The frame body of a Fast Transition Request contains the information shown in Figure Error: Reference source not found.

The Category field shall be set to the value given in section 7.3.1.11 for Fast BSS Transition Action Frames.

The Action field shall be set to 0.

NOTE - to make consistent with Authentication frames (numbered 1-2-3-4), this value of Action field should be 1.

The TTAP field shall be set to the BSSID value of the target AP.

The Count IE specifies the number of IEs protected by the MIC in the EAPKIE (the Count IE itself, and succeeding IEs up through and including the EAPKIE).

The TRIE field shall be set to the Fast Transition Resource Information Element with the resource policy it is negotiating.

The TSIE field shall be set to the Fast Transition Security Information Element with the security policy it is negotiating.

The RSNIE field is only present when RSN is enabled and shall be set to the RSN IE as defined in section 7.3.2.25. When present, the RSN IE defines the security policy negotiated by the TSTA. In addition, the TSTA must provide the R2Name used to generate a fresh PTKSA.

The EAPKIE field is only present when RSN is enabled and the RSN IE is specified. The EAPKIE must provide a random SNonce in the KeyNonce field.

Fast Transition Response

The FT Response Action frame is transmitted by the current BSSID as a response to the TSTA’s Fast Transition Request. The frame body of the Fast Transition Response is as follows:.

NOTE - a Status code from this message is referenced in section 8A.4.2. It needs to be added.

The frame body of a Fast Transition Request contains the information shown in Figure Error: Reference source not found.

The Category field shall be set to the value given in section 7.3.1.11 for Fast BSS Transition Action Frames.

The Action field shall be set to 1.

NOTE - to make consistent with Authentication frames (numbered 1-2-3-4), this value of Action field should be 2.

The TTAP field shall be set to the BSSID value of the target AP.

The Count IE specifies the number of IEs protected by the MIC in the EAPKIE (the Count IE itself, and succeeding IEs up through and including the EAPKIE).

The Time Interval IE shall be set to the reassociation deadline time.

NOTE - proposal text was “Time interval information element used to convey either the reassociation deadline time.” Confirmation requested that the above text is accurate.

The TRIE field shall be set to the Fast Transition Resource Information Element with the resource policy it is negotiating.

The TSIE field shall be set to the Fast Transition Security Information Element with the security policy it is negotiating.

The RSNIE field is only present when RSN is enabled and shall be set to the RSN Information Element as defined in section 7.3.2.25. When present, the RSN Information Element defines the security policy advertised by the TTAP’s beacons. In addition, the TTAP must provide the R2Name used to generate a fresh PTKSA.

The EAPKIE field is only present when RSN is enabled and the RSN Information Element is specified. The EAPKIE must provide a random ANonce in the KeyNonce field and a KeyLifetime for the FT Key Hierarchy.

Fast Transition Confirm

The FT Confirm Action frame is used by the TSTA to confirm to the TTAP receipt of the ANonce and to initiate both the liveness of the PTKSA and, if required, to request QoS resources. The FT Confirm action frame body format is as follows:

The frame body of a Fast Transition Confirm contains the information shown in Figure Error: Reference source not found.

The Category field shall be set to the value given in section 7.3.1.11 for Fast BSS Transition Action Frames.

The Action field shall be set to 3.

NOTE - proposal section 4.3.1 said this value should be 2, but section 4.3.1.3 has value 3. Editor believes the sequence 1-2-3-4 (matching the Authentication frames) is preferred, and that this frame should have value 3.

The TTAP field shall be set to the BSSID value of the target AP.

The Count IE specifies the number of IEs protected by the MIC in the EAPKIE (the Count IE itself, and succeeding IEs up through and including the EAPKIE).

The TRIE field shall be set to the Fast Transition Resource Information Element with the resource policy it is negotiating.

The TSIE field shall be set to the Fast Transition Security Information Element with the security policy it is negotiating.

The RSNIE field is only present when RSN is enabled and shall be set to the RSN Information Element as defined in section 7.3.2.25. When present, the RSN Information Element defines the security policy specified in the FT Request frame body.

The RIC-Request field is present only if QoS resources are to be requested. This field shall be a sequence of RIC Information Elements (RRIE, followed by one or more RDIE(s)), as specified in section .

The EAPKIE field is only present when RSN is enabled and the RSN Information Element is specified. The EAPKIE must echo the ANonce in the KeyNonce field and authenticate all fields commencing with the Count IE and including the EAPKIE. The authentication tag shall be transmitted in the EAPOL-Key MIC field and use the same algorithms as specified in section 8.5.2.

Fast Transition Acknowledgement

The FT Acknowledgement Action frame is transmitted by the current BSSID as a response to the TSTA’s Fast Transition Confirm. The FT Acknowledgement action frame body format is as follows:.

NOTE - a Status code from this message is referenced in section 8A.4.2. It needs to be added.

The frame body of a Fast Transition Acknowledgement contains the information shown in Figure Error: Reference source not found.

The Category field shall be set to the value given in section 7.3.1.11 for Fast BSS Transition Action Frames.

The Action field shall be set to 4.

NOTE - proposal section 4.3.1 said this value should be 3, but section 4.3.1.4 has value 4. Editor believes the sequence 1-2-3-4 (matching Authentication frames) is preferred, and that the field value for FT ACK should be 4.

The TTAP field shall be set to the BSSID value of the target AP.

The Count IE specifies the number of Information Elements protected by the MIC in the EAPKIE (the Count Information Element itself, and succeeding Information Elements up through and including the EAPKIE).

The Time Interval Information Element shall contain the reassociation deadline time.

The TRIE field shall be set to the Fast Transition Resource Information Element with the resource policy it is negotiating.

The TSIE field shall be set to the Fast Transition Security Information Element with the security policy it is negotiating.

The RSNIE field is only present when RSN is enabled and shall be set to the RSN Information Element as defined in section 7.3.2.25. When present, the RSN Information Element defines the security policy specified in the FT Response frame body.

The RIC-Response field is present only if QoS resources are to be requested. This field shall be a sequence of RIC Information Elements (RRIE, followed by one or more RDIE(s)), as specified in section .

The EAPKIE field is only present when RSN is enabled and the RSN Information Element is specified. The EAPKIE must echo the SNonce in the KeyNonce field and authenticate all fields commencing with the Count Information Element and including the EAPKIE. The authentication tag shall be transmitted in the EAPOL-Key MIC field and use the same algorithms as specified in section 8.5.2.

Security

Insert new clause 8.5A after end of 8.5, renumbering the following clauses, figures, and tables as needed.

Key distribution for Fast BSS Transition

While the use of the key distribution mechanism defined in clause 8.5 can facilitate a BSS transition, the implication is that the TSTA must have established a fresh PMK with each AP prior to association. The imposition of establishing a PMK by means of a full IEEE 802.1X EAP authentication can often be a time expensive operation that would prohibit a fast BSS transition.

This clause describes a new key hierarchy and its supporting architecture that enables Fast BSS transitions while obviating the need to have the STA (and each AP it transitions to) execute multiple IEEE 802.1X EAP authentications. Note however that while this new key hierarchy enables the optimal BSS transition, to ensure backward compatibility all of the key distribution methods as defined by clause 8.5 are also supported.

Overview

To accommodate different 802.11 based network topologies, a four level key hierarchy is introduced. This ensures key separation between the potential key holders, which are the authorized agents to hold the keying material. The multi-layer key hierarchy expedites the distribution of keys between the network entities prior to, or during, a transition.

As shown in Figure Error: Reference source not found, each entity holding a key is identified by a 16 octet identifier. The first level of the hierarchy (R0KH-ID) generates the PMK-R0 from either the PSK or from the AAA-Key resulting (per RFC 3748) from a successful 802.1X EAP Authentication between the AS and the SPA.

NOTE - remember to fix all the occurrences of SPA to whatever they are supposed to be.

The lifetime of the keys is bound to the lifetime of the PSK or AAA-Key. For example, the RADIUS protocol communicates key lifetime through the Session-Timeout attribute. It is important to note that any keys derived from PMK-R0 cannot have a lifetime that is greater than its parent.

The FBT key hierarchy derives its keys using a new Key Derivation Function (KDF) as defined in Section  with separate labels to further distinguish derivations. Each level of the hierarchy derives keys for the level below it. The distribution of keys within the PMK key holders is outside the scope of this specification. However, to ensure security, this new key hierarchy requires that keys only be distributed down the key hierarchy using a secure transport protocol.

The FBT key hierarchy is shown in Figure Error: Reference source not found:

Fast BSS Transition Key Hierarchy

NOTE - something went wrong in transferring this figure into Frame. Need to go back to original source and fix.

As different entities, whether physical or logical, may hold keying material used to derive PTKs, the four level key hierarchy is enforced to ensure that compromise of such keying material is isolated to only that branch of the hierarchy. For example, the AAA server on which the top level key is established will be able to derive the keys used by every AP (in the hierarchy) and have the knowledge to decrypt any session on any AP. Thus, if the AAA server is compromised, then every key in the hierarchy is compromised and their resulting sessions must be deleted.

However, if the third level (R2KH in Figure Error: Reference source not found) is compromised, only its corresponding key PMK-R2 and the keys derived from that PMK-R2 are compromised; only the sessions on APs using the keys derived from that PMK-R2 are compromised and must be deleted. Furthermore, since the key hierarchy lacks freshness, they must never be derived again from higher layer keys. For ease of implementation, APs may delete the higher layer keys to simplify its session states.

The mechanisms for ensuring keys are deleted and not derived again after a compromise in the key hierarchy is outside the scope of this specification. However, to ensure that keys are not deleted without warrant, there should be some mechanism to ensure authorization for key distribution as well as key deletion.

Key Hierarchy

As shown in Figure Error: Reference source not found, the FBT key hierarchy consists of four levels whose keys are derived using a new KDF described in Section . The key derivation hierarchy is shown in Figure Error: Reference source not found below.

PMK-R0 - the top level of the FBT key hierarchy; this key is derived as a function of the MSK and SSID and stored by the PMK-R0 key holder, R0KH. This key is mutually derived by the SPA and the R0KH and named by the SSID, SPA and R0KH.

PMK-R1 - the second level of the key hierarchy; this key is mutually derived by the SPA and the R1 Key holder, R1KH. This key is named by the SPA, R0KH and the R1KH.

PMK-R2 - the third level of the key hierarchy; this is mutually derived by the SPA and the R2 key holder, R2KH. This key is named by the SPA, R0KH, R1KH and R2KH.

PTK - the last level of the key hierarchy that defines the 802.11 and 802.1X protection keys. The PTK is mutually derived by the SPA and the TTAP. This key is named by SNonce, ANonce, SPA, and BSSID.

NOTE - numbering of the levels was consistently made PMK-R1 at second level, PMK-R2 at third level. Editor requests confirmation of this intent.

NOTE - AA (presumably the AP MAC address, same as BSSID) is included in the formulas later for R2Name. Should be added to item #3 above.

NOTE - No formula is given later for a PTKName. Several refs later for a PTKName, but not clear if it is needed or used. Need to keep item #4 in list above consistent with definition of PTKName (or lack thereof).

While FBT defines a four level key hierarchy, the last level ultimately results in a PTK to be consumed in the same manner as defined by Section 8.5.

FBT Key Derivation Hierarchy

NOTE - PTK formula in 8.5A.8 has the string “PTK Key derivation” instead of “PTKey derivation”, and has AA followed by SPA. The figure above should be consistent with the text.

Key derivation function

A new key derivation function, KDF, is used to address sliding parameter vulnerabilities existing in the PRF function of 8.5.1.1. The KDF function is a variant of the PRF function, and defined as follows:

Algorithm kdf

Output = KDF-Length (K, label, Context) where

Input:		K, a 256 bit key derivation key

label, a string identifying the purpose of the keys derived using this KDF

Context, a bit string that provides context to identify the derived key

Length, the length of the derived key in bits

Output:		a Length-bit derived key

result = ""

iterations =  (Length+159)/160

do i = 1 to iterations

result = result || HMAC-SHA1(K, i || label || 0x00 || Context || Length)

od

return first Length bits of result, and securely delete all unused bits

In this algorithm, i and Length are encoded as 16 bit unsigned integers, represented using the bit ordering conventions of Clause 7.1.1.

PMK-R0

To ensure policy consistency in both security and QoS, the TSTA can only use the Fast BSS Transition mechanisms to APs that retain the same SSID, security policy and similar QoS capabilities as the AP with which the TSTA is currently associated. Thus, the top level FBT Key Hierarchy key, PMK-R0 binds the SPA, top level key holder (R0KH-ID) and SSID with the keying material resulting from the negotiated AKM (e.g. the AAA-Key for 802.1X EAP authentication, or PSK). The PMK-R0 is the top level 256 bit keying material used to derive the next level keys (PMK-R1s):

PMK-R0 = KDF-256(MSK, "R0 Key Derivation", SSID || SPA)

where

KDF-256 is the KDF function as defined in Section  used to generate a key of length 256 bits.

MSK is the Master Session Key from the 802.1X EAP authentication.

"R0 Key Derivation" is the literal string consisting of the sequence of letters 'R', '0', ' ', 'K', 'e', 'y', ' ', 'D', 'e', 'r', 'i', 'v', 'a', 't', 'i', 'o', and 'n' (no null terminator).

SSID is the service set identifier.

SPA is the STA’s MAC address.

The PMK-R0 is referenced and named as follows:

R0Name = SHA-256(PMK-R0 || "R0 Key Name" || SSID || R0KH-ID || SPA)

The entity that stores this key is typically the NAS that is identified by a 16 octet string referred to as the R0KH-ID. The R0KH-ID must be advertised by the TTAP in the beacons and probe responses.

Informative Note: the R0KH-ID may also be advertised by 11k, but its definition for this is out of scope for this specification.

PMK-R1

The second level key of the FBT Key Hierarchy, PMK-R1, is a 256 bit key used to derive the third level keys (PMK-R2s). The PMK-R1 binds the SPA, top and second level key holders:

PMK-R1 = KDF-256(PMK-R0, "R1 Key Derivation", R0KH-ID || R1KH-ID || SPA)

where

KDF-256 is the KDF function as defined in Section  used to generate a key of length 256 bits.

PMK-R0 is the top level key in the FBT Key Hierarchy.

"R1 Key Derivation" is the literal string consisting of the sequence of letters 'R', '1', ' ', 'K', 'e','y', ' ', 'D', 'e', 'r', 'i', 'v', 'a', 't', 'i', 'o', and 'n' (no null terminator).

R0KH-ID is the 16-octet identifier of the holder of PMK-R0.

R1KH-ID is the 16-octet identifier of the holder of PMK-R1.

SPA is the STA’s MAC address.

The PMK-R1 is referenced and named as follows:

R1Name = SHA-256(R0Name || R0KH-ID || R1KH-ID || SPA)

This level enables network topologies where a device is connected to a NAS and to one or many APs, and provides the facility of caching TSTA's keys. The entity that stores this key is identified by a 16 octet string referred to as the R1KH-ID. The R1KH-ID must be advertised by the TTAP in the beacons and probe responses.

Informative Note: the R1KH-ID may also be advertised by 11k, but its definition for this is out of scope for this specification.

PMK-R2

The third level key in the FBT Key Hierarchy, PMK-R2, is a 256 bit key used to derive the PTK. The PMK-R2 binds the SPA, top, second and third level key holders:

PMK-R2 = KDF-256(PMK-R1, "R2 Key Derivation", R0KH-ID || R1KH-ID || R2KH-ID || AA || SPA)

where

KDF-256 is the KDF function as defined in Section  used to generate a key of length 256 bits.

PMK-R1 is the second level key in the FBT Key Hierarchy.

"R2 Key Derivation" is the literal string consisting of the sequence of letters 'R', '2', ' ', 'K', 'e','y', ' ', 'D', 'e', 'r', 'i', 'v', 'a', 't', 'i', 'o', and 'n' (no null terminator).

R0KH-ID is the 16-octet identifier of the holder of PMK-R0.

R1KH-ID is the 16-octet identifier of the holder of PMK-R1.

R2KH-ID is the 16-octet identifier of the holder of PMK-R2.

AA is the AP’s MAC address.

SPA is the STA’s MAC address.

The PMK-R2 is referenced and named as follows:

R2Name =  SHA-256(R0Name || R0KH-ID || R1KH-ID || R2KH-ID || AA || SPA)

The entity that stores this key is the TTAP's 802.1X Authenticator and is identified by a 16 octet string referred to as the R2KH-ID. The R2KH-ID must be advertised by the TTAP in the beacons and probe responses.

Informative Note: the R2KH-ID may also be advertised by 11k, but its definition for this is out of scope for this specification.

PMK Distribution within a Security Mobility Domain

A mechanism has been defined for caching and distributing PMKs, using the above described logical key hierarchy. A TSTA first acquires PMK-R0 keys for use in fast transitioning ("first contact") via the Security Domain of the AP with which it associated. This Security Domain is part of an Security Mobility Domain, and this Security Mobility Domain may contain additional Security Domains. A TSTA may transition to any of the Security Domains within a Security Mobility Domain. The R1 Key Holders of the other Security Domains acquire PMK-R1 from the PMK-R0 Key Holder in the original Security Domain.

This logical hierarchy is illustrated in figure below.

Key Distribution within Security Mobility Domain

When the 802.1X AKM is used to establish keys, the PMK-R0 Key Holder acquires PMK-R0 from the AAA Client, which may be co-located with the PMK-R0 Key Holder or may be a separate device. When Pre-Shared Keys are used, the AAA Client is omitted and the PMK-R0 Key Holder uses an alternate means of acquiring PMK-R0 as a derivation of the PSK.

Each PMK-R0 Key Holder is responsible for deriving PMK-R1 and transmitting it to the appropriate PMK-R1 Key Holder in the Security Domain with which the station is associated. This may be the Security Domain of which it is a member, or it may be a different Security Domain.

Each PMK-R1 Key Holder is responsible for deriving PMK-R2 and transmitting it to the appropriate PMK-R2 Key Holder in its own Security Domain.

Each PMK-R2 Key Holder is responsible for conducting the negotiation with the station to derive the PTK.

PTK

The fourth level of the key hierarchy is the PTK. This key is mutually derived by the SPA and the TTAP with the key length being a function of the negotiated cipher suites as defined by 802.11i. Using the KDF construction defined in Section , the PTK derivation is as follows:

PTK = KDF-PTKLen(PMK-R2, "PTK Key derivation", SNonce || ANonce || R0KH-ID || R1KH-ID || R2KH-ID || AA || SPA)

where

KDF-PTKLen is the KDF function as defined in Section  used to generate a PTK of length PTKLen.

PMK-R2 is the key that is shared between the Supplicant and the Authenticator

"PTK Key derivation" is the literal string consisting of the sequence of letters 'P', 'T', 'K', ' ', 'K', 'e', 'y', ' ', 'd', 'e', 'r', 'i', 'v', 'a', 't', 'i', 'o', and 'n' (no null terminator).

SNonce is a 256 bit random bit string contributed by the Supplicant

ANonce is a 256 bit random string contributed by the Authenticator

SPA is the Supplicant’s (STA’s) MAC address

AA is the Authenticator’s (AP's) MAC Address (BSSID).

PTKlen is the total number of bits to derive, e.g. number of bits of the PTK. The length is dependent on the negotiated cipher suites as defined by 802.11i

NOTE - need a ref to somewhere in the spec, rather than just to 802.11i

NOTE - later sections (e.g. 8A.4.1, 8A.4.2) refer to a PTKName, which is not defined. Should PTKName be defined?

Fast Transition Authentication frame sequence

NOTE - the frame sequence described in this section is mapped into several different message sets. (1) 802.11 Authentication/Reassociation messages for the base mechanism, (2) 802.11 Authentication messages for over-the-air with reservation, (3) Fast Transition Action messages for over-the-DS with reservation. Could possibly also be (4) Fast Transition Action followed by Reassociation messages for a mixed over-the-DS/over-the-air transition. So the statement that they are all “Message Type: Management” and “Message Subtype: Authentication” seems overly restrictive. Some more general wording is needed for the frame descriptions in this section. Editor suggests dropping the “Message Type: Management” and “Message Subtype: Authentication” from the descriptions, and making these procedures based on a sequence of IEs (e.g. Count, TRIE, TSIE, TIE, RIC, others, EAPKIE), which can then be placed in any of the message sequences.

The FT Authentication frame sequence is invoked to initiate a Fast BSS Transition. In a RSNA enabled negotiation, the first two FT Authentication frames are used to allow each TSTA and TTAP to provide their random contributions SNonce and ANonce respectively. These values must be random and are used to generate the PTK.    The first two message sequences are used to enable the TTAP to provision the PMK-R2 and for the TSTA and TTAP to compute the PTK. As the PTKSA must be established to protect the resource reservation in a RSNA enabled TSTA, no resources may be requested in the first two FT Authentication frames. If no RSNA is negotiated between the TSTA and TTAP, these first two frames are the equivalent of the Open Authentication sequence.

The third and fourth FT Authentication frames are used to prove liveness of the PTK and to enable authenticated resource reservation. The FT Authentication frame sequence is always initiated by the TSTA and responded by the TTAP.

FT Authentication frame: 1st frame

The first frame of the FT Authentication frame sequence is used by the TSTA to initiate a Fast BSS Transition. When RSNA is enabled, the TSTA shall, in the TSIE, include the corresponding key names of the key hierarchy it is using to generate the PTK. By including the key names and their respective key holders, the TTAP can use this information to ensure it has the corresponding PMK-R2 or request for the appropriate PMK-R2. Additionally, the TSTA shall also include the SNonce as its random contribution to ensure the derivation of a fresh PTK. Using the conventions of Clause 8:

Message Type: Management

Message Subtype: Authentication

Information Items:

Station Identity Assertion (in the SA field of the header)

Target AP Identity Assertion (in the DA field of the header)

Authentication Algorithm Identification = "Fast Transition"

Authentication Transaction Sequence number = 1

Authentication Algorithm dependent information:

TSIE must be included to negotiate the Fast Transition Capabilities

If RSNA is enabled the EAPKIE must be included with the SNonce and RSNIE values filled in, all other fields shall be zero.

If RSNA is enabled the RSNIE must include a PMKID count of 1 and the R2Name in the PMKID field.

Direction of message: From TSTA to TTAP

FT Authentication frame: 2nd frame

The second frame of the FT Authentication frame sequence is used by the TTAP to respond to the requesting TSTA. When RSNA is enabled, the TTAP shall also, in the TSIE, echo the key holders and key names used to generate the PTK. Additionally it shall also include the ANonce.

The response shall also include a status and when RSNA is enabled, it may return with a key lifetime (for the FT key hierarchy) and a (re)association deadline. The reassociation deadline is the time allotted for the TSTA to initiate reassociation. Using the conventions of Clause 8:

Message Type: Management

Message Subtype: Authentication

Information Items:

Station Identity Assertion (in the SA field of the header)

Target AP Identity Assertion (in the DA field of the header)

Authentication Algorithm Identification = "Fast Transition"

Authentication Transaction Sequence number = 2

Authentication Algorithm dependent information:

TSIE must be included in response to the Fast Transition Capabilities request

If RSNA is enabled the EAPKIE must be included with the ANonce and RSNIE values filled in, all other fields shall be zero.

If RSNA is enabled the RSNIE must include a PMKID count of 1 and the R2Name in the PMKID field.

If RSNA is enabled, FT Key lifetime must be included

(Re)association Deadline

Status

Direction of message: From TTAP to TSTA

FT Authentication frame: 3rd frame

The third frame of the FT Authentication frame sequence is used by the TSTA to assert, in a RSNA-enabled negotiation that it has a valid PTK and to authenticate a resource request. If no resources are required, then it may omit inclusion of the TSPEC IEs. Similarly, if no RSNA is enabled, the RSNIE and MIC values are omitted. Using the conventions of Clause 8:

Message Type: Management

Message Subtype: Authentication

Information Items:

Station Identity Assertion (in the SA field of the header)

Target AP Identity Assertion (in the DA field of the header)

Authentication Algorithm Identification = "Fast Transition"

Authentication Transaction Sequence number = 3

Authentication Algorithm dependent information:

TRIE and TSIE must be included to confirm to the Fast Transition Capabilities

If RSNA is enabled the EAPKIE must be included with the ANonce and RSNIE values filled in, all other fields shall be zero. The MIC shall protect the contents of the FT Authentication frame starting with the IE count up to and including the EAPKIE.    Additionally, the following bits shall be enabled: security, MIC, A (response is required) and Install.

If RSNA is enabled the RSNIE must include a PMKID count of 1 and the R2Name in the PMKID field.

If resources are being requested, the RIC resource request shall be included

Direction of message: From TSTA to TTAP

FT Authentication frame: 4th frame

The fourth frame of the FT Authentication frame sequence is used by the TTAP to respond to the requesting TSTA. The fourth message serves as final confirmation of both liveness of the PTK to the TSTA and resource availability. Note however that the EAPOL-IE may be absent if RSNA is disabled; similarly the RIC-IE will be absent if no resources are requested.

The response shall also include a status and when RSNA is enabled, it may return with a key lifetime (for the FT key hierarchy) and a (re)association deadline. The reassociation deadline is the time allotted for the TSTA to initiate reassociation. Using the conventions of Clause 8:

Message Type: Management

Message Subtype: Authentication

Information Items:

Station Identity Assertion (in the SA field of the header)

Target AP Identity Assertion (in the DA field of the header)

Authentication Algorithm Identification = "Fast Transition"

Authentication Transaction Sequence number = 4

Authentication Algorithm dependent information:

TRIE and TSIE must be included to acknowledge the Fast Transition Capabilities

If RSNA is enabled the EAPKIE must be included with the SNonce, RSNIE values filled in, all other fields shall be zero. The MIC shall protect the contents of the FT Authentication frame starting with the IE count up to and including the EAPKIE.    Additionally, the following bits shall be enabled: security, MIC and Install.

If RSNA is enabled the RSNIE must include a PMKID count of 1 and the R2Name in the PMKID field.

If RSNA is enabled, FT Key lifetime must be included in the Time Interval IE

If resources are being requested, the RIC resource response shall be included along with (Re)association Deadline

Status

Direction of message: From TTAP to TSTA

Insert a new section following Section 8:

Fast BSS Transition

Overview of transition mechanisms

NOTE - Text from proposal section 3 and section 5 is here; is 8A a reasonable spot? Possibly better as 9A? 11A?

NOTE - This section really hits the ground running. It needs some introduction text. Contributions solicited.

With the reservation mechanisms being policy driven, it is feasible to have deployments that do not have pre-reservation. Thus, two Fast BSS Transitions are defined:

Base Fast BSS Transition: this mechanism is executed when a TSTA must transition to a TTAP and does not require a reservation prior to its transition.

Pre-reservation Fast BSS Transition: this mechanism is executed when a TSTA needs assurances that the required security and QoS resources be available prior to a transition.

APs must advertise both their capabilities and policies for supporting the Fast BSS Transition mechanisms.

Capability Advertisement

Fast Transition capability will be advertised in the Beacon and Probe Response frames. Separate Fast Transition Information Elements are defined, one each for resource and security. The resource and security Information Elements are present only if QoS and security, respectively, are enabled on the network.

NOTE - original text first sentence was “Fast Transition capability will be advertised in the extended capability bit Information Element”. That doesn’t seem to be defined in the proposal. It seems that the presence of TRIE and TSIE advertise fast transition capability. Editor requests confirmation.

Base Mechanism

Systems capable of Fast BSS Transition must support the Fast BSS Transition Base Mechanism. The base mechanism enables TSTAs to transition to a TTAP in the event that it must transition without expending costs for invoking a reservation or in the event that the deployment policy only enforces the base mechanism.

The Fast BSS Transition (FBT) base mechanism optimizes the number of exchanges required to establish either the PTKSA or the QoS resources. Information elements are included to allow for the PTKSA and QoS resources provisionment. The base mechanism is shown in Figure Error: Reference source not found:

Fast BSS Transition: Base Mechanism

NOTE - Reassociation Response should contain SNonce, not ANonce, according to 7.2.3.7. Confirmation requested.

NOTE - CIE (presumed to mean Count IE) hardly needs a subscript. How about just “Count”?

NOTE - ordering of the IEs in the sequence of messages should be consistent.

As shown in Figure Error: Reference source not found, the FBT base mechanism defines a new Authentication algorithm, FT. This enables the TSTA and TTAP to specify the PTKSA to be established, and provides the respective SNonce and ANonce to precompute the PTK. This occurs prior to the TTAP committing QoS resources. The PTKSA is established to enable the integrity protection of the QoS resources specified in the (re)association exchange.

Note that the PTKSA request and response as well as the SNonce and ANonce triggers may be conducted either over-the-air as shown in Figure Error: Reference source not found, or over-the-DS using the FBT Action Frame.

Pre-Reservation Mechanism

Networks capable of Fast BSS Transition may allow TSTA's to pre-reserve PTKSAs and QoS resources prior to (re)association. The FBT pre-reservation mechanism decouples the allocation of the security and QoS resources from the TSTA's critical path, e.g. (re)association.   The decoupling allows TSTAs better assurances of meeting the Fast BSS Transition time.

The Resource Information Container (RIC) expresses resource requirements and various combinations of the resource requests. A RIC is essentially a collection of Information Elements that express the number of resources, the specification of the resource requirements and any relationship between these resource requirements.

Similar to the FBT Base Mechanism, the FBT pre-reservation mechanism uses the FBT 802.11 Authentication algorithm to enable the pre-reservation of both the security and QoS resources. Figure Error: Reference source not found demonstrates the message flow for a TSTA employing the FBT Pre-Reservation mechanism using the FBT 802.11 Authentication mechanism.

Fast BSS Transition using Pre-reservation over the air

NOTE - Authentication ACK should contain SNonce, not ANonce, according to 8.5A.9.4. Confirmation requested.

NOTE - Reassociation Response should contain SNonce, not ANonce, according to 7.2.3.7. Confirmation requested.

NOTE - CIE (presumed to mean Count IE) hardly needs a subscript. How about just “Count”?

NOTE - ordering of the IEs in the sequence of messages should be consistent.

To further optimize potential delays and latencies incurred in channel switching and messaging over the air, FBT Pre-Reservation can also be invoked using the current AP to relay the pre-reservation exchange to the TTAP. The figure below depicts the message flow for the FBT Pre-Reservation mechanism using the current AP.

Fast BSS Transition using Pre-Reservation over the wire

NOTE - FT Action ACK should contain SNonce, not ANonce, according to 7.4.3.4. Confirmation requested.

NOTE - Reassociation Response should contain SNonce, not ANonce, according to 7.2.3.7. Confirmation requested.

NOTE - CIE (presumed to mean Count IE) hardly needs a subscript. How about just “Count”?

NOTE - ordering of the IEs in the sequence of messages should be consistent.

NOTE - Why is FT Action Response a “EAPKeyIE”, while the others are EAPKIE? I’d assume EAPKIE. Confirmation requested.

NOTE - FT Action ACK has a RIC-ID-Response, instead of RIC-IE-Response. Remember to fix.

Notation

The following notation is used throughout the remainder of this clause to represent management frames of subtype Authentication:

Auth-FT(Alg, Seq, SC, Data)

where

Alg	is the Authentication Algorithm Number field representing FT.

Seq	is the Authentication Transaction Sequence Number field.

SC	is the Status Code field.

Data	is additional data carried in the payload of the message.

The Fast BSS Transition extends the EAPOL-Key frames, as defined in Section 8. The extension allows the EAPOL-Key frames to convey and authenticate information about the security and resource contexts. The new information about security and resource contexts is conveyed in the Key Data portion of the EAPOL Key frame. The following notation is used throughout the remainder of this clause to represent this updated EAPOL-Key frames:

EAPOL-Key-FT(S, M, A, I, K, KeyRSC, ANonce/SNonce, MIC, RSNIE, GTK, N, KeyName, TSIE, TRIE, KeyLifetime)

where the parameters are as defined in section 8.5.2.2, with the following additions:

KeyName	is the name of the Key being referenced by this exchange e.g. R2Name.  This is in the RSNIE PMKID field.

TSIE	is the Fast BSS Transition Security Information Element. This is in the Key Data field.

TRIE	is the Fast BSS Transition Resource Information Element. This is in the Key Data field.

KeyLifetime	is the lifetime of the TGr Key Hierarchy.  That is, it is the lifetime of all the PMK keys, PMK-R0, PMK-R1 and PMK-R2.  This is present in the Key Data field.  This field is represented as the Time Interval IE (TIE) of type KeyLifetime.

When the EAPOL-Key frame is encapsulated as an EAPKIE, then there may be other associated data that is protected by the MIC inside the EAPOL-Key frame.

NOTE - Not clear what the difference is between EAPOL-Key (of 8.5.2.2) and EAPOL-Key-FT (here). If thre is no difference, change all of the occurances of “EAPOL-Key-FT” to “EAPOL-Key”.

First Contact

The Fast Transition mechanisms optimize the transitions across an ESS under many different conditions. In RSN enabled networks a new key hierarchy is defined to reduce the number of transactions required to establish a fresh PTKSA. In QoS enabled networks a pre-reservation mechanism is defined to reduce the time required to establish QoS attributes for media streams. To obtain the full potential of Fast Transition mechanisms, a TSTA must enable the Fast Transition procedures during (re)association. This enabling is referred to as first contact to ensure that all potential TTAPs to which a TSTA may transition will affect the appropriate policies and store relevant information to enable Fast Transition.

Thus, on first contact, the TSTA asserts both the security and QoS policies it will employ throughout the duration of the session. This is achieved using the same message flows as defined by the 802.11-2003 specification and including 802.11i when RSN is enabled and 802.11e when QoS is enabled.   The only distinction to affect an FT session is to assert the use of FT by including the TRIE and TSIE in the (re)association negotiation. The message flow is depicted in Figure Error: Reference source not found.

NOTE - Need to remove refs to 802.11-2003, 802.11i and 802.11e, since they are all part of the same spec that this will be joining. What is proper wording to do this? 802.11i could probably be replaced by a ref to somewhere in section 8. What is equivalent ref for 11e? What is equivalent ref for 802.11-2003? Editor needs some help.

First Contact to enable TSTA’s Fast BSS Transition

As shown in Figure Error: Reference source not found, on first contact with the ESS within which the TSTA intends to use Fast BSS Transitions, the packet exchange between the TSTA and the TTAP is similar to that defined by 802.11-2003, 802.11i and 802.11e. In particular, with RSN-enabled, the TSTA first performs an 802.11 Authentication using the Open System Authentication Algorithm. Upon successful 802.11 open authentication, TSTA then sends a (re)association request to the TTAP and includes the TRIE and TSIE to assert the use of Fast BSS Transitions for future transitions. Additionally, it includes its security capabilities in the RSN Information Element. The R2Name is set to 0's (zeroes) as no PMKSA has been negotiated yet.

TSTA->TTAP:	Auth-FT (0, 0, 0, 0)

TTAP->TSTA:	Auth-FT (0, 1, SC, 0)

If the SC returned by the TTAP is 0, indicating success, the TSTA continues with (Re)Association:

TSTA->TTAP:	(Re)Association Request (TRIESTA, TSIESTA, RSNIESTA)

TTAP->TSTA:	(Re)Association Response ( )

On successful (re)association, the SPA on the TSTA and the 802.1X Authenticator on the TTAP then proceed with an 802.1X EAP authentication. The 802.1X EAP exchange is sent between the TSTA and TTAP using EAPOL messages carried in 802.11 data frames.

Upon successful completion of the 802.1X EAP authentication, the 802.1X Authenticator receives the required information to define its PMKSA:

TTAP's BSSID,

PMK-R2,

R0Name,

R1Name,

R2Name,

R0KH-ID,

R1KH-ID,

R2KH-ID,

SPA, and

FT KeyLifetime

While the TTAP's 802.1X Authenticator receives most of the information from its parent (R2KH-ID), the TSTA holds all the information to derive the PMKSA directly.    Thus, following a successful 802.1X EAP authentication, the TTAP and TSTA then perform a FT 4-way handshake similar to the 802.11i handshake. The EAPOL-Key-FT frame notation is defined in Section  and 8.5.2.2.

TTAP->TSTA:	Data(EAPOL-Key-FT(0, 0, 1, 0, P, 0, ANonce, 0, RSNIEAP, 0, 0, R2Name, 0, 0, KeyLifetime))

TSTA->TTAP:	Data(EAPOL-Key-FT(0, 1, 0, 0, P, 0, SNonce, MIC, RSNIESTA, 0, 0, R2Name, TRIESTA, TSIESTA, 0))

TTAP->TSTA:	Data(EAPOL-Key-FT(1, 1, 1, 1, P, 0, ANonce, MIC, RSNIEAP, GTK, N, R2Name, TRIEAP, TSIEAP, TIE[KeyLifetime]))

TSTA->TTAP:	Data(EAPOL-Key-FT(1, 1, 0, 0, P, 0, 0, MIC, 0, 0, 0, 0, 0, 0, 0))

The "Data()" indicates the message is an 802.11 data frame.

As in 802.11i, the EAPOL-Key messages are encapsulated in 802.11 Data frames. On a successful 802.1X EAPOL-Key 4-way handshake, the 802.1X port is opened on both the TSTA and the TTAP, data connectivity has been established, and the TSTA can perform negotiation of QoS or other features if desired.

On successful completion of the 4-way handshake the Key Replay Counter shall be initialized to zero and the subsequent EAPOL-Key-FT frames shall use the Key Replay Counter to ensure they are not replayed.

Base Mechanism

The Fast BSS Transition base mechanism commences when a TSTA has determined its TTAP; that is, it has completed all the discovery and selection criteria for the TSTA to transition to the TTAP.

When a TSTA wishes to move from it's Current AP (CAP) to a Transition Target AP (TTAP) utilizing a Fast BSS Transition without Reservation, the TSTA exchanges nonces with the TTAP and establishes a PTKSA prior to a (re)association exchange.

When a TSTA wishes to perform an Over-the-Air Fast BSS Transition without Reservation to a TTAP, the TSTA and TTAP perform the following exchange:

TSTA->TTAP:	Auth-FT(FT, 0, SC, TRIESTA, TSIESTA, RSNIESTA, EAPOL-Key-FT(0, 0, 1, 0, P, 0, SNonce, 0, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Auth-FT(FT, 1, SC, TRIEAP, TSIEAP, RSNIEAP, EAPOL-Key-FT(0, 0, 0, 0, P, 0, ANonce, 0, 0, 0, 0, 0, 0, 0, TIE[KeyLifetime]))

NOTE - original text had “SC” in the first TSTA->TTAP message. There is no status code in the first message of the sequence. Editor suggests changing to “0” in first message, and requests confirmation.

If the SC returned by the TTAP is 0, indicating success, the TSTA continues with (Re)Association.

On successful FT Authentication, the TSTA completes State 1 and proceeds to State 2. Note that while the TSTA is 802.11 Authenticated, the 802.1X port remains blocked. This behavior is consistent to that defined by 802.11i.

If the TTAP does not have the key identified by R2Name, it attempts to retrieve that key from the "infrastructure". The method by which is retrieves the key is beyond the scope of this specification.

The TSTA and the TTAP compute the PTK and PTKName using PMK-R2, R2Name, and the exchanged nonces as specified in Section .

NOTE - There is no definition of PTKName. If it is needed, then section 8.5A.8 needs a definition. Editor requests guidance.

The TSTA must send a Reassociation Request to the TTAP within the Time Interval IE's Reassociation Deadline specified in milliseconds or the TTAP will delete the PTKSA.

TSTA->TTAP:	Reassociation Request(TRIESTA, TSIESTA, RSNIESTA, RIC-Request, EAPKIE(0, 1, 1, 1, P, 0, ANonce, MIC, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Reassociation Response(TRIEAP, TSIEAP, RSNIEAP, RIC-Response, EAPKIE(1, 1, 0, 1, P, 0, SNonce, MIC, 0, GTK, N, 0, 0, 0, TIE[KeyLifetime]))

The TSTA must echo the FT and security policies in the TSIESTA and RSNIESTA as it specified in the FT 802.11 Authentication Request.   In the same reassociation request, the TSTA may request for resources by including a RIC-Request. To authenticate the request, an encapsulated EAPOL-Key frame (EAPKIE) is included that echoes the TTAP's ANonce and authentication tag in the respective Key Nonce and MIC fields. Finally, the TTAP specifies the FT Key Hierarchy lifetime as part of the Key Data field.

At this point, the 802.1X port is opened on both the TSTA and the TTAP and data connectivity with QoS has been established.

Pre-Reservation Mechanism

When an TSTA wishes to move from it's Current AP (CAP) to a Transition Target AP (TTAP) utilizing a Fast BSS Transition with Reservation, the TSTA establishes a PTK with the TTAP and reserves resources using one of two methods:

"Over-the-Air". In this case, the TSTA communicates directly with the TTAP using 802.11 Authentication frames with the Authentication Algorithm set to Fast BSS Transition.

"Over-the-DS". In this case, the TSTA communicates with the TTAP via the CAP. The communication between the TSTA and the TTAP are carried in Fast BSS Transition Action frames between the TSTA and the CAP, and between the CAP and TTAP via an encapsulation method beyond the scope of this specification. The CAP converts between the two encapsulations.

NOTE - original text said encapsulation method was TBD. Is it intended to be defined here? Editor assumed not, and requests confirmation.

In either mechanism it is feasible for a TSTA to invoke multiple Pre-reservations but only commit to one TTAP. That is, it may invoke the pre-reservation mechanism as defined in this section with more than one BSSID but only invoke the (re)association exchange with a single TTAP.

Over-the-Air Fast BSS Transition with Reservation

When a TSTA wishes to perform an Over-the-Air Fast BSS Transition with Reservation to a TTAP, the TSTA and TTAP perform the following exchange:

TSTA->TTAP:	Auth-FT(FT, 0, SC, TRIESTA, TSIESTA, RSNIESTA, EAPOL-Key-FT(0, 0, 1, 0, P, 0, SNonce, 0, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Auth-FT(FT, 1, SC, TRIEAP, TSIEAP, RSNIEAP, TIE[ReassocDeadline], EAPOL-Key-FT(0, 0, 0, 0, P, 0, ANonce, 0, 0, 0, 0, 0, 0, 0, TIE[KeyLifetime]))

NOTE - original text had “SC” in the first TSTA->TTAP message. There is no status code in the first message of the sequence. Editor suggests changing to “0” in first message, and requests confirmation.

If the SC returned by the TTAP is 0, indicating success, the TSTA continues with the next exchange in the FT Authentication. Class 2 frames are not yet allowed as the authentication exchange has not been completed.

If the TTAP does not have the key identified by R2Name, it attempts to retrieve that key from the "infrastructure". The method by which is retrieves the key is beyond the scope of this specification.

The TSTA and the TTAP compute the PTK and PTKName using PMK-R2, R2Name, and the exchanged nonces as specified in Section .

NOTE - there is no definition of PTKName in 8.5A.8. If it is needed, then section 8.5A.8 needs a definition. Editor requests guidance.

The TSTA must complete the remainder of the FT Authentication exchange and send a Reassociation Request to the TTAP within Reassociation Deadline milliseconds or the TTAP will delete the PTKSA and release any reserved resources.

TSTA->TTAP:	Auth-FT(FT, 2, 0, TRIESTA, TSIESTA, RSNIESTA, RIC-Request, EAPKIE(0, 1, 1, 1, P, 0, ANonce, MIC, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Auth-FT(FT, 3, SC, TRIEAP, TSIEAP, RSNIEAP, RIC-Response, TIE[ReassocDeadline], EAPKIE(1, 1, 0, 1, P, 0, SNonce, MIC, 0, 0, 0, 0, 0, 0, TIE[KeyLifetime]))

Note - original text did not include the status code field in these messages. Added by editor. Editor requests confirmation.

On successful completion of the FT Authentication exchange for pre-reservation, the PTKSA has been established and proven live. Thus, the Key Replay Counter shall be initialized to zero and the subsequent FT EAPOL-Key frames shall use the Key Replay Counter to ensure they are not replayed.

Similarly, on successful completion of the FT Authentication exchange for pre-reservation, both the PTKSA and QoS resources are guaranteed to be reserved until the Reassociation Deadline.

If the SC returned by the TTAP is 0, indicating success, the 802.1X port remains blocked and Class 1 & 2 frames are now allowed between the TSTA and the TTAP, and the TSTA continues with (Re)Association:

TSTA->TTAP:	Reassociation Request(TRIESTA, TSIESTA, RSNIESTA, RIC-Request, EAPKIE(0, 1, 1, 1, P, 0, ANonce, MIC, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Reassociation Response(TRIEAP, TSIEAP, RSNIEAP, RIC-Response, EAPKIE(1, 1, 0, 1, P, 0, SNonce, MIC, 0, GTK, N, 0, 0, 0, TIE[KeyLifetime]))

The TSTA must echo the FT and security policies in the TSIESTA and RSNIESTA as it specified in the FT 802.11 Authentication Confirm.   In the same reassociation request, the TSTA asserts the reserved resources by including a RIC-Request as provided in the FT Authentication Confirm. To authenticate the request, an encapsulated EAPOL-Key frame (EAPKIE) is included that echoes the TTAP's ANonce and authentication tag in the respective Key Nonce and MIC fields. Finally, the TTAP specifies the FT Key Hierarchy lifetime as part of the Key Data field.

At this point, the 802.1X port is opened on both the TSTA and the TTAP and data connectivity with QoS has been established.

Over-the-DS Fast BSS Transition with Reservation

When a TSTA wishes to perform an Over-the-DS Fast BSS Transition with Reservation to a TTAP, the TSTA and TTAP perform the following exchange via the CAP:

TSTA->CAP:	FT Action Request (TTAP, TRIESTA, TSIESTA, RSNIESTA, EAPOL-Key-FT(0, 0, 1, 0, P, 0, SNonce, 0, 0, 0, 0, 0, 0, 0, 0))

CAP->TSTA:	FT Action Response (TTAP, TRIEAP, TSIEAP, RSNIEAP, TIE[ReassocDeadline], EAPOL-Key-FT(0, 0, 0, 0, P, 0, ANonce, 0, 0, 0, 0, 0, 0, 0, TIE[KeyLifetime]))

TSTA->CAP:	FT Action Confirm (TTAP, TRIESTA, TSIESTA, RSNIESTA, RIC-Request, TIE[ReassocDeadline], EAPKIE(0, 1, 1, 1, P, 0, ANonce, MIC, 0, 0, 0, 0, 0, 0, 0))

CAP->TSTA:	FT Action Acknowledgement (TTAP, TRIEAP, TSIEAP, RSNIEAP, RIC-Response, TIE[ReassocDeadline], EAPKIE(1, 1, 0, 1, P, 0, SNonce, MIC, 0, 0, 0, TIE[KeyLifetime]))

If the TTAP does not have the key identified by R2Name, it attempts to retrieve that key from the "infrastructure" via method beyond the scope of this specification.

NOTE - original text had “TBD” above. But in the section on over-the-air reservations, the original text said “beyond the scope of this specification”. Editor requests confirmation of this change.

The TSTA and the TTAP compute the PTK and PTKName using PMK-R2, R2Name, and the exchanged nonces as specified in Section .

NOTE - There is no definition of PTKName in 8.5A.8. If it is needed, then section 8.5A.8 needs a definition. Editor requests guidance.

On successful completion of the FT Action frame exchange for pre-reservation, the PTKSA has been established and proven live. Thus, the Key Replay Counter shall be initialized to zero and the subsequent FT EAPOL-Key frames shall use the Key Replay Counter to ensure they are not replayed.

Similarly, on successful completion of the FT Authentication exchange for pre-reservation, both the PTKSA and QoS resources are guaranteed to be reserved until the Reassociation Deadline.

NOTE - following paragraph mentions a status code from the FT Action frame. But there is no Status Code field in those frames. Editor requests clarification or a contribution.

If the SC returned by the TTAP is 0, indicating success, the 802.1X port remains blocked and Class 1 & 2 frames are now allowed between the TSTA and the TTAP, and the TSTA continues with (Re)Association:

TSTA->TTAP:	Reassociation Request(TRIESTA, TSIESTA, RSNIESTA, RIC-Request, EAPKIE(0, 1, 1, 1, P, 1, ANonce, MIC, 0, 0, 0, 0, 0, 0, 0))

TTAP->TSTA:	Reassociation Response(TRIEAP, TSIEAP, RSNIEAP, RIC-Response, EAPKIE(1, 1, 0, 1, P, 1, SNonce, MIC, 0, GTK, N, 0, 0, 0, TIE[KeyLifetime]))

The TSTA must echo the FT and security policies in the TSIESTA and RSNIESTA as it specified in the FT 802.11 Authentication Confirm.   In the same reassociation request, the TSTA asserts the reserved resources by including a RIC-Request as provided in the FT Authentication Confirm. To authenticate the request, an encapsulated EAPOL-Key frame (EAPKIE) is included that echoes the TTAP's ANonce and authentication tag in the respective Key Nonce and MIC fields. Finally, the TTAP specifies the FT Key Hierarchy lifetime as part of the Key Data field.

NOTE - original text said an echo of the TTAP’s “authentication tag” would be in the MIC field. Is the MIC in this frame not really a MIC? Editor requests guidance.

At this point, the 802.1X port is opened on both the TSTA and the TTAP and data connectivity with QoS has been established.

QoS Procedures

Constructing a RIC for Request

To reserve resources the station must create a Resource Information Container (RIC) and insert it in an appropriate request message. The request message is then sent to the target AP, either directly (over-the-air), or via the Current AP (over-the-DS).

A RIC construction would be done at the time of (re)-association. If pre-reservation is used, a RIC construction would be done at the time of pre-reservation, and a second RIC construction would be done at the time of (re)-association.

To construct a RIC:

the STA considers the set of resources that the STA requires at the next AP. This set will structure the resource entities, such as TSPEC IEs, to be placed in the RIC, within RDIEs.

Each RDIE may contain one or more individual resource entities, such as TSPECs. The RDIE mandatory bit, when set to 1, indicates that one of the TSPECs within the RDIE must be allocated for the resource reservation to be successful. The TSTA may not be able to transition when its RDIEs with mandatory bit set to 1 are not accepted.

When multiple resource entities, like TSPECs, are inserted within the same RDIE, then this indicates that a logical "OR" relationship exists between those resource entities (TSPECs) within the same RDIE. Only one of these TSPECs, within an RDIE, need to be accepted.

RIC processing by the target AP

When a Resource Information Container appears in a Resource Request, the AP shall attempt to allocate or check for allocatibility of one resource for each RDIE in the RIC in the order appearing in the RIC.

In case the RIC appeared in the Re-association message, the AP needs to check if the resources have been previously allocated by a pre-reservation request. This is done by checking the sequence number in each RDIE against the sequence numbers in the previous RIC request.

The behavior of the AP is described below:

Select next RDIE (if none finish - request succeeds)

Select first TSPEC after RDIE

Check for allocatibility/Attempt to allocate TS using TSPEC

If succeeds go to step 1

If fails -> is there another TSPEC for this RDIE?

If yes, select next TSPEC and go to line 3

If no, check state of Mandatory bit

if clear go to line 1

if set finish - request fails.

NOTE - this would look better as a flowchart. Editor to-do list.

The AP constructs a response message containing a list of resource requests that succeeded and a list of resource requests that failed. If a mandatory resource request fails, the AP need not process RDIEs that follow but may choose to do so in order to provide suggestions to the TSTA. If the request fails, no resources are actually allocated.

Interpreting RIC in the response message

The AP replies to the STA request with a response RIC where the RRIE identifier matches the request. The response RIC comprises the RRIE and one RDIE for each resource that the AP has assigned or attempted to assign. The RDIEs shall be in the same order as the request and include the original RDIE sequence numbers. All RDIEs processed by the AP must be present. The "confirm" bit in the RDIE shall be set according to the result of the allocation request as follows:

Confirm = '1' indicates that the resource has been allocated or could have been allocated. The RDIE will also be followed by TSPECs that were accepted.

Confirm = '0' indicates that the resource could not be allocated. In this case the AP may optionally include a single TSPEC following the RDIE indicating a suggested TSPEC that could have been allocated. The TSPEC count field shall be set to '0' or '1' depending whether the suggested TSPEC is attached.

Absence of a RDIE in the response will indicate that the RDIE and related TSPECs were not considered by the AP.

QoS procedures at the non-AP QSTA on (Re)-Association

The following procedures apply to a non-AP TSTA that is also a QSTA.

A non-AP TSTA may choose to initiate the (Re)-Association mechanism when it deems fit. When sending the Re-association Request message, the TSTA constructs a RIC to express the resources required for the transition to be successful. If the TSTA does not require QoS related resources, then this RIC is omitted. If a pre-reservation of resource had been successfully done, and the TSTA is invoking the (Re)-association within the required time, the following aditional rules apply to the creation of the RIC:

for resources that were successfully pre-reserved, and still required, the RDIEs from the previous request are included, in order and with the old sequence numbers, to confirm the resources at the time of transition. TSPECs need not be included.

for resources that were successfully pre-reserved, but no longer required, the sequence number in the previous RDIE is excluded from this RIC.

for resources that were successfully pre-reserved, but the TSPEC is now different, the sequence number of the previous RDIE is included in an RDIE in this RIC, and the revised TSPEC is included in this request.

for new resources that were not previously reserved, a new RDIE is included with the new TSPEC and a new sequence number.

In the simplest case, a STA may choose to initiate an (Re)-Association without any prior reservation phase. In this case the STA needs to negotiate the QoS with the new AP as part of the connection, and must include a fully specified RIC in the Re-association message. This would indicate to the AP that the STA wants the QoS resources specified in the RIC.

A re-association request with a RIC to an AP that has not advertised support for Fast Transition is not defined. The response to such request is beyond the scope of this document.

If the AP is able to meet the resource request included (or implied) by a RIC in the re-association request, and if the re-association is otherwise successful, then the AP shall include a RIC in the re-association response.

If the AP is unable to meet the resource request then it must fail the re-association request with a status code of RESOURCES_UNAVAILABLE. If the status code is (1) Invalid TSPEC, or (2) QoS resources not available, then the AP may include a RIC in the response indicating a set of TSPECs as a suggestion. This set of suggested TSPECs may include one or more original TSPECs sent in the reassociation message. The STA should expect to receive the suggested set of TSPECs (which could be just a replica of the original set, from a TTAP) but is not required to act on the information. Pre-reserved resources shall remain until the expiry time indicated upon issue.

QoS procedures at the AP on (Re)-Association

The procedures below apply to an AP that is also a QAP. An AP that is not a QAP should ignore any RIC in an association request message.

If the TSTA had sent a RIC in a pre-reservation request, but the TSTA did not invoke (Re)-association within the required time, then the resources that had been “accepted” are released.

If the RIC contained in the (re)-association request contains only the RRIE and the RIC identifier matches that of the pre-reservation, the resources that were "accepted" in the reservation phase are deemed "admitted". If the RIC of the association request contains RDIEs, then only the resources corresponding to those RDIEs shall be deemed admitted. If a RDIE is followed by a TSPEC then this shall override the previously allocated resource for the specified sequence number.

If the (re)-association message contains a fully specified RIC with a different RIC identifier, the TTAP treats the request as a new request and takes into consideration any TSPECs for the pre-reservation when allocating resources for the new TSPECs (as an implementation optimization the TTAP may "transfer" resources from the prior reservation to the new reservation in a way that is transparent to the non-AP TSTA.).

If reservation for any new TSPECs fail, the TTAP shall include in the re-association response an appropriate status code and the RRIE with RIC identifier matching the prior reservation. This is to notify the non-AP TSTA that reservation for the new request has failed and the non-AP TSTA may try to associate with the prior reservation if the holding time for the prior reservation has not expired. If the holding time for the prior reservation has expired, the TTAP shall notify the non-AP TSTA with a status code of RESOURCES_UNAVAILABLE in the re-association response.

If the admission of TSPECs succeeds and there is no other association related error, the AP shall send an association response message with a status code of SUCCESS. The response shall contain a RIC. If it is necessary to provide updated information about TSPECs then the RIC shall contain RDIEs with such updated TSPECs indicating, for example, the parameters of the TS that the AP has admitted. The response TSPECs may contain information like Medium Time, Service Period etc.

If the requirements of the RIC fail, the (Re)-Association mechanism would fail with a status code of (1) Invalid TSPECs, or (2) QoS resources not available, in the association response: In case of such failures, the AP may suggest certain TSPECs that it is likely to accept for each RRIE. If the AP does not have a suggestion for a RRIE, it must include the original TSPEC in response.

In case where the reassociation request fails for reasons other than TSPEC admission failure, the status code for the other reason will supersede.

Reservation Procedures at the non-AP TSTA

A non-AP QSTA may initiate resource reservation as it deems fit to an AP that has advertised support for resource reservation in the capability field. A reservation request to an AP that has not advertised such suport is not defined. The response to such request is beyond the scope of this document.

The intent of the resource reservation request is to enable a non-AP QSTA to reserve resources based on specified TSPECs before the non-AP QSTA actually associates with the new TTAP. The TSPECs in the request need not belong to only active TSIDs. The non-AP QSTA can send TSPECs for any TSIDs that it intends to use after the transition.

Resource reservation can be performed either over-the-air, or over-the-DS. To initiate an over-the-air reservation, the TSTA transmits request to the TTAP. To initiate over-the-DS resource reservation, a non-AP QSTA transmits a reservation request (which is an 802.11 management action frame) to its current AP.

Following a successful resource reservation, it is the responsibility of the non-AP QSTA to initiate a (Re)-Association session to associate with the new TTAP within the specific time indicated in the resource reservation response.

If a non-AP QSTA is declined for a resource reservation request, it does not mean that the non-AP QSTA has been denied service. The non-AP QSTA may still attempt to associate with new TTAP using the (Re)-Association mechanism or the standard association mechanism without any FBT enhancements.

A response to the reservation request is considered "SUCCESS" by a non-AP QSTA if the status code SUCESS is returned in the response message.

A response to the reservation request is considered "FAILED" if the status code is equal to any of the following QoS Failure codes:

Invalid TSPECs

QoS resources not available

Other types of policy violation

A response to a successful resource reservation shall contain the amount of time for which the resources will be locked down (at a TTAP and the BSS) and timestamp indicating the end of the lock-down period. The non-AP QSTA should associate with the new TTAP within the specified amount of time. Failure to do so will result in the release of the resources by the TTAP.

Reservation Procedures at the Target TTAP

This section describes the behavior at the TTAP when it receives a RIC from a neighbor TTAP or a non-AP QSTA that has not yet associated with it.

The underlying assumption is that an SME on a TTAP receives some TSPECs, encapsulated in a RIC, from another AP to which a non-AP QSTA is associated. The SME receives these TSPECs in an over-the-wire message on RRSAP. The SME then looks at these TSPECs and generates MLME.RESERVATION.request that passes down the TSPECs to the HC MAC. The SME may also send these TSPECs to an external entity such as a back-end QoS module for its consideration; these procedures are beyond the scope of this specification. The HC MAC shall respond with MLME.RESERVATION.indication that will indicate whether the HC MAC has accepted the TSPECs or not. The acceptance of a TSPEC by a new TTAP results in resource allocation for a traffic stream at the TTAP. The SME translates this response into an appropriate Resource Response message to the old AP originating the TSPECs via the RRSAP. Alternatively, the SME may choose to wait for response from an external QoS module before sending out the Resource Response.

By the end of a successful Resource Request /Response sequence, the HC MAC would have reserved resources for each Traffic Stream denoted in the TSPECs it received in the MLME.RESERVATION.request. However, the TSs shall remain in the Inactive state, and neither the inactivity nor the suspension timer would be started. The TSs will be moved to Active state only after the non-AP QSTA that originated the TSPECs associates with the new TTAP within specified timeframe, and the inactivity and the suspension timers will be started thereafter.

Reservation Procedures at the Current TTAP

This section applies to over-the-DS resource reservations only.

Upon receiving the reservation request, the RRSAP shall invoke the Broker Function. The Broker Function, after appropriate processing as described previously, will forward the request to the RRSAP which in turn will send it to the RRSAP of the target AP.

The RRSAP shall listen for responses to the reservation responses on the RRSAP. The TTAP shall then invoke the BF.

MLME-Reservation.Request

NOTE - apparently no refs to the above figure.

Traffic Stream Cleanup on Reservation TImeout

NOTE - apparently no references to the above figure.

TS Active on successful Transition

NOTE - apparently no references to the above figure.

Broker Function

A broker function in the TTAP is defined to facilitate policy enforcement and reservation request response co-ordination. This is shown in Figure Error: Reference source not found. The Broker Function is only invoked at the Current TTAP when an over-the-DS reservation request is to be performed. Following are the tasks performed by a Broker function:

At the minimum, the BF shall re-format the reservation request from the SME for consumption by the RRSAP. Similarly, it shall be able to re-format the response from the RRSAP to send it to the SME.

A BF may choose to apply any policy dependent processing on the reservation request before forwarding it to the RRSAP. A BF may, similarly choose to apply any policy dependent processing on the resource response before forwarding it to the SME.

Broker Function for Over-the-DS Communication

Layer Management

MLME SAP Interface

Reassociate

MLME-Reassociate.Request

Semantics of the Service Primitive

Change the text of 10.3.7.1.2 as follows:

The primitive parameters are as follows:

MLME-Reassociate.request 	(

NewAPAddress,

ReassociateFailureTimeout,

CapabilityInformation,

ListenInterval,

SupportedChannels,

Snonce,

Anonce,

Transition IE content,

RSN,

KeyLifetime,

ReservationItems,

MIC

)

Add the following rows to the table in 10.3.7.1.2:

NOTE - “ReassociationDeadline” appears in table, but not on list of parameters. Should it be a parameter, or drop from table? Doesn’t seem likely that it would be provided by the STA in a request for reassociation.

NOTE - Need content for several cells in above table, for type (e.g. “As defined in frame format”), valid ranges (e.g. “As defined in frame format”), and descriptions.

MLME-Reassociate.Confirm

Semantics of the Service Primitive

Change the text of 10.3.7.2.2 as follows:

The primitive parameters are as follows:

MLME-Reassociate.confirm 	(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

Transition IE content,

ReservationResponses,

MIC

)

Add the following rows to the table in 10.3.7.2.2:

NOTE - Need content for several cells in above table, for type (e.g. “As defined in frame format”), valid ranges (e.g. “As defined in frame format”), and descriptions.

NOTE - Proposal section 6.1.5 has SupportedRates followed by AssociationID, while 11ma has AssociationID followed by SupportedRates. Editor assumed there was no intention to reverse the order of these two parameters. Need confirmation of this assumption.

MLME-Reassociate.Indication

Semantics of the Service Primitive

Change the text of 10.3.7.3.2 as follows:

The primitive parameters are as follows:

MLME-Reassociate.indication 	(

PeerSTAAddress,

CurrentAPAddress

CapabilityInformation,

ListenInterval,

SSID

SupportedRates,

Snonce,

Anonce,

Transition IE content,

RSN,

KeyLifetime,

ReservationItems,

MIC

)

NOTE - Proposal section 6.1.5 has first parameter of MLME-Reassociate.indication as “PeerTSTAAddress”. If this is intentional, then a row is needed in the following table describing it. Editor assumed that there was no intended change from 11ma’s “PeerSTAAddress”. Need confirmation of this assumption.

Add the following rows to the table in 10.3.7.3.2:

NOTE - “ReassociationDeadline” appears in table, but not on list of parameters. Should it be a parameter, or drop from table? Doesn’t seem likely that it would be provided by the STA in a request for reassociation.

NOTE - Need content for several cells in above table, for type (e.g. “As defined in frame format”), valid ranges (e.g. “As defined in frame format”), and descriptions.

MLME-Reassociate.Response

Semantics of the Service Primitive

Change the text of 10.3.7.4.2 as follows:

The primitive parameters are as follows:

MLME-Reassociate.response 	(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

Transition IE content,

ReservationResponses,

MIC

)

NOTE - Proposal section 6.1.5 has first parameter of MLME-Reassociate.indication as “PeerTSTAAddress”. If this is intentional, then a row is needed in the following table describing it. Editor assumed that there was no intended change from 11ma’s “PeerSTAAddress”. Need confirmation of this assumption.

Add the following rows to the table in 10.3.7.4.2:

NOTE - Need content for several cells in above table, for type (e.g. “As defined in frame format”), valid ranges (e.g. “As defined in frame format”), and descriptions.

Insert the following subclauses after end of 10.3.30.

NOTE - 11ma ends at 10.3.23; 11e added 10.3.11 through 10.3.15 (which will likely become 10.3.24 through 10.3.28); 11k added 10.3.24 and 10.3.25 (which will likely become 10.3.29 and 10.3.30). This amendment assumes 10.3.31 is next.

MLME SAP Interface for Resource Reservation

MLME-Reservation.request

Function

This primitive is used to enact the security and QoS reservation method with a specified peer MAC entity.

Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation.request 	(

PeerMACAddress,

Message Sequence Number,

Snonce,

Transition IE content,

RSN IE content,

Anonce,

Challenge,

ReservationItems,

MIC

)

When generated

This primitive is generated by the SME at a non-AP STA to send either the first or third frames of the reservation handshake.

Effect of receipt

Upon receipt of this primitive, the MLME constructs the appropriate authentication frame and causes it to be transmitted to the PeerMACAddress.

MLME-Reservation.indicate

Function

This primitive is used to enact the security and QoS reservation method with a specified peer MAC entity.

Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation.indicate 	(

PeerMACAddress,

Message Sequence Number,

Snonce,

Transition IE content,

RSN IE content,

Anonce,

Challenge,

ReservationItems,

MIC

)

When generated

This primitive is generated by the MLME at an AP to indicate either the first or third frames of the reservation handshake has been received.

Effect of receipt

Upon receipt of this primitive, the SME examines the Transition IE and RSN IE contents and responds to the PeerMACAddress using the MLME-Reservation.response primitive.

MLME-Reservation.response

Function

This primitive is used to enact the security and QoS reservation method with a specified peer MAC entity.

Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation.response 	(

PeerMACAddress,

Message Sequence Number,

Anonce,

Transition IE content,

RSN IE content,

KeyLifetime,

ReservationResponses,

MIC

)

When generated

This primitive is generated by the SME at an AP to cause the transmission of either the second or fourth frame in the reservation handshake.

Effect of receipt

Upon receipt of this primitive, the MLME constructs the appropriate authentication frame and causes it to be transmitted to the PeerMACAddress. When the Message Sequence Number is 4, the AP shall reserve the bandwidth

MLME-Reservation.confirm

Function

This primitive is used to enact the security and QoS reservation method with a specified peer MAC entity.

Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation.confirm 	(

PeerMACAddress,

Message Sequence Number,

Anonce,

Transition IE content,

RSN IE content,

KeyLifetime,

ReservationResponses,

MIC

)

When generated

This primitive is generated by the MLME on receipt of either the second or fourth frame in the reservation handshake.

Effect of receipt

Upon receipt of this primitive, the SME examines the content of the message and either responds to the PeerMACAddress with the next message in the reservation sequence or completes its processing of the reservation request.

MLME

Note - sections 11.4 through 11.7 were added by 802.11e, with instructions to move sections 11.4, 11.5, and 11.6 (as appear in 11ma) to 11.8, 11.9, and 11.10. Assume what will happen here is that sections 11.3A, 11.3B, 11.3C, and 11.3D will be added when amendment 11e is incorporated. The changes below refer to the section numbered 11.4 by 11e, and 11.3A after incorporation.

Traffic Stream Operation

Introduction

Change the third paragraph of section 11.4.1 as follows:

TSPEC and the optional TCLAS elements are transported on the air by the ADDRS, in the corresponding QoS Action frame and across the MLME SAP by the MLME-ADDRS primitives. In addition, a Traffic Stream could be created if a TSTA sends a resource request to a TTAP prior to initiating a transition to that TTAP.

TS Lifecycle

Change figure 148A as follows:

Note - 11ma last figure in section 11.3 is 148. 11e called its first figure 68.1, which will become 148A when incorporated into the standard.

NOTE - Need an additional path out of “Inactive” by way of an oval marked “FBT Resource Request” to a hexagon labeled “Accepted”. From “Accepted” by way of an oval marked “Reassociation” to “Active”. Updated figure to be done.

Insert after the second paragraph of section 11.3A.3 (second paragraph starts “Initially a TS is inactive ...”):

The TS would be successfully created if the TTAP determines that it can honor the TSPECS in case the RSTA in question transitions to it. A TS thus created will be in Accepted State. The TS would be considered an "accepted TS". The following properties hold true for such an accepted TS:

The TS will be in Accepted State. This state is a intermediate state between Inactive and Active State. The Accepted State may not be used in all instances of TS setup, e.g in case of a regular ADDTS request/response, a TS will directly jump from Inactive to Active state upon successful negotiation.

The inactivity and suspension timer would not be started for the TS.

The scheduler and any other resource manager in the AP will take the resource/timing requirements of the TS into consideration before assigning any further resources to other admitted/accepted TS.

In case of HCCA based TS, the HC would not generate CF-Poll for the TS.

The TS is moved to Active State once the RSTA transitions to the AP in question and confirms in the Re-association message, using the procedures defined in section 5.x.x.x, that it wants to retain the TS upon transition. The inactivity and suspension timers are started at this point.

If the reservation timer times out and the TS is not yet in Active State, the TS is deleted.

NOTE - This section requires more editorial work to match the style of the text in 11e for this section.

Inset new Annex O after Annex N:

(informative)

Example Deployments using multiple SDs in an SMD

This logical hierarchy key hierarchy may be physically realized in a variety of ways. Some examples are provided below.

Controller Architecture

A centralized Controller that manages multiple APs may include the AAA Client, R0 Key Holder and R1 Key Holder.

Each AP managed by the Controller may be an R2 Key Holder. Alternatively, if the Controller is also the authenticator for the purpose of PTK derivation, the Controller will be the R2 Key Holder for each of its APs, and transmit the PTK to the AP, which serves as Data Termination point. It is also possible for Data Termination to be aggregated in the Controller, with the AP transmitting encrypted data between station and Controller.

Each Controller and its associated APs thus constitute an SD, and multiple Controllers may be associated with each other to form an SMD. When a station transitions to an AP managed by a different Controller, the new Controller acquires PMK-R1 from the Controller with which the station first established keys.

Standalone AP Architecture

A deployment may comprise multiple APs that are not managed by central Controllers. In this case, each AP comprises the AAA Client and R0/R1/R2 Key Holders.

Each AP constitutes an SD, and multiple APs may be associated with each other to form an SMD. When a station transitions to a new AP, the new AP acquires PMK-R1 from the AP with which the station first established keys.

Key Holder and SMD Naming

Each AP advertises the following names in its beacon and/or probe response:

SMD-ID

R0KH-ID

R1KH-ID

R2KH-ID

To ensure global uniqueness, the first three octets of an ID must be set to an Organizationally Unique Identifier (OUI). The remainder of the binary sequence is managed by the vendor named in the OUI as its own private namespace, within which it must guarantee uniqueness.

One option for selecting an ID is the MAC address of an interface of the device, zero-filled to 16 octets. Another option is to append 13 random octets to the OUI.

Typically, R2KH-ID might be set to the BSSID of the AP; however, this is not a requirement and there may be architectures in which R2KH-ID and BSSID are different values.

For key derivation and key naming purposes, R2KH-ID and R1KH-ID are always the names of the R2 and R1 Key Holders in the current SD which with the station is associated, and R0KH-ID is the name of the R0 Key Holder of the original SD in which the security association was established. Thus, R0KH-ID may or may not be the name of the R0 Key Holder of the current SD with which the station is associated.

Broker Function and Policies

A TTAP may choose to incorporate variety of policies in the Broker function. This provides a vendor some flexibility and control over the behavior it wants to mete to the resource reservation requests originating in its network. As mentioned earlier, however, a BF shall at least be able to relay the request to its local RRSAP and receive the responses from the RRSAP.

Over and above the minimal functionality a TTAP vendor may choose to impose certain restrictions and/or preferences by injecting some policy processing in the Broker Function.

Examples of this include:

A BF may choose to limit the number of resource request/queries per STA.

A BF may choose to spoof the over-the-DS resource allocation if it has knowledge of the overall system and can make decisions on behalf of the target TTAP.

A BF may choose to introduce a bias in the responses, for various reasons like load balancing etc.

A BF may want to check for the mobility domain of the target APs before sending out request to the APs. The BF may reject one or more request if it has enough knowledge of the mobility domain to which the target APs belong

Transition Mechanism during Voice Flow

The figure below depicts a scenario where the FT proposal could be used. We assume that a 802.11 enabled station had associated with AP1.

It originates a voice call after reserving air resources by using 802.11e enhancements (ADDTS etc).

The voice call signaling and the RTP path setup occurs while the phone is associated with AP1.

During the voice-call the phone tries to "roam".

It finds that AP2 is a potential AP to roam to.

It triggers a resource reservation mechanism.

A FT association is initiated to roam to the new AP2

The phone now connects to the back-end via AP2.

Transition Mechanism in a Voice Call Flow

NOTE - any blank pages that follow (beyond one to finish the double sided-ness of the last text page) are not intentional