IEEE P802.11
Wireless LANs

| LB73 Hidden Station Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-May-19 |  |  |  |  |
| Author(s): Simon Barber |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Barber | Devicescape Software, Inc | 1000 Marina Blvd, #400, Brisbane, CA | 650-829-2618 | simon@devicescape.com |

Abstract

This document addresses comments  from LB73.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

##### 3. Definitions

A hidden station is a station that is known to exist, since MPDUs with DA set to the hidden station’s MAC address are received, however frames with TA set to the MAC address of the hidden station are not received.

##### 7.3.2.22.8 Hidden Station Report

The format of the Measurement Report field of a Hidden Station Report is shown in Figure k1.

|  |  |  |  |  | Zero or more entries |
| --- | --- | --- | --- | --- | --- |
|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Hidden Station Entry |
| Octets: | 1 | 1 | 8 | 2 | 10 |

Figure k1—Measurement Report field format for a Hidden Station Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Hidden Station Report was measured, expressed in TUs.

|  | Hidden Station Address | Number of Frames | Missing ACKs |
| --- | --- | --- | --- |
| Octets: | 6 | 2 | 2 |

Figure k2—Hidden Station Entry

Hidden Station Address contains the MAC address of the hidden station being reported (the DA from the observed MPDU for which no ACK was seen).

Number of frames is a count of the unique unicast MPDUs, received from any station, where the MPDU is an initial transmission attempt (not a retransmission) and an immediate ACK is expected. A count of 65535 is used to indicate frame counts of 65535 or greater.

Missing ACKs is a count of the above frames where an immediate ACK was not observed and a subsequent retransmission attempt was also not observed. A count of 65535 is used to indicate frame counts of 65535 or greater.

#### 11.7.8.5 Hidden Station Report

If a station accepts a Hidden Station Request it shall respond with a Radio Measurement Report frame containing one or more Measurement (Hidden Station) Report elements.

A measuring STA detects a potential hidden station if it receives a unicast frame destined for a station other than itself for which an acknowledgement is required and it does not receive the acknowledgement. To minimize false alarms due to frame errors, the measuring STA shall only count frames when the detected frame is an initial transmission, and shall not count the missing ACK if a subsequent retransmission of the original frame is seen. By maintaining both the count of the number of frames where an ACK is expected, and also the number of frames where an ACK was not observed it is possible to calculate the ratio of good to missing frames, and account for a number of ACKs not received due to data errors. Hidden Station Entrys shall only be reported where the number of ACKs not received due to data errors is non zero.

-- ********************************************************************

-- * dot11HiddenStationReport TABLE

-- ********************************************************************

dot11HiddenStationReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HiddenStationReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Hidden Station Reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 5 }

dot11HiddenStationReportEntry OBJECT-TYPE

SYNTAX Dot11HiddenStationReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11HiddenStationReportTable Indexed by

dot11HiddenStationRprtIndex."

INDEX { dot11HiddenStationRprtIndex }

::= { dot11HiddenStationReportTable 1 }

Dot11HiddenStationReportEntry ::=

SEQUENCE {

dot11HiddenStationRprtIndex Unsigned32,

dot11HiddenStationRprtRqstToken OCTET STRING,

dot11HiddenStationRprtIfIndex InterfaceIndex,

dot11HiddenStationMeasuringSTAAddr MacAddress,

dot11HiddenStationRprtChanNumber INTEGER,

dot11HiddenStationRprtRegulatoryClass INTEGER,

dot11HiddenStationRprtActualStartTime TSFType,

dot11HiddenStationRprtMeasurementDuration Unsigned32,

dot11HiddenStationRprtHiddenSTAAddr MacAddress,

dot11HiddenStationRprtNumFrames INTEGER,

dot11HiddenStationRprtMissingACKs INTEGER }

dot11HiddenStationRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Request elements in dot11HiddenStationReportTable, greater

than 0."

::= { dot11HiddenStationReportEntry 1 }

dot11HiddenStationRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11HiddenStationReportEntry 2 }

dot11HiddenStationRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Hidden Station Report has been received on."

::= { dot11HiddenStationReportEntry 3 }

dot11HiddenStationMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Hidden Station report."

::= { dot11HiddenStationReportEntry 4 }

dot11HiddenStationRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The channel number on which the this measurement report was made."

::= { dot11HiddenStationReportEntry 5 }

dot11HiddenStationRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

Hidden Station report has been conducted."

REFERENCE

"IEEE 802.11k, Error: Reference source not found"

::= { dot11HiddenStationReportEntry 6 }

dot11HiddenStationRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11HiddenStationReportEntry 7 }

dot11HiddenStationRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Hidden Station

Report was measured."

UNITS

"TUs"

::= { dot11HiddenStationReportEntry 8 }

dot11HiddenStationRprtHiddenSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the destination of the frame received by the

measuring STA without detecting the subsequent acknowledgement."

::= { dot11HiddenStationReportEntry 9 }

dot11HiddenStationRprtNumFrames OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of frames is a count of the frames sent to the hidden station

address."

::= { dot11HiddenStationReportEntry 10 }

dot11HiddenStationRprtMissingACKs OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of frames is a count of the frames sent to the hidden station

address for which no ACK was received."

::= { dot11HiddenStationReportEntry 11 }

-- ********************************************************************

-- * End of dot11HiddenStationReport TABLE

-- ********************************************************************