IEEE P802.11
Wireless LANs

| PTKName, other key name issues |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |
| Jesse Walker | Intel Corporation | JF3-206, 2111 NE 25th Ave, Hillsboro, OR 97124 | 503-712-1849 | jesse.walker@intel.com |

Abstract

This contribution proposes a definition for PTKName in 11r section 8.5A.8.

It also (1) fixes a size problem with R2Name being too long to fit in the PMKID field in the RSNIE, and (2) similarly changes the formulas for R0Name and R1Name.

Recent 11r conference calls indicated a need for text explaining the proper use of key names versus key values.  This contribution proposes adding a normative statement about each key value, rather than a simple informative statement.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Overview

The text in 11r section 8.5A many places refers to a PTKName, but it is never defined.  Rather than delete all the references, a value of PTKName should be defined.

The value of R2Name is to be placed in the PMKID field of the RSN Information Element in several messages.  However, the formula for R2Name in 11r section 8.5A.6 gives a 32-byte value (256 bits), where the PMKID is only 16 bytes.  Therefore a revised formula is needed.

Document Changes

Change section 8.5A.4 as follows:

R0Name = Truncate-128(SHA-256(PMK-R0 || “R0 Key Name” || SSID || R0KH-ID || SPA))

Where Truncate-128 (x) returns the first 128 bits of its argument, and securely destroys the remainder.

Add the following at the end of section 8.5A.4:

R0Name is a value that may appear (as needed) in non-secured output from the STA or AP, while PMK-R0 is intended to be kept secure. An equal comparison of values of R0Name confidently shows matching values of PMK-R0.

Change section 8.5A.5 as follows:

R1Name = Truncate-128(SHA-256(R0Name || R0KH-ID || R1KH-ID || SPA))

Add the following at the end of section 8.5A.5:

R1Name is a value that may appear (as needed) in non-secured output from the STA or AP, while PMK-R1 is intended to be kept secure. An equal comparison of values of R1Name confidently shows matching values of PMK-R1.

Change section 8.5A.6 as follows:

R2Name = Truncate-128(SHA-256(R0Name || R0KH-ID || R1KH-ID || R2KH-ID || AA || SPA))

Add the following at the end of section 8.5A.6:

R2Name is a value that may appear (as needed) in non-secured output from the STA or AP, while PMK-R2 is intended to be kept secure. An equal comparison of values of R2Name confidently shows matching values of PMK-R2.

Add the following at the end of section 8.5A.8:

The PTK is referenced and named as follows:

PTKName = Truncate-128(SHA-256(R2Name || AA || SPA || ANonce || SNonce))

PTKName is a value that may appear (as needed) in non-secured output from the STA or AP, while PTK is intended to be kept secure. An equal comparison of values of PTKName confidently shows matching values of PTK.