IEEE P802.11
Wireless LANs

| Methodology for Employing Variable Attenuators in a Conducted Test Environment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emmelmann, Marc | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49 (0)30/314-24580 | emmelmann@ieee.org |
| Langgärtner, Tim | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49 (0)30/314-23823 | langgaertner@tkn.tu-berlin.de |
| Rathke, Berthold | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49 (0)30/314-23832 | rathke@tkn.tu-berlin.de |
| Egner, Peter | Telefunken Radio Communication Systems GmbH & Co KG | Eberhard-Finck-Strasse 55 89075 Ulm-Boefingen Germany | +49 (0)731/1553 - 219 | Peter.Egner@tfk-racoms.com |

Abstract

This document provides a methodology for employing variable attenuators in a conducted test environment. The purpose of this document is to define a procedure that will result in repeatable, reliable, and comparable measurement results. This methodology shall be applied for tests using a conducted environment whenever the attenuation of the communication channel is varied while measuring metrics and sub-metrics. All text presented is to be inserted into the TGT recommended practice document with editorial changes.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1 Methodology for Employing Variable Attenuators in a Conducted Test Environment

- 1.1 Introduction and Purpose

This section outlines a methodology to be used in conducted test environments employing variable attenuators. It is to be used for measurements of metrics and sub-metrics as outlined in the 802.11T specifications.

The presented methodology shall be used for measurements of metrics and sub-metrics which require that the attenuation imposed to the communication signal is varied during the measurement. Possible metrics and sub-metrics to which this methodology applies include but are not limited to “BSS transition time” and “rate vs. range.”

The general setup for tests using this methodology includes shielded enclosures  separating all wireless devices (DUT and “reference AP”) and variable attenuators.

- 1.3 Test Configuration

- 1.3.1 Resource Requirements

The following equipment is required in order to conduct tests using this methodology. It may be extended by any means in order set up a conducted test environment appropriate for measuring the considered metric or sub-metric:

- Shielded enclosures (one for each wireless device);
the shielded enclosure shall protect the wireless device from extraneous signals; the enclosure shall ensure that the communication test signal is only propagated via the conducted test environment; isolation in the frequency band being tested between the inside and outside of the shielded enclosure should exceed -95dB

- DUT, respectively STA,
device under test

- Reference AP,
participant of communication with DUT; may either be another DUT or a calibrated communication partner / AP

- Variable attenuators,
device inserted in the conducted communication path in between DUT and “reference AP” that allows to change the attenuation experienced in the transmission path while the test is running.  The level of the imposed attenuation at any given time during the test shall be reproducible.

- Attenuator controller
an optional entity that is steering / controlling the imposed signal degradation of the employed variable attenuators according to a channel model; may be co-located with the variable attenuator

- Channel model,
algorithm how to vary the imposed attenuation during the measurement.

- Supplementary equipment
Additional (calibrated) equipment to connect the DUT and Reference AP via the variable attenuator(s), e.g.: cables, splitters, combiner, etc.

- 1.3.3 Permissible Error Margins and Reliability of Test

Prior to beginning of the test, the test equipment described above shall be calibrated, and all test software be verified. The calibration date of all equipment should be noted in the test report for a given metric. The test setup may be monitored during the test to ensure proper functionality of the employed equipment.

- 1.4 Approach

- 1.4.1 Configuration Parameters

This test methodology applies to any configuration in which variable attenuators are employed in a conducted test environment.

- 1.4.1.1 Baseline Configuration

The baseline setup to be configured, measured, and reported whenever this methodology is applied, is performed as follows:

A linear fading channel model shall be imposed by the variable attenuators while measuring the considered metric or sub-metric. At least two sets of measurements shall be conducted employing two speeds (v1 and v2, given in dB/s) at which the imposed attenuation is changed during the measurement such that v1 << 2*v2.

For a metric or sub-metric to be considered independent of the employed speed at which the imposed attenuation is changed, reported results shall be the same within a given accuracy for v1 and v2. Otherwise, the metric or sub-metric shall be considered dependent on the velocity at which the imposed attenuation changes.

- 1.4.1.2 Modifiers

The baseline setup may be modified as follows to enable additional trials to be performed for tests using this methodology. Only one variation should be tested at a time.

Modifies include:

- the channel model,

- speed at which the imposed attenuation is changed

If the channel model is changed, the baseline configuration shall be extended by all employed channel models.

- 1.4.2 Test Conditions

This methodology describes a test environment and as such does not have a particular test condition. Other test conditions can be applied in addition to the test environment suggested.

- 1.4.3 Procedure

- 1.4.3.1 Preparation of Environment

Each device shall be placed in a separate shielded enclosure. The communication test signal shall be propagated via the conducted test environment only. Variable attenuators may be placed in the transmission path of the communication test signal.

Measurement point(s) may be inserted in order to monitor the communication test signal, interference test signal, internal interference signal, or RF test signal.

- 1.4.3.2 Calibrating Environment

The variable attenuators shall be calibrated. The calibration shall assure an imposed attenuation according to the specified channel model. Besides, the imposed attenuation and its variation during the test, i.e. the speed at which the attenuation is changed at any given test time, shall be reproducible in time and location.

- 1.4.3.4 Measuring Performance

The attenuation imposed by the variable attenuators should be monitored during the test. Additional measurements may be performed according to the measured metric or sub-metric.

- 1.4.4 Reported Results

This section outlines a methodology and therefore does not have traditional results to report. Reported results should include all required measurements according to the baseline configuration as defined in 1.4.1.1.

- 1.4.4.1 Special Reporting Requirements

Reported results of tests employing this methodology shall specify for each experiment

- the used channel model and

- the attenuation imposed on the communication test signal between the DUT and “reference AP” at any given time during the experiment.

References: