IEEE P802.11
Wireless LANs

| Resolutions for Selected TGm Letter Ballot Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Johnny Zweig | Apple Computer | 1 Infinite Loop Cupertino, CA 95014 | (408) 974-7831 | justjohnny@apple.com |

Abstract

Resolutions for Darwin Engwer’s Letter Ballot 75 comments (numbers 131 and 132) concerning problems with the descriptions of WDS and the four-address frame format were agreed-upon in the TGm session on Wednesday afternoon (July 20, 2005 from 4:00 to 6:00pm). Proposed changes to P802.11REV-ma-D2.0 are shown here.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Proposed changes to Document P802.11REV-ma-D2.0 to Resolve Comment 131

Comment 131, clause 7.1.3.1.4

Comment: WDS is not defined in the standard; the existing references are

very confusing and too limiting.

In order to resolve Comment 131, it is necessary to change text in clause 7..1.3.1.3 and 7.1.3.1.4 as well as text in Table 2 (in 7.1.3.1.4) in order for both clauses to be consistent.

In Draft 2.0, these sections read thus:

7.1.3.1.3 To DS field

The To DS field is 1 bit in length and is set to 1 in data type frames destined for the DS. This includes all

data type frames sent by STAs associated with an AP. The To DS field is set to 0 in all other frames.

7.1.3.1.4 From DS field

The From DS field is 1 bit in length and is set to 1 in data type frames exiting the DS. It is set to 0 in all other

frames.

The permitted To/From DS bit combinations and their meanings are given in Table 2.

Table 2—To/From DS combinations in data type frames

| To/From DS values | Meaning |
| --- | --- |
| To DS = 0 From DS = 0 | A data frame direct from one STA to another STA within the same IBSS, as well as all management and control type frames. |
| To DS = 1 From DS = 0 | Data frame destined for the DS. |
| To DS = 0 From DS = 1 | Data frame exiting the DS. |
| To DS = 1 From DS = 1 | Wireless distribution system (WDS) frame being distributed from one AP to another AP. |

Proposed modifications to accept Comment 131 make them read thus:

7.1.3.1.3 To DS field

The To DS field is 1 bit in length and is set to 1 in data type frames destined for the DS. This includes all data type frames sent by STAs associated with an AP.

7.1.3.1.4 From DS field

The From DS field is 1 bit in length and is set to 1 in data type frames exiting the DS

The permitted To DS and From DS bit combinations and their meanings are given in Table 2.

Table 2—To/From DS combinations in data type frames

| To DS and From DS values | Meaning |
| --- | --- |
| To DS = 0 From DS = 0 | Data frame direct from one STA to another STA within the same IBSS, as well as all management and control type frames. |
| To DS = 1 From DS = 0 | Data frame destined for the DS or being sent by a STA associated with an AP to the Port Authenticator Entity in that AP. |
| To DS = 0 From DS = 1 | Data frame exiting the DS or being sent by the Port Authenticator Entity in an AP. |
| To DS = 1 From DS = 1 | Data frame using the four-address WDS format. This standard does not define procedures for using this combination of field values. |

# Proposed Changes to Document P802.11REV-ma-D2.0 to Resolve Comment 132

Comment 132, clause 7.2.2

Comment: WDS is not defined in the standard; the existing references are

very confusing and too limiting.

To accept this comment, modify the text in Clause 7.2.2 thus:

The DA field is the destination of the MSDU (or fragment thereof) in the frame body field.

The SA field is the address of the MAC entity that initiated the MSDU (or fragment thereof) in the frame

body field.

The RA field is the address of the STA that is the immediate intended

receiver of the frame or the address of the STAs that are the immediate intended receivers of the frame.

The TA field is the address of the STA that is transmitting the frame.