IEEE P802.11
Wireless LANs

| TGv Objectives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily H. Qi | Intel Corporation | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |

Major contributions were received from the following individuals:

Jesse Walker		Marian Rudolf		Joe Kwak		Joe Epstein

John Klein 		Pat R. Calhoun		Tim Olson		Necati Canpolat

Stuart Golden		Roger Durand		Simon Black		Mikko Jaakkola

Jari Jokela

Abstract

This document captures the objectives for IEEE 802.11 Task Group v (TGv, Wireless Network Management). It proposes overall design requirements for Wireless Network Management, the type of management services to be provided, and what management frameworks are to be implemented. This document is intended to be a working document during TGv requirement discussion time frame, and will be finalized when the TGv requirement discussion is closed.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Objectives Summary

This section provides a point by point summary of the main objectives. The rationale behind these objectives, along with secondary objectives and desirable features, is given in subsequent sections.

## Req 1000: Backwards Compatibility

All defined TGv mechanisms shall be compatible with the operation of 802.11 stations that do not support 802.11v.

## Req 1010: Enterprise, Home, and Hot spots

All defined TGv mechanisms shall provide flexibility for the different usages: Enterprise, Home, and Hot Spots.

## Req 1020: ESS and IBSS mode

All defined TGv mechanisms shall provide flexibility to operate in the usage scenarios according to Req 1010 in infrastructure mode. Applicability or extension of this protocol to the IBSS case shall be taken into consideration as appropriate.

## Req 1300: Advertisement

TGv capabilities must be advertised and (/or) communicated amongst all STAs.

## Req 1310: Compliance

Some TGv objectives may provide a compliance specification along with the associated management request.

## Req 1400: Access Control Mechanism

TGv shall define its own Access Control mechanism for certain management objectives.

## Req 1410: Management Message Timeliness

TGv shall provide a mechanism for checking management message timeliness to prevent processing of stale management requests.

## Req 1500: Client Management Protocol

TGv shall define a framework to allow a STA to extract and provide managed information from another STA.

## Req 1900: Regulatory Requirements

This standard shall not prevent FIPS certification.

## Req 2000: Dynamic Channel Selection

TGv shall support dynamic channel selection to permit devices to compensate for interference.

## Req 2010: Power Saving

TGv shall support one or more power saving schemes to optimize the STA’s power consumption during prolonged time periods of no transmission or reception activity in the order of several hundred’s of ms’s or above.

## Req 2020: AP Firmware Update

TGv shall support firmware update for AP.

## Req 2030: AP Load Balancing

TGv shall facilitate AP load balancing to improve wireless network throughput and QoS effectiveness.

## Req 2040: Deferred Management

TGv shall support Transmit Power control, Energy Detection Threshold (EDT) control and Clear Channel Assessment (CCA) control.

## Req 2041: Spectrum Etiquette

TGv shall provide a mechanism for dynamic multi-level transmitter power control.

## Req 2050: Access Point Coordination

TGv shall facilitate Access Point Coordination in order to meet various objectives.

Req2060:  Virtual Aceess Point

TGv shall support MAC extensions to improve support of virtual access points.

Req2070:  Diagnostics and Troubleshooting

TGv shall support MAC extensions to improve wireless network diagnostics and troubleshooting.

Req2071: Contention

TGv shall provide a mechanism for notification of performance degradation due to contention issues in multi-radio devices.

Req2080:  Advanced Antennas

TGv shall define mechanisms to provide better support of Advanced Antennas.

Req2090: Location

TGv will provide a mechansism to coordinate the gathering and possibly generation of data to support various location methods such as time of arrival, time difference of arrival, and signal strength.

- Introduction

This document defines objectives for Wireless Network Management in IEEE802.11. The objectivess relate to the work of task group ‘v’ which has the following scope and purpose:

Scope: To provide Wireless Network Management enhancements to the 802.11 MAC, and PHY, to extend prior work in radio measurement to effect a complete and coherent upper layer interface for managing 802.11 devices in wireless networks.

Purpose: To provide amendments to the IEEE 802.11 PHY/MAC layers that enable management of attached stations in a centralized or in a distributed fashion (e.g. monitoring, configuring, and updating) through a layer 2 mechanism. While the 802.11k Task Group is defining messages to retrieve information from the station, the ability to configure the station is not in its scope. The proposed Task Group will also create an Access Port Management Information Base (AP MIB).

This document defines requirements and objectives in the areas of:

- General requirements affecting all solutions.

- Capability advertisement and compliance negotiation: Requirements for stations and APs to establish mutual expectations.

- Security requirements in addition to the protection mechanisms that 802.11w provides.

- General management frameworks and interfaces affecting all services.

- Service and function objectives: the kind of services and functions that 802.11v should provide.

This document may be used as the basis for a call for technical proposals for Task Group v.

# Assumptions

It is assumed that all stations supporting 802.11v will as a minimum also support 802.11k, 802.11i, and 802.11w. However, this shall not preclude the future 802.11v amendment to contain management extensions that address or use specific management needs of other 802.11 amendments such as for example 802.11e and 802.11r if implemented on the station.

# Overall Design Requirements

Solution shall have the following attributes:

[Req1000] Backwards Compatibility requires that it shall be possible for an access point or station to operate in a mixed environment of 802.11v and non-802.11v stations. In other words, it shall be possible for stations to operate using 802.11v without disrupting the operation of stations that do not support 802.11v. This does not preclude policy-driven operations in which an access point may refuse to associate with stations that do not support 802.11v.

## [Req1010] Enterprise, Home and Hot Spots shall be the use cases for Wireless Network Management (TGv). The solution shall have the flexibility applying to these different usages.

……

# Policy and Negotiation Requirements

Solution shall have the following capabilities:

[Req1300] Advertisement  requires that a station or access point be able to determine whether its peer supports 802.11v prior to attempting a connection. Access Point shall advertise its capability.  Client STA must be able to provide its capability to an Access Point.

[Req1310] STAs should be able to negotiate the level of compliance for an individual command, control sequence or other requested operation. The requested compliance parameters can be generalized such that they can be applied to any of the objectives within TGv’s scope.

# Security Requirements

In addition to the requirements that are provided to 802.11w, solution shall provide the mechanisms listed below.

[Req1400] TGv requires a separate Access Control Mechanism for some of its objtectives to prevent an unauthorized entity from attempting some management operations by assuming the identity of an authorized entity. This mechanism is above and beyond the general Layer 2 Authentication and Authorization Protection shall be provided by 802.11w.

[Req1410] Some TGv objectives require a mechanism for Management Message Timeliness validation.  Timeliness validation can prevent the processing of stale management requests making the system operate more efficiently and possibly prevent certain security threats.

# Framework and Interface Objectives

[Req1500] TGv should define a framework to allow a STA to extract and provide information from another STA, which could be useful for load balancing, pushing configuration, retrieving statistics, etc.  TGv would have to identify the specific managed elements that are of interest. This would allow for client management without any reliance on an SNMP client or agent on the STA

- Service and Function Objectives

Solutions shall define mechanisms to provide the service listed below.

[Req2000] TGv shall support Dynamic Channel Selection, to allow STAs to avoid interference. Solution shall be able to change the operating channel (and/or band) for the entire BSS during live system operation and be done seamlessly with no intermittent loss of connectivity from the perspective of an associated STA. Solution shall not define algorithm for channel selection.

[Req2010] TGv shall support a “long term” Power Saving management feature; to optimize a STA’s power consumption during prolonged time periods of no transmission or reception activity in the order of several hundred’s of ms’s or above.

[Req2020] TGv shall support AP Firmware Update. Solution shall provide the streamlined means to upgrade AP firmware automatically. Solution shall operate over wired/wireless infrastructure, and be secure, reliable, and compatible with CAPWAP.

[Req2030] TGv shall support AP Load Balancing. An AP uses load balancing to suggest that one or more of its associated STAs transition to another AP. Solution shall provide the means by which the AP service load can be balanced between a congested AP and an under-utilized AP to optimize the use of radio resource and to increase network aggregate throughput and QoS effectiveness. A STA and AP cooperative approach is highly recommended.

[Req2040] TGv shall support “Deferral Management”to improve throughput and reduce interference. Solution should provide the control capabilities with Transmit Power control, Energy Detection Threshold (EDT) control, Clear Channel Assessment (CCA) control or other PHY tuning parameters.   For Deferral Management to reach its full potential, solution shall be coherent and cohesive across all the APs and STAs.

[Req2041] TGv shall support Transmitter Power Control that is specifically enhanced to include the aspects of “dynamic multi-level transmitter power control. For example, in the neighbor overlapping RF coverage zones, both AP and STA may conduct dynamic multi-level Transmit Power control.

[Req2050] TGv shall support Access Point Coordination. The possible coordination between APs may include Time Coordination for Resource Management, Site Survey Mode, Power Coordination across APs, alignment of QoS policies, Neighbor Learning Process, Antenna Control, and Report Sharing.

[Req2060] TGv shall support MAC extensions to improve support of Virtual Access Points, in order to conserve air time as well as enhanced integration with radio measurement extensions, such as TGk. One example would be to allow a single AP to advertise multiple SSIDs through a single beacon, where each SSID is mapped to a unique BSSID that may have a separate set of policies. Other alternatives to tie a virtual AP to a physical AP would also be acceptable.

[Req2070] TGv shall support MAC extensions to improve wireless network Diagnostics and Troubleshooting; TGv shall provide a mechanism to allow STAs to report alerts when a certain failure occurs or performance degrades; TGv shall provide an extensible framework for STAs to request the execution of diagnostics and report diagnostic results and other event log data.

[Req2071] TGv shall provide a mechanism for notification of performance degradation due to Contention issues in multi-radio devices

[Req2080] TGv shall define mechansims to support better use of Advanced Antennas.  For example the mechanisms defined in 11-05/0280r2 would be one approach.

[Req2090]

TGv will provide a mechansism to coordinate the gathering and possibly generation of data to support various location methods such as time of arrival, time difference of arrival, and signal strength.

8. References

11-05-0224-03-000v-tgv-requirements.doc

11-05-0459-02-000v-minutes-tgv-cairns-meeting-may-05.doc

11-05-0521-00-0ads-requirements-management-protection.doc

11-05-0369-03-000v-secure-wnm-requirements.ppt

11-05-0370-02-000v-bss-load-balancing-requirements.ppt

11-05-0501-00-000v-load-balancing-kwak-rudolf.ppt

11-05-0497-00-000v-dcs-kwak-rudolf.ppt

11-05-0498-01-000v-tpc-edt-kwak-rudolf.ppt

11-05-0499-00-000v-power-saving-kwak-rudolf.ppt

11-05-0507-00-000v-firmware-updates.ppt

11-05-0504-00-000v-different-uses.ppt

11-05-0500-00-000v-11v-control-kwak-rudolf.ppt

11-05-0506-00-000v-tx-power-requirements.ppt

11-05-0505-00-000v-access-point-coordination.ppt

11-05-0720-02-000v-virtual-ap-requirements.ppt

11-05-0732-00-000v-client-management-protocol.ppt

11-05-0905-00-000v-diagnostics-and-troubleshooting.ppt

11-05-0945-00-000v-diagnostics.ppt

11-05-0280-02-000v-advanced-antenna-support.ppt

11-05-0891-00-000v-enabling-localization-in-wlan-by-time-stamp-differences-tsd.ppt

11-05-0932-00-000v-coexistence-in-multi-radio-devices.ppt

11-05-0906-00-000v-spectrum-etiquette.ppt

11-05-0927-00-000v-client-management-protocol-details.ppt