IEEE P802.11
Wireless LANs

| Merge TRIE and TSIE to create FTIE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel | 2111 NE 25th Ave, JF3-206, Hillsboro OR 97124 | +1-503-264-3759 | Kapil.sood@intel.com |
| Nancy Cam-Winget | Cisco Systems | 3625 Cisco Way, San Jose CA 95134 | +1-408-853-0532 | ncamwing@cisco.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Rationale for merging:

- A unified mobility domain has been defined, which includes security and resource domains for fast transitions.

- A new Mobility Domain IE has been created that leaves TRIE and TSIE smaller and hence, bit-wise in-efficient.  A new IE called Fast Transition IE (FTIE) has been defined.

- Policy configurations for security and QoS resource are static and localized, and typically used together.  So, merging both makes for a simpler data structure.

- For cases for Security and QoS: All fields will be used.

- For cases that deploy security and no QoS: The Resource mechanisms bits will be reset to 0.

Note: Asscociated presentation is in Doc#: 11-05-0903-00-000r-TRIE-TSIE-Integration-FTIE

The following document describes detailed changes to the TGr draft to combine the TSIE and TRIE information elements.  The recommended changes are based on TGr draft 0.05.

Replace Sections 7.3.2.40 and 7.3.2.41 with the following:

7.3.2.40 Fast Transition Information Element (FTIE)

The Fast Transition Information Element (FTIE) is defined to enable the advertisement of network infrastructure resource reservation and security policy information. This can assist a TSTA in deciding which transition mechanism(s) to use. A TAP can advertise the FTIE through a Beacon frame and through a Probe Response frame. This information element is defined in Figure 80Z.

NOTE - Beacon and Probe Response frames are not yet defined. Need to keep the above paragraph consistent with their eventual definition.

The Length field must be set to 33 (0x21).

The Fast Transition Resource Mechanism value is as defined in Table 31J.

| Table 31J- Fast Transition Resource Mechanism value |  |
| --- | --- |
| Bit | Meaning |
| 0: Reservation over air | AP supports Reservation over the air, which is protected using a separate handshake protocol |
| 1: Reservation over DS | AP supports Reservation over the DS |
| 2: Reserve Option | Set if reservation is mandatory, clear if reservation is optional. |
| 3-6: Reservation Limit | Number of APs at which a STA can reserve |
| 7: Reserved | Reserved |

The STA can use information from the FTIE to determine the resource and security transition mechanisms supported and recommended by the AP and infrastructure. The choice of executing any specific transition mechanism is left as discretion of the STA.

Replace mentions of TRIE and/or TSIE with FTIE in the draft

Replace rows in 7.3.2 Table 22 by collapsing TSIE and TRIE into one FTIE, and re-number accordingly

Replace Section 5.7 Fast Transition Security and Resource Information with Fast Transition Information

Replace rows in 7.2.3.6 Table 9 by collapsing TSIE and TRIE rows into one FTIE

Replace rows in 7.2.3.7 Table 10 by collapsing TSIE and TRIE rows into one FTIE

Replace rows in 7.2.3.10 Table 13 by collapsing TSIE and TRIE rows into one FTIE

Replace rows in 7.2.3.10 Table 14 by merging TSIE and TRIE into one FTIE

Replace in 7.4.7 Fig 85H, 85I, 85J, 85K by replacing TRIE and TSIE fields with a FTIE field

Replace in 8.5.2.2 TSIE and TRIE in Key Data field with FTIE

Replace in 8.5A.8 Table 35A rows with TSIE and TRIE into a single row with FTIE

Replace in 8.5A.8.1 lines 35 through 49, with the following:

The Fast Transition IE shall contain:

- Resource mechanisms shall indicate the capabilities of the TSTA

- Key Holder IDs (R0KH-ID and R1KH-ID) shall indicate the values obtained from the current AP during re-association.

Replace in 8.5A.8.2 lines 34 through 47, with the following:

The Fast Transition IE shall contain:

- Resource mechanisms shall indicate the capabilities of the target TAP

- Key Holder IDs (R0KH-ID and R1KH-ID) shall echo the key holder identifiers provided by the TSTA in the first message

Replace in 8.5A.8.3 lines 30 through 48, with the following:

The Fast Transition IE shall contain:

- Resource mechanisms shall indicate the capabilities of the target TAP

- Key Holder IDs (R0KH-ID and R1KH-ID) shall echo the key holder identifiers provided by the TSTA in the first message

- The FTIE shall be identical to the FTIE presented in the first message of this sequence.

Replace in 8.5A.8.4 lines 42 through 57, with the following:

The Fast Transition IE shall contain:

- Resource mechanisms shall indicate the capabilities of the target TAP

- Key Holder IDs (R0KH-ID and R1KH-ID) shall echo the key holder identifiers provided by the TSTA in the first message

- FTIE shall be identical to the FTIE presented in the second message of the sequence.

Replace in Section 8A.1.2, Figures 121C, 121D, 121E, and 121F, and 8A.2 Figure 121G, and 8A.4.3.1 Fig 121H, combine TRIE and TSIE with a unified FTIE.  Keep subscript same as in the figure.

Replace in Section 8A.2, mentions of TSIE and TRIE with a unified FTIE keeping the same subscripts.

Replace in Sections 8A.3 and 8A.4, mentions of TSIE with FTIE