IEEE P802.11
Wireless LANs

| Virtual AP Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Partha Narasimhan | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA | 408-754-3018 | partha@arubanetworks.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA | 408-754-1213 | subbu@arubanetworks.com |
| Dorothy Stanley | Aruba Networks | Warrenville, IL | 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains substantive text to satisfy the “Virtual AP” TGv objective, #2060. The solution uses a single beacon to efficiently advertise multiple BSSIDs and SSIDs, while optionally inheriting selected attributes of the transmitted BSSID beacon. The Probe Request frame is augmented to probe for multiple SSIDs in a single frame. The Probe Response frame is extended in the same way as the beacon frame, to include the attributes of multiple SSIDs.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 1. Req 2060 – Virtual Access Points

[Req2060] TGv shall support MAC extensions to improve support of Virtual Access Points, in order to conserve air time as well as enhanced integration with radio measurement extensions, such as TGk. One example would be to allow a single AP to advertise multiple SSIDs through a single beacon, where each SSID is mapped to a unique BSSID that may have a separate set of policies. Other alternatives to tie a virtual AP to a physical AP would also be acceptable.

Insert new elements in Table 5 as indicated below:

#### 7.2.3.1 Beacon frame format

- Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 22 | Multiple BSSID | The Multiple BSSID element is present only within Beacon frames generated by APs which support multiple BSSIDs. |
| 23 | Multiple BSSID-TIM | The Multiple BSSID-TIM element is present only within Beacon frames generated by APs which support multiple BSSIDs. |
| 242 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.2.3.8 Probe Request frame format

- Insert a new row into table 11 as follows:

- Table 11—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Multiple SSID | The Multiple SSID element is present when the STA requests information on multiple SSIDs. |
| 56 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.2.3.9 Probe Response frame format

- Insert a new row into table 12 as follows:

- Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 21 | Multiple BSSID-PR | The Multiple BSSID-PR element is present only within Probe Response frames generated by APs supporting the Multiple BSSID capability. |
| 221 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.3.2 Information Elements

Insert the following new information elements in clause 7.3.2, renumbering as appropriate:

| Information Element | Element ID |
| --- | --- |
| Multiple BSSID | ANA Assigned |
| Multiple SSID | ANA Assigned. |
| Multiple BSSID-PR | ANA Assigned |
| Multiple BSSID-TIM | ANA Assigned |

- Insert the following new clauses, renumbering the Vendor Specific Information Element as 7.3.2.28.:

#### 7.3.2.25 Multiple BSSID Element

- The format of the Multiple BSSID element is shown in Figure xx below.

|  | Element ID | Length | BSSID indicator | Multiple BSSID |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure xx —Multiple BSSID element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the information element in octets.

The BSSID Indicator shall be n, where 2n is the maximum number of BSSIDs supported by the AP, including the transmitted BSSID.

The non-transmitted BSSID values shall be derived from the transmitting AP’s BSSID by incrementing the least significant n bits of the transmitted BSSID, changing only the least significant n bits.

The Multiple BSSID field shall include the SSID element for each of the supported BSSIDs. The beacon element values of the transmitted beacon are inherited by the non-transmitted BSSIDs, with exceptions indicated in the Multiple BSSID Beacon field. As shown in the table xyz, the Multiple BSSID field may include the Capability, Supported Rates, DS Parameter Set, CF Parameter Set,  Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN and Vendor Specific information element fields for each non-transmitted BSSID. The Timestamp, Beacon Interval, FH Parameter Set, IBSS Parameter Set, TIM, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information information element values for each non-transmitted BSSID are not included in the Multiple BSSID element, and shall be the values in the transmitted beacon.

Table xyz – Multiple BSSID Elements

| 1 | Timestamp | Not Included |
| --- | --- | --- |
| 2 | Beacon Interval | Not Included |
| 3 | Capability | Optional |
| 4 | Service Set Identifier (SSID) | Included |
| 5 | Supported Rates | Optional |
| 6 | Frequency Hopping (FH Parameter Set | Not Included |
| 7 | DS Parameter Set | Optional |
| 8 | CF Parameter Set | Optional |
| 9 | IBSS Parameter Set | Not Included |
| 10 | Traffic Indication Map (TIM) | Not Included |
| 11 | Country | Not Included |
| 12 | FH Parameters | Not Included |
| 13 | FH Pattern Table | Not Included |
| 14 | Power Constraint | Optional |
| 15 | Channel Switch Announcement | Not Included |
| 16 | Quiet | Optional |
| 17 | IBSS DFS | Not Inlcluded |
| 18 | TPC Report | Optional |
| 19 | ERP Information | Not Included |
| 20 | Extended Supported Rates | Optional |
| 21 | RSN | Optional |
| 22 | Multiple BSSID | Not Included |
| 23 | Multiple BSSID-TIM | Included |
| 24 | Vendor Specific | Optional |

The Multiple BSSID-TIM field includes the DTIM Count and DTIM Period for the non-transmitted BSSIDs. When the Multiple BSSID-TIM field is present, the Partial Virtual Bitmap field in the transmitted beacon shall indicate the presence or absence of traffic to be delivered to all stations supported by the AP.

#### 7.3.2.26 Multiple BSSID-PR Beacon Element

- The format of the Multiple BSSID element is shown in Figure xy below.

|  | Element ID | Length | BSSID indicator | Multiple BSSID |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure xy —Multiple BSSID-PR element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the information element in octets.

The BSSID Indicator shall be n, where 2n is the maximum number of BSSIDs supported by the AP, including the transmitted BSSID.

The non-transmitted BSSID values shall be derived from the transmitting AP’s BSSID by incrementing the least significant n bits of the transmitted BSSID, changing only the least significant n bits.

The Multiple BSSID field shall include the SSID information element for each of the supported BSSIDs. The beacon element values of the transmitted beacon are inherited by the non-transmitted BSSIDs, with exceptions indicated in the Multiple BSSID Beacon field. As shown in the table xz, the Multiple BSSID Beacons field may include the Capability, Supported Rates, DS Parameter Set, CF Parameter Set, Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN and Vendor Specific information element fields for each Multiple BSSID. The Timestamp, Beacon Interval, FH Parameter Set, IBSS Parameter Set, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information information element values for each Multiple BSSID shall be the values in the transmitted beacon.

Table xz – Multiple BSSID-PR Field Elements

| 1 | Timestamp | Not Included |
| --- | --- | --- |
| 2 | Beacon Interval | Not Included |
| 3 | Capability | Optional |
| 4 | Service Set Identifier (SSID) | Included |
| 5 | Supported Rates | Optional |
| 6 | Frequency Hopping (FH Parameter Set | Not Included |
| 7 | DS Parameter Set | Optional |
| 8 | CF Parameter Set | Optional |
| 9 | IBSS Parameter Set | Not Included |
| 10 | Country | Not Included |
| 11 | FH Parameters | Not Included |
| 12 | FH Pattern Table | Not Included |
| 13 | Power Constraint | Optional |
| 14 | Channel Switch Announcement | Not Included |
| 15 | Quiet | Optional |
| 16 | IBSS DFS | Not Inlcluded |
| 17 | TPC Report | Optional |
| 18 | ERP Information | Not Included |
| 19 | Extended Supported Rates | Optional |
| 20 | RSN | Optional |
| 21 | Multiple BSSID-PR | Not Included |
| 22 | Vendor Specific | Optional |

#### 7.3.2.27 Multiple SSID Element

- The format of the Multiple SSID element is shown in Figure xyz below.

|  | Element ID | Length | SSID List |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure xyz —Multiple SSID element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the information element in octets.

The SSID List is a list of the SSID elements for which the STA is requesting information.

#### 7.3.2.28 Multiple SSID-TIM Element

- The format of the Multiple SSID-TIM element is shown in Figure xz below.

|  | Element ID | Length | DTIM Count | DTIM Period |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure xz —Multiple SSID-TIM element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the information element in octets.

The DTIM Count is the DTIM count element for the BSSID.

The DTIM Period is the DTIM period element for the BSSID.

References: