IEEE P802.11
Wireless LANs

| Interpretation Response 01 01/05 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: January 17, 2005 |  |  |  |  |
| Author(s) |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bob O’Hara | Airespace | 110 Nortech Pkwy, San Jose, CA 95134 | 408 635 2025 | bob@airespace.com |

Abstract

.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at < patcom@ieee.org>

Interpretation Number: 1-01/05 (Service field content)

Topic: Use of the Service field by

Relevant Clauses: Annex G

Classification: Conflicts with clause 17

Interpretation Request

I am requesting an interpretation of the IEEE 802.11g PLCP SERVICE field

header.

1.Designation of the standard, including the year of publication.

IEEE Std 802.11g-2003

IEEE Std 802.11-1999 (Reaff 2003)

2.The specific subsection being questioned.

Clause 19.3.2.1 Long preamble PPDU format

Clause 15.2.3.4 PLCP IEEE 802.11 SERVICE field

3. The applicable conditions for the case in question.

Clause 19.3.2.1 states that within the SERVICE field "Bit b2 is used to indicate that the transmit frequency and symbol clocks are derived from the same oscillator. For all ERP systems, the Locked Clock Bit shall be set to 1."

However, clause 15.2.3.4 states "The 8-bit IEEE 802.11(r) SERVICE field

shall be reserved for future use. The value of X'00' signifies IEEE

802.11(r) device compliance."

Is a Clause 19 ERP device non-conformant if it sends a 1 or 2 Mbps frame

without the SERVICE field bit B2 set?

There is also a known interoperability issue of legacy Clause 15 devices

discarding frames which have SERVICE field bit B2 set as discussed in

the attached email below.

“As described in section 18.2.3.4 of the High Rate Supplement to 802.11, the Service field in the PLCP contains several new bit assignments. Bit position b2 is defined as the Locked Clocks Bit. When set to a "1", this bit indicates a PHY implementation where the transmit frequency and the symbol clock are derived from the same oscillator. As such, several implementations will be setting this bit. The problem is that previously, the Service field was set to all zeros and an implementation would be justified in rejecting frames transmitted with a non-zero Service field. In fact, I discovered that cards employing the XXX MAC, and hence its firmware, indeed do reject frames transmitted with bit b2 set.”

Interpretation Response

The standard is unambiguous  in this case.  A PHY implementing clause 19 shall set bit B2 in the Service field, as required by 19.3.2.1 in all transmissions.  A PHY implementing clause 15 that is dependent upon a particular value in a reserved field for its correct operation is not compliant with the standard.  All implementations must ignore values of reserved fields upon reception.  The editor of the standard will be instructed to remove the sentence “The value of X'00' signifies IEEE 802.11(r) device compliance.” from clause 15.2.3.4.