IEEE P802.11
Wireless LANs

| Load Balancing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-Jan-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wang Bin | ZTE | Electronic West Road 3#, Electronic Industry Zone, Xi’an, P.R.China | +86-29-88723141 | Wang.binxa @zte.com.cn |
| Li Feng | ZTE | Electronic West Road 3#, Electronic Industry Zone, Xi’an, P.R.China | +86-29-88723130 | li.feng8@zte.com.cn |
| Li Jie | ZTE | Electronic West Road 3#, Electronic Industry Zone, Xi’an, P.R.China | +86-29-88723148 | li.jiexa@zte.com.cn |

- Abstract

This document contains a normative text proposal in support of an AP load balancing protocol to improve wireless network throughput in accordance with REQ2030.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 11. MAC sublayer management entity

### 11.X.5 Specific management actions

#### 11.X.5.1 Roaming Management for Network Load Balancing

- Load balancing should be divided into two sections. One section should deal with the problem among neighbor APs. The other should deal with the problem in ESS which has more APs beyond one neighbor scope.

##### 11.x.5.1.1 Neighbor Load Balancing

The load balance process between neighbor APs should keep the following steps.

##### 1. Load difference discovery.

Each AP in ESS exchange the load information with the others which are neighbor of the AP in position relationship, when the central AP discovery the existence of load difference and the difference has been greater than dot11LoadDifference，which defined the threshold of two AP’s load difference to init a load balancing process. Then the AP would init a load balance process.

The dot11LoadDifference should be set in the MIB, and can be changed adaptively by other function such as frequent handover avoid.

##### 2. AP implements the candidate neighbor information exchanging

The central AP should select one or more candidates from neighbor APs based on the candidate neighbor selection algorithm, however, the candidate neighbors selection algorithm is beyond the standard scope. The selected AP will format to be a candidate list and be sent to the STAs belongs to the central AP. The detail frame format and interactive sequence should be same as the description in document “11-05-1064-00-000v-normative-text-proposal-load-balancing.doc”.

##### 3. STA makes roaming decision.

The detail frame format and interactive sequence should be same as the description in document “11-05-1064-00-000v-normative-text-proposal-load-balancing.doc”.

##### 11.x.5.1.2 ESS Load Balancing

(In some circumstance, such as hotspot, the load of ESS will be spread in a distance beyond the scope of neighbor AP, which lead to the gradate distribution of load, which means the load difference between neighbor APs less than doc11LoadDifference, but some AP who are not in neighbor position satisfy the load difference limit. This situation would not lead to the initiation of neighbor load balancing process.)

If the unbalance happened between the APs whose position relationship beyond the neighbor APs, then the other APs who are in the middle position of the unbalancing APs would init ESS load balancing, and try to pass the heavy one’s load to the light one or the light direction.

The load balance process beyond neighbor APs should keep the following steps.

##### 1. Candidate AP select and create candidate list

After the heavy load AP init a load balancing process, it should select candidate target AP in the ESS, whether the candidate is in the neighbor position of the heavy load AP or not.

##### 2. Pass load to the light AP

The heavy load AP should try to pass it’s load to the lightest AP or the AP in the direction. Each Pass Load Process has a time expire, after the Pass Load Process, the heavy AP would calculate success rate. if the success rate of once Pass Load Process below some threshold, the target AP should be deleted from the candidate list, and the heavy AP would select a lightest AP from the remain candidate list as a newer target.

The Pass Load Process would be marked in the target AP, and after the process, this target AP would try to init a new Neighbor Load Balancing between neighbor AP and try to pass it’s load to the lighter direction.

The Pass Load Process will be terminated only if the candidate list has been consumed or the load of the heavy AP has been successful passed.