IEEE P802.11
Wireless LANs

| Location Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Richard Barnwell | Pango Networks | Framingham, MA | 508-626-8900 | Richard.Barnwell@pangonetworks.com |
| Stuart Golden | Intel Corporation | Portland, OR | 503-677-5292 | Stuart.Golden@intel.com |
| Tomi Heinonen | Ekahau, Inc | Helsinki, Finland | 358 20 743 5931 | Tomi.Heinonen@ekahau.com |
| Graham Mostyn | G2 Microsystems | 408 Euclid Ave Oakland, CA 94610 | 408-255-3880 | Graham.Mostyn@grmicrosystems.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA 94089 | 408-754-1213 | subbu@arubanetworks.com |
| Peter Single | G2 Microsystems | Eveleigh, NSW, 1430 Australia | +61 2 9418 3203 | psingle@g2microsystems.com |
| Dorothy Stanley | Aruba Networks | Warrenville, IL | 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains substantive text to satisfy the “Location” TGv objective, #2090. The solution provides a mechanism for a STA to alert the network of its presence.and to receive instructions from the network for providing further alerts.  A location-determination algorithm is NOT specified, and the STA may know, but is not assumed to know its location.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 1. TGv Req 2090 – Location

Req2090: Location

TGv will provide a mechansism to coordinate the gathering and possibly generation of data to support various location methods such as time of arrival, time difference of arrival, and signal strength.

# 2. Definitions

“Location Methods” – Algorithms for determining the location of a STA. Algorithms can be based on time of arrival, time difference of arrival, or signal strength. Location can also be provisioned for stationary devices, or determined via GPS.

“Location Data” – Data associated with a STA- geospacial (latitude, longitude, altitude) or civil (street address). The IETF geopriv Working Group documents describe security and privacy issues surrounding the use of location data, recommend data formats for location objects, and specify mechanisms for carrying location objects in Radius.

“Location-based Services” – Services which use location data. Examples of location-based services include navigation, emergency services, and management of equipment in the field.

# 3. IEEE 802.11 Location Support

Current and planned IEEE 802.11 capabilities which support the transport of location information, and enable the gathering of information used to determine the location of a STA include:

- The draft IEEE 802.11k Location Configuration Information Report, which references RFC 3825, and includes latitude, longitude, altitude and azimuth, with accuracy resolution indicators for each. The ability to request the location of a local or remote STA is provided.

- Mechanisms are defined for STAs to report RSSI and to gather additional RF measurements.

- Discussions are ongoing within TGu and TGv regarding the support of E-911 services in WLAN systems. For the purposes of this submission, E-911 is viewed as a location-based service, which may utilize the presence reporting mechanisms that are defined here. E-911 services require that the location of the calling station be known. How the location (longitude, latitude, altitude or street address) is determined is outside the scope of this document.

Requirements on an IEEE 802.11 TGv solution:

- Support a variety of end devices – high functionality and low functionality, very low power.

- Lowest end devices cannot calculate their own location, they report “presence”, and can receive reporting instructions. Lowest end devices must be able to “sleep” for very long periods of time (10s of minutes).

- Support multiple location based services, including navigation, emergency services, management of equipment in the field.

- Provide a secure mechanism for the gathering of location data. An insecure method may also be provided.

- Be backwards compatible with existing non-location enabled STAs.

The following new TGv location mechanisms are included in this proposal:

- Define and transport basic presence information

- Deliver presence reporting instructions and information to a STA

- Support low-end devices, which can send Wireless Network Management Action Presence frames without being authenticated or associated.

## Substantive Text - Location

Insert a new list item for presence management frames, as follows: Note- The text in 5.4.3.7 is being added by TGw.

#### 5.4.3.7 Management frame protection

The management frame protection mechanism defines the means by which a STA may provide confidentiality and integrity services to certain “Robust” 802.11 management frames. Only management frames within an infrastructure BSS sent in State 3 shall be deemed Robust and are limited to the following frames:

- Deauthentication

- Action with category:

    - Spectrum management

    - QoS

    - DLP

    - Block Ack

    - Wireless Network Management

- Dissassociation

{NOTE: This document assumes the TGw amendment is ratified before the TGv amendment.  If the TGk and TGr amendments ratify before TGv/TGw, then the following action management frame subtypes shall also be included:

- Radio Measurement

- Fast BSS Transition}

#### 5.6 Relationships between services

Change list item a)2)vi as follows:

vi) Presence Notification and Presence Notification Response Wireless Network Management Action Frames

Within an IBSS, action frames are class 1.

#### 7.2.3.1 Beacon frame format

Insert a new element in Table 5 as indicated below:

- Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| xx | Presence Parameters | The Presence Parameters element is present only within Beacon frames generated by APs which support Wireless Network Management Presence Reporting. |

#### 7.3.1.11 Action field

- Insert the following new row into table 21 and update the reserved value as shown:

- Table 21—Category values

| Name | Value | See clause |
| --- | --- | --- |
| Spectrum Management | 0 | 7.4.1 |
| Wireless Network Management | x | 7.4.x |
| Reserved | x-127 | - |
| Error | 128-255 | - |

### 7.3.2 Information Elements

Insert the following element ID into table 20 and change the reserved row accordingly:

Table 22—Element IDs

| Information Element | Element ID |
| --- | --- |
| Presence Parameters | ANA Assigned |
| Reserved |  |

### 7.3.2.xx Presence Parameters Information Element

Table xy Presence Parameters Information Element

| Element ID | Information Element |
| --- | --- |
| 1 | Presence Reporting Parameters |
| 2 | Presence Reporting Channels |
| 3 | Timing Measurements |
| 4 | Radio Information |
| 5 | Motion |
| 6 | Battery Information |
| 7 | Vendor Specific |
| 8-255 | Reserved |

All of the Presence Parameters information elements are optional. The Beacon frame may include only the Presence Reporting Parameters and the Presence Reporting Channels information elements.

### 7.3.2.xx Presence Reporting Parameters Element

Table xx Presence Reporting Parameters

| EID (1) | Length (10) | Stationary Report Interval | Stationary number of frames per channel | In-Motion Report Inerval | In-Motion number of frames per channel | Inter--frame Interval | Triggered Event Indication | Triggered Event Data |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets:1 | 1 | 2 | 2 | 2 | 1 | 1 | 1 | 1 |

The Stationary Report Interval is the time interval, expressed in minutes at which the STA shall report its presence, when the STA is stationary.

The Stationary number of frames per channel is the number of Presence Notification or Presence Report frames per channel to be sent by the STA at each Stationary Report Interval.

The In-Motion Report Interval is the time interval, expressed in seconds at which the STA shall report its presence, when the STA is in motion. If motion detection is not supported, this value shall be 0.

The In-Motion number of frames per channel is the number of Presence Notification or Presence Report frames per channel to be sent by the STA at each In-Motion Report Interval. If motion detection is not supported, this value shall be zero.

The Inter-frame interval is the time interval, expressed in milliseconds between the transmissions of each of the Stationary or In-Motion frames per channel.

Table xx Triggered Event Indication

| Triggered Event Indication | Meaning |
| --- | --- |
| 0 | Scheduled Event |
| 1 | Triggered Event |
| 2 | Vendor Specific |
| 3-255 | Reserved |

The Triggered event indication field contains indicated whether the presence frame was sent as a result of a scheduled event, or a triggered event, as indicated in Table xx.

Triggered event data is a single octet which may contain data related to the triggered event.  If no triggered event data is available, this field is set to 0.

### 7.3.2.xx Presence Reporting Channels Element

Table 10 Presence Reporting Channels Element

| EID (2) | Length (number of channels + 1) | Number of channels | Channel 1 | …Channel n |
| --- | --- | --- | --- | --- |
| Octets:1 | 1 | 1 | 1 | 1 |

The Presence Reporting Channels element specifies the channels in which a STA shall report its presence.

When included in the Beacon, the Presence Reporting Channels element shall indicate the list of channels on which the AP listens for presence frames.

When included in the Presence Notification and Presence Request Frame, this element shall indicate the list of channels on which the STA sends presence frames. When included in the Presence Response and Presence Notification Response Frames, this element shall indicate the list of channels on which the receiving STA shall send presence frames.

### 7.3.2.xx Timing Measurements Element

Table 13 Timing Measurements

| EID (3) | Length(12) | Timestamp Difference | Received Timestamp |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | 8 |

The Timing Measurements Element shall not be included in Presence Parameters information element in the Beacon, Presence Notification and Presence Report frames.

The Timestamp Difference field contains the time difference in tenths of nanoseconds between the time that the corresponding Presence Notification or Presence Request frame was received from a STA and the time that the corresponding ACK frame was sent to the STA. The Timestamp Difference field may be included in the Presence Notification Response and Presence Response Frames.

The Received Timestamp field includes the time that the corresponding Presence Notification or Presence Request frame was received from a STA. The Received Timestamp field may be included in the Presence Notification Reponse and Presence Response Frames.

### 7.3.2.xx Radio Information Element

Table xx Radio Information IE

| EID (4) | Length (5) | Transmit Power | Antenna ID | Antenna Gain | Received RSNI | RCPI |
| --- | --- | --- | --- | --- | --- | --- |
| Octets:1 | 1 | 1 | 1 | 1 | 1 | 1 |

The transmit power is the transmit power of the radio transmitting the presence frame and is reported in dBm.

The antenna gain is the antenna gain of the antenna over which the presence frame will be transmitted and is reported in dBi.

The received RSNI value shall be included in Presence Notification Response and Presence Response frames and includes the RSNI value (dBm) for the corresponding received Presence Notification or Presence Request frame.  The received RSNI value shall be set to 0 in Presence Notification and Presence Request frames. Note: RSNI, and Antenna ID are as defined in draft 802.11k.

The RCPI value is as defined in Clause xxx (draft 802.11k).

### 7.3.2.xx Motion Element

The Motion element is used by WLAN devices with motion detection capabilities.

Table xx Motion IE

| EID (5) | Length (18) | Motion Indicator | LCI | Velocity |
| --- | --- | --- | --- | --- |
| Octets:1 | 1 | 1 | 16 | 1 |

Table xx Motion Indicator Values

| Motion Indicator | Meaning |
| --- | --- |
| 0 | Stationary |
| 1 | Start of motion |
| 2 | In motion |
| 3 | End of motion |
| 4-255 | Reserved |

The LCI field contains the Location Configuration Information Field, defined in clause 7.3.2.21.11 (TGk).

The Velocity field contains the velocity or speed of the STA expressed in meters per second.

### 7.3.2.xx Battery Information Element

Table 9 Battery Information Element

| EID (6) | Length (3) | Battery Remaining | Battery Age |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 2 |

The Battery remaining field indicates the percentage of battery remaining in the STA, specified in percent.

The Battery Age field indicates the age, or calendar days of usage of the battery, and is specified in days.

The Battery Information Element shall not be included in in the Presence Parameters element in Beacon, Presence Notification Response, and Presence Response frames.

### 7.4. Action frame format details

### 7.4.1 Spectrum management action details

### 7.4.x Wireless Network Management action details

Four presence related action frame formats are defined for wireless network management. An Action Value field, in the octet field immediately after the Category field, differentiates the four formats. The Action Value field values associated with each frame format are defined in Table aa.

Table aa — Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 0 | Presence Notification |
| 1 | Presence Notification Response |
| 2 | Presence Request |
| 3 | Presence Response |
| 3-255 | Reserved |

STAs may transmit presence management frames on pre-provisioned channels which are valid in the location specific regulatory domain.

#### 7.4.x.1 Presence Notification frame format

The Presence Notification frame uses the Action frame body format and is transmitted by a STA to report its presence. The format of the Measurement Request frame body is shown in Figure yy.

The Category field shall be set to x (representing wireless network management).

The Action field shall be set to 0 (representing a Presence Notification frame).

The Dialog Token field shall be set to a nonzero value chosen by the STA sending the presence notification to identify the notification frame. The dialog token shall be unique for each presence notification frame sent to a given destination MAC address.

The Response Requested field shall be set to 1 if a Presence Notification Response frame is requested. The Response Requested field shall be set to 0 if a Presence Notification Response frame is not requested.

The Presence Parameters element field shall contain the presence parameters element, described in Clause 7.3.2.xx.

|  | Category | Action | Dialog Token | Response Requested | Presence Parameters Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure yy— Presence Notification frame body format

A STA may periodically advertise its presence by sending presence notification frames. The Presence Notification frame may be a broadcast or unicast frame, and may be sent prior to authentication and association, similar to a Probe Request frame.

#### 7.4.x.2 Presence Notification Response frame format

The Presence Notification Response frame uses the Action frame body format and shall be transmitted by a STA upon receipt of a Presence Notification frame with the response requested bit set to 1. The format of the Presence Notification Response frame body is shown in Figure zzx.

|  | Category | Action | Dialog Token | Timestamp | Presence Parameters Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 8 | variable |

Figure zzx— Presence Notification Response frame body format

The Category field shall be set to x (representing wireless network management).

The Action field shall be set to 1 (representing a Presence Notification Response frame).

The Dialog Token field shall be set to the nonzero value received in the Presence Notification frame to identify the request/response transaction.

The Timestamp field shall indicate the time at which the Presence Notification Response frame is sent. The Timestamp field is in units of nanoseconds. If the time is not available, the field shall be set to 0.

The Presence Parameters element field shall contain the presence parameters element, described in Clause 7.3.2.xx.

#### 7.4.x.3 Presence Request frame format

The Presence Request frame uses the Action frame body format and is transmitted by a STA to deliver presence reporting information. The Presence Request frame shall be a unicast frame, sent in State 3. The format of the Presence Request frame body is shown in Figure zx.

|  | Category | Action | Dialog Token | Presence Parameters Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure zx— Presence Request frame body format

The Category field shall be set to x (representing wireless network management).

The Action field shall be set to 2 (representing a Presence Request frame).

The Dialog Token field shall be set to a nonzero value, to identify the Presence Request/Response transaction. The dialog token shall be unique for each presence notification frame sent to a given destination MAC address.

The Presence Parameters element field shall contain the presence parameters element, described in Clause 7.3.2.xx.

#### 7.4.x.4 Presence Response frame format

The Presence Response frame uses the Action frame body format and is transmitted by a STA in response to the receipt of a Presence Request frame. The format of the Presence Response frame body is shown in Figure zz.

The Category field shall be set to x (representing wireless network management).

The Action field shall be set to 3 (representing a Presence Response frame).

The Dialog Token field shall be set to the nonzero value received in the Presence Request frame to identify the request/response transaction.

The Timestamp field shall indicate the time at which the Presence Response frame is sent. The Timestamp field is in units of nanoseconds. If the time is not available, the field shall be set to 0.

The Management Action Pending field shall be set to 0 if no management action is pending for the destination STA, and set to 1 if there is a management action pending for the destination STA.

The Presence Parameters element field shall contain the presence parameters element, described in Clause 7.3.2.xx.

|  | Category | Action | Dialog Token | Timestamp | Management Action Pending | Presence ParametersElement |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 8 | 1 | variable |

Figure zz— Presence Response frame body format

The Presence Management Response frame provides presence reporting instructions to the STA.