IEEE P802.11
Wireless LANs

| LB78 Clause 11.1.3.2.2 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-Jan-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Floyd Simpson | Motorola, Inc. | 8000 W. Sunrise Blvd. Plantation, FL 33467 | 954-723-5269 | Floyd.Simpson@Motorola.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for clause 11.1.3.2.2.  This document resolves comments: 90,256,779,1291

Note to Editor:

This normative text is referenced to 11k d3.0 and the text replaces normative text in the TGk draft from page 59  line 13 to page 59 line 22.

All new text relative to the base draft is identified in blue color and underlined. All deleted text is identified by strikethrough.

##### 11.1.3.2.2 Active scanning procedure

Insert the following text at the end of this clause:

If dot11RadioMeasurementEnabled is true and a Probe Request with a Request Information element which contains the RCPI element ID is received then a RCPI element shall be included in the Probe Response. If a RCPI element is received in a Probe Response frame, the RCPI value shall be included in the RCPIMeasurement parameter of the BSSDescription in the MLME-SCAN.confirm.

If dot11RadioMeasurementEnabled is true and if the Request Information element of the Probe Request includes the RCPI element ID, an AP shall include a RCPI element in the Probe Response. An AP may measure RCPI on the received Probe Request frame and include the result in the RCPI element of the Probe Response. If no measurement result is available the RCPI value shall be set to the eight bit value which indicate that a measurement is not available.