IEEE P802.11
Wireless LANs

| LB78 Clause 0 Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-12-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Richard Paine | Boeing |  | 206-854-8199 | richard.h.paine@boeing.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for clause 0/General.  This document resolves comments 277, 498, 499, 834, 874, and 900.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[Change the amendment number 7 on page 1, line 9  to amendment number 9]

Amendment 9: Radio Resource Measurement

[Add an Introduction Section in the boilerplate (starting at page iii) as follows:]

This introduction is not part of IEEE Std 802.11-2005, IEEE Standard for Information Technology – Telecommunications and Information Exchange Between Systems – LAN/MAN Specific Requirements – Part 11:  Wireless Medium Access Control (MAC) and physical layer (PHY) specifications.

Radio Resource Measurement (RRM) is a key enabler to the next generation of WLANs.  RRM addresses some of the existing issues in using unlicensed radio environments to meet the requirements of emerging technologies.  In addition, RRM provides knowledge about the radio environment to improve performance and reliability.

The existing 802.11 measurements and information about them are inadequate to move ahead to the next generation of Wireless Local Area Network (WLAN).  The proposed 11k approach, from the PAR and 5 criteria, is to add measurements that extend the capability, reliability, and maintainability of WLANs through measurements and provide that information to upper layers in the communications stack.  In addition, there are new technologies needing more measurement and capability.  These technologies include VOIP, Video Over IP, and location as well as mitigation of harsh radio environment places where WLANs are operated (multifamily dwellings, airplanes, factories, municipalities, etc).

IEEE 802.11k addresses the MAC, the SME, the MLME, primitives, the MIB and the interface to upper layers.  The 11k scope is:  Define Radio Resource Measurement enhancements to provide mechanisms to higher layers for radio and network measurements.  The group has provided the mechanisms using request/response queries and the MIB using an Object Identifier (OID) interface to upper layers.

The 11k specification adopted the TGh layer management request/response model to gather the information.  Also, by recognizing the mobility requirements of the new technologies, such as VOIP and video streaming, the group specified measurements, the channel load report, and the neighbor report as a means of developing pre-handoff information gathering that drastically speeds up the ability to handoff between cells within the same ESS.  By accessing and using this information, the STAs (either in the APs or in the individual devices) can make decisions about the most effective way to utilize the available spectrum, power, and bandwidth for its desired communications.

The Radio Resource Measurement specification:

- Enables better diagnostics of problems

  - Using info that is easy and cheap to gather

- Enables better frequency planning, optimized network performance

  - Enables automated frequency planning

- Enables new services

  - Location based services

  - Voice Over IP (VOIP)

  - Video Over IP

The specification makes lists of known stations (STAs, both APs and devices) visible to the requestor; it includes link statistics including counts of MSDU/MPDU and channel utilization in total microseconds; it includes the data rates and modulation and link margin as seen by other stations.  It includes optional events to notify management stations of authentication and association events and can provide TRAPS on association and authentication failures.  It also provides reports on all the RSN pre-authentication and de-authentication events as well as normal association and de-association events.

So, radio measurement, as offered by 11k, enables automatic radio configuration and enables radio-aware performance by offering the measurements to allow monitoring, roaming, and handoff.  Radio measurements allow the 802.11 community to:

  - Simplify and/or automate WLAN radio configuration

  - Achieve better performance in dense BSS deployments

  - Better utilize radio resources across client stations

  - Alert WLAN administrator to problems

  - Notify client station users of current radio status

  - Vendors use measurements to add value

The request/response measurements are:

Beacon

Frame

Channel Load

Noise Histogram

STA Statistics

Location Configuration Information

Neighbor Report

Link Measurement

The request-only mechanism is:

Measurement Pause

The following list of measurements gives the reader an idea of the type of measurements obtained through the 11k mechanisms.  Measurements include country, power constraints, Transmitter Power Control (TPC) report, Access Point (AP) Channel, power capability, neighbor report element ID, TSF information, Received Channel Power Indicator (RCPI), DS parameter set, capability information, channel number, regulatory class, randomization interval, measurement duration, Basic Service Set Identifier (BSSID), reporting condition, threshold/offset, , measurement modes (passive/active or beacon table), group identity, STA counters, late bit, incapable bit, refused bit, location, RPI density, measurement start time, measurement duration, PHY type, parent TSF, target TSF, beacon interval, capability information, number of frames, transmitted fragment count, multicast transmitted frame count, failed count, retry count, multiple retry count, frame duplicate count, RTS success count, RTS failure count, ACK failure count, received fragment count, FCS error count, transmitted frame count, latitude resolution, latitude, longitude resolution, longitude, altitude type, altitude resolution, altitude, datum, element ID, length, channel list, BSSID information, channel number, PHY options, TSF information, reachability, security, key scope, capabilities unknown, capabilities, spectrum management capability, QoS capability, TSF offset, action, dialog token, number of repetitions, frame restart delay, time unit, restart delay, neighbor TBTT offset type, transmit power, link margin,

The measurement pairs include the beacon request/report, the frame request/report, the channel load request/report, the noise histogram request/report, the STA statistics request/report, the Location Configuration Indication (LCI) request/report, the measurement pause, the neighbour request/report, the link measurement request/report, and the QoS Metrics request/report.  This ability to exchange measurements and make them available to upper layers is critical to the advancement of 802.11 as the pre-eminent wireless networking standard in the world.

Beacon

The Beacon request/report pair enables a STA to inquire of another STA to provide a list ofAPs it can receive on a specified channel or channels.  This measurement may be done by active mode (like active scan), passive mode (like passive scan), and beacon table modes.  If the measurement request is accepted and is in passive mode, a duration timer is set and the measuring STA monitors the requested channel, measures beacon and probe response power levels (RCPI), and logs all received beacons and probe responses within the measurement duration.  If the measurement request is in active mode, the measuring STA sends a probe request on the requested channel at the beginning of the measurement duration, then monitors the requested channel, measures beacon and probe response power levels (RCPI), and logs all received beacons and probe responses within the measurement duration. .  If the request is beacon table mode, then the measuring STA returns the current contents of any stored beacon information without monitoring any channel. At the end of the duration, the measuring STA prepares and sends a measurement report of all the beacon or probe response information..

Frame

The frame request/report pair enables one to get a picture of all the channel traffic and a count of all the frames received at the measuring STA.  For each unique Transmitter Address, the STA reports the Transmitter Address, number of frames received from this transmitter, average power level (RCPI) for these frames,  and BSSID of the transmitter.

Channel Load

The channel load request/report pair enables you to get the channel utilization measurement as observed by the measuring STA.

Noise Histogram

The noise histogram request/report pair returns non-802.11 noise power measurement obtained by sampling the channel only when CCA indicates idle.

STA Statistics

The STA statistics request/report pair returns groups of values for STA counters and for BSS load.  The STA counter values include: transmitted fragment counts, multicast transmitted frame counts, failed counts, retry counts, multiple retry counts, frame duplicate counts, Request to Send (RTS) success counts, RTS failure counts, Acknowledgement (ACK) failure counts, received fragment counts, received fragment counts, multicast received frame counts, FCS error counts, and transmitted frame counts.  BSS load values include: AP service load, average access delay for each access category, associated STA count, and channel utilization

Location

The Location request/report pair returns a requested location in terms of latitude, longitude, and altitude.  It includes types of altitude such as floors and permits various reporting resolutions. The requested location may be the location of the requestor (e.g. Where am I?) or the location of the reporting STA (e.g. Where are you?)

Measurement Pause

This measurement request is defined, but no report comes back from this request.  The measurement pause permits the inclusion of a quantified delay between the execution of individual measurements which are provided in a series within a measurement request frame.  The measurement pause used as the last measurement in a frame provides control of the measurement period when measurement request frames are to be repeated..

Neighbor

The neighbour report request/report pair is sent by an AP and returns information about known neighbour APs that are candidates for a BSS transition.  Reports contain information from the table dot11RRMNeighborReportTable in the MIB concerning neighbour APs.  This request/report pair enables a STA to gain information about the neighbours of the associated AP to be use as potential roaming candidates.

Link Measurement

The link measurement request/report pair is like an RF ping in which a STA requests another STA to measure and report the link path loss and estimation of the link margin.  This enables understanding the instantaneous capabilities of a link for QoS-type requirements.

The 11k standard enables the next generation of WLAN by providing the ability of a STA to query its radio environment to make appropriate decisions about its health and efficiency.  It is the first step in making the Wireless LAN (WLAN) smart and capable of making appropriate decisions for fast handoff, for mesh connectivity, and for managing the radio environment for the good of all Part 15 devices.